3GPP TSG-RAN WG4 Meeting #114                                                                     R4-2500217

# Greece, Athens, 17th – 21st Feb., 2024

Agenda item:	7.16.2

Source:	Orange

Title:	Performance comparison of high and low-resolution codebook under different CDL models and AAV configurations

Release:	Rel-19

Related WI:	FS_NR_demod_SCM

Document for:	Discussion

# Introduction

The goal of this contribution is to assess the SU-MIMO performance gap between high (eType-II) and low resolution (Type-I) codebook (CB) with respect to the CDL model (either 901 or 827) and the Antenna Array Virtualization (AAV) configuration (subarray or fully connected). We have witnessed in our previous contribution [1] that the legacy RAN4 TDL channel was not able to discriminate between these two types of CB in a SU-MIMO setting due to its inherent spatial deficiency. However, even with CDL spatially consistent channel, depending on the number of antenna elements (AEs), number of Antenna Ports (APs), CDL model (901, 827), and AAV methods, the performance difference between the two CBs can be rendered negligible. Since, the SCM SI goal is to provide a spatially consistent channel for MIMO performance requirements in the future, it is important to identify the configurations where the performance gap between eType-II and Type-I is sufficient for discriminating their respective implementation. The first part of this contribution stick to the simulation parameters agreed in the last RAN4 meeting Way Forward. The second part deviates from the agreements to investigate the effect of increasing the number of AEs and/or APs. Finally, we question the assumption of RAN4 CSI-RS EPRE with respect to what is assumed for DMRS EPRE and propose to open the discussion on this parameter within the SCM SI.

Our simulation results rely on the MIESM PHY abstraction as described in [2]. We generally assume perfect channel estimation except for the RAN4 CSI-RS EPRE discussion.

# WF compliant simulations

In the following simulations, our goal is to analyse the performance gap between Type-I and eType-II for UMa CDL-C 827 and 901 with configurations described in the WF [3].

The detailed simulation parameters are given in Appendix B and the adopted options are:

- Option 1: with subarray Antenna Array Virtualisation (AAV)

  - (M,N,P,Ms,Ns)  = (8,8,2,8,2) for 8Tx CSI-RS Ports (option X)

  - (M,N,P,Ms,Ns)  = (8,4,2,8,1) for 8Tx CSI-RS Ports (option Y)

- Option 2: with fully connected AAV

  - (M,N,P,Ms,Ns) = (8,8,2,8,8) for 8Tx CSI-RS Ports

## UE speed of 3km/h – 10 Hz Doppler shift

### Rank 4

#### Option 1X: 8 Tx (8,8,2,8,2) subarray AAV

| CDL-C 901, 1X | CDL-C 827, 1X |
| --- | --- |

#### Option 2 – CDL 827: 8 Tx (8,8,2,8,8) fully connected AAV

CDL-C 827, Option 2

#### Option 1Y: 8 Tx (8,4,2,8,1) subarray AAV.

| CDL-C 901, 1Y | CDL-C 827, 1Y |
| --- | --- |

Observation 1: For CDL-C 827, Option 2, the performance of Type-I follow PMI is close to the one given by the random PMI wideband bound. We conclude that applying a wideband DFT CB on a fully connected AAV, which pre-selects geometrical directions, does not bring substantial gains. On the other hand, the eType-II CB can improve the performance since a sum of DFT can approach the eType-II Port Selection behaviour.

Observation 2: The performance gap between eType-II and Type-I CB is larger for CDL-C 901, 1X than for CDL-C 901, 1Y. This can be explained by the fact that 1X AAV has a higher number of AEs in the horizontal dimension.

Observation 3: CDL-C 901, 1X, yields a larger performance gap between eType-II and Type-I CB than CDL-C 827 1X. We can conclude The CDL-C, 901 has more “direction diversity” which Type-I cannot capture.

### Rank 2 vs. rank 4

#### Option 1X – CDL 901: 8 Tx (8,8,2,8,2) subarray AAV

| CDL-C 901, 1X, Rank 4 | CDL-C 901, 1X, Rank 2 |
| --- | --- |

#### Option 2 – CDL 827: 8 Tx (8,8,2,8,8) fully connected AAV

| CDL-C 827, Option 2, Rank 4 | CDL-C 827, Option 2, Rank 2 |
| --- | --- |

Observation 4: For CDL-C 901, 1X, eType-II CB yields better performance than its CDL-C, 827, Option 2 counterpart for both rank 4 and rank 2. It confirms that CDL-C 901 has more “direction diversity” to be exploited by high resolution CSI CB.

Observation 5: The performance gap between eType-II and Type-I CB is larger with rank 4 for CDL-C 901, 1X. On the other hand, it is the opposite for CDL-C 827, Option 2. We can conclude that CDL-C 827, Option 2 is less favourable to high rank transmission than CDL-C 901, 1X.

## UE speed of 30km/h – 100 Hz Doppler shift

### Rank 4

#### Option 1X: 8 Tx (8,8,2,8,2) subarray AAV.

| CDL-C 901, 1X | CDL-C 827, 1X |
| --- | --- |

#### Option 2 – CDL 827: 8 Tx (8,8,2,8,8) fully connected AAV

CDL-C 827, Option 2

#### Option 1Y: 8 Tx (8,4,2,8,1) subarray AAV

| CDL-C 901, 1Y | CDL-C 827, 1Y |
| --- | --- |

Observation 6: Same observations as for 3km/h, the curves are slightly translated to higher SNR values.

# Additional simulations

## Number of antenna elements

### CDL 901: 	8 Tx subarray AAV

| CDL-C 901, 1Y- (8,4,2,8,1) | CDL-C 901, 1Y-(12,4,2,12,1) |
| --- | --- |

### CDL 827: 	8 Tx subarray AAV

| CDL-C 827, 1Y - (8,4,2,8,1) | CDL-C 827, 1Y - (12,4,2,12,1) |
| --- | --- |

Observation 7: Increasing the number of AEs in the vertical dimension, while keeping a single AP per vertical dimension, does not help for discriminating the performance between Type-I and eType-II CB.

## Number of antenna ports

### Increasing the number of antenna ports in the vertical dimension

#### CDL 901: 	subarray AAV

| CDL-C 901, 1Y, 8 Tx - (12,4,2,12,1) | CDL-C 901, 16 Tx - (12,4,2,6,1) |
| --- | --- |

#### CDL 827: subarray AAV

| CDL-C 827, 1Y, 8 Tx - (12,4,2,12,1) | CDL-C 827, 16 Tx - (12,4,2,6,1) |
| --- | --- |

Observation 8: Increasing the number of APs in the vertical dimension helps to discriminate between Type-I and eType-II CB performance for CDL-C 901. However, this effect is not observed for CDL-C 827. This may be attributed to the fact that 827 has less “direction diversity”, as it has fixed zenith of arrival at 90° for all clusters with per-cluster zenith spread of arrival of 0.

### Increasing the number of antenna ports in the horizontal dimension

#### CDL 901: 	subarray AAV

| CDL-C 901, 1X, 8 Tx - (8,8,2,8,2) | CDL-C 901, 1Y, 16 Tx - (8,8,2,8,1) |
| --- | --- |

#### CDL 827: subarray AAV

| CDL 827, 1X, 8 Tx - (8,8,2,8,2) | CDL 827, 1Y, 16 Tx - (8,8,2,8,1) |
| --- | --- |

Observation 10: CDL 827 has less “direction diversity” than 901, since Type-I yields the same performance for both Type-I and eType-II CB.

### Increasing the number of antenna ports in both dimensions for modified CDL 827

| CDL 827, 1X, 8 Tx - (12,8,2,12,2) | CDL 827, 1Y, 32 Tx - (12,8,2,6,1) |
| --- | --- |

Observation 11: Increasing the number of APs in both vertical and horizontal dimensions helps to discriminate between Type-I and eType-II for both CDL-C 901 and 827, as it provides additional degrees of freedom to select beams.

Proposal 1: Consider a higher number of CSI-RS antenna ports and antenna elements for eType-II performance evaluation, e.g., an antenna panel and subarray AAV (M,N,P,Ms,Ns)= (12,8,2,6,1).

# Discussion on CSI-RS EPRE assumption

In TS38.211, the following notations are introduced:

- k,lp,μ is the Resource Element (RE) with frequency-domain index k and time-domain index l for AP p and subcarrier spacing configuration μ,

- ak,l(p,μ)	is the value of the RE (k,l) for AP p and subcarrier spacing configuration μ.

The Energy Per Resource Element (EPRE) of an RE k,lp,μ is defined before the mapping from APs to TXRUs and with reference to an EPRE E0. Since a RE k,lp,μ belongs to a resource grid associated to an AP, the EPRE is always per AP. The mapping from AP to TXRU is carried out with a spec-transparent precoder E0W∈CNTXRU×N with NTXRU the number of TXRUs and N the number of APs. The only precoder property known for the generation of the APs is that WHW=1NIN with IN the identity matrix of dimension N. This assumption allows to define the EPRE before the precoding independently of the number of APs, the normalization of the total EPRE being taken into account by the implicit precoder W.

In general, the standard set by a RRC parameter the ratio β2 of reference signal (CSI-RS, DMRS) EPRE to the reference transmit EPRE E0 before precoding.  After the precoding for the REs k,lp,μ  p∈{1,…,N} , the transmit signal z can be written:

z=E0Wak,l(p1,μ)ak,l(p2,μ)⋮ak,l(pN,μ).

The EPRE per AP after the precoding is E0Nβ2 assuming Eak,lp,μ2=β2 . If all the APs are active, the maximum total EPRE after precoding that can be reached is E0β2=EzHz.

The DMRS/CSI-RS are reference signal known to the receiver designed to estimate the precoded channel E0HW=h1⋯hN ∈CNR×N with NR the number of receive APs, the pth column being   hp=h1,p⋮hNR,p, p∈{1,…,N}. Note that H is the non pre-coded channel of dimension NR× NTXRU which is unknown to the UE.

## DMRS EPRE

The number of APs is N=ν , which is equal to the number of spatial layers of the PDSCH (the DMRS are part of the PDSCH channel and are pre-coded in the same way). The DMRS EPRE before precoding is βPDSCHDMRS2 and after precoding  E0νβPDSCHDMRS2. The DMRS RE value can be expressed as ak,l(p,μ)=βPDSCHDMRSck,lp with  ck,lp2=1.

TS38.211 defines CDM groups which are set of REs used by the reference signal associated to a given AP. For a given CDM group of size L, the reference signal of AP p is spread by OCC over the REs indexed by 1 to L in the following for simplicity. The associated spreading sequence is cp=c1p, c2p,…, cLpTwith cpHcp'=Lδp,p' where δp,p'=1 for p=p' and δp,p'=0 for p≠p'. There can be up to L APs that are code division multiplexed on the same (k,l) radio resource, the set of APs belonging to the same CDM group is denoted by G.

The received signal for antenna r can be written as

yr,1⋮yr,L=p'∈Ghr,p'βPDSCHDMRSc1p'⋮cLp'rp',m+wr,1⋮wr,L

where m is an index that depends on the PRB number as well as the position of the CDM group within the PRB.

It is stated in TS38.214 that βPDSCHDMRS2 is equal to the number of CDM groups (#CDM-groups) since for a given non-zero power RE for DMRS there is #CDM-groups-1 RE set to zero.

The ratio of PDSCH EPRE to DMRS EPRE - table 4.1-1 TS38.214 [3]

## CSI-RS EPRE

The EPRE before the precoding is βCSI-RS2 after precoding it becomes E0βCSI-RS2N. The value of the RE allocated can be written as ak,l(p,μ)=βCSI-RSck,lpwith  ck,lp2=1, similarly to DMRS. The standard gives the ratio βCSI-RSPDSCH2 of the PDSCH EPRE to the CSI-RS EPRE, it yields βCSI-RS=1βCSI-RSPDSCH.

Similarly, to the DMRS βCSI-RS2 can be taken as equal to the number of CDM groups in an OFDM symbol  (#CSI-RS CDM group in an OFDM symbol) located over different sub-carriers.

In RAN4, if we refer to Table C.3.1-1 of TS38.101-4, βCSI-RS2 is defined as 1/L (in linear scale) for a CSI-RS belonging to a CDM group of size L which means that the EPRE after precoding is E0NL where E0 is the reference SSS EPRE (which is equal to the PDSCH EPRE or the nominal EPRE).

We propose to fix βCSI-RS for row 17, 32 APs, L=4, (#CSI-RS CDM group in an OFDM symbol=4 as

|  | RAN4 | DMRS like |
| --- | --- | --- |
| EPRE before precoding (βCSI-RS)² | 1/L=1/4 | #CSI-RS CDM group in an OFDM symbol = 4 |

Observation 12: The CSI-RS EPRE assumed by RAN4 is too low with respect to the DMRS EPRE assumption.

- CDM aims at improving the quality of the Least Square channel estimate at the receiver side (thanks to de-spreading), the CDM de-spreading gain at the receiver side is cancelled by the adoption of 1/L EPRE in Table C.3.1-1 of TS38.101-4,

- The number of CDM groups in the frequency dimension (#CSI-RS CDM group in an OFDM symbol) allows boosting the CSI-RS EPRE  since for a given non-zero power RE for CSI-RS there is #CSI-RS CDM group in an OFDM symbol-1 RE set to zero, this power boosting is ignored in Table C.3.1-1 of TS38.101-4.

We propose to change Table C.3.1-1 of TS38.101-4 in the following way

|  | RAN4 | Proposal |
| --- | --- | --- |
| EPRE before precoding (βCSI-RS)² in [dB] | -10log10(L) | 0 if no power boosting in FD 10log(#CSI-RS CDM group in an OFDM symbol) if enabling power boosting in FD |

Proposal 2: Modify the value of the EPRE ratio of CSI-RS to SSS Table C.3.1-1 of TS38.101-4 to 0 if no power boosting is allowed in the frequency domain or 10log(#CSI-RS CDM group in an OFDM symbol) if power boosting is allowed.

### Simulation results

#### CDL 901 – 32 Tx (12,8,2,6,1) subarray AAV

| DMRS like EPRE (EPRE before precoding=4) | RAN4 EPRE (EPRE before precoding=1/4) |
| --- | --- |

#### CDL 827 - 32 Tx (12,8,2,6,1) subarray AAV

| DMRS like EPRE (EPRE before precoding=4) | RAN4 EPRE (EPRE before precoding=1/4) |
| --- | --- |

Observation 13: eType-II CB performance is more sensitive to the CSI-RS channel estimation quality compared to Type-I.

Proposal 2: Modify the value of the EPRE ratio of CSI-RS to SSS Table C.3.1-1 of TS38.101-4 to 0 if no power boosting is allowed in the frequency domain or 10log(#CSI-RS CDM group in an OFDM symbol) if power boosting is allowed.

# Conclusion

The CDL models and AAV configurations should be carefully selected in order to be able to discriminate the performance between low- and high-resolution CBs. For instance, the antenna panel and subarray AAV (M,N,P,Ms,Ns)= (12,8,2,6,1) allows discriminating these two types of CB for both CDL-C 827 and 901. We showed that 901 has intrinsically more “direction diversity” than 827 which yields larger performance gap between Type-I and Type-II CB. Additionally, the RAN4 CSI-RS EPRE needs to be discussed, it appears to be too pessimistic in view of the DMRS EPRE assumption.

Proposal 1: Consider a higher number of CSI-RS antenna ports and antenna elements for eType-II performance evaluation, e.g., an antenna panel and subarray AAV (M,N,P,Ms,Ns)= (12,8,2,6,1).

Proposal 2: Modify the value of the EPRE ratio of CSI-RS to SSS Table C.3.1-1 of TS38.101-4 to 0 if no power boosting is allowed in the frequency domain or 10log(#CSI-RS CDM group in an OFDM symbol) if power boosting is allowed.

# References

- R4-2415285, “TDL and CDL comparison with respect to the CSI resolution feedback,” Orange, BT, Telecom Italia, Vodafone, Bouygues Telecom, Fraunhofer, Deutsche Telekom, 3GPP RAN4#112b, Oct., 2024.

- X. Fu, D. Le Ruyet, R. Visoz, V. Ramireddy,  M. Grossman, M. Landmann, W. Quiroga , "A Tutorial on Downlink Precoder Selection Strategies for 3GPP MIMO Codebooks," in IEEE Access, vol. 11, pp. 138897-138922, 2023 (URL).

- R4-2419782, “Way Forward for [113][320] NR_SCM,” Nokia, 3GPP RAN4#113, Nov., 2024.

- 3GPP. 5G; Study on channel model for frequencies from 0.5 to 100 GHz. Technical Specification (TS) 38.901, 3rd Generation Partnership Project (3GPP), 01 2024. Version 17.1.0.

- R4-2402277, “Discussion on 8rx general demodulation aspects and spatial channel models,” Nokia, BT, 3GPP TSG-RAN WG4 Meeting #110, February 2024

# Annex A: SNR computation

The receive signal per RE without precoding (for a given subcarrier f∈{0,…,NFFT-1} ) is

y(f)=H(f)x(f)+n(f)

where H(f)∈CNR×NT is the MIMO channel at subcarrier f assuming NT=2N1N2 transmit and NR receive antennas.

For notation simplicity, we omit the frequency dependance when it is obvious. We suppose that the transmit power PT=Exx†=1 or Ext2=12N1N2. The noise is circularly symmetric with covariance K=Enn†=σ2INR .

The receive power by definition is

PR=Ex†H†Hx
=Et,t'xt*H†Ht,t'xt' .
Since the symbols belonging to different spatial layers are independent, it yields

PR=Etxt2H†Ht,t 
=12N1N2t,rEhr,t2,

where hr,t is the (r, t)-th element of H . Since we use the channel power normalization
σh2=12N1N2NRt,rEhr,t2=1,

it comes that the received power per RE is PR=NR. Finally, the receive power per receive antenna is P=1. The SNR defined by 1σ2  where σ2 is the noise variance corresponds to the SNR per receive antenna per RE which is what RAN4 has defined.

Let the time domain channel with τ taps be hr,t,ii=0τ-1  , then hr,t(f)=i=0τ-1hr,t,ie-j2πif/NFFT  , it yields that Ehr,tf2=i=0τ-1E{hr,t,i 2}  (independence between taps)

In the time domain we have

12N1N2NRt,rEhr,tf2=12N1N2NRt,ri=0τ-1E{hr,t,i 2}=1

The CDL/TDL channel is normalized in the time domain such that hr,t,i  is normalized to 1Chr,t,i with

C=12N1N2NRt,ri=0τ-1E{hr,t,i 2}

Numerical evidence confirms that σh2f=1 ∀f∈{0,…,NFFT-1} .

# Annex B: Simulation parameters

| BS antenna configuration | WF compliant |
| --- | --- |
|  | Option 1Y: M,N,P,Ms,Ns=8,4,2,8,1-8Tx |
|  | Option 2: M,N,P,Ms,Ns=8,4,2,8,8- 8Tx |
|  | Option 1X: M,N,P,Ms,Ns=8,8,2,8,2- 8Tx |
|  | Non WF compliant |
|  | M,N,P,Ms,Ns=(12,4,2,12,1) - 8Tx |
|  | M,N,P,Ms,Ns=(12,4,2,6,1) - 16Tx |
|  | M,N,P,Ms,Ns=(8,8,2,8,1) - 16Tx |
|  | M,N,P,Ms,Ns=(12,8,2,6,1) -32Tx |
| BS antenna spacing | dH,dV=0.5,0.5 λ |
| UE antenna configuration | M,N,P,Ms,Ns=1,2,2,1,1-4Rx |
| UE antenna spacing | dH,dV=0.5, - λ |
| BS antenna polarization | (+45,-45°) |
| UE antenna polarization | (0,90°) |
| BS antenna radiation pattern | Defined in TR38.901 Table 7.3-1 |
| UE antenna radiation pattern | Omnidirectional |
| BS height | 25m |
| UE height | 1.5m |
| BS antenna orientation | α=0°, β=10°,γ=0° |
| UE antenna orientation | α=180°, β=0°,γ=0° |
| Distance BS-UE (D2D) | 100m |
| UE speed | 3km/h or 30 km/h |
| UE travel direction | ϕv,θv=(65°,90°) |
| MCS/RI/PMI delay | 6.5ms |
| FR/carrier frequency | FR1/3.5 GHz |
| Subcarrier Spacing | μ=30 kHz |
| Number of Sub-bands (SB) | N3=14 |
| Number of PRBs per SB | NSB=8 |
| Total number of PRBs (40MHz, SCS=30kHz) | TBW=106 |
| Rank | rank 4 or 2 |
| Modulation and coding scheme (MCS) | WF compliant |
|  | MCS 7 from Table 2 for PDSCH (corresponding to MCS 13 from Table 1) |
| Retransmission type | Chase |
| Max. number of retransmissions | NRTmax=4 |
| Channel model | 3GPP 38.901/38.827 UMa CDL-C RMS Delay Spread=365ns |
| Detector at the receiver | LMMSE |
| eType-II/Type-I spatial domain basis (W1) | WF compliant |
|  | N1,N2,O1,O2=(4,1,4,1) |
|  | Non WF compliant |
|  | N1,N2,O1,O2=(4,2,4,4) for (12,4,2,6,1) 16 Tx |
|  | N1,N2,O1,O2=(8,1,4,1) for (8,8,2,8,1) 16 Tx |
|  | N1,N2,O1,O2=(8,2,4,4) for (12,8,2,6,1) 32 Tx |
| eType-II configuration | paramCombination-r16=5 L=4,pν=1/4,β=3/4 |
| Channel Estimation | Perfect or LMMSE based CSI-RS channel estimation based on row 17 for 32Tx. |

# Annex C: CDL-C 901 and 827 channel generation

Step 1: generate arrival and departure angles for both azimuth and elevation.

For CDL 901, the azimuth angle of arrival for sub-path m of cluster n is given as

ϕn,m,AOA=ϕn,AOA+cASAαm,

where ϕn,m,AOA is the cluster AOA and cASA is the cluster-wise rms azimuth spread of arrival angles given by Tables 7.7.1.1 – 7.7.1.5 - [4], and αm denotes the ray offset angles within a cluster given by Table 7.5-3 - [4]. The generation of AOD (ϕn,m,AOD), ZOA (θn,m,ZOA), and ZOD (θn,m,ZOD) follows a similar procedure. [4]

One CDL only represents a single channel realization, the predefined angle values in the CDL models can be generalized by introducing angular translation and scaling. By translation, mean angle can be changed to μϕ,desired and angular spread can be changed to ASdesired by scaling. The angular scaling and translation are optional in CDL 901 and applied according to Clause 7.7.5.1 - [4]. The translated and scaled ray angles can be obtained according to the following equation:

ϕn,m,AOA,scaled=ASdesiredASmodelϕn,m,AOA-μϕ,model+μϕ,desired,

where ASmodel is the rms angular spread of the tabulated CDL 901 including the offset ray angles, given in [4].

ASmodel=-2lnnmexpjϕn,mPn,mnmPn,m,

and μϕ,model is the mean angle of the tabulated CDL given as

μϕ,model=argnmexpjϕn,mPn,m.

Here Pn,m is the power for the m-th sub-path of the n-th path and ϕn,m is the sub-paths angle in radians. The values for angular spreads presented in [5] are given below.

Table C1: Original angle spreads of CDL models UMi and UMa (K-factor 9 dB) - Table 7.2-2 [5]

| Model | ASmodel [deg] |  |  |  |
| --- | --- | --- | --- | --- |
|  | ASD | ASA | ZSD | ZSA |
| CDL-A | 73.6985 | 85.2676 | 28.5575 | 21.0831 |
| CDL-B | 41.5917 | 59.3326 | 5.9633 | 10.3818 |
| CDL-C | 39.0949 | 71.1175 | 4.0666 | 10.4245 |
| CDL-D | 15.6771 | 17.3604 | 2.4462 | 1.5362 |
| CDL-E | 13.1544 | 37.5640 | 1.4577 | 2.4601 |

In TS 38.827, the desired mean angles equal the original CDL mean angles, i.e.,

μϕ,model=μϕ,desired,

and the desired angular spreads are given in [5] as

Table C2: Desired AS for UMi and UMa at 3.5 GHz (FR1) - Table 7.2-4 [5]

| Model | ASdesiredASdesired [deg] |  |  |  |
| --- | --- | --- | --- | --- |
|  | ASD | ASA | ZSD | ZSA |
| UMi NLOS (CDL-A, B, C) | 23.9751 | 57.2457 | 0.7762 | 7.8320 |
| UMi LOS (CDL-D, E) | 15.0432 | 47.6149 | 0.6166 | 4.6204 |
| UMa NLOS (CDL-A, B, C) | 25.7620 | 74.1138 | 4.8978 | 18.2050 |
| UMa LOS (CDL-D, E) | 14.0180 | 64.5654 | 3.4674 | 8.9125 |
| Note: For UMa frequency fc = 6 as stated in [2], and other parameters hUMa = 25, hUMi = 10, hUT = 1.5, and D2D = 100. |  |  |  |  |

Note: The azimuth angles may need to be wrapped around to be within [0, 360] degrees, while the zenith angles may need to be clipped to be within [0, 180] degrees.

Step 2: coupling of rays within a cluster for both azimuth and elevation.

For CDL 901, we couple randomly AOD angles ϕn,m,AOD to AOA angles ϕn,m,AOA within a cluster n, we couple randomly ZOD angles θn,m,ZOD with ZOA angles θn,m,ZOA using the same procedure, and we couple randomly AOD angles ϕn,m,AOD with ZOD angles θn,m,ZOD within a cluster n. While for CDL 827, we use fixed coupling given by the table below.

Table C3: Fixed coupling pattern of ray angles to be applied for each cluster – Table 7.2-6 [5]

|  | m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 6 | 12 | 5 | 10 | 8 | 11 | 16 | 14 | 18 | 9 | 20 | 4 | 2 | 15 | 7 | 13 | 19 | 17 | 3 | 1 |
|  | 20 | 9 | 12 | 1 | 13 | 18 | 10 | 4 | 8 | 2 | 6 | 14 | 11 | 19 | 7 | 3 | 17 | 5 | 15 | 16 |
|  | 2 | 16 | 3 | 11 | 18 | 9 | 5 | 17 | 4 | 19 | 15 | 20 | 13 | 7 | 10 | 1 | 8 | 12 | 6 | 14 |
|  | 15 | 18 | 13 | 1 | 12 | 9 | 6 | 7 | 5 | 3 | 2 | 8 | 14 | 17 | 19 | 16 | 11 | 20 | 10 | 4 |

Step 3: initial phases

For CDL 901, we draw random initial phases {Φn,mθθ,Φn,mθϕ,Φn,mϕθ,Φn,mϕϕ} for each ray m of each cluster n and for four different polarization combinations θθ,θϕ,ϕθ,ϕϕ. The distribution for initial phases is uniform within -π,π. While for CDL 827, use fixed initial phases. The set of fixed initial phases can be same for all clusters for all four polarization combinations, i.e.

Φn,mθθ=Φmθθ, ∀n=1,…,N.

The 20×4 initial phase values are specified by the Table C4. The initial phases for all four polarization combinations can also be set to

Φn,mθθ=Φmθθ+ϕn,m, ∀n=1,…,N.

Here ϕn,m is a scalar random initial phase term drawn from uniform distribution [-π,π]. This term is shared for all BB visible Tx/Rx branch pairs of the current cluster and ray combination.

Table C4: Fixed initial phases for 2x2 polarization matrices. These values are drawn from uniform distribution – Table 7.2-8 [5]

| m | Φmθθ [rad] | Φmϕθ [rad] | Φmθϕ [rad] | Φmϕϕ [rad] |
| --- | --- | --- | --- | --- |
| 1 | 1.7609 | -0.6928 | -1.6230 | -0.6037 |
| 2 | -2.5356 | -2.3124 | 2.7775 | 2.8660 |
| 3 | 0.4725 | -2.7660 | -1.6664 | -0.9226 |
| 4 | 2.0181 | -3.0448 | -2.8713 | -2.0798 |
| 5 | 0.9369 | 1.4560 | 0.9283 | -0.3084 |
| 6 | 0.2954 | -1.2798 | 1.5375 | -1.9544 |
| 7 | 1.1735 | -1.9886 | -0.8263 | 0.7893 |
| 8 | 1.7607 | -2.6319 | 2.6979 | 1.7324 |
| 9 | -0.0830 | -0.4030 | -0.3344 | -1.2167 |
| 10 | 0.0535 | 0.0677 | 1.9957 | 1.8525 |
| 11 | 0.9068 | -0.7627 | 1.9577 | 0.2062 |
| 12 | -0.9379 | 2.7583 | 2.3621 | 0.3151 |
| 13 | 0.7695 | 0.5469 | -1.8363 | -1.2488 |
| 14 | -0.1827 | -1.6934 | 2.1634 | -1.9179 |
| 15 | -1.7221 | -2.0690 | -1.7111 | -0.4040 |
| 16 | -1.1869 | 2.6602 | -0.4385 | -1.9804 |
| 17 | 2.5439 | 3.0143 | -0.3841 | -2.4434 |
| 18 | -1.5201 | -0.5735 | 0.5962 | -1.4941 |
| 19 | 0.6462 | 1.3271 | -1.7483 | -2.4038 |
| 20 | -1.2775 | -1.1386 | -0.4765 | 0.0494 |

Step 4: Generate the cross-polarization power ratios

Generate the cross-polarization power ratios (XPR) κn,m for each ray m of each cluster n as

κn,m=10X/10

with X = 7 dB for both CDL-C 901 and 827.

Step 5: Generate channel coefficients for each cluster n and each receive and transmit antenna elements (u, s)

For CDL 901 and 827, before AAV, we can write the channel coefficient at time t for transmit antenna s, receive antenna u and cluster n as the sum of the coefficients of all the sub-paths, that is

hu,s,nt=mαn,mejβs,n,m+βu,n,m+γn,mt

- Generate coefficients αn,m

αn,m=PnMFrx,u,θθn,m,ZOA,ϕn,m,AOAFrx,u,ϕθn,m,ZOA,ϕn,m,AOATexpjΦn,mθθ κn,m-1expjΦn,mθϕκn,m-1expjΦn,mϕθexpjΦn,mϕϕFtx,s,θθn,m,ZOD,ϕn,m,AODFtx,s,ϕθn,m,ZOD,ϕn,m,AOD

- Frx,u,θθn,m,ZOA,ϕn,m,AOAFrx,u,ϕθn,m,ZOA,ϕn,m,AOA is the UE antenna element field pattern in GCS.

- Ftx,s,θθn,m,ZOD,ϕn,m,AODFtx,s,ϕθn,m,ZOD,ϕn,m,AOD is the BS antenna element field pattern in GCS.

- expjΦn,mθθ κn,m-1expjΦn,mθϕκn,m-1expjΦn,mϕθexpjΦn,mϕϕ is the polarization matrix.

Ftx,s,θθn,m,ZOD,ϕn,m,AODFtx,s,ϕθn,m,ZOD,ϕn,m,AOD=RψtxF'tx,s,θ'θ'n,m,ZOD,ϕ'n,m,AODF'tx,s,ϕ'θ'n,m,ZOD,ϕ'n,m,AOD

Where Rψtx is the rotation matrix (LCS to GCS conversion) and the field pattern in BS LCS are given by

F'tx,s,θ'θ'n,m,ZOD,ϕ'n,m,AODF'tx,s,ϕ'θ'n,m,ZOD,ϕ'n,m,AOD=ABS'θ'n,m,ZOD,ϕ'n,m,AODcosξBSsinξBS

Here  θ'n,m,ZOD and ϕ'n,m,AOD represent the zenith and azimuth angles of departure in BS local coordinate system (LCS) (converted from global coordinate system (GCS) to BS LCS), respectively,  ABS'θ'n,m,ZOD,ϕ'n,m,AOD is the BS antenna radiation power pattern, and the polarization slant ξBS=±45° for cross-polarization.

- Generate coefficients βs,n,m /βme,ne,n,m

βs,n,m=2πλrn,mD.dstx,

where the vector of transmit antenna element s is given by

dstx=0nHsΔHBSnVsΔVBST ,

and the spherical unit vector of departure in BS LCS is

rn,mD=sinθn,m,ZOD'cosϕn,m,AOD'sinθn,m,ZOD'sinϕn,m,AOD'cosθn,m,ZOD'T .

- Generate coefficients βu,n,m

βu,n,m=2πλrn,mA.durx,

where the vector of receive antenna element u is given by

durx=0nHuΔHUEnVuΔVUET,

and the spherical unit vector of arrival in UE LCS is

rn,mA=sinθn,m,ZOA'cosϕn,m,AOA'sinθn,m,ZOA'sinϕn,m,AOA'cosθn,m,ZOA'T.

- Generate coefficients γn,m

γn,m=2πλrn,mAv,

where the spherical unit vector of arrival in GCS is given by

rn,mA=sinθn,m,ZOA cosϕn,m,AOA sinθn,m,ZOA sinϕn,m,AOA cosθn,m,ZOA T ,

and the velocity vector is defined in GCS as

v=Vm sinθvcosϕvsinθvsinϕvcosθv,

here Vm is the UE velocity and (ϕv,θv) represents the travelling direction.

Step 6: Generate cluster delays

In TS 38.827, absolute delays are given for a rms delay spread (DS) of 365 ns, while for CDL 901, the delays need to be scaled by the rms DS.

Step 7: Generate channel impulse response for each receive and transmit antenna elements (u, s)

The channel impulse response for transmit antenna s and receive antenna u is given as

hu,st,τ=nhu,s,ntδτ-τn

Step 8: Antenna array virtualization

- Option 1: subarray AAV

The channel impulse responses corresponding to the antenna elements belonging to a given sub-array are summed with ones as weights for all antenna elements.

- Option 2: fully connected AAV

According to TS 38.827, the co-polarized elements of the array are combined to a single RF port. The weight vector applied for the first and second polarizations is.

as,1,1…as,Me,Ne =1MeNeexp-j2πrtx,max,s.dtx,me,neλ0,

where dtx,me,ne is the location vector of transmit antenna element (me,ne) in BS LCS and rtx,max,s is a spherical unit vector of departure in BS LCS denoting the target beam direction s=1,…,S.

rtx,max,s=sinθs,ZOD'cosϕs,AOD'sinθn,m,ZOD'sinϕs,AOD'cosθs,ZOD'T.

Hence, the BS transmits downlink signals with S beams, and the channel coefficient for instant t, receive antenna u, transmit beam s, and cluster n is defined by

hu,s,nt=mnemeαn,mas,me,neejβme,ne,n,m+βu,n,m+γn,mt,

here (me,ne) denotes the antenna elements indices at BS, the coefficients αn,m, ϕn,m, βme,ne,n,m, βu,n,m, and γn,m are defined above.

Note: The beam directions (θ's,ZOD ,ϕs,AOD') in BS LCS are calculated based on the angles in GCS (θs,ZOD ,ϕs,AOD ) defined hereafter. In TS 38.827, the way the beamforming directions are obtained for FR1 is explained as: a codebook of 60 fixed beams is constructed to a grid of five elevation angles from -20° to +20° with 10° steps and 12 azimuth angles from -80° to +80° with approximately 15° steps.

For 8x8 MIMO, the four strongest transmitting beams for UMa CDL-C given in [5] are:

- Strongest beam: AoD: -7.27°, ZoD: 100°

- 2nd strongest beam: AoD: -21.82°, ZoD: 100°

- 3rd strongest beam: AoD: -36.36°, ZoD: 100°

- 4th strongest beam: AoD: -50.91°, ZoD: 100°

The power, delays, and angles (AOA, AOD, ZOA, ZOD) are given for CDL-C 901 and 827 by Tables C5 and C6, respectively.

Table C5: parameters for CDL-C 901 - Table 7.7.1-3 - [4]

Table C6: parameters for CDL-C 827 - Table 7.2.1-8 - [5]