3GPP TSG-RAN WG4 Meeting #114							         			R4-2500218

Athens, GR, 17th – 21st February 2025

Title: 	Simplification of HPUE 2UL DeltaMSD

Source: 	Skyworks Solutions, Inc.

Agenda Item:	7.1.1.1.1	General aspects (incl. duty cycle solutions, MSD rules) [NR_ENDC_RF_Ph4-Core]

Document for:	Approval

# 1	Background

In RAN#113, we discussed in [1,2] the status of the MSD for HPUE inter-band combinations for 2UL IMDs and made some early proposals for simplification of the existing Rel-17/18 cases. Together with other contributors, a way forward [3] mostly recorded the cases to be analyzed for the following meeting. In this contribution, we provide our analysis for all nine HPUE cases (and sub-cases) for MSD to own band for two-band combinations and MSD to third band for a three-band combination. The document also provides the associated assumptions and short-cuts to reduce the number of cases/equations and, although not 100% accurate, it is more reasonable and accurate than the current process.

# Discussion

## 2.1 WF inputs

The way forward [2] provides a summary of the cases to be studied and aspects to be considered for the simplification.

Up to ten different UL-CA (2UL) power class configurations have been identified for Rel-19 [7].

| case # |  | 1 Baseline | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ULCA power class | Type | PC3 2Tx | PC2 2Tx | PC2 2Tx | PC2 3Tx | PC2 2Tx | PC2 3Tx | PC1.5 2Tx | PC1.5 3Tx | PC1.5 3Tx | PC1.5 3Tx |
|  | PCmax | 23 | 26 | 26 | 26 | 26 | 26 | 29 | 29 | 29 | 29 |
| per band power class | Bd1 | PC3 1Tx | PC3 1Tx | PC3 1Tx | PC3 1Tx | PC2 1TX | PC2 1TX | PC2 1TX | PC2 1TX | PC3 1Tx | PC2 1TX |
|  | Bd2 | PC3 1Tx | PC3 1Tx | PC2 1Tx | PC2 2Tx | PC2 1Tx | PC2 2Tx | PC2 1Tx | PC2 2Tx | PC1.5 2Tx | PC1.5 2Tx |
| Valid Release |  | R15 | R17 | R17 | R18 | R19 | R19 | R19 | R19 | R18 | R19 |

Agreement:

- Companies are invited to

  - Provide 2UL HPUE MSD simplification rules to cover the Rel-19 supported 2UL configurations using either equations/LUT or single value de-ratings relative to the default power class 3 MSD,

  - Study means to minimize the number of LUTs / set of equations that cover all cases.

  - Check if all Rel-19 cases have been correctly captured and to look at the initial proposals in [4] and [7].

  - FFS if only IMD orders are sufficient to design LUT/ set of equations or if both the IMD order and the IMD indexes should be accounted for.

  - FFS to account for other IMD contributors than just the power amplifier.

Beyond the agreements copied above, we also made the analysis for both MSD to own band for two-band combinations and MSD to third band for a three-band combination.

## 2.2 Detailed description of the cases and high-level assumptions

To analyze the different cases, it is essential to get the details of how the interference level increases and how it compares with the PA linearity (e.g., is the linearity staying the same or increasing with the interference level, and is it the same for each UL band or not).

Table 1 provides the details of each PA in each band and the associated 2UL power:

- The first two columns provide a case number and which release introduced such combinations.

- The next two columns provide the inter-band 2UL ower class and the associated total number of Tx.

- Then, parameters are provided per each UL band that are essential for the analysis of the IMD increases, as follows:

  - The power class per band

  - The number of Tx per band

  - The 1UL max power per band

  - The 2UL power per band (sum is limited to inter-band power class)

  - Associated PA back-off per band in 2UL mode.

- Finally, the last two columns provide the potential maximum power with both bands at their maximum and whether this opens for the increased power option. Note that increased power above PC1.5 may need further discussion, but this is not in the scope of this contribution.

Table 1: Detailed description of the different inter-band HPUE 2UL combinations

| case | Rel | ULCA PC | #Tx | FDD/TDD Types (R19 only) | /Bd PC |  | /Bd #Tx |  | /Bd 1UL Pwr |  | /Bd 2UL power |  | /Bd 2UL BO |  | Max Tot Pwr | Inc Pwr option |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Bd1 | Bd2 | Bd1 | Bd2 | Bd1 | Bd2 | Bd1 | Bd2 | Bd1 | Bd2 |  |  |
| 1 | R15 | PC3 | 2 | F+T, T+T, F+F | PC3 | PC3 | 1 | 1 | 23 | 23 | 20 | 20 | -3 | -3 | 26 | N(+3dB) |
| 2 | R17 | PC2 | 2 |  | PC3 | PC3 | 1 | 1 | 23 | 23 | 23 | 23 | 0 | 0 | 26 | N |
| 3 | R17 | PC2 | 2 | F+T, T+T, F+F | PC3 | PC2 | 1 | 1 | 23 | 26 | 23 | 23 | 0 | -3 | 27.8 | Y(+1.8dB) |
| 4 | R18 | PC2 | 3 |  | PC3 | PC2 | 1 | 2 | 23 | 2x23 | 23 | 2x20 | 0 | -3 | 27.8 | Y(+1.8dB) |
| 5 | R18 | PC2 | 2 | F+T, T+T, F+F | PC2 | PC2 | 1 | 1 | 26 | 26 | 23 | 23 | -3 | -3 | 29 | N(+3dB) |
| 6 | R19 | PC2 | 3 |  | PC2 | PC2 | 1 | 2 | 26 | 2x23 | 23 | 2x20 | -3 | -3 | 29 | N(+3dB) |
| 7 | R19 | PC1.5 | 2 |  | PC2 | PC2 | 1 | 1 | 26 | 26 | 26 | 26 | 0 | 0 | 29 | N |
| 8 | R19 | PC1.5 | 3 |  | PC2 | PC2 | 1 | 2 | 26 | 2x23 | 26 | 2x23 | 0 | 0 | 29 | N |
| 9 | R18 | PC1.5 | 3 | F+T, T+T | PC3 | PC1.5 | 1 | 2 | 23 | 2x26 | 23 | 2x24.7 | 0 | -1.3 | 30 | Y(+1dB) |
| 10 | R19 | PC1.5 | 3 |  | PC2 | PC1.5 | 1 | 2 | 26 | 2x26 | 26 | 2x23 | 0 | -3 | 30.8 | Y(+1.8dB) |

Observation: One first observation is that the cases in yellow (including the baseline PC3 case) would allow the  implementation of the next power class of inter-band HPUE provided no other UE limitation applies:

- PC3 case 1 implementation can support PC2 case 2

- PC2 case 5 can support PC1.5 case 7

- PC2 case 6 can support PC1.5 case 8

- This may be a later option for simplification to merge case 5/6 with 7/8 respectively.

To derive the IMD increase, after the details are known, one must make some assumption in term of the dominating path for the IMD. Based on experience and existing analysis, we propose to use the assumptions in Table 2 for MSD to own band or to third band depending on the FDD/TDD types of the two bands and the number of Tx per band:

- T+T means 2UL is TDD+TDD => only the third band may be affected by MSD.

- F+T means 2UL is FDD+TDD => only the FDD band can be affected by MSD for two bands

- F+F means 2UL is FDD+FDD => both bands may be affected by MSD for two bands

- #DL denotes the number of DL bands in the inter-band combination

Proposal: The following table is proposed as assumptions for IMD increase calculations

Table 2: Proposed assumptions on dominating IMD generator versus 2/3 band DL, TDD/FDD and 2/3Tx

| #DLB | 2UL | Assumptions on dominating IMD | 3Tx assumption |
| --- | --- | --- | --- |
| 2 | F+T | Only FDD band is victim => dominated by FDD PA | If 2Tx on TDD => Only one TDD PA dominates coupling to FDD PA |
|  |  |  | If 2Tx on FDD => Only one FDD PA dominates IMD |
|  | F+F | Dominated by FDD PA associated with victim DL band | if 2Tx on victim FDD => Only one PA dominates IMD |
|  |  |  | if 1Tx on victim FDD => Only one of 2Tx PA dominates coupling to FDD PA |
| 3 | F/T+F/T | Dominated by victim band LNA | Only one LNA IMD is dominated by both UL |

Beyond these assumptions on the dominating IMD generator, there are other aspects where assumptions and possibly short cuts are needed:

- For the PA and the LNA even if the power levels are the same at the two PA outputs, all IMDs will be asymmetric in level because:

  - A PA sees only reverse IMD, thus its own signal and a small coupled signal from the other PA

  - The LNA sees each PA with a different coupling loss

- Asymmetric IMD is ignored as there is no way to set a generic imbalance and, even if that is the case, one would then have to look at which side IMD is the interfering one. Currently, even with a thorough analysis, there is no chance that the assumptions and levels would be the same than the one used for the PC3 analysis. In most cases, this issue is ignored regardless.

- When there is asymmetry in the level in each PA, this is considered in the IMD increase calculation by retaining the worst-case side.

- For a thorough analysis beyond the IMD order, one would have to account for the IMD index, especially when there is asymmetry in the two UL band levels.

- We also propose to ignore the IMD index since the asymmetry is not known or generic. Thus, we assume that the IMD increase is only proportional to the IMD order. Only the asymmetry of the power per band is considered and the worst-case IMD retained.

Proposal on IMD asymmetry and indexes: Since there is no way to adopt a generic approach in terms of UL band level asymmetry, we propose to ignore asymmetric levels and IMD indexes in the IMD increase calculations. However, when the level in each UL band is asymmetric, the worst IMD side is considered.

Finally, there could be as many architectures and antenna sharing schemes as one would like. The assumption here is the most common and, in most cases, the worst-case: the two UL bands share the main path antenna and thus the main antenna PA (1Tx or 2Tx) associated with the victim band dominates the IMD generation. Also, the levels at the victim LNA input are larger in the main path.

Proposal on architecture assumption: The main path of each UL band shares the same antenna and IMD interference dominates in the main DL path and is generated by the main PA associated with the victim band.

## 2.2 IMD increase calculations for 2B DL/2B UL

Using the assumptions described in the previous section, we have calculated the IMD increase for all nine HPUE cases and sub-cases by comparing step by step between the different cases. In this case, the IMD generator is the PA associated with the victim FDD band that shares its antenna.

These calculations are summarized in Table 3:

- The first seven columns provide the case description:

  - Col1: Case number and related release

  - Col2: Inter-band power class and number of Tx

  - Col3-7: Band1 or band 2 parameters: Power class per band / Number of Tx per band/Output power per band in 2UL mode / Back-off per band in 2UL mode

- The eighth column shows the IMD increase versus the PC3 baseline in dB and in proportion of the IMD order “Nxorder” in bold. Then, it shows the increase versus another case which is often useful to check how IMD evolves iteratively.

  - In cases 3, 4, 6, and 8, some sub-cases are needed to account on which band (power class, number of Tx) dominated the IMD when the two bands are of different types.

- Finally, the last four columns provide the maximum applicable DeltaMSD per IMD order.

Table 3: DeltaMSD calculations summary for 2B DL/2B UL

| case /Rel | ULCA PC /#Tx | Bd | /Bd PC | /Bd #Tx | /Bd 2UL power | /Bd 2UL BO | 2DL2UL interference level inrease vs Ref / vs case(x) [dB] | ∆MSD max [dB] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | 2 | 3 | 4 | 5 |
| 1 | PC3 | Bd1 | PC3 | 1Tx | 20 | -3 | Ref | 0 | 0 | 0 | 0 |
| R15 | 2Tx | Bd2 | PC3 | 1Tx | 20 | -3 |  |  |  |  |  |
| 2 | PC2 | Bd1 | PC3 | 1Tx | 23 | 0 | +3xorder / =(1)+3xorder | 6 | 9 | 12 | 15 |
| R17 | 2Tx | Bd2 | PC3 | 1Tx | 23 | 0 |  |  |  |  |  |
| 3 | PC2 | Bd1 | PC3 | 1Tx | 23 | 0 | if PC3 is FDD victim band: +3xorder / =(2) | 6 | 9 | 12 | 15 |
| R17 | 2Tx | Bd2 | PC2 | 1Tx | 23 | -3 | if PC2 is FDD victim band: +3 / =(1)+3 | 3 | 3 | 3 | 3 |
| 4 | PC2 | Bd1 | PC3 | 1Tx | 23 | 0 | if PC3 is FDD victim band: +3x(order-1)=+3xorder-3 / =(1)+3 Bd1 | 3 | 6 | 9 | 12 |
| R18 | 3Tx | Bd2 | PC2 | 2Tx | 2x20 | -3 | if PC2 is FDD victim band: +3x(order-1)=+3xorder-3 / =(1)+3 Bd1 | 3 | 6 | 9 | 12 |
| 5 | PC2 | Bd1 | PC2 | 1TX | 23 | -3 | +3 / =(1)+3 => since this could be PC1.5: | 3 | 3 | 3 | 3 |
| R19 | 2Tx | Bd2 | PC2 | 1Tx | 23 | -3 |  |  |  |  |  |
| 6 | PC2 | Bd1 | PC2 | 1TX | 23 | -3 | if 1Tx is FDD victim band: +3-3x(order-2)=-3xorder+9 / =(5)-3 Bd2 | 3 | 0 | -3 | -6 |
| R19 | 3Tx | Bd2 | PC2 | 2Tx | 2x20 | -3 | if 2Tx is FDD victim band: +3x(order-1)=+3xorder-3 / =(1)+3 Bd1 | 3 | 6 | 9 | 12 |
| 7 | PC1.5 | Bd1 | PC2 | 1TX | 26 | 0 | +3xorder+3 / =(2)+3 | 9 | 12 | 15 | 18 |
| R19 | 2Tx | Bd2 | PC2 | 1Tx | 26 | 0 |  |  |  |  |  |
| 8 | PC1.5 | Bd1 | PC2 | 1TX | 26 | 0 | if 1Tx is FDD victim band: +3x(order-1)+3=3xorder / =(4)+3dB | 6 | 9 | 12 | 15 |
| R19 | 3Tx | Bd2 | PC2 | 2Tx | 2x23 | 0 | if 2Tx is FDD victim band: +3*(order+order-1)=6xorder-3 / =(2)+3*(order-1) | 9 | 15 | 21 | 27 |
| 9 | PC1.5 | Bd1 | PC3 | 1Tx | 23 | 0 | +3*order+1.7*(order-1)=+4.7xorder-1.7/ =(2)+1.7*(order-1) | 7.7 | 12.4 | 17.1 | 21.8 |
| R18 | 3Tx | Bd2 | PC1.5 | 2Tx | 2x24.7 | -1.3 |  | =>9 | =>12 | =>18 | =>21 |
| 10 | PC1.5 | Bd1 | PC2 | 1TX | 26 | 0 | +3*(order-1)+3=3xorder / =(4)+3dB | 6 | 9 | 12 | 15 |
| R19 | 3Tx | Bd2 | PC1.5 | 2Tx | 2x23 | -3 |  |  |  |  |  |

Observation:

- The worst-case IMD increase versus PC3 is 27dB for IMD5 of PC1.5=PC2+PC2 2Tx when 2Tx is the victim band. This is high but one needs to consider that PC3 IMD5MSD levels are low, thus 27dB is highly unlikely.

- There are cases where the MSD would decrease (red highlight) for the PC2=PC2+PC2 2Tx case. This is because the levels are asymmetric and the PA generating the IMD has higher capability, thus higher-order IMDs would decrease. Since this is difficult to account for and the same implementation is capable of PC1.5, we propose to clamp the negative values at zero.

- Where yellow highlight appear, the UL level on one band is not increased by a 3dB step, resulting in IMD increases that are not a multiple of 3dB. Here, we suggest rounding to the closest 3dB increment.

- Overall, for any IMD increases, the maximum DeltaMSD is from 0 to 27dB in 3dB increments. The higher values are for higher IMD orders where the PC3 MSD is small, thus the maximum DeltaMSD is not reached.

- It should therefore be noted that higher order IMDs (6,7..?) could be of concern for the inter-band highest power classes.

## 2.3 IMD increase calculations for 3B DL/2B UL

Using the assumptions for the third DL band MSD being dominated by the victim LNA IMD, we have calculated the IMD increase for all nine HPUE cases by comparing step by step between the different cases. These calculations are summarized in Table 4, which is ordered in the same fashion as Table 3.

Table 4: DeltaMSD calculations summary for 3B DL/2B UL

| case /Rel | ULCA PC /#Tx | Bd | /Bd PC | /Bd #Tx | /Bd 2UL power | /Bd 2UL BO | 2DL2UL interference level inrease vs Ref / vs case(x) [dB] | ∆MSD max [dB] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | 2 | 3 | 4 | 5 |
| 1 | PC3 | Bd1 | PC3 | 1Tx | 20 | -3 | Ref | 0 | 0 | 0 | 0 |
| R15 | 2Tx | Bd2 | PC3 | 1Tx | 20 | -3 |  |  |  |  |  |
| 2 | PC2 | Bd1 | PC3 | 1Tx | 23 | 0 | +3xorder / =(1)+3xorder | 6 | 9 | 12 | 15 |
| R17 | 2Tx | Bd2 | PC3 | 1Tx | 23 | 0 |  |  |  |  |  |
| 3 | PC2 | Bd1 | PC3 | 1Tx | 23 | 0 | +3xorder / =(1)+3xorder | 6 | 9 | 12 | 15 |
| R17 | 2Tx | Bd2 | PC2 | 1Tx | 23 | -3 |  |  |  |  |  |
| 4 | PC2 | Bd1 | PC3 | 1Tx | 23 | 0 | +3x(order-1)=+3xorder-3 / =(1)+3 Bd1 | 3 | 6 | 9 | 12 |
| R18 | 3Tx | Bd2 | PC2 | 2Tx | 2x20 | -3 |  |  |  |  |  |
| 5 | PC2 | Bd1 | PC2 | 1TX | 23 | -3 | +3xorder / =(1)+3xorder | 6 | 9 | 12 | 15 |
| R19 | 2Tx | Bd2 | PC2 | 1Tx | 23 | -3 |  |  |  |  |  |
| 6 | PC2 | Bd1 | PC2 | 1TX | 23 | -3 | +3x(order-1)=+3xorder-3 / =(1)+3 Bd1 | 3 | 6 | 9 | 12 |
| R19 | 3Tx | Bd2 | PC2 | 2Tx | 2x20 | -3 |  |  |  |  |  |
| 7 | PC1.5 | Bd1 | PC2 | 1TX | 26 | 0 | +6xorder / =(2)+3xorder | 12 | 18 | 24 | 30 |
| R19 | 2Tx | Bd2 | PC2 | 1Tx | 26 | 0 |  |  |  |  |  |
| 8 | PC1.5 | Bd1 | PC2 | 1TX | 26 | 0 | +3x(order)+3*(order-1)=6xorder-3 / =(2)+3 Bd1 | 9 | 15 | 21 | 27 |
| R19 | 3Tx | Bd2 | PC2 | 2Tx | 2x23 | 0 |  |  |  |  |  |
| 9 | PC1.5 | Bd1 | PC3 | 1Tx | 23 | 0 | +3*order+1.7*(order-1)=+4.7xorder-1.7/ =(2)+1.7*(order-1) | 7.7 | 12.4 | 17.1 | 21.8 |
| R18 | 3Tx | Bd2 | PC1.5 | 2Tx | 2x24.7 | -1.3 |  | =>9 | =>12 | =>18 | =>21 |
| 10 | PC1.5 | Bd1 | PC2 | 1TX | 26 | 0 | +3x(order)+3*(order-1)=6xorder-3 / =(2)+3 Bd1 | 9 | 15 | 21 | 27 |
| R19 | 3Tx | Bd2 | PC1.5 | 2Tx | 2x23 | -3 |  |  |  |  |  |

Observation:

- The worst-case IMD increase versus PC3 is 30dB for IMD5 of PC1.5=PC2+PC2. This is because the LNA power capability does not increase.

- Where yellow highlights appear, the UL level on one band is not increased by a 3dB step resulting in IMD increases that are not a multiple of 3dB. Here, we suggest rounding to the closest 3dB increment.

- Overall, for any IMD increase, the maximum DeltaMSD is from 0 to 30dB in 3dB increments. The higher values are for higher IMD orders where the PC3 MSD is small, thus the maximum DeltaMSD is not reached.

- It should therefore be noted that higher order IMDs (e,g. 6,7..?) could be of concern for the inter-band highest power classes.

## 2.4 Summary of the number of curves/equations needed for all ten cases and four IMD orders.

Since the IMD increase equation and maximum DeltaMSD is applicable to multiple cases, we propose re-arranging the cases and assigning them to the related maximum DeltaMSD values in 3dB increments.

This is seen in Table 5 where:

- Col1: MSD to own DL band out of 2DL/2UL combination or third band in a 3DL / 2UL combination.

- Col2: Inter-band power class, Col3: Number of Tx, Col4: Per band power class (when needed), Col5: Power class of victim band.

- Col6: Corresponding cases in previous tables.

- Finally, to which maximum DeltaMSD curve each IMD order applies per cases.

Proposal on cases and DeltaMSD curves: The following table is used to map the cases and IMD order to a given DeltaMSD curve.

Table 5: DeltaMSD curve assignment per IMD order and 2DL/3DL 2UL cases.

| #DL band MSD | Inte-band PC | #Tx | per-band PC | Victim band PC | case | ∆MSDmax [dB] /IMD order |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 0 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 |
| 2DL | PC2 | 2Tx |  | PC3 | 2/3a |  |  | 2 | 3 | 4 | 5 |  |  |  |  |  |
|  |  |  |  | PC2 | 3b/5 |  | 2345 |  |  |  |  |  |  |  |  |  |
|  | PC1.5 |  |  | PC2 | 7 |  |  |  | 2 | 3 | 4 | 5 |  |  |  |  |
|  | PC2 | 3Tx | PC3/PC2+PC2 2Tx | PC3 1Tx/PC2 2Tx | 4/6b |  | 2 | 3 | 4 | 5 |  |  |  |  |  |  |
|  |  |  | PC2+PC2 2Tx | PC2 1Tx | 6a | 345 | 2 |  |  |  |  |  |  |  |  |  |
|  | PC1.5 |  | PC2+PC2/PC1.5 2Tx | PC2 1Tx | 8a/10 |  |  | 2 | 3 | 4 | 5 |  |  |  |  |  |
|  |  |  |  | PC2 2Tx | 8b |  |  |  | 2 |  | 3 |  | 4 |  | 5 |  |
|  |  |  | PC3+PC1.5 | N/A | 9 |  |  |  | 2 | 3 |  | 4 | 5 |  |  |  |
| 3DL | PC2 | 2Tx |  |  | 2/3/5 |  |  | 2 | 3 | 4 | 5 |  |  |  |  |  |
|  | PC1.5 |  |  |  | 7 |  |  |  |  | 2 |  | 3 |  | 4 |  | 5 |
|  | PC2 | 3Tx |  |  | 4/6 |  | 2 | 3 | 4 | 5 |  |  |  |  |  |  |
|  | PC1.5 |  |  | PC2 | 8/10 |  |  |  | 2 |  | 3 |  | 4 |  | 5 |  |
|  |  |  |  | PC3 | 9 |  |  |  | 2 | 3 |  | 4 | 5 |  |  |  |

Observations:

- Multiple cases fall on the same DeltaMSD curve, especially for 3DL and some are common between 2DL and 3DL.

- Although it looks like a high number of cases to be mapped, all the curves have a 3dB scaling to the others.

In the following section, we calculate the curves and show how they can be modelled and scaled with a single number.

## 2.4 DeltaMSD curve calculations, modelling and scaling

The DeltaMSD curves can be calculated by adding the IMD increase (Maximum Delta MSD value) to the PC3 interference value and recalculating the post MRC MSD assuming a 9dB imbalance between the main and diversity path. Table 6 shows such calculations for the curves corresponding to a maximum Delta MSD of 3/6/9/12dB. The same calculation exists for all 3dB steps up to 30dB.

Table 6: Delta MSD curves calculations for cases with maximum delta MSD of 3/6/9/12dB versus PC3 MSD (0.1-29dB)

The DeltaMSD values versus PC3 MSD for the same cases are plotted in a plain line in Figure 1 together with a model curve in the dashed line.

Figure 1: 3/6/9/12dB DeltaMSD curves vs PC3 MSD and their model (dashed)

The proposed modelling for the DeltaMSD curves fits well with the calculated MSD and has several advantages versus the linear approach previously proposed or a single value:

- In general, the curve fitting is better than linear approach. The worst-case error is 0.5/0.8/0.8/1.5dB for the 3/6/9/12dB curves respectively. The error increases for the higher DeltaMSD curves, but only for the very small values of PC3 MSD <0.3dB which are not used regardless.

- Compared with the linear approach where the MSD value would already be quite high for small PC3 MSDs, this model has 0dB DeltaMSD for 0.1dB PC3 MSD.

- The model is pessimistic in the 0.5-6dB PC3 region but only for the low DeltaMSD curves. This part can be changed by changing the assumed main to diversity interference imbalance, so it should not be a concern.

- The model directly scales with the maximum DeltaMSD of a curve.

The proposed Delta MSD model is as follows:

- Input parameter:

  - ΔMSDmax: Value from the last columns of Table 5 based on the case and IMD order

  - PC3MSD: PC3 MSD value for the corresponding IMD and case.

- Equation:

ΔMSD=Min(0.2*ΔMSDmax*(LN(PC3MSD)+2),ΔMSDmax)

- At this stage we do not propose further simplification as there is only one equation for all the cases that uniquely scales with the maximum ΔMSD. Some cases may be further simplified since, for some IMD orders, the PC3 MSD may exercise only the upper or lower part of the curve.

- Table 5 can be used to map the ΔMSD curve to the IMD order, power class type, and 2DL or 3DL MSD.

Observation: We recognize that some of the analysis provided in this contribution would require more space for detailed explanations. We hope to provide more explanations online, offline, or in adhocs in the coming meeting.

# Conclusion

In this contribution, we continued to analyze the delta MSD for the 2UL IMDs for HPUE inter-band combinations. We had to base our work on the following assumptions.

Proposal: The following table is proposed as assumptions for IMD increase calculations

Table 2: Proposed assumptions on dominating IMD generator versus 2/3 band DL, TDD/FDD and 2/3Tx

| #DLB | 2UL | Assumptions on dominating IMD | 3Tx assumption |
| --- | --- | --- | --- |
| 2 | F+T | Only FDD band is victim => dominated by FDD PA | If 2Tx on TDD => Only one TDD PA dominates coupling to FDD PA |
|  |  |  | If 2Tx on FDD => Only one FDD PA dominates IMD |
|  | F+F | Dominated by FDD PA associated with victim DL band | if 2Tx on victim FDD => Only one PA dominates IMD |
|  |  |  | if 1Tx on victim FDD => Only one of 2Tx PA dominates coupling to FDD PA |
| 3 | F/T+F/T | Dominated by victim band LNA | Only one LNA IMD is dominated by both UL |

Proposal on IMD asymmetry and indexes: Since there is no way to adopt a generic approach in terms of UL band level asymmetry, we propose to ignore asymmetric levels and IMD indexes in the IMD increase calculations. However, when the level in each UL band is asymmetric, the worst IMD side is considered.

Proposal on architecture assumption: The main path of each UL band shares the same antenna and IMD interference dominates in the main DL path and is generated by the main PA associated with the victim band.

Based on those assumptions, the different DeltaMSD curves were calculated, mapped and modelled in the following proposals.

Proposal on cases and DeltaMSD curves: The following table is used to map the cases and IMD order to a given DeltaMSD curve.

Table 5: DeltaMSD curve assignment per IMD order and 2DL/3DL 2UL cases.

| #DL band MSD | Inte-band PC | #Tx | per-band PC | Victim band PC | case | ∆MSDmax [dB] /IMD order |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 0 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 |
| 2DL | PC2 | 2Tx |  | PC3 | 2/3a |  |  | 2 | 3 | 4 | 5 |  |  |  |  |  |
|  |  |  |  | PC2 | 3b/5 |  | 2345 |  |  |  |  |  |  |  |  |  |
|  | PC1.5 |  |  | PC2 | 7 |  |  |  | 2 | 3 | 4 | 5 |  |  |  |  |
|  | PC2 | 3Tx | PC3/PC2+PC2 2Tx | PC3 1Tx/PC2 2Tx | 4/6b |  | 2 | 3 | 4 | 5 |  |  |  |  |  |  |
|  |  |  | PC2+PC2 2Tx | PC2 1Tx | 6a | 345 | 2 |  |  |  |  |  |  |  |  |  |
|  | PC1.5 |  | PC2+PC2/PC1.5 2Tx | PC2 1Tx | 8a/10 |  |  | 2 | 3 | 4 | 5 |  |  |  |  |  |
|  |  |  |  | PC2 2Tx | 8b |  |  |  | 2 |  | 3 |  | 4 |  | 5 |  |
|  |  |  | PC3+PC1.5 | N/A | 9 |  |  |  | 2 | 3 |  | 4 | 5 |  |  |  |
| 3DL | PC2 | 2Tx |  |  | 2/3/5 |  |  | 2 | 3 | 4 | 5 |  |  |  |  |  |
|  | PC1.5 |  |  |  | 7 |  |  |  |  | 2 |  | 3 |  | 4 |  | 5 |
|  | PC2 | 3Tx |  |  | 4/6 |  | 2 | 3 | 4 | 5 |  |  |  |  |  |  |
|  | PC1.5 |  |  | PC2 | 8/10 |  |  |  | 2 |  | 3 |  | 4 |  | 5 |  |
|  |  |  |  | PC3 | 9 |  |  |  | 2 | 3 |  | 4 | 5 |  |  |  |

The proposed Delta MSD model is as follows:

- Input parameter:

  - ΔMSDmax: Value from the last columns of Table 5 based on the case and IMD order

  - PC3MSD: PC3 MSD value for the corresponding IMD and case.

- Equation:

ΔMSD=Min(0.2*ΔMSDmax*(LN(PC3MSD)+2),ΔMSDmax)

- At this stage we do not propose further simplification as there is only one equation for all the cases that uniquely scales with the maximum ΔMSD. Some cases may be further simplified since, for some IMD orders, the PC3 MSD may exercise only the upper or lower part of the curve.

- Table 5 can be used to map the ΔMSD curve to the IMD order, power class type, and 2DL or 3DL MSD.

Observation: We recognize that some of the analysis provided in this contribution would require more space for detailed explanations. We hope to provide more explanations online, offline, or in adhocs in the coming meeting.

# References

[1] R4-2417793 Observations on HPUE 2UL IMD MSDs, Skyworks Solutions Inc., RAN4#113

[2] R4-2417794 Simplification of HPUE 2UL IMD MSDs, Skyworks Solutions Inc., RAN4#113

[3] R4-2420380 WF on MSD rule, Skyworks, Murata, Nokia, Qualcomm, RAN4#113