3GPP TSG-RAN WG4 Meeting #114                                R4-2500228

# Athens, GR, 17th – 21st February, 2025

Agenda Item:	7.19.2

Source:	Apple

Title:	Discussion on CSI Prediction: CSI reporting requirement framework for CSI prediction

Release:       Rel-19

Document for:	Discussion

# Introduction

Rel-19 Work Item (WI) was approved on the Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface (WID in [1]). The application of AI/ML techniques to NR air interface has been studied in FS_NR_AIML_Air.

This work item provides normative support for the general framework of AI/ML concerning air interfaces. It also enables the implementation of recommended use cases outlined in the previous study. Furthermore, several study objectives within this project aim to address outstanding issues identified during the study, with the goal of enhancing understanding in preparation for future normative effort.

The current agreements on how to perform the RAN4 study on general issues for AI/ML, and issues related to interoperability/testing have been captured in the latest TR [2]

In this contribution, we provide our viewpoints on some of the interoperability and testability aspects for AI/ML for NR air interface for CSI compression and prediction.

# Metric for RAN4 Requirements

In the previous meeting RAN4#112, the following agreements was reached for CSI prediction.

Agreement:

Use relative throughput as requirement metric for CSI prediction

- companies are invited to bring contribution on how the requirement could be defined:

  - what is the baseline that throughput would be compared to

using relative throughput as the test metric for inference, the baseline denominator for relative throughput still requires further clarification.

In the current PMI test case, RAN4 uses random PMI as the baseline to evaluate whether the UE-reported PMI is accurate enough. This approach is analogous to a scenario where no UE reporting occurs.

Issue 2-1: Channel model for CSI prediction tests

Agreement:

- For channel model for CSI prediction tests, TDL channel models will be used.

  - FFS on whether CDL channel model is needed for generalization test.

Issue 2-2: Reference throughput

Agreement:

- reference throughput is obtained based on random PMI with Rel-15 Type I single panel codebook

Issue 2-3: Testing conditions

Agreement:

- use static conditions as baseline.

  - FFS on whether to have non-static condition

Issue 2-5: Prediction delay requirements

Agreement:

- Prediction delay includes:

  - Measurement for prediction

    - FFS on whether the legacy measurement delay can be reused

  - Inference delay

  - Other components are FFS

According to the agreement, relative throughput should be tested for prediction accuracy. The γ value comparing predicted PMI versus random PMI can be used in the test. To demonstrate the performance gain of AI-based CSI prediction, the γ value from the AI-based prediction would differ from the γ value observed in previous non-AI tests.

Proposal 1: The γ value comparing predicted PMI to random PMI can be used in the test for CSI prediction. To infer the gains of AI/ML-based CSI prediction, the γ values can be compared between AI/ML and non-AI/ML cases.

During RAN4#112bis it was discussed whether the new requirements on AI/ML-based functionality should be worse or better than the requirements on the legacy CSI feedback:

Issue 1-4: AI/ML Performance Agreement: - For CSI prediction, AI/ML based performance requirement should not be worse than the legacy (non-AI/ML based) if a relevant equivalent legacy requirement/feature exists. - FFS which legacy requirement/feature are relevant - FFS whether AI/ML based performance should be better than the legacy - FFS on target for AI/ML based beam management

Given that the requirements for legacy non-AI/ML CSI feedback mechanisms are well-established, it would be advantageous and practical to evaluate the performance of AI/ML-based CSI prediction using the same test parameters that have already been defined.

- Proposal 2: RAN4 should assess the performance of AI/ML-based CSI prediction using the same test configurations and conditions as those defined for non-predicting eTypeII and/or eTypeII Doppler-r18.

For CSI monitoring, since SGCS is the direct KPI of the AI/ML model, SGCS may be considered as a monitoring KPI metric. The relevant KPIs and evaluation metrics for CSI prediction from RAN1, as outlined in TR 38.843 section 7.1.2, are noted below.

7.1.2 CSI feedback enhancement Performance monitoring: For CSI prediction using UE side model use case, at least the following aspects have been proposed by companies on performance monitoring for functionality-based LCM: … - Type 2: - UE reports predicted CSI and/or the corresponding ground-truth - NW calculates the performance metrics. - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting).

As outlined above, for NW-side performance monitoring (Type 2) in functionality-based LCM, predicted CSI along with its corresponding ground truth is sent to the network. The same mechanism can be applied for testing, where the UE provides both the current channel information and the predicted channel information. Subsequently, the TE can calculate the SGCS by comparing the current channel from the latest report with the predicted channel from the previous report.

Observation 1: In the context of CSI prediction performance monitoring, RAN1 is already exploring a network-side mechanism that relies on UE-reported ground truth. This same approach can be utilized to evaluate the accuracy of CSI prediction as a performance KPI within RAN4

Proposal 3: Same mechanism used for NW-side performance monitoring can be employed for testing where the UE provides both the current channel information and the predicted channel information. Subsequently, the TE can calculate the SGCS by comparing the current channel from the latest report with the predicted channel from the previous report.

- On Rel-19 AIML-based CSI reporting

Since time domain prediction reporting mechanism is already introduced in Rel-18, RAN1 at 116bis meeting agreed to assume R18 eType II doppler codebook for AI/ML-based prediction:

Agreement For Rel-19 study on CSI prediction, companies are encouraged to evaluate throughput performance by comparing performance with non-AI/ML based CSI prediction. - R18 eType II doppler codebook is assumed for CSI report for both AI/ML and Non AI/ML prediction. - Companies to report the assumption for N4, which could be 1, 2, 4, 8. Note: Non-AI/ML based CSI prediction (Benchmark 2) can include statistical model based CSI prediction (e.g., based on Kalman filter, Wiener filter, Auto-regression). Conclusion For Rel-19 study on CSI prediction only, consider UE-sided model only. Agreement - For the AI/ML based CSI prediction, for CSI report, adopt following as a baseline for evaluation purpose - N4 value: 1, 4 - Others can be additionally submitted, e.g., 2, 8 - paramCombination-Doppler-r18: 6,7 or paramCombination -r16 = 5,6 (for Benchmark 1) - Others can be additionally submitted. - Note: The same selected parameter combination shall be applied for benchmarks. - CSI report periodicity: 5ms, 20ms (encouraged) - Others can be additionally submitted, e.g., 10ms

- Proposal 4: RAN4 to assume R18 eType II Doppler codebook for reporting AI/ML based CSI predictions.

# Conclusion

In conclusion the following observations and proposals were discussed in this contribution:

Observation 1: In the context of CSI prediction performance monitoring, RAN1 is already exploring a network-side mechanism that relies on UE-reported ground truth. This same approach can be utilized to evaluate the accuracy of CSI prediction as a performance KPI within RAN4

Proposal 1: The γ value comparing predicted PMI to random PMI can be used in the test for CSI prediction. To infer the gains of AI/ML-based CSI prediction, the γ values can be compared between AI/ML and non-AI/ML cases.

- Proposal 2: RAN4 should assess the performance of AI/ML-based CSI prediction using the same test configurations and conditions as those defined for non-predicting eTypeII and/or eTypeII Doppler-r18.

Proposal 3: Same mechanism used for NW-side performance monitoring can be employed for testing where the UE provides both the current channel information and the predicted channel information. Subsequently, the TE can calculate the SGCS by comparing the current channel from the latest report with the predicted channel from the previous report.

- Proposal 4: RAN4 to assume R18 eType II Doppler codebook for reporting AI/ML based CSI predictions.

# Reference

[1] RP-242399 – “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”

[2] 3GPP TR 38.843, “Technical Specification Group Radio Access Network;Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”