3GPP TSG-RAN WG4 Meeting #114                                                         R4-2500233

Athens, GR, 17th – 21st February, 2025

Title:	Study of impacts on RAN4 requirements for AIML Mobility in NR

Source:	Apple

Agenda item:	7.21.2

Document for:	Discussion

Introduction

RAN4 discussion on the AI (Artificial Intelligence)/ML (Machine Learning) for mobility in NR (FS_NR_AIML_Mob study item) starts from RAN4#112bis. The recent RAN#105 plenary has agreed the updated version of the SID with the following RAN4 objectives [1]:

- Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] - Study the impacts on requirements based on RAN2 assumptions, and coordinate with RAN2 if needed - Study the testability and interoperability based on RAN2 framework (e.g., number of cells to measure, beams etc.) - NOTE 4: Leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability. - NOTE 5: Avoid the overlaps with RAN2 work for evaluation

During this meeting, it is expected the following aspects to be discussed.

- Work Plan for RAN4 Discussions on AI/ML for Mobility in NR

- AI/ML Mobility Scenarios Evaluation and RRM Requirements

- Testability and Interoperability Issues in AI/ML Mobility

In this contribution, we provide our views and discuss different aspects of AI/ML Mobility Scenarios, Evaluation and RRM Requirements based on the RAN2 framework.

Discussion

During RAN4#112bis meeting the following agreements were captured:

Issue 1-1-2: Scenario/sub-use-case priorities

Agreement: RAN4 to start the discussion for scenario 2, 3, 4. Further clarification can be discussed for the scenarios.

| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| --- | --- | --- | --- | --- |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |

Agreement:

  - Scenario 2, 3 and 4, the 3 cases can be discussed:

    - Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated

    - Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s)

    - Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

    - Follow the latest definition of the sub use-cases in RAN2. Further clarification on the sub use-cases from RAN4 perspective can be discussed.

Issue 2-1-1: Performance Metrics/KPIs for Measurement Prediction use case

Agreement:

- RAN4 to first study the impacts on requirements for the following case:

  - L3 cell level RSRP (Point C) prediction

  - FFS L1 beam-level (Point A1)

  - FFS L3 beam-level (Point E)

  - Other cases can be considered based on RAN4 further discussion and RAN2 further progress.

Figure 1: 3GPP TS 38.300 Measurement Model

Based on the current RAN2 agreements, following scenarios and use cases have been initially considered for AI/ML in mobility

- Measurement Prediction,

- Handover and Radio Link Failure Prediction, and

- Measurement Event Prediction.

During last meeting RAN4#112bis the following agreement was reached:

Issue 1-1-1: Priorities of different use cases

Agreement:

  - RAN4 to start the discussion with RRM measurement prediction use-case from RAN4 #112bis meeting.

Agreement:

  - RAN4 to only start the discussion of Measurement event prediction use-case and RLF use-case if sufficient progress in RAN2.

  - The earliest time to start the discussion for Measurement event prediction use-case is RAN4#114 meeting (Feb 2025).

  - The earliest time to start the discussion for RLF use-case is from RAN4#114bis meeting (Apr 2025).

RRM Measurement Prediction

In RAN2#125bis the following sub-use cases were agreed for RRM measurement prediction:

- Sub-case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then cell-level measurement result is generated

- Sub-case 2: Cell-level measurement result(s) is predicted based on actual cell-level measurement result(s)

- Sub-case 3: Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

In the draft of TR 38.744, the two main identified study goals are:

- First Goal; Measurement reduction. Achieve the same mobility performance, but with reduced DL radio measurements that the UE needs to take and report, which improves UE energy efficiency, reduces the required signalling, and network overhead of transmitting reference signals (RSs).

- Second Goal; Mobility optimization. Predict future UE radio measurements and use them to optimize HO performance and mobility decisions.

Three domains (or a combination them) have been identified for mobility predictions:

- Spatial: when UE is not measuring every cell and/or beam

- Frequency domain: The UE is not measuring every frequency carrier/bands

- Temporal: when UE may skip and predict some of the measurement time steps

The primary focus of the spatial domain prediction is deciding about a beam/cell based on the measurement of another cell/beam. Based on the current RAN2 framework, spatial domain prediction is used for FR2 intra-cell intra-frequency scenario targeting to reduce the measurement effort.

In the case of frequency domain prediction, the UE measures a specific frequency carrier or band and predicts the measurements for another carrier or band. This approach helps minimize the number of measurements and reduces the required measurement gap, thereby decreasing communication interruptions.

For temporal domain measurement prediction,

In RAN2#125bis meeting, it has been agreed as:

- For temporal domain measurement prediction, we will consider the AI-PHY beam management Case A and Case B from the RAN1 AI/ML PHY TR and it applies to both beam level and cell level. As baseline we will focus on pure temporal predication.

Intra-frequency temporal domain case A: continuous measurement results in prediction window are predicted by continuous historical measurement result(s) in observation window. Then observation window and prediction window slide forward with either one or more sampling periods (with sliding L1/L3 filtering option) or one or more measurement periods (with non-sliding L1/L3 filtering option), where measurement result(s) are measured before sliding. In RRM temporal prediction case A, there is no measurement reduction within observation window (OW). When sliding forward, to produce new predictions, the UE measures at least part of the samples of the previous predicted window (PW). There is no measurement samples reduction in Case A and it is used for achieving the second goal (i.e., mobility optimization such as handover prediction).

Figure 2: Illustration of temporal domain prediction – case A

Intra-frequency temporal domain case B: measurement results in prediction window are predicted by historical measurement result(s) in observation window. Then observation window and prediction window slide forward with either sampling period(s) (with sliding L1/L3 filtering option) or measurement period(s) (with non-sliding L1/L3 filtering option) and measurement result(s) in previous prediction window is/are skipped during window sliding. Case B design target is for the first goal of measured reduction, where the measurement of the previously predicted samples in previous PW is skipped (either by sliding window or applying sample and hold method) for the new predicted window, enabling measurement reduction (e.g., 50% Figure 3).

Figure 3: Illustration of temporal domain prediction – case B

It is expected that the performance of different approaches (spatial, temporal case A, temporal case B) to be dependent on several factors such as input samples, reductions patterns, predicted window length, etc. Some of the factors need standardization and some need RAN2 signaling for coordination between UE and the network.

Regarding scenario prioritization, the following prioritization was agreed upon during RAN2#126, as outlined in Table 1:

Table 1: Prioritization of evaluation scenarios (Table 5.2.1-1 in TR 38.744 draft)

| scenario number | Priority | Evaluation scenario | Target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |

AI/ML Measurement Prediction with Measurement Reduction (UE-Side, first goal)

Measurement reduction in Scenario #2:

(i.e. FR1 to FR1 intra-frequency temporal domain case B), the goal is to reduce measurement overhead by reducing the number of measured samples. To avoid additional complexity, i.e., in the design of new reporting mechanisms, in the 1st goal of measurement reduction, the network is expected to use the predicted measurements reported by a UE, or measurements derived from predicted and measured values, as it uses the current measurements reports. The reported predicted measurements or measurements derived from predictions may include:

- Beam-level layer-1 filtered measurements (A1)

- Cell-level layer-3 filtered measurements (C)

- Beam-level layer-3 filtered measurements (E)

For predicted measurement, one of the key issues which needs to be investigated is predicted measurement accuracy. Based on the current RAN2 progress on AI/ML for Mobility, for measurement prediction accuracy Mean Absolute Error (MAE) which is the average difference between the predicted values and the actual values (e.g., predicted L3 filtered cell-level measurement result and actual L3 filtered cell-level measurement) has been agreed to be used as the performance metric.

However, in RAN4 AI/ML for Air Interface, single or a combination of options listed in Table 3 are under investigation for accuracy. [3]

Table 3: RAN4 metric options for beam management in AI/ML Air Interface WI

- Option 1: RSRP accuracy - Option 2: Beam prediction accuracy - Top-1 (%) : the percentage of "the Top-1 strongest beam is Top-1 predicted beam" - Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams" - Top-1/K (%) : the percentage of "the Top-1 predicted beam is one of the Top-K strongest beams" - Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB, - Related measurement accuracy can be considered to determine x - Option 4: combinations of above options

For scenario 2 we have the following sub-use cases:

Agreement:

  - Scenario 2, 3 and 4, the 3 cases can be discussed:

    - Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated

    - Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s)

    - Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

    - Follow the latest definition of the sub use-cases in RAN2. Further clarification on the sub use-cases from RAN4 perspective can be discussed.

Sub-use case 1: To predict beam level results, then generate cell level results based on the predicted beam results

Sub-case 1 can be conceptually modeled as a concatenation of an AI/ML model for beam management and the consolidation of beam prediction results. Optionally a single AI/ML model can generate cell-level predictions. However, the sub-case may be transparent to the NW. Figure 4 shows in more detail the AI/ML role/functionality inside the measurement model.

Figure 4: Scenario #2 and Sub-use case 1

The beam level Layer 1 is similar to Rel-18 AI/ML L1-RSRP for beam temporal prediction with exception that the subsequent filtering processing is needed, while AI model is not impacted. Therefore,

- The UE side AI/ML model may make use of a Rel-18 AI/ML model for beam measurement predictions to obtain the predicted L1-RSRP of the same cell (serving cell-serving cell and neighbouring cell-neighbouring cell)

- The framework/simulation platform for AI-PHY beam management can be utilized as much as possible

However, in TR 38.843 it is mentioned that all the following cases are considered for BM-Case2:

- Set B is a subset of Set A

- Set B is not a subset of Set A

- Set B is the same as Set A

It is our understanding that the AI/ML model in Fig 4 performs only temporal prediction and not spatial. That is, scenario #2 for beam level prediction is identical to AI/ML BM-Case when set A= set B.

Observation 1: Scenario #2 for AI/ML in Release-19 Mobility is similar to Release-18 AI/ML beam prediction (Case-2), except that BM Case-2 allows set B to differ from set A. They are identical when set B = set A

Regarding the report of the predictions and the prediction requirements the following agreement was reached:

Agreement:

- RAN4 to first study the impacts on requirements for the following case:

  - L3 cell level RSRP (Point C) prediction

  - FFS L1 beam-level (Point A1)

  - FFS L3 beam-level (Point E)

  - Other cases can be considered based on RAN4 further discussion and RAN2 further progress.

Beam management/measurement is a L1 procedure and only for serving cell, so network configures L1 measurement and report to UE and therefore UE reports L1 measurement result (beam level) as in Rel-18 AI/ML L1-RSRP beam prediction.  In legacy case, L3 is for mobility and RRM purpose, and network configures L3 measurement/report to UE, and UE reports L3 beam or cell result. If LTM is considered, then neighbour cell measurement can be L1 and then we don’t need to follow this diagram in TS38.300. Legacy L3 framework doesn’t support L1 report (beam level point A1) during the L3 procedure. If network configures to let UE report all the beams in point E, i.e., Point F and point E report are both technically feasible.

In TS 38.133, RAN4 has defined measurement accuracy as a fixed limit (e.g. +/-X dB). As an example, absolute SS-RSRP, and relative SS-RSRP accuracy requirements for intra-frequency FR1 band in some scenarios are expressed as a fixed limit (e.g. +/-X dB), during the tests assuming a Gaussian distribution of the error, the limits will have to be set at +/-3.29 if the probability of failing a "good device under test" in a single test is to be kept at 0.1% (i.e., 99.9% of the measurements based on the assumed Gaussian distribution of errors) will fall within the specified range.

Requirements for the accuracy of predicted measurements may follow the requirements for baseline measurements, but RAN4 should study if there are potentially new or updated requirements when AI/ML-based predicted measurements are reported for measurement reduction purpose. [3]

RAN4 will need to study accuracy requirements for AI/ML predicted cell-level L3-filtered measurements for measurement reduction purpose. FFS on predicted beam level L3-filtered measurements (point E)

Proposal 2: For beam-level L1 requirements and given that L1 reporting is not supported within the L3 mobility framework, bypass L3 filtering and utilize a single Tx beam for testing purposes. (point A1 equivalent to C)

Additionally, as discussed, RAN4 may have different requirement KPIs (metrics) than RAN2, e.g., because of the availability of the ground truth/reference values and their accuracy in the tests. Evaluation of new RAN4 requirements can be associated with a need to perform new simulations, such as errors/accuracy of the inputs/measurements, latencies of the procedures, new metrics, etc. RAN2 has progressed in defining simulation assumptions and setup for the RRM measurement prediction and RLF prediction. To ensure that these simulations accurately reflect RAN4 RRM requirements and KPIs, it is important to cross-check them with the RAN4 requirements.

Proposal 3: Investigate the translation of RAN2 KPI requirements into RAN4 RRM measurement accuracy requirements to assess the feasibility of the RAN2 study. This translation is essential for aligning performance metrics and ensuring that mobility optimization goals are met effectively. Additionally, explore the impact of RRM measurement requirements on prediction accuracy across various RRM measurement prediction cases and evaluation scenarios given side conditions of OW and PW, SNR.

Regarding Sub-use case 2: To directly predict cell level results based on cell level results, the AI/ML model role is shown in Fig 5.

Figure 5:  Scenario #2 and Sub-use case 2

RAN2 agreements define measurement prediction accuracy for cell-level RRM measurement prediction as the average L3 RSRP difference between the predicted and actual L3-filtered cell-level measurement results for the same cell. For FR2, the RX beamforming gain at the UE is implementation-dependent and thus unknown, therefore more clarification is needed.  For AI/ML-based beam management, one candidate test metric is RSRP prediction accuracy which was concluded to be defined as= predicted L1-RSRP of beam index i – ground truth of L1-RSRP of beam index i. The index i may be any beam index in top-K beams based on ground truth L1-RSRP.

As for the ideal measured L1-RSRP for FR2, it was agreed that the ground truth for the predicted RSRP is the ideal measurement of RSRP on the predicted Tx beam. To solve the uncertainty due to RX beamforming gain, it is agreed in RAN4 that the ground truth is the approximate as the reported RSRP measurement under the certain SNR on the predicted Tx beam.

Considering the above, cell-level RSRP prediction accuracy can be defined as the difference between the predicted RSRP and the ideal measured RSRP. For FR1, the ideal measured RSRP is the transmitted RSRP (assuming conducted test and AWGN channel during the test). For FR2, the ideal RSRP approximates the reported RSRP measurement when the SNR is sufficiently high to ensure accurate reporting

During RAN4#113 the following agreement was reached:

Issue 2-1-9a: L3 RSRP prediction accuracy definition

Agreement:

- For testing, Absolute L3 Predicted RSRP Accuracy = reported predicted L3-RSRP – ground truth of L3-RSRP

  - FFS Ground truth definition of L3-RSRP for FR1 and FR2

Here is our proposal:

Proposal 4: The proposed cell-level RSRP prediction accuracy is defined as follows:

- Absolute RSRP Accuracy for Cell-Level = predicted L3-RSRP of cell i - ideal received L3-RSRP of cell i

  - In FR2: The ideal received L3-RSRP is assumed to approximate the reported RSRP measurement, provided the SNR is sufficiently high for reliable RSRP accuracy.

  - In FR1: The ideal received L3-RSRP is taken to be equal to the transmitted power (assuming conducted test and AWGN channel during the test)

The role of AI/ML model in the measurement model is shown in Fig 6 for sub-use case 3.

Figure 6: Scenario #2 and Sub-use case 3

Measurement reduction in Scenario #3:

Scenario#3 (i.e., FR1 to FR1 inter-frequency in frequency domain) focuses on an additional use case in the inter-cell mobility measurement reduction which is inter-frequency measurement prediction. It can be used to reduce the required inter-frequency measurements a UE needs to take, which in turn can reduce the number of measurement gaps that the UE needs to be configured with. Reducing measurements gaps can improve the data throughput and reduce the latency. The required accuracy of the inter-frequency measurement prediction needs to be studied based on how the predictions are used. Additionally, RAN4 to study any impacts due to coordinating the measurement gaps between the network and the UE.

In Scenario 3, which involves inter-frequency inter-cell prediction from FR1 to FR1 (frequency domain), RAN2 concentrates on predicting a single cell within one inter-frequency layer. The advantage of this scenario lies in the ability to utilize a much longer Measurement Gap Repetition Periodicity (MGRP) or to skip certain gaps for inter-frequency measurements. However, if in RAN4 the UE only predicts measurement results for a single cell, it will not be possible to reduce the MGRP. This is because the predictability of one cell does not imply that all other cells within the same frequency are also predictable or can be skipped. Additionally, the Measurement Gap (MG) is configured to monitor multiple inter-frequency layers, not just one [6] So the output will be cell-level RSRP of multiple cells.

The reduction of MGRP is unlikely to be explicitly reflected in the specifications, as it may depend on network implementation. Decisions regarding the extension of the MGRP or the possibility of skipping certain gap occasions should consider all frequency layers.

Encourage companies to bring proposals on the definition of relative RSRP and absolute RSRP.

Subsequently, companies may also consider following candidate options to introduce RSRP accuracy requirements:

- Option 1: RAN4 to consider relative RSRP accuracy requirements only.

- Option 2: RAN4 to consider absolute RSRP accuracy requirements only.

- Option 3: RAN4 to consider both absolute and relative RSRP accuracy requirements.

Proposal 5: To support the increase of MGRP, RAN4 should investigate the feasibility of predicting measurement results for multiple cells per frequency layer. Additionally, it should examine the impact of adjusting the measurement gap period or selectively skipping gap occasions to reduce measurement gaps.

Proposal 6: In Scenario 3 of RRM measurement prediction, further explore the decision to establish absolute and/or relative accuracy requirements for the predicted/generated L3 cell-level RSRP. Additionally, discuss methods for conducting relative measurements, such as those referencing to an anchor cell or calculating pairwise differences between cells.

AI/ML Measurement Prediction Without Measurement Reduction (mobility optimization, second goal)

Another aspect of measurement prediction is the enhancement of mobility and handover (HO) performance when the time and spatial density of measurements remain unchanged, meaning there is no reduction in measurements. In this use case, future measurements predicted using AI/ML are leveraged to optimize mobility decisions.

The measurements may be predicted by the UE and either reported to the network or utilized within the UE to derive further predictions, such as forecasting handovers, radio link failures, or measurement events. The specific use cases and decisions regarding how the predicted measurements are employed to optimize mobility performance, along with their signaling impact, are still under further study in RAN2

Ultimately, the interplay between achievable prediction accuracy and mobility performance is crucial. The evaluation of the benefits provided by AI/ML-assisted mobility should consider KPIs related to handovers (e.g., ping-pong handovers, handover failures/radio link failures, time of stay, handover interruptions, prediction accuracy, and measurement reduction). These factors will shape the formulation of RAN4 requirements, as the primary objective is to enhance the robustness of mobility procedures and minimize failures

Proposal 7: Investigate the translation of RAN2 KPI requirements into RAN4 RRM measurement accuracy and prediction measurement accuracy requirements to assess the feasibility of the RAN2 study. The evaluation of the benefits provided by AI/ML-assisted mobility should consider (KPIs) related to handovers (e.g., ping-pong handovers, handover failures/radio link failures, time of stay, handover interruptions, prediction accuracy, and measurement reduction). These factors will determine the formulation of RAN4 requirements, as the primary objective is to enhance the robustness of mobility procedures and minimize failures.

Impact of measurement error on AI/ML Prediction Accuracy for Mobility

- In the context of mobility scenarios, it is inevitable that measurement inaccuracies will occur, affecting the data used as input or ground truth for AI/ML systems. Such discrepancies can have a substantial negative effect on network performance, leading to incorrect predictions that compromise mobility management, hinder effective handover processes. Performance of AI/ML-based measurement prediction is highly dependent on the accuracy of the input data.

- Based on RAN4#122bis an agreement was reached to evaluate the effect of measurement errors (simulation methodology on R4-2417211) to the AI/ML spatial prediction BM case 1. RAN4 to leverage the simulation methodology and evaluation results for the impact of measurement errors in the predicted measurements for mobility.

Issue 2-1-3: Impacts of measurement error on prediction accuracy

- Candidate options:

  - Option 1: RAN4 should study the impact of measurement error on the prediction accuracy during SI phase

  - Option 2: RAN4 to not study the impact of measurement error on the prediction accuracy during SI phase

  - Option 3: RAN4 should wait for evaluation outcomes of RAN2 AI/ML for mobility (option3) and RAN4 AI/ML for beam management (option 4)

  - Option 4: RAN4 to provide input to RAN2 to consider impact of error on measurement used for prediction on the accuracy of the predicted measurement/event

- Proposal 8: RAN4 to investigate the impact of measurement errors in the input data of AI/ML model on the accuracy of predicted measurements. Additionally, RAN4 should leverage the simulation methodologies and evaluation results (from AI/ML BM Case 1) to assess how these measurement errors affect the accuracy of predictions related to mobility.

Life Cycle Management (LCM) Related Requirements

With the integration of AI/ML models into essential network functions—such as managing mobility, facilitating handovers, and optimizing radio resource allocation—it is important to ensure these models operate efficiently in real-time, adapting to the diverse and dynamic conditions of wireless channels. This consideration is crucial for both UE and network-side implementations. To ensure the sustained effectiveness of these AI/ML models, it is important to establish and monitor KPIs like prediction accuracy and latency, which are indicative of their real-time operational performance. [3]

When an AI/ML model's performance drops below specified thresholds, it's critical for either the network or the UE to implement corrective actions. This could involve updating the model, tweaking its configurations, or modifying the input data. If performance significantly deteriorates, it may be necessary to disable the AI/ML model entirely and revert to traditional methods, such as legacy measurement and reporting systems, to ensure consistent and reliable network operation. This proactive approach helps to maintain network integrity, ensuring that the use of AI/ML does not undermine performance during instances of model failure or reduced efficacy

- Proposal 9:  RAN4 should leverage the LCM progress and requirements discussed in R-19 WI related to AI/ML Air Interface and investigate enhancements to these LCM procedures to RRM for mobility.

Conclusion

This paper provides an overview of the Rel-19 AI/ML-based mobility scenarios and RRM requirements under study within the RAN4 working group. The following Observations and Proposals were made:

Observation 1: Scenario #2 for AI/ML in Release-19 Mobility is similar to Release-18 AI/ML beam prediction (Case-2), except that BM Case-2 allows set B to differ from set A. They are identical when set B = set A

- Proposal 1: RAN4 will need to study accuracy requirements for AI/ML predicted cell-level L3-filtered measurements for measurement reduction purpose. FFS on predicted beam level L3-filtered measurements (point E)

Proposal 2: For beam-level L1 requirements and given that L1 reporting is not supported within the L3 mobility framework, bypass L3 filtering and utilize a single Tx beam for testing purposes. (point A1 equivalent to C)

Proposal 3: Investigate the translation of RAN2 KPI requirements into RAN4 RRM measurement accuracy requirements to assess the feasibility of the RAN2 study. This translation is essential for aligning performance metrics and ensuring that mobility optimization goals are met effectively. Additionally, explore the impact of RRM measurement requirements on prediction accuracy across various RRM measurement prediction cases and evaluation scenarios given side conditions of OW and PW, SNR.

Proposal 4: The proposed cell-level RSRP prediction accuracy is defined as follows:

- Absolute RSRP Accuracy for Cell-Level = predicted L3-RSRP of cell i - ideal received L3-RSRP of cell i

  - In FR2: The ideal received L3-RSRP is assumed to approximate the reported RSRP measurement, provided the SNR is sufficiently high for reliable RSRP accuracy.

  - In FR1: The ideal received L3-RSRP is taken to be equal to the transmitted power (assuming conducted test and AWGN channel during the test)

Proposal 5: To support the increase of MGRP, RAN4 should investigate the feasibility of predicting measurement results for multiple cells per frequency layer. Additionally, it should examine the impact of adjusting the measurement gap period or selectively skipping gap occasions to reduce measurement gaps.

Proposal 6: In Scenario 3 of RRM measurement prediction, further explore the decision to establish absolute and/or relative accuracy requirements for the predicted/generated L3 cell-level RSRP. Additionally, discuss methods for conducting relative measurements, such as those referencing to an anchor cell or calculating pairwise differences between cells.

Proposal 7: Investigate the translation of RAN2 KPI requirements into RAN4 RRM measurement accuracy and prediction measurement accuracy requirements to assess the feasibility of the RAN2 study. The evaluation of the benefits provided by AI/ML-assisted mobility should consider (KPIs) related to handovers (e.g., ping-pong handovers, handover failures/radio link failures, time of stay, handover interruptions, prediction accuracy, and measurement reduction). These factors will determine the formulation of RAN4 requirements, as the primary objective is to enhance the robustness of mobility procedures and minimize failures.

- Proposal 8: RAN4 to investigate the impact of measurement errors in the input data of AI/ML model on the accuracy of predicted measurements. Additionally, RAN4 should leverage the simulation methodologies and evaluation results (from AI/ML BM Case 1) to assess how these measurement errors affect the accuracy of predictions related to mobility.

- Proposal 9:  RAN4 should leverage the LCM progress and requirements discussed in R-19 WI related to AI/ML Air Interface and investigate enhancements to these LCM procedures to RRM for mobility.

- References

- RP-242393, Revised SID on AIML for mobility in NR , OPPO, 3GPP TSG RAN Plenary #105, Melbourne, Australia, September 9-12, 2024.

- R2-2406309, Text proposal of 38.744, OPPO, RAN2#127, Maastricht.

- R4-2416449, On AIML Mobility Scenarios Evaluation and RRM Requirements Impacts, NOKIA Hefei, China, Oct 14 – Oct 18, 2024

- R4-2415577, General discussion on RRM requirement impact analyses on AI mobility, Samsung, Hefei, China, Oct 14 – Oct 18, 2024

- R4-2415033, Study of impacts on RAN4 requirements for AI mobility, OPPO , Hefei, China, Oct 14 – Oct 18, 2024

- R4-2414931, Discussion on RRM requirements of AI mobility, MediaTek , Hefei, China, Oct 14 – Oct 18, 2024