# 3GPP TSG-RAN WG4 Meeting #114	R4-2500241

# Athens, GR, 17th – 21st February, 2025

Agenda item:	7.10.3.4

Source:	Apple

Title:	On less than 5MHz for NTN RRM

Document for:	Discussion

# Introduction

The WF for RRM of NTN with channel BW less than 5MHz [1] has been agreed in RAN4#113, and the open issues have been summarized in the topic summary for [113][216] NR_IoT_NTN_req_test_enh [2].

In this contribution, we continue the discussion on the RRM requirement for less than 5MHz in NR NTN.

# Discussion

### Issue 1-1-2: Measurement(SSB index detection requirements)

Wayforward: - Option 1: - The number of SSB samples for Rel-18 NTN measurement requirements (Tidentify_xxx_with_index) can be extended to will be extended by [4] and [3] for intra-frequency and inter-frequency cases respectively. - Option 2: Two sets of requirements - Set 1: The number of SSB samples for Rel-18 NTN measurement requirements (Tidentify_xxx_with_index) can be extended to will be extended by [4] and [3] for intra-frequency and inter-frequency cases respectively. - Set 2: smaller delay with higher SNR side condition. - FFS on the exact number of delay to be extended base on the simulation results under the same simulation assumptions for TN less than 5MHz [R4-2306400] - FFS on the separate UE capability to support set 2 requirements

In the current TN requirement for less than 5MHz, it’s specified that sample extension is X=4 and Y=3 for intra-frequency and inter-frequency cases respectively. In R17 NTN requirement, we used TN measurement delay and accuracy as baseline, and therefore here, we propose also to use the extension values from TN less than 5MHz as baseline. We don’t prefer to have two sets of delay requirement based on different SNR conditions, since delay requirement is mainly for functionality (shall be general) and RAN4 has no such multiple sets of requirements for legacy TN less than 5MHz either.

Proposal 1: support option 1: The number of SSB samples for Rel-18 NTN measurement requirements (Tidentify_xxx_with_index) can be extended to will be extended by [4] and [3] for intra-frequency and inter-frequency cases respectively.

# Conclusion

In this contribution, we continue the discussion on the RRM requirement for less than 5MHz in NR NTN.

Proposal 1: support option 1: The number of SSB samples for Rel-18 NTN measurement requirements (Tidentify_xxx_with_index) can be extended to will be extended by [4] and [3] for intra-frequency and inter-frequency cases respectively.

# References

[1] R4-2420091, WF on RRM requirements for NR_IoT_NTN_req_test_enh, Xiaomi, RAN4#113

[2] R4-2418274, Topic summary for [113][216] NR_IoT_NTN_req_test_enh, Xiaomi, RAN4#113