3GPP TSG-RAN4 Meeting  #114	R4-2500243

Athens, GR, 17th – 21st February, 2025

Agenda item:	7.6.2

Source:	Apple

Title:	On methods for reducing the number of UE Rx chains for fragmented carriers

Document for:	Discussion

# 1	Introduction

The objective of this study is as follows:

-	Identify methods for reducing the number of UE Rx chains (e.g. from separate RF chains per CC to shared RF chains ) needed for single DL band with frequency span ≤ 100 MHz, containing two non-contiguous CCs within a CA combination for the inter-operator co-located scenario, considering:

-	Which RF requirements could be adjusted for the inter-operator co-located BS scenario, e.g. existing UE RF requirements such as ΔRIBNC, ACS and in-band blocking [RAN4];

- Other requirements are not precluded, if identified

-	The ability to semi-statically switch hardware resources (i.e. Rx chains) [RAN4, RAN2 – See note 2];

-	Up to 6 dB DL received power spectral density imbalance between the two non-contiguous CCs [RAN4];

-	Determine a reasonable level for the power spectral density difference between carriers of co-located adjacent channel operators for study [RAN4]

-	Impacts on DL performance [RAN4] ;

-	Means for a UE to inform the network of appropriate CA configuration it can support with adjusted RF requirements [RAN4, RAN2 – See note 2].

At RAN#104, a R19 SI on fragmented carriers was approved [1], with the following objective:

At the last RAN4 meeting, a WF was agreed that captures the agreements and open issues [2]. In this contribution, we share our views on the open issues.

# 2	Discussion

Prerequisite conditions

To allow the UE to support the two non-contiguous CCs with common/shared RF chains instead of separate RF chains, as analyzed in our companion contribution [3], the blocker power level in the gap needs to be manageable to the UE. In other words, the blocker power level needs to be at an appropriate level that the UE can handle with common RF chain. It can either be a specified value in the spec or based on UE capability reporting. The Rx chain configuration/reconfiguration may be based on some assistant information by UE measurement of the blocker.

Proposal 1: To allow the UE to support the two non-contiguous CCs with common/shared RF chains instead of separate RF chains, the blocker power level in the gap needs to be less than a defined level threshold that the UE can handle with common RF chain given a wanted signal level.

Signaling aspects

At the last meeting, it was captured in the WF that companies are encouraged to provide analysis with pros and cons on preferred signalling solutions so that they can be discussed in next meeting. We provide our analysis below.

Usually, the CA configurations supported by a UE are reported to the network when the UE camps on a cell and receives capability inquiry from the network. In the case of RF chain switching (between common RF chains and separate RF chains), however, the UE may need to update its supported CA configuration.

There are at least three alternatives as shown in the table below.

Table 1: Three alternatives of signaling solutions with pros and cons analysis

|  | How it works | Pros | Cons |
| --- | --- | --- | --- |
| Alt. 1: Legacy band combination reporting scheme + Rx sharing capability indication | UE reports to network that it is capable of Rx chain sharing for the supported band combination by additional signaling on top of existing band combination reporting. An example is the UE supports CA_n7(2A)-n25(2A) with Rx sharing capability and supports CA_n7A-n25(2A) without. | The current CA capability reporting is reused. | New UE capability is needed to indicate support of Rx chain sharing. |
| Alt. 2: Band combination capability update with the supported number of CCs | The UE first reports a high CA combination assuming non-continuous CCs can be handled by common RF chain. If the blocker power is too high, the UE can inform the network that only fewer CCs can be supported. Similarly, the UE first reports it supports CA_n7(2A)-n25(2A), and later if the blocker in the gap between two non-contiguous CCs in n25 is too high, either the UE can use UAI or other mechanism to release CCs to CA configuration of CA_n7A-n25(2A) or the network can release CCs to CA configuration of CA_n7A-n25(2A) | The current CA capability reporting is reused. No new UE capability is introduced. |  |
| Alt. 3: New band combination definition e.g. CA_n25(f_2A) + new set of signaling | A new syntax/notation, say CA_n25(f_2A), is used for indicating support of fragmented carrier operation in for CA in band n25 with common RF chain. Later the UE may indicate it cannot support it when the blocker power becomes too high for the UE to do so. | It is visible and clear in RAN4 specification that a CA configuration is supported by UE with RF sharing capability. | A new syntax/notation is needed for both RAN4 and RAN2. |

We also note that there are more aspects related to UE capability and implementation options. Accordingly, more consideration is needed for the signaling design.

Proposal 2: In terms of signaling, at least three alternatives can be considered for band combination reporting for UE capable of Rx chain sharing, with the analysis of pros and cons in Table 1. More alternatives can be further considered.

- Alt. 1: Legacy band combination reporting scheme + Rx sharing capability indication

- Alt. 2: Band combination capability update with the supported number of CCs

- Alt. 3: New band combination definition e.g. CA_n25(f_2A) + new set of signaling

# 3	Conclusions

In this contribution, we make the following proposals.

Proposal 1: To allow the UE to support the two non-contiguous CCs with common/shared RF chains instead of separate RF chains, the blocker power level in the gap needs to be less than a defined level threshold that the UE can handle with common RF chain given a wanted signal level.

Proposal 2: In terms of signaling, at least three alternatives can be considered for band combination reporting for UE capable of Rx chain sharing, with the analysis of pros and cons in Table 1. More alternatives can be further considered.

- Alt. 1: Legacy band combination reporting scheme + Rx sharing capability indication

- Alt. 2: Band combination capability update with the supported number of CCs

- Alt. 3: New band combination definition e.g. CA_n25(f_2A) + new set of signaling

# 4	References

RP‑241669, "New SID: Study on NR FR1 DL Fragmented Carriers," MediaTek

R4-2420337, "WF on fragmented DL carriers study," MediaTek

R4-2418532, "On UE RF requirements for fragmented carriers," Apple