3GPP TSG-RAN WG4 Meeting #114		R4-2500252

Athens, GR, 17th – 21st February, 2025

Agenda item:	4.5.1

Source:	Apple

Title:	(NR_RRM_enh-Core) On gradual timing adjustment and MTTD for CA and DC

Release:	Rel-15

Document for:	Discussion

# 1. Introduction

In Rel-15 gradual timing adjustment requirements were introduced.  In this contribution we propose to clarify the limits to gradual timing requirements and MTTD in CA and DC similar to our proposal in [1] in RAN4#113.

# 2. Discussion

The gradual timing adjustment requirement in §7.1.2.1 is defined as:

The gradual timing adjust requirements are applicable for CA and DC. The UE applied gradual timing adjustment until the transmission timing error between UE and the reference timing is within ±Te for each TAG in DC and CA case. The gradual timing adjust is applied until the timing error with respect to reference timing is within ±Te for each TAG. UE is also expected to meet MTTD requirements between pairs of TAGs or PCell and PScell depending on the configuration. It is possible that with gradual timing adjustment applied on each TAG independently the TTD between the pair of TAGs exceeds MTTD requirement and could impact the UL transmission in certain scenarios.

While the UE is applying gradual timing adjustment, the TTD for the pair of TAGs or between PCell-PSCell could exceed the MTTD requirement

In certain cases, there might be an impact to UL transmission if the MTTD is exceeded.

The MTTD requirements are defined for DC and CA for the following:

Inter-band EN-DC: between E-UTRA PCell and PSCell

Async – mandatory, except in FDD-FDD with overlapping spectrum with LTE with capability

Sync - mandatory

Intra-band EN-DC: between E-UTRA PCell and PSCell

Async – optional with capability

NR CA: between pairs of TAGs configured in SA and DC modes

Inter-band NE-DC: between PCell and E-UTRA PSCell

Async FDD-FDD with UE capability

Inter-band NR-DC: between PCell and PSCell

Sync and Async based on UE capability

For EN-DC, NE-DC, NR-DC if asynchronous operation is supported, if the TTD exceeds MTTD might not be an issue. But if asynchronous operation is not supported, then there will be an impact to UL transmission if TTD exceeds MTTD.

In case of NR CA if MTTD is exceeded, there would be an impact to UL transmission

For NR-DC, EN-DC and NE-DC if asynchronous mode is supported there might not be issue with UL transmission if TTD exceeds MTTD.

For NR-DC, EN-DC and NE-DC if asynchronous mode is not supported there would be an impact to UL transmission if TTD exceeds MTTD.

For NR CA if TTD exceed MTTD, there would be an impact to UL transmission.

In the cases above listed above where there would be an impact to UL transmission if TTD exceeds MTTD, we propose that the gradual timing adjust should stop if the transmit timing difference would exceed MTTD with the adjustment, which is also aligned with the behavior in LTE.

From 36.133 §7.1.2

In LTE when gradual timing adjustment results in TTD exceeding MTTD requirement, UE stops gradual timing adjustment on that TAG.

In RAN4#113 discussion another UE behavior for MTTD requirements in LTE was brought up as a potential way to handle the UE gradual timing adjustment limit above.

We have the following options to define behaviour when UE gradual timing adjustment exceeds MTTD:
Option 1: UE stops gradual timing adjustment.
Option 2: UE stops UL transmission on the secondary UL carrier

36.133 §7.9.2:

It must be pointed out that in LTE if the MTTD is exceeded because of a new TA command, but not gradual timing adjustment by UE, the UE is not expected to transmit on the secondary UL carrier.  If the same approach is used for handling when MTTD is exceeded with gradual timing adjustment, the reason for stopping UL transmission would not be known to the network. It could be mistaken to be link failure of some kind. If UE continues transmission, network would be able to send new TA command.

In LTE if the MTTD is exceeded because of a new TA command, but not gradual timing adjustment by UE, the UE is not expected to transmit on the secondary UL carrier.

With the approach of stopping UL transmission when MTTD is exceeded due to gradual timing adjustment, the reason would not be known to the network and could be mistaken as link failure

The timing issue would be resolved with new TA command on the TAG

If the gradual timing adjustment on PCell causes TTD to exceed MTTD, stopping UL transmission on PCell would be detrimental, while stopping UL transmission on secondary cell would not help the NW realize the issue.

Figure 1: Illustration of gradual timing adjustment and MTTD issue

Hence, we propose the following:

- The gradual timing adjustment is applied in the until timing error is within ±Te or the MRTD requirement depending on the configuration is not exceeded in the following cases:
       Inter-band CA
       Inter-band EN-DC with spectrum overlapping between LTE and NR, and UE doesn’t support       asynchronous operation 
       Intra-band EN-DC if UE doesn’t support asynchronous operation
       Inter-band NE-DC if UE doesn’t support asynchronous operation 
       Inter-band NR-DC if UE only supports synchronous operation

The above behavior to stop UL transmission when TA command results in MTTD being exceeded is missing in NR spec. We propose to add the following UE behavior in NR:

If MTTD is exceeded due to application of new TA command, the UE shall stop transmission on Scell or PSCell in the following cases:

- Inter-band CA

- Inter-band EN-DC with spectrum overlapping between LTE and NR, and UE doesn’t support asynchronous operation

- Intra-band EN-DC if UE doesn’t support asynchronous operation

- Inter-band NE-DC if UE doesn’t support asynchronous operation

- Inter-band NR-DC if UE only supports synchronous operation

- If MTTD is exceeded due to application of new TA command, the UE shall stop transmission on SCell or PSCell in the following cases:
       Inter-band CA
       Inter-band EN-DC with spectrum overlapping between LTE and NR, and UE doesn’t support       asynchronous operation 
       Intra-band EN-DC if UE doesn’t support asynchronous operation
       Inter-band NE-DC if UE doesn’t support asynchronous operation 
       Inter-band NR-DC if UE only supports synchronous operation

Our proposals are captured in our companion CR [2].

# 3. Conclusion

In this paper, we propose an update to the gradual timing requirements in CA and DC. Our observations and proposals are captured below:

- While the UE is applying gradual timing adjustment, the TTD for the pair of TAGs or between PCell-PSCell could exceed the MTTD requirement

In certain cases, there might be an impact to UL transmission if the MTTD is exceeded.

For NR-DC, EN-DC and NE-DC if asynchronous mode is supported there might not be issue with UL transmission if TTD exceeds MTTD.

For NR-DC, EN-DC and NE-DC if asynchronous mode is not supported there would be an impact to UL transmission if TTD exceeds MTTD.

For NR CA if TTD exceed MTTD, there would be an impact to UL transmission.

In LTE when gradual timing adjustment results in TTD exceeding MTTD requirement, UE stops gradual timing adjustment on that TAG.

We have the following options to define behaviour when UE gradual timing adjustment exceeds MTTD:
Option 1: UE stops gradual timing adjustment.
Option 2: UE stops UL transmission on the secondary UL carrier

In LTE if the MTTD is exceeded because of a new TA command, but not gradual timing adjustment by UE, the UE is not expected to transmit on the secondary UL carrier.

With the approach of stopping UL transmission when MTTD is exceeded due to gradual timing adjustment, the reason would not be known to the network and could be mistaken as link failure

The timing issue would be resolved with new TA command on the TAG

If the gradual timing adjustment on PCell causes TTD to exceed MTTD, stopping UL transmission on PCell would be detrimental, while stopping UL transmission on secondary cell would not help the NW realize the issue.

- The gradual timing adjustment is applied in the until timing error is within ±Te or the MRTD requirement depending on the configuration is not exceeded in the following cases:
       Inter-band CA
       Inter-band EN-DC with spectrum overlapping between LTE and NR, and UE doesn’t support       asynchronous operation 
       Intra-band EN-DC if UE doesn’t support asynchronous operation
       Inter-band NE-DC if UE doesn’t support asynchronous operation 
       Inter-band NR-DC if UE only supports synchronous operation

- If MTTD is exceeded due to application of new TA command, the UE shall stop transmission on SCell or PSCell in the following cases:
       Inter-band CA
       Inter-band EN-DC with spectrum overlapping between LTE and NR, and UE doesn’t support       asynchronous operation 
       Intra-band EN-DC if UE doesn’t support asynchronous operation
       Inter-band NE-DC if UE doesn’t support asynchronous operation 
       Inter-band NR-DC if UE only supports synchronous operation

# Reference

- R4-2418540, “On gradual timing adjustment for CA and DC”, Apple.

- R4-2500254, “(NR_RRM_Enh-Core) CR to 38.133 for gradual timing adjustment and MTTD-R16”, Apple.