3GPP TSG-RAN WG4 Meeting #114		R4-2500263

Athens, GR, 17th – 21st February, 2025

Agenda item:	7.16.2

Source:	Apple

Title:	Simulation results for Spatial Channel Modeling study

Release:	Rel-19

Document for:	Discussion

# 1. Introduction

In RAN4#113 spatial channel model for demodulation requirements was discussed and way forward [1] was approved. In this contribution present simulation results for spatial channel model study.

# 2. Simulation Results

For the initial simulation parameters below, we present our results.

Table 1: Simulation Parameters for Spatial Channel Model Study

| Parameter |  | SU-MIMO PDSCH (FR1) | SU-MIMO PMI (FR1) |
| --- | --- | --- | --- |
| Duplex mode |  | TDD |  |
| TDD Slot Configuration Pattern |  | 7DS2U |  |
| FR / Carrier frequency |  | FR1 / 3.5 GHz |  |
| UE receiver type |  | IRC Baseline | IRC Baseline |
| Number of Tx Ports |  | 4 | 8 |
| Number of Rx Ports/Antennas |  | 4 | 4 |
| Number of layers |  | 4 | 4,2 |
| PMI |  | Random | Follow, Random |
| Waveform |  | CP-OFDM with normal CP |  |
| Channel Bandwidth/SCS |  | 40MHz/30kHz |  |
| MCS |  | 13 (64 QAM table) |  |
| PDSCH configuration | Mapping type | Type A |  |
|  | k0 | 0 |  |
|  | Starting symbol (S) | 2 |  |
|  | Length (L) | 12 |  |
|  | PDSCH aggregation factor | 1 |  |
|  | Resource allocation type | Type 0 |  |
|  | VRB-to-PRB mapping type | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size | N/A |  |
| PDSCH DMRS configuration | DMRS Type | Type 1 |  |
|  | Number of additional DMRS | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS | 2 (for rank > 4) 1 (for rank <= 4) |  |
| Codebook configuration | CodebookType | type-I-SP | type-I-SP, eType-II |
|  | Codebook configuration | For 8Tx (N1,N2,O1,O2) = (4,1,4,1) For 4Tx (N1,N2,O1,O2) = (2,1,4,1) | For typeI-SP: (N1,N2,O1,O2) = (4,1,4,1) For eType-II: (N1,N2,O1,O2) = (4,1,4,1) [ paramCombination-r16=5 (L=4, pν=1/4, β=3/4) ] |
| PDSCH DMRS Precoding Configuration |  | For Random precoding: for every PRB Bundle (size=2) | For typeI-SP (random and follow): Wideband PMI For eType-II with follow: SB PMI |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type | Aperiodic |  |
|  | Number of CSI-RS ports (X) | 4 CSI-RS Ports (2,1) for 4 Layer | 8 CSI-RS Ports (4,1) |
|  | Density (ρ) | 1 |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) | (13) |  |
| CQI/RI/PMI delay |  | N/A | 6.5 ms |
| Number of HARQ Processes |  | 8 |  |
| Maximum HARQ transmissions |  | 4 |  |

Channel Models:

TDLA30

TDLC300

UMa-CDL-C: CDL (TR 38.753) with Table 7.2.1-8 in TR 38.827

Doppler: 10, 100 Hz

AAV Configuration: Pass through – AAV 3

  - (M,N,P,Ms,Ns) = (1,4,2,1,1) for 8Tx CSI-RS Ports

  - (M,N,P,Ms,Ns) = (1,2,2,1,1) for 4Tx CSI-RS Ports

SU-MIMO PDSCH Performance

From the results above, we don’t observe better performance with CDL channel model compared to TDL with low, XP-Med correlation.

- The performance with CDL channel model is not better than existing TDL channel models with Low, Med correlation for 4x4 with 4 layers.

SU-MIMO PMI-PDSCH Performance

Figure 1: PMI Reporting performance with follow vs random PMI

The TP gain with Type I and eType II are presented in the table below.

Table 2: TP Gain with PMI reporting

| Channel/ Doppler | # Layers | g for Type I | g for eType II |
| --- | --- | --- | --- |
| UMa-CDL-C; 10 Hz | 2 | 1.97 | 2.29 |
| UMa-CDL-C; 10 Hz | 4 | 1.83 | 1.58 |
| UMa-CDL-C; 100 Hz | 2 | 1.73 | 2.00 |
| UMa-CDL-C; 100 Hz | 4 | 1.80 | 1.48 |

For eType II we compare performance between different channel models – existing TDL channel models and Uma-CDL-C.

Figure 2: Follow PMI with eType II performance comparison

Based on the results in figure 2, the performance with follow PMI with eType II is not significantly better than existing TDL channel models with/ without correlation.

- The performance of follow PMI with eType II is not significantly better than existing TDL channel models.

# 3. Conclusion

In this paper, we provide our simulation results for study of spatial channel models. Our observations are captured below:

SU-MIMO PDSCH performance

- The performance with CDL channel model is not better than existing TDL channel models with Low, Med correlation for 4x4 with 4 layers.

SU-MIMO PMI PDSCH performance

- The performance of follow PMI with eType II is not significantly better than existing TDL channel models.

# Reference

- R4-2419782, “Way Forward for [113][320] NR_SCM”, Nokia.