3GPP TSG-RAN WG4 Meeting #114                                       	                         R4-2500275

# Athens, Greece, February 17 – February 22, 2025

Agenda item:	7.1.1.3.2

Source:	Apple Inc.

Title:	Views on Maximum Number of MIMO Layers for 6Rx UEs

Release:	Rel-19

Document for:	Discussion

# Introduction

A new WID on RF enhancements for NR FR1/FR2 and EN-DC has been created for Release 19. The WID covers three main areas: High power UE (HPUE) for CA in terrestrial network (TN), power boosting and/or MPR reduction, and 6Rx for handheld and FWA UEs. The main objectives for 6Rx, as highlighted in the WID [1], are captured below:

Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario

  - Example bands: n41, n77/n78, n79, n104

  - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers

  - Specify the Rx requirements including reference sensitivity requirements for support 6Rx

    - Note: the specified requirements can be applicable to both handheld UE and FWA devices

  - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, t4r6 depending on UE capability

  - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution

Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding

In this discussion paper, we share our views on the maximum number of MIMO layers for 6Rx handheld UEs. Enabling 6Rx for large form-factor handheld UEs has the potential of improving DL coverage and throughput. However, it is important to understand if supporting 6 layers vs. 4 layers can produce practical throughput gains. We continue discussing our views below, including simulation results supporting our conclusions.

# Scope

During the previous RAN4#112bis meeting in Hefei, China, and during RAN4#113 meeting in Orlando, FL, discussion continued on the general considerations for the MIMO layers evaluation for 6Rx UEs, and in addition, specific outcomes on 6 layers support. As captured in the previous WF [2], we highlight the discussion outcome below:

## Sub-topic 2-1: General considerations for MIMO layer evaluation for 6Rx UE

Issue 2-1-1: Tx EVM assumptions for 6-layer feasibility evaluation

Way Forward: RAN4 to further discuss the following options.

-	Option 1: Use Tx EVM = 1%

-	Option 2: Use Tx EVM = 3% for all of modulation order in MCS Table 2 for performance evaluation

-	Option 3: Consider TxEVM of 6% for 64QAM and TxEVM 3% for 256QAM

Issue 2-1-2: 6-Layer Feasibility Evaluation Assumptions

Way Forward: RAN4 to further discuss and align on simulation assumptions provided in Annex A and discuss possible options for refinement and down selection for key parameters as identified in company proposals as summarised below. The bullet list included is not intended to be a complete set of options and it does not specifically down select or preclude other options for refinement. It is only intended to summarize the options presented.

-	Antenna Configuration:

-	Limit the transmit/receive antenna configuration to 8Tx/6Rx and exclude 16Tx/6Rx and 32Tx/6Rx antenna configurations.

-	Consider both 8Tx/6Rx and 32Tx/6Rx antenna configurations

-	DMRS Configuration:

-	Consider DMRS Configuration Option 1: Rel-18 1+1(6L), Rel-15 1+1(4L) from Annex A table.

-	Consider DMRS Configuration Option 3: Rel-15 1+1(4L), Rel-15 2+2(6L) from Annex A table.

-	Consider DMRS Configuration Option 1: Rel-18 1+1(6L), Rel-15 1+1(4L) and Option 3: Rel-15 1+1(4L), Rel-15 2+2(6L) from Annex A table.

-	Consider additional DMRS Configuration Option 1a: Rel-18 1+1(6L), Rel-18 1+1(4L)

-	Rank

-	Include only Rank 4 and Rank 6 configurations

-	Include Rank 5, in addition to Rank 4 and Rank 6

-	Modulation and Coding Schemes for Adaptive MCS

-	Consider Table 2 only, 64QAM + 256QAM

-	Consider Table 1 only, 64QAM

-	Consider Table 1 and Table 2

-	Additional margins

-	Apply at least 3dB correction to the SNR values reported by companies in baseband simulation results

-	A 2~3 dB additional margin or 2% Rx EVM shall be considered

-	BS Antenna Correlation

-	RAN4 to use BS correlation matrix with alpha = 0.1

-	RAN4 shall use no correlation on BS Tx Ports

-	RAN4 shall not use BS correlation matrix other than 3GPP specified ones

Issue 2-1-3: Performance requirements for 6Rx

Agreement: RAN4 shall discuss the performance requirements for 6Rx once the performance part commences:

Issue 2-1-4: Way forward for performance requirements for 6-layers for handheld UE and FWA

Way Forward: RAN4 to further discuss the following options.

-	Option 1: Once the gain and feasibility for handheld UE with 6 MIMO layers are verified, then 6 MIMO layers related requirements defined for handheld UE could be applicable for FWA devices. Otherwise, no 6 MIMO layers related requirements should be defined for neither handheld UE nor FWA devices.

-	Option 2: RAN4 to introduce 6-layer performance requirements for handheld and FWA devices.

-	Option 3: RAN4 shall conclude that whether to introduce 6 layers for both FWA and handheld UE depends on the outcome of 6 MIMO layers evaluation on handheld UE.

-	Option 4: From WF in R4-2417209, companies to further discuss the following aspect of 6-layer support.

-	If the handheld feasibility is confirmed, then the handheld and FWA devices will share the same 6-layer requirements.

-	If the handheld feasibility is not confirmed, 6-layer requirements will be specified for FWA device only.

-	Option 5: Do not specify 6-layer requirements for handheld and FWA devices in Rel-19.

Issue 2-1-5: Baseband complexity

Way Forward: RAN4 to determine if RAN4 needs to analyse the increased baseband complexity and power consumption for 6Layers compared to 4Layers including LDPC decoder, channel estimation, DMRS based interference estimation and MIMO detector.

Issue 2-1-6: Issues for UE Device Types

Way Forward: RAN4 to further discuss the following proposals and determine any necessary actions.

-	Proposal 1: For the foldable UE assumed for the 6 layers feasibility study, the following aspects should be considered for evaluation with 1~2 dB margin:

-	When the phone is folded, the spatially overlapped antenna could be impacted which would lead to degraded receiving performance.

-	If the diversity branches are not on the main board, there could be power imbalance issue since the insertion loss would be high for the path across the hinge.

-	The hinge would partially occupy the edge of each screen in order to guarantee the structural stability, hence unlike phablet at least the occupied space cannot be used for antenna placement anymore. Then adding more antenna could make the antenna correlation worse.

-	Proposal 2: RAN4 should clarify the nature of the proposed ECC-based correlation models and specify the types of devices that they represent

## Sub-topic 2-2: 6-layer Support

Issue 2-2-1: 6-layer Support Feasibility

Way Forward: RAN4 to further discuss the following options.

-	Option 1: 6-layer support is feasible for handheld and FWA based on some company evaluations considering realistic antenna correlation assumptions and deployment scenarios.

-	Option 2: 6-layer support is feasible for FWA devices

-	Option 3: Companies to further discuss align on the feasibility criteria and simulation assumptions to evaluate 6-Layer feasibility for handhelds using the WF in R4-2417209 and any updates related to Issue 2-1-2 above.

-	Option 4: 6-layer support is not feasible for handheld UEs

Issue 2-2-2: 6-layer Support as optional feature

Way Forward: RAN4 to further discuss if 6-Layer support should be considered an optional feature per the following.

-	Introduce 6 MIMO layers support as an optional feature, if 6-layer performance requirements are introduced for 6Rx.

In addition to this, Annex A in [2] corresponds to the progress on agreed simulation assumptions, which will be discussed below.

# Discussion

One of the primary objectives of this WID is twofold: 1) to establish 4 MIMO layers as the baseline configuration, and 2) to evaluate the potential performance gains and feasibility of supporting 6 MIMO layers with a 6-receive antenna (6Rx) user equipment (UE). During the previous meeting, we reached several foundational agreements to quantitatively assess whether supporting up to 6 MIMO layers is viable. Specifically, we agreed that: 1) feasibility criteria should be based on throughput performance comparisons between 4-layer and 6-layer configurations, 2) only link-level simulations are relevant for this evaluation, and 3) a preliminary set of detailed simulation assumptions was documented in Annex A of the previous WF [2]. While this represents significant progress, we propose narrowing the parameter space further to streamline the decision-making process. Additionally, based on our own simulation results, we conclude that supporting 6 MIMO layers on a handheld UE is not feasible, a position that is shared by several other companies.

Simulation Parameters:

In Annex A, one of the assumptions in discussion is the transmit antenna configuration. During RAN4#112bis it was agreed to 1) Consider {8Tx, 6Rx} as baseline, and 2) Interested companies can submit results with {32Tx, 6Rx} and specify the precoding type.

Observation #1: There is insufficient justification to increase the number of transmit antennas beyond 8 when considering only 6 MIMO layers.

Proposal #1: Limit the transmit/receive antenna configuration to 8Tx/6Rx.

Proposal #2: Exclude 16Tx/6Rx and 32Tx/6Rx antenna configurations from the feasibility analysis.

Regarding rank, the previous RAN4#112bis/113 meetings concluded that: 1) Rank 4 and Rank 6 should be included in the feasibility assessment, and 2) companies expressing interest may submit results for Rank 5. In this context, we maintain that the primary intent of the WID is to assess whether six MIMO layers provide a performance advantage over four MIMO layers, despite other potential interpretations based on the specific wording of the WID. For instance, if a scenario arises where five MIMO layers outperform four layers, rather than six, the key question remains whether this feature would offer sufficient benefit to justify deployment. Our position is that, in such a case, it would not.

Observation #2: The primary objective of the WID is to assess whether 6 MIMO layers can outperform 4 MIMO layers.

Observation #3: If only Rank 5 is feasible, and not Rank 6, this would further expand concerns of whether the increase in UE complexity of 2-CW vs single-CW operation is justified.

Proposal #3: Include only Rank 4 and Rank 6 configurations in the feasibility evaluation.

Regarding the selection of MCS values for the analysis, it was agreed to use Adaptive MCS for performance comparison. As per the agreement in the previous meeting, the focus will be on the 64QAM and 256QAM modulation schemes. For this analysis, Table 2 provides all necessary data points.

Observation #4: MCS Table 2 already considers the full range of code rates and spectral efficiency to be evaluated for 64QAM.

Proposal #4: Consider only MCS Table 2 for the feasibility evaluation.

Regarding the DMRS configuration, four options were still under consideration during the previous meeting, one of which includes Rel-18 DMRS eType-1. We believe that the most equitable comparison is to focus on Option 3: [1+1 (4L), 2+2 (6L)], as these configurations represent the typical deployment scenarios. It is anticipated that the majority of field deployments will use 1+1 for 4 layers, as this is sufficient for Rel-15 DMRS Type-1.

Observation #5: The typical DMRS configuration used by operators is 1+1 for 4 layers, while 2+2 is typically used for 6 layers due to RAN1 design requirements.

In adition, some companies propose and try to justify comparing Rank 6 vs Rank 4 performance in an equal-overhead approach. This is, 4L 2+2 vs 6L 2+2, or 4L 1+1 vs 6L 2+0. Considering 4L 2+2 implies an additional artificial penalty to Rank 4 operation, since imposing this feature should operate only under Rel-18 is not practical or beneficial. On the other hand, 6L 2+0 is unlikely to provide any benefits since such configuration would hinder UE mobility since additional DMRS cannot be allocated dinamically via DCI. This means that once the UE starts increasing its mobility 1) the network needs to detect this (likely via HARQ retransmissions at the minimum), and 2) perform an RRC reconfiguration, with all the signaling overhead and slow reaction time that this implies. At the moment, there’s no DCI-based dynamic mechanism to configure additional DMRS positions on a slot-by-slot basis.

Observation #6: The typical DMRS configuration used by operators is 1+1 for 4 layers, while 2+2 is typically used for 6 layers due to RAN1 design requirements. A 6L 2+0 would severly hinder mobility, which is not practical.

Proposal #5: Limit the analysis to Rel-15 DMRS Type-1 for this feasibility study, considering Rel-18 only for informational purposes.

Proposal #6: Use only Option 3: [1+1 (4L), 2+2 (6L)] as the DMRS configuration, while excluding 4L 2+2 and 6L 2+0 DMRS configurations.

Regarding the assumption on TxEVM impairments, we have proposed a typical value based on prior experience with demodulation performance requirements. Specifically, we suggested 6% for 64QAM and 3% for 256QAM, which already represent lower values than the minimum base station (BS) requirements. We do not support custom tightening of these values for the purposes of this specific work item, but rather advocate for a general tightening of the requirements if multiple companies can demonstrate that TxEVM performance in the field consistently exceeds the minimum specifications. Furthermore, no tightening was agreed upon during the Rel-18 8Rx evaluation, and we believe the same approach should be applied here. In addition, it is worth noting that considering TxEVM in this evaluation should be accompanied by the corresponding assumption on RxEVM, which today is missing in the analysis, and that it has been common practice in previous feasibility evaluations.

Observation #7: If multiple companies support the claim that TxEVM in the field exceeds the minimum BS RF requirements, then the requirements should be tightened. However, this would fall outside the scope of this work item.

Proposal #7: Do not relax TxEVM for the purposes of this feasibility study. Consider a TxEVM value of 3% for up to 256QAM. In addition, consider a RxEVM value of at least 2%.

Custom ECC based MIMO Correlation Models:

Given that some companies are willing to share ECC measurements, it was agreed during RAN4#112bis to at least consider custom MIMO correlation models as part of the evaluation options. From our perspective, however, we remain concerned about the methodology used to derive these correlation models, including the types of devices they represent and other related factors, as they may present an overly optimistic scenario.

Observation #8: The exact conditions under which ECC-based correlation models were derived remain unclear.

Proposal #8: RAN4 should clarify the nature of the proposed ECC-based custom correlation models and specify the types of devices they represent.

In addition, during RAN4#112bis (and further explained during RAN4#113) it has been agreed that since custom correlation models are considered, also antenna efficiency should be part of the analysis. Antenna efficiency, plus other practical factors, increase the SNR

Observation #9: During RAN4#112bis it was agreed that antenna efficiency could be considered in the feasibility analysis, and companies are encouraged to provide antenna efficiency data.

Observation #10: Results submitted by companies during RAN4#112bis and earlier are based on baseband simulations and do not account for antenna efficiency or impairment factors.

Proposal #9: Apply at least a 3 dB correction to the SNR values reported by companies in baseband simulation results.

# Simulation Results

As the starting point for our analysis, it is important to note that a comprehensive calibration exercise involves comparing the performance of 4 MIMO layers versus 6 MIMO layers under ideal conditions. Specifically, this assumes no correlation at both the Tx and Rx sides, and no impact from TxEVM. This approach allows us to establish the best-case scenario, after which we can proceed with analyzing how factors like correlation and TxEVM may influence our conclusions. In Figure 1 below, we present a comparison between Rank 4 and Rank 6 using MCS adaptation across the full range of 64QAM and 256QAM (MCS Table 2).

Figure 1: Absolute throughput comparison between Rank 4 and Rank 6 under the assumptions of no antenna correlation and no impact of TxEVM. Results obtained using link adaptation over 64QAM and 256QAM MCS values in MCS Table 2. Baseband results.

At this stage, it is already clear that any potential performance gain between 6 MIMO layers and 4 MIMO layers would only be observable at exceptionally high SNR values, which could be considered impractical or overly optimistic relative to the SNR levels achievable in real-world deployments.

Observation #11: Performance gains between 6 MIMO layers and 4 MIMO layers are only noticeable at very high SNR (> 30 dB) under idealized conditions with no correlation and no TxEVM.

At this point, conclusion seems quite clear. However, we will present results based on agreed options from RAN4#112bis for completeness. First, we will analyze simulation results based on custom correlation models based on ECC measurements. As shown in Figure 6 below, the inclusion of these models in comparison to no antenna correlation show that they are likely very optimistic. If we pay attention to Oppo’s correlation matrix and its very optimistic coefficients, it’s basically saying that there’s virtually no correlation in their model. A more balanced approach could be obtained via using Samsung’s based on 2.5GHz measurements. Here we see a 1-2dB difference with the case of no correlation.

Figure 6: Absolute throughput comparison between ideal zero correlation (3GPP Low) vs Custom ECC-based correlation matrices, for Rank 6. Values of TxEVM considered for both 64QAM and 256QAM modulations at 1% and 3% respectively. Baseband results.

In any case, even if these models are fully realistic, the conclusion remains in that relatively modest performance gains are only observed at very high SNRs.

Observation #12: Under custom ECC-based models and ideal conditions (no TxEVM impairments), 6 MIMO layers would only outperform 4 MIMO layers at very high SNR (> 30 dB).

In addition, we also agreed to explore what would be the hypothetical consequences of assuming the same DMRS overhead between Rank 4 and Rank 6. Even though we do not believe this is a valid and relevant scenario to consider, we show some of these results in Figure 7 below.

Figure 7: Absolute throughput comparison between Rank 4 1+1 vs Rank 4 2+2 vs Rank 6 2+2. Here we consider no TxEVM and TxEVM values of 1%/2%/3%. Baseband results obtained using Samsung’s ECC-based proposal.

In Figure 7 above we can still observe that if we hinder Rank 4 to the same overhead as Rank 6, Rank 6 outperforms Rank 4 still at high SNR values. In addition to this, we need to consider both RxEVM impairments and implementation aspects into the equation, which are not simulated yet, but will only strengthen our position.

Observation #13: Under custom ECC-based models, non-zero TxEVM impairments, and an hypothetical equal DMRS overhead between Rank 4 and Rank 6, 6 MIMO layers would still only outperform 4 MIMO layers at very high SNR.

Proposal #10: RAN4 to confirm that 6 MIMO layers with 6Rx is not feasible for handheld UE scenarios.

# Conclusions

Our observations and proposals are summarized below:

Observation #1: There is insufficient justification to increase the number of transmit antennas beyond 8 when considering only 6 MIMO layers.

Proposal #1: Limit the transmit/receive antenna configuration to 8Tx/6Rx.

Proposal #2: Exclude 16Tx/6Rx and 32Tx/6Rx antenna configurations from the feasibility analysis.

Observation #2: The primary objective of the WID is to assess whether 6 MIMO layers can outperform 4 MIMO layers.

Observation #3: If only Rank 5 is feasible, and not Rank 6, this would further expand concerns of wether the increase in UE complexity of 2-CW vs single-CW operation is justified.

Proposal #3: Include only Rank 4 and Rank 6 configurations in the feasibility evaluation.

Observation #4: MCS Table 2 already considers the full range of code rates and spectral efficiency to be evaluated for 64QAM.

Proposal #4: Consider only MCS Table 2 for the feasibility evaluation.

Observation #5: The typical DMRS configuration used by operators is 1+1 for 4 layers, while 2+2 is typically used for 6 layers due to RAN1 design requirements.

Observation #6: The typical DMRS configuration used by operators is 1+1 for 4 layers, while 2+2 is typically used for 6 layers due to RAN1 design requirements. A 6L 2+0 would severly hinder mobility, which is not practical.

Proposal #5: Limit the analysis to Rel-15 DMRS Type-1 for this feasibility study, considering Rel-18 only for informational purposes.

Proposal #6: Use only Option 3: [1+1 (4L), 2+2 (6L)] as the DMRS configuration, while excluding 4L 2+2 and 6L 2+0 DMRS configurations.

Observation #7: If multiple companies support the claim that TxEVM in the field exceeds the minimum BS RF requirements, then the requirements should be tightened. However, this would fall outside the scope of this work item.

Proposal #7: Do not relax TxEVM for the purposes of this feasibility study. Consider a TxEVM value of 3% for up to 256QAM. In addition, consider a RxEVM value of at least 2%.

Observation #8: The exact conditions under which ECC-based correlation models were derived remain unclear.

Proposal #8: RAN4 should clarify the nature of the proposed ECC-based custom correlation models and specify the types of devices they represent.

Observation #9: During RAN4#112bis it was agreed that antenna efficiency could be considered in the feasibility analysis, and companies are encouraged to provide antenna efficiency data.

Observation #10: Results submitted by companies during RAN4#112bis and earlier are based on baseband simulations and do not account for antenna efficiency or impairment factors.

Proposal #9: Apply at least a 3 dB correction to the SNR values reported by companies in baseband simulation results.

Observation #11: Performance gains between 6 MIMO layers and 4 MIMO layers are only noticeable at very high SNR (> 30 dB) under idealized conditions with no correlation and no TxEVM.

Observation #12: Under custom ECC-based models and ideal conditions (no TxEVM impairments), 6 MIMO layers would only outperform 4 MIMO layers at very high SNR (> 30 dB).

Observation #13: Under custom ECC-based models, non-zero TxEVM impairments, and an hypothetical equal DMRS overhead between Rank 4 and Rank 6, 6 MIMO layers would still only outperform 4 MIMO layers at very high SNR.

Proposal #10: RAN4 to confirm that 6 MIMO layers with 6Rx is not feasible for handheld UE scenarios.

# References

- RP-240720 “New WID: UE RF enhancements for FR1/FR2 and EN-DC, Phase 4”, by RAN4 chair (Huawei), 3GPP TSG-RAN Meeting #103, Maastricht, Netherlands, March 18-21, 2024

- R4-2420376 “WF on 6Rx UE requirements”, AT&T, 3GPP TSG-RAN WG4 Meeting #113, Orlando, FL USA, November 18-22, 2024