3GPP TSG-RAN WG4 Meeting #114	R4-2500308

Athens, Greece,  17th February – 21st February 2025

Agenda item:	7.22.2

Source:	Apple

Title:	On UE RF impact of 3Tx UL MIMO

WI/SI:	NR_MIMO_Ph5-Core

Release:	Rel-19

Document for:	Approval

# 1	Introduction

The scope of the Rel-19 WI [1] is the introduction of PC2 and 40MHz channel bandwidth for NR band n28. It is encouraged to evolve the existing dual duplexer architecture assumption to meet the current industry demand. This contribution discusses the introduction of 40MHz channel bandwidth.

# 2  Discussion

##### 2.1 Open item on applicable UE type

RAN1 responded RAN4 with a reply LS [2] and answered the question on output power for UL full power transmission mode 0:

Question 2: RAN4 would like to check with RAN1 whether a 3Tx UE supporting UL full power transmission mode 0 needs to achieve the same maximum output power with both 1Layer and 2Layers. Regarding Questions 2 and 3, RAN1 would like to inform RAN4 that UL full power Mode 0 delivers full power for any precoder in the 3-port codebook, regardless of the number of layers in the precoder. Reply for Question 2: Yes, a 3Tx UE supporting UL full power transmission mode 0, delivers the same maximum output power with both 1- and 2-layer precoders.

The answer confirms the decision that 3Tx PC1.5 cannot be supported for full power mode 0 in Rel-19.

##### 2.1 Open item on MPR application

At last meeting the question on 3Tx PC1.5 MPR could not be settled which lead to the following way forward:

Checking on clause 6.2D.2 of TS 38.101-1 it can be observed that there exists PC1.5 A-MPR tables dedicated to dual Tx and 4Tx. It would make sense to assume that a dedicated table would be required for 3Tx. Before this conclusion can be made an architecture check is needed. With the current available amplifier technology there are two available power classes, PC3 and PC2.

To reach PC1.5 with 3Tx there can only be two architecture options:

- PC1.5 is achieved with PC3+PC3+PC2

- PC1.5 is achieved with PC2+PC2+PC2

It has already been observed that the first option is not possible as the individual layers would have unequal power which is currently not applicable. Therefore, this option can be excluded. Option 2 uses 3xPC2 and does not require the individual amplifiers to operate at maximum output power when transmitting at 29dBm. Instead, the amplifiers can operate at roughly 1.8dB back-off. Comparing the dual Tx for 20dB antenna isolation table with the 4Tx table for 20dB antenna isolation we can observe that the larges difference is 3dB for edge allocations at 10dB antenna isolation. The inherent back-off cannot absorb all the increased power back-off need. This means that a 3xPC2 architecture requires a dedicated 3Tx MPR table.

Observation: With the assumption of achieving PC1.5 with 3xPC2 each amplifier can operate at roughly 1.8dB back-off at maximum output power. Comparing the dual Tx for 20dB antenna isolation table with the 4Tx table for 20dB antenna isolation we can observe that the larges difference is 3dB for edge allocations at 10dB antenna isolation. The inherent back-off cannot absorb all the increased power back-off need. This means that a 3xPC2 architecture requires a dedicated 3Tx MPR table.

Proposal 1: Discuss the introduction of a 3Tx table for PC1.5

Similar observation can be made for the open item on 3Tx PC2 MPR. In this case 3xPC3 can be used to achieve PC2 over the air and all layers have equal power. The amplifiers operate at roughly 1.8dB back-off. Comparing the 1Tx and 2Tx MPR it can be found that the largest difference is 3dB for Pi/2 BPSK edge allocations. The inherent back-off cannot absorb all the increased power back-off need. This means that a 3xPC3 architecture requires the use of the dual Tx MPR table.

Proposal 2: If  a UE implements 3Tx PC2 with PC3 chains then the use of dual Tx MPR is required.

# 4	Conclusions

In this contribution, we share our views on the UE RF impact of 3Tx UL MIMO which includes power classes and PA configurations and their implication on the MPR requirements. The following observations are made:

Observation: With the assumption of achieving PC1.5 with 3xPC2 each amplifier can operate at roughly 1.8dB back-off at maximum output power. Comparing the dual Tx for 20dB antenna isolation table with the 4Tx table for 20dB antenna isolation we can observe that the larges difference is 3dB for edge allocations at 10dB antenna isolation. The inherent back-off cannot absorb all the increased power back-off need. This means that a 3xPC2 architecture requires a dedicated 3Tx MPR table.

Proposal 1: Discuss the introduction of a 3Tx table for PC1.5

Proposal 2: If  a UE implements 3Tx PC2 with PC3 chains then the use of dual Tx MPR is required.

# 4	References

- R4-2414310, “WF on UE RF requirements for Rel-19 MIMO enhancement “, Samsung, 3GPP TSG-RAN WG4 Meeting #112

- R4-2500005, “Reply to RAN4 LS on UE RF issues for Rel-19 MIMO Enhancement”, RAN1, 3GPP TSG-RAN WG4 Meeting #114