3GPP TSG-RAN WG4 Meeting #114	R4-2500309

Athens, Greece,  17th February – 21st February 2025

Agenda item:	6.17.1

Source:	Apple

Title:	n68: Introduction of the new band and NS_26 and NS_36

WI/SI:	NR_band_n68-Core

Release:	Rel-19

Document for:	Approval

# 1	Introduction

The WF [1] captures agreements for further simulations  on A-MPR for band n68. It was agreed to re-run simulations with a certain duplex filter characteristic. This contribution provides simulations results including the agreed duplex characteristic.

# 2  Discussion

##### 2.1 Simulation assumptions

The following assumptions and requirements for the simulations are applied:

- Power Class 3

- NR Waveform

- Calibration: 1dB MPR: DFT-s-OFDM QPSK 20MHz, 100RB0

- Carrier Leakage: 28dBc

- Image: 28dBc

- CIM3: 60dBc

- CIM5: 70dBc

- Requirements from NS_26 and NS_36

##### 2.2 Simulation results for NS_26

This section presents the QPSK simulation results for 5MHz, 10MHz and 15MHz channels.

Fig 1: Simulation results for NS_26 at lower band edge with QPSK modulation

To check whether A-MPR is needed for channels placed with a gap to the lower band edge we conducted simulations with a gap of CBW/2. The 5MHz channel and the 10MHz channel do not require additional power back-off. The 15MHz channel required a larger gap of  9.7MHz.

Table 1: RB regions for NS_26

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 5 MHz | 700.5 ≤ FC ≤ 725.5 | ≤0.36 | > 0 | A1 |
| 10 MHz | 703.0 ≤ FC ≤ 723.0 | ≤2.16 | > 0 | A2 |
| 15 MHz | 705.5 ≤ FC ≤ 720.5 | ≤4.5 | > 0 | A2 |

Table 2: A-MPR for NS_26

| Modulation/Waveform |  | A1 | A2 |
| --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | 2.0 | 4.0 |
|  | QPSK | 2.5 | 4.5 |
|  | 16 QAM | 3.0 | 5.0 |
|  | 64 QAM | 4.5 | 5.0 |
|  | 256 QAM | 4.5 | 5.0 |
| CP-OFDM | QPSK | 3.5 | 6.0 |
|  | 16 QAM | 3.5 | 6.0 |
|  | 64 QAM | 3.5 | 6.0 |
|  | 256 QAM | 6.5 | 6.5 |

Fig 2: NS_26 RB allocations regions

Proposal 1: Consider Table 1 and Table 2 when specifying RB regions and A-MPR for NS_26

##### 2.3 Simulation results for NS_36

Fig 3: Simulation results for NS_36 at lower band edge with QPSK modulation

Table 3: RB regions for NS_26

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 5 MHz | 700.5 ≤ FC ≤ 725.5 | ≤1.08 | > 0 | A1 |
|  |  | >1.08 | ≥ 1.8 | A2 |
| 10 MHz | 703.0 ≤ FC ≤ 723.0 | ≤2.16 | >0 | A3 |
|  |  | >2.16 | ≥ 3.96 + (RBstart*12*SCS -2.34MHz) | A4 |
|  |  | >2.16, ≤4.32 | ≥ 1.8 + ceil((RBstart*12*SCS -2.34)/2) < 3.96 + (RBstart*12*SCS -2.34) | A5 |
|  |  | >4.32 | ≥ 2.88 - (RBstart*12*SCS -4.32) | A6 |
| 15 MHz | 705.5 ≤ FC ≤ 720.5 | ≤4.14 | > 0 | A3 |
|  |  | >4.14 | ≥ 6.3 + (RBstart*12*SCS -4.14) | A1 |
|  |  | >4.14, ≤ 7.2 | ≥ 2.34 + ceil((RBstart-4.14)/2) and < 6.3 + (RBstart*12*SCS -4.14) | A5 |
|  |  | > 7.2 | ≥ 3.78 - (RBstart*12*SCS -7.2) | A6 |

Table 4: A-MPR for NS_26

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 | A6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | 8.5 | 4.0 | 13.5 | 7.0 | 5.0 | 5.0 |
|  | QPSK | 9.0 | 4.5 | 14.0 | 7.5 | 5.5 | 5.0 |
|  | 16 QAM | 9.5 | 5.0 | 14.5 | 8.0 | 6.0 | 5.0 |
|  | 64 QAM | 9.5 | 5.0 | 14.5 | 8.0 | 6.0 | 5.0 |
|  | 256 QAM | 9.5 | 5.0 | 14.5 | 8.0 | 6.0 | 5.0 |
| CP-OFDM | QPSK | 10.5 | 6.0 | 15.5 | 10.0 | 8.0 | 5.0 |
|  | 16 QAM | 10.5 | 6.0 | 15.5 | 10.0 | 8.0 | 5.0 |
|  | 64 QAM | 10.5 | 6.0 | 15.5 | 10.0 | 8.0 | 5.0 |
|  | 256 QAM | 10.5 | 6.5 | 15.5 | 10.0 | 8.0 | 5.0 |

Fig 4: NS_36 RB allocations regions

Proposal 2: Consider Table 3 and Table 4 when specifying RB regions and A-MPR for NS_36.

# 4	Conclusions

This contribution provides simulation results for NS_26 and NS_36. This paper makes the following proposals:

Proposal 1: Consider Table 1 and Table 2 when specifying RB regions and A-MPR for NS_26

Proposal 2: Consider Table 3 and Table 4 when specifying RB regions and A-MPR for NS_36.

# 4	References

- R4-2417080, “WF on A-MPR for NR band n68”, Ericsson, 3GPP TSG-RAN WG4 Meeting # 112bis