3GPP RAN TSG-RAN4 Meeting # 114		R4-2500311

Athens, GR, February 17 – 21, 2025

Agenda item:	7.1.1.3.4

Source:	Apple

Title:	On SRS Antenna Switching IL Imbalance Solution for 6Rx

WI/SI:	NR_ENDC_RF_Ph4-Core

Release:	Rel-19

Document for:	Discussion

# 1	Introduction and Scope

In the previous RAN4 meeting (Meeting #113, Orlando, USA, November 18–22, 2024), discussions were held on the requirements for 6Rx for handheld and FWA, which are parts of the new  Release 19 WID on RF enhancements for NR FR1/FR2 and EN-DC [1]. As highlighted in the meeting WF [2], one of the remaining issues to be addressed is the SRS insertion loss imbalance issue. Captured below is a summary of the outcome of the discussion related to this issue taken from the WF of the WID.

# Topic 3: SRS IL imbalance issue ## Sub-topic 3-1: General considerations for SRS IL imbalance issue Issue 3-1-1: Whether to solve SRS IL imbalance issue in Rel-19 Way Forward: RAN4 to further discuss the following options. - Option 1: RAN4 should not continue the discussion on how to solve the SRS IL imbalance issue. - Option 2: Continue to pursue a solution to the SRS IL imbalance issue based on the candidate solution framework with minimal impact to the specification and to indicate the specific impacts to RAN1, RAN2, and RAN4 specifications and performance gain based on the outcome of Issue 3-2-1. Issue 3-1-2: Initial Considerations for SRS IL imbalance issue Way forward: RAN4 to focus on refining the compromised solution framework in Issue 3-2-1. ## Sub-topic 3-2: SRS IL imbalance issue solutions Issue 3-2-1: Candidate solutions for the SRS IL imbalance issue Way forward: RAN4 to further discuss the candidate solution framework below and present proposals concerning the details. Companies are encouraged to provide expected benefit/performance gains and draftCRs so that the impacts to RAN1, RAN2, and RAN4 specifications (for RAN1 and RAN2, a description of the necessary changes is sufficient if draftCRs are not possible) can be further discussed. Candidate solution framework: - UE performs self-compensation of SRS IL up to its configured maximum output power. - FFS on whether condition would introduce any additional RAN4 performance requirement. No RAN1 specification impact. - UE provides assistance to the network on SRS IL compensation (semi-static and/or dynamic), e.g. - Per SRS resource power headroom reporting - Configured maximum output power per SRS resource reporting - SRS insertion loss value reporting (per SRS resource or per UE) - Other - ΔPPowerClass reporting is proposed as a package with per SRS resource power headroom and configured maximum output power per SRS resource reporting. - Others - Details of SRS IL self-compensation and assistance framework are FFS

In this discussion paper, we will share our views on the different sub-topics of the SRS IL imbalance shown above.

# Discussion

  - On whether to solve the SRS IL issue in Rel-19

The SRS insertion loss imbalance issue has been highly debated in RAN4 and RAN1 meetings since the introduction of the 8Rx WID in Rel-18. Over the past RAN4 meetings, several discussions have been held on the issue with no clear solution identified yet, although most companies see the need for a solution.

Based on simulation results from several companies, it was recognized that, if this issue is not addressed, there could be a potential performance degradation in user throughput [4-9]. In RAN4#106 meeting (Athens, Greece, February 27 – March 3, 2023), an LS [3] from RAN4 was sent to RAN1 describing the SRS IL imbalance issue for 8Rx and its potential impact on inaccurate channel estimation at the gNB receiver, which could lead to an incorrect DL CSI estimation. In response to the RAN4 LS, a lot of companies provided simulation results to highlight the potential impacts of IL imbalance on the degradation of overall system performance[4]. It can be observed from those simulation results that, without any reporting or compensation, SRS IL imbalance can result in a non-negligible performance loss for both SU-MIMO and MU-MIMO because it will negatively affect the channel estimation, which will result into  poor DL performance.

Observation #1: Based on simulation results from several companies, it is recognized that, if the SRS IL imbalance issue is not addressed, there could be a potential performance degradation in user throughput.

We therefore support Option 2 of issue 3-1-1 above.

Proposal #1: We support Option 2: RAN4 should continue the discussions on how to solve the SRS insertion loss balance issue and find a compromised solution in Rel-19.

A potential solution for the SRS IL imbalance issue should also be used for 2Rx/4Rx/6Rx/8Rx.

Proposal #2: A potential solution for the SRS IL imbalance issue should also be used for 2Rx/4Rx/6Rx/8Rx.

  - On candidate solutions for SRS IL imbalance issue

Most of the solutions provided so far by numerous companies are all centered around UE power self-compensation and IL imbalance reporting to the NW

    - UE self-compensation

RAN4 identified in Meeting # 110-Bis three cases that determine the effectiveness of applying the compensation [9]:

- Case 1(non near max Tx power): In this scenario, the UE is not power-limited and should be able to compensate for the IL loss imbalance across the SRS ports if the PA headroom is higher than the maximum IL imbalance.

- Case 2 (near max Tx power): For this case, the UE is close to be power-limited so performing a reliable compensation for this case could be a challenge because there could be not enough PA headroom remaining. However, this would depend on UE implementation.

- Case 3 (max Tx power): The UE is power-limited in these conditions and would probably not be do power compensation.  the UE is close to be power-limited so performing a reliable compensation for this case could be a challenge because there could be not enough PA headroom remaining.

As we discussed in one of our previous contributions to this topic[11], the UE should always perform self-compensation on every SRS resource (SRS antenna ports in this case) when it can. This is when it has enough headroom relative to its maximum configured output power to overcome the ΔTRxSRS for each SRS antenna port. No RAN1 specification will be impacted if the UE self-compensates.

Proposal #3: The UE should always perform self-compensation on every SRS resource (SRS antenna port) whenever it is possible to do so ( when it has enough headroom relative to its maximum configured output power to overcome the ΔTRxSRS for each SRS antenna port.

    - UE reporting

UE self-compensation as described in the previous section can work when there is enough headroom to do so. However, it will not work properly in power-limited conditions where the UE may not have enough power to compensate for the imbalances. UE reporting has also been proposed as an alternative solution when the self-compensation does not work. In this case, the UE provides some assistance to the network (either statically or dynamically) on SRS IL imbalance and the network performs the compensation. Several UE performance metrics, as enumerated in  the introduction section above, have been proposed to be used for reporting by different companies: per SRS resource power headroom reporting, the configured maximum output power per SRS resource reporting, the SRS insertion loss value reporting (per SRS resource or per UE) and other metrics.

- By 3GPP definition, Type 3 power headroom is the difference between the nominal UE maximum transmit power and the estimated power for SRS transmission per activated serving sell. The specified tolerance on the configured UE maximum power (Table 6.2.4-1 of TS38.101-1) are shown in Table 2.2.2-1 below. As can be seen, the tolerance on PCMAX can be quite high. This will translate into poor accuracy for the power headroom reporting and the configured maximum output power per SRS resource reporting.

Table 2.2.2-1: PCMAX Tolerance (from TS38.101-1)

- The SRS insertion loss value reporting (per SRS resource or per UE) seems to be the simplest parameter to be used for the NW assistance. This is in line with our previous contribution: In RAN4 Meeting #112 (Maastricht, Netherlands, August 19-23, 2024), we proposed a hybrid solution based on both UE self-compensation and reporting where the UE always compensates when it is capable to do so in a non-power limited situations and does some reporting to the network when the power at an SRS port is above certain threshold [11].

Proposal #4: For UE assistance to the network on SRS IL compensation (semi-static and/or dynamic), we prefer that the actual insertion loss values (per SRS resource or per UE) to be reported to the network.

The ∆TRxSRS per SRS resource reporting may be either static or dynamic.

# 3	Conclusions

Observation #1: Based on simulation results from several companies, it is recognized that, if the SRS IL imbalance issue is not addressed, there could be a potential performance degradation in user throughput.

Proposal #1: We support Option 2: RAN4 should continue the discussions on how to solve the SRS insertion loss balance issue and find a compromised solution in Rel-19.

Proposal #2: A potential solution for the SRS IL imbalance issue should also be used for 2Rx/4Rx/6Rx/8Rx

Proposal #3: The UE should always perform self-compensation on every SRS resource (SRS antenna port) whenever it is possible to do so ( when it has enough headroom relative to its maximum configured output power to overcome the ΔTRxSRS for each SRS antenna port.

Proposal #4: For UE assistance to the network on SRS IL compensation (semi-static and/or dynamic), we prefer that the actual insertion loss values (per SRS resource or per UE) to be reported to the network.

# 4	References

RP-240828 “New WID: UE RF enhancements for FR1/FR2 and EN-DC , Phase 4”, by RAN4 chair (Huawei),  3GPP TSG-RAN Meeting #103, Maastricht, Netherlands, March 18-21, 2024

R4-2420376, “WF on 6Rx requirements”, by AT&T, 3GPP TSG-RAN WG4 Meeting #113, Orlando, USA, November 18-22, 2024

R4-2303519, “LS on the UE SRS IL imbalance issue”, 3GPP TSG-RAN WG4 Meeting #106, Athens, Greece, February 27 – March 3, 2023

R1-2306200, “FL summary on UE SRS IL imbalance issue”, 3GPP TSG RAN WG1 #112bis-e, Incheon, Korea, May 22nd – May 26th, 2023

R1-2304643, “Discussion on the UE SRS IL imbalance issue”, by Huawei, HiSilicon, 3GPP TSG-RAN WG1 Meeting #113,  Incheon, Korea, 22 – 26 May 2023

R1-2304433, “On SRS Imbalance”, by InterDigital, Inc., 3GPP TSG-RAN WG1 Meeting #113,  Incheon, Korea, 22 – 26 May 2023

R1-2305646, “Evaluation of SRS antenna switching with insertion loss”, by MediaTek, 3GPP TSG-RAN WG1 Meeting #113,  Incheon, Korea, 22 – 26 May 2023

R4-2300696, “8RX UE RF requirements”, by Qualcomm Inc., 3GPP TSG-RAN WG4#106 Athens, Greece, February 27th ‒ March 3rd, 2023

R1-2305838, “Discussion the UE SRS IL imbalance issue”, by vivo, 3GPP TSG-RAN WG1 Meeting #113,  Incheon, Korea, 22 – 26 May 2023

R4-2406585, “WF on requirements for 6Rx”, by AT&T, 3GPP TSG-RAN WG4 Meeting #110-bis,  Changsha, China, 15th Apr 2024 - 19th Apr 2024

R4-2411151, “On SRS IL imbalance issue for 6Rx”, by Apple, 3GPP RAN TSG-RAN4 Meeting # 112, Maastricht, NL, August 19 – 23, 2024

R4-2411883, “Further discussion on 6Rx Reference sensitivity requirements”, by ZTE Corporation, Sanechips,  3GPP TSG-RAN WG4 Meeting #110-bis,  Changsha, China, 15th Apr 2024 - 19th Apr 2024