3GPP RAN WG4 Meeting #114		R4-2500317

Athens, Greece, February 17th – 21st, 2025

Agenda item:	7.1.1.1.3

Source:	Apple

Title:	On Rel-19 HPUE inter-band UL CA DC configurations

WI/SI:	NR_ENDC_RF_Ph4-Core

Release:	Rel-19

Document for:	Approval

# 1	Introduction

The Rel-19 RAN4 non-spectrum WID/SID package has been approved in RAN #103 meeting. In consideration that the uplink coverage is crucial for 5G and 5G-adavanced roll-out and the continued strong interests from operators in higher UE maximum output power such as PC1.5 for their band combinations, one of the key objectives under the new Rel-19 “UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4” work item [1] is to specify the generic requirements of HPUE for NR uplink CA in FR1 and EN-DC with NR FR1 bands where the features include,

    - PC1.5 for intra-band contiguous UL CA with and without UL MIMO

    - PC1.5 Intra-band non-contiguous UL CA without UL MIMO

    - PC1.5 2-band inter-band UL CA with 2Tx or 3Tx for handheld UE and FWA

    - PC2 and PC1.5 2-band inter-band EN-DC with 2Tx or 3Tx for handheld UE and FWA UE

    - Increasing UE transmission power limit up to the sum of maximum output power per band for NR inter-band UL CA and EN-DC HPUE

In last RAN4 meeting, companies had provided their further views on the respective features above where the additional agreements were captured in the approved “WF on HPUE for CA in TN” [2]. For HPUE inter-band UL CA/EN-DC, upon the agreement to work on the feasible unified Equations/LUT/Simplified guidelines for HPUE MSD as normative requirements as captured in the approved “WF on MSD rule” [3], RAN4 also saw a need to create and maintain a configuration table listing all configurations that can be considered in Rel-19 for information purpose and to identify what configurations may require new MSD framework to facilitate the new “HPUE MSD rule” development.

In this contribution, we summarize the lists of new HPUE inter-band UL CA/DC configurations which can be considered in Rel-19 including:

    - PC2 inter-band UL CA with 2Tx or 3Tx (not explicitly included in the WID objectives)

    - PC2 inter-band EN-DC with 2Tx or 3Tx

    - PC1.5 inter-band UL CA with 2Tx or 3Tx

    - PC1.5 inter-band EN-DC with 2Tx or 3Tx

# Discussion

## 2.1	PC2 inter-band UL CA with 2Tx or 3Tx

Table 2.1-1 summarizes the potential new UL configurations for PC2 inter-band UL CA with 2Tx or 3Tx which can be considered in Rel-19. Some of the configurations may require new MSD framework as indicated in the table.

| 1st band | 2nd band | 2Tx or 3Tx | New MSD framework |
| --- | --- | --- | --- |
| PC3 TDD 1Tx | PC2 FDD 1Tx | 2Tx | No |
| PC3 TDD 1Tx | PC2 FDD 2Tx | 3Tx | Yes |
| PC3 FDD 1Tx | PC2 FDD 1Tx | 2Tx | No |
| PC3 FDD 1Tx | PC2 FDD 2Tx | 3Tx | Yes |

Table 2.1-1 Potential new UL configurations for PC2 inter-band UL CA with 2Tx or 3Tx

## 2.2	PC2 inter-band EN-DC with 2Tx or 3Tx

Table 2.1-2 summarizes the potential new UL configurations for PC2 inter-band EN-DC with 2Tx or 3Tx which can be considered in Rel-19. Some of the configurations may require new MSD framework as indicated in the table.

| E-UTRA band | NR band | 2Tx or 3Tx | New MSD framework |
| --- | --- | --- | --- |
| PC3 TDD 1Tx | PC2 FDD 1Tx | 2Tx | No |
| PC3 TDD 1Tx | PC2 FDD 2Tx | 3Tx | Yes |
| PC3 FDD 1Tx | PC2 FDD 1Tx | 2Tx | No |
| PC3 FDD 1Tx | PC2 FDD 2Tx | 3Tx | Yes |
| PC2 FDD 1Tx | PC3 TDD 1Tx | 2Tx | No |
| PC2 FDD 1Tx | PC3 FDD 1Tx | 2Tx | No |
| NOTE: PC2 FDD band for E-UTRA has only been specified for Band 14. |  |  |  |

Table 2.1-2 Potential new UL configurations for PC2 inter-band EN-DC with 2Tx or 3Tx

## 2.3	PC1.5 inter-band UL CA with 2Tx or 3Tx

Table 2.1-3 summarizes the potential new UL configurations for PC1.5 inter-band UL CA with 2Tx or 3Tx which can be considered in Rel-19. Some of the configurations may require new MSD framework as indicated in the table.

| 1st band | 2nd band | 2Tx or 3Tx | New MSD framework |
| --- | --- | --- | --- |
| PC2 TDD 1Tx | PC2 TDD 1Tx | 2Tx | No |
| PC2 TDD 1Tx | PC2 TDD 2Tx | 3Tx | No |
| PC3 TDD 1Tx | PC1.5 TDD 2Tx | 3Tx | No |
| PC2 TDD 1Tx | PC1.5 TDD 2Tx | 3Tx | No |
| PC2 TDD 1Tx | PC2 FDD 1Tx | 2Tx | Yes |
| PC2 TDD 1Tx | PC2 FDD 2Tx | 3Tx | Yes |
| PC2 FDD 1Tx | PC2 TDD 2Tx | 3Tx | Yes |
| PC2 FDD 1Tx | PC2 FDD 1Tx | 2Tx | Yes |
| PC2 FDD 1Tx | PC2 FDD 2Tx | 3Tx | Yes |
| PC2 FDD 1Tx | PC1.5 TDD 2Tx | 3Tx | Yes |
| PC5 TDD 1Tx | PC1.5 TDD 2Tx | 3Tx | No |

Table 2.1-3 Potential new UL configurations for PC1.5 inter-band UL CA with 2Tx or 3Tx

## 2.4	PC1.5 inter-band EN-DC with 2Tx or 3Tx

Table 2.1-4 summarizes the potential new UL configurations for PC1.5 inter-band EN-DC with 2Tx or 3Tx which can be considered in Rel-19. Some of the configurations may require new MSD framework as indicated in the table.

| E-UTRA band | NR band | 2Tx or 3Tx | New MSD framework |
| --- | --- | --- | --- |
| PC3 TDD 1Tx | PC1.5 TDD 2Tx | 3Tx | No |
| PC3 FDD 1Tx | PC1.5 TDD 2Tx | 3Tx | No |
| PC2 TDD 1Tx | PC2 TDD 1Tx | 2Tx | No |
| PC2 TDD 1Tx | PC2 TDD 2Tx | 3Tx | No |
| PC2 TDD 1Tx | PC2 FDD 1Tx | 2Tx | Yes |
| PC2 TDD 1Tx | PC2 FDD 2Tx | 3Tx | Yes |
| PC2 TDD 1Tx | PC1.5 TDD 2Tx | 3Tx | No |
| PC2 FDD 1Tx | PC2 TDD 1Tx | 2Tx | Yes |
| PC2 FDD 1Tx | PC2 TDD 2Tx | 3Tx | Yes |
| PC2 FDD 1Tx | PC2 FDD 1Tx | 2Tx | Yes |
| PC2 FDD 1Tx | PC2 FDD 2Tx | 3Tx | Yes |
| PC2 FDD 1Tx | PC1.5 TDD 2Tx | 3Tx | Yes |
| NOTE: PC2 FDD band for E-UTRA has only been specified for Band 14. |  |  |  |

Table 2.1-4 Potential new UL configurations for PC1.5 EN-DC with 2Tx or 3Tx

Proposal: RAN4 to consider the lists of new HPUE inter-band UL CA/DC configurations as summarized in Table 2.1-1, Table 2.1-2, Table 2.1-3, and Table 2.1-4 to develop the corresponding specifications in Rel-19.

# 3	Conclusion

In this contribution, we summarize the lists of new HPUE inter-band UL CA/DC configurations which can be considered in Rel-19.

Proposal: RAN4 to consider the lists of new HPUE inter-band UL CA/DC configurations as summarized in Table 2.1-1, Table 2.1-2, Table 2.1-3, and Table 2.1-4 to develop the corresponding specifications in Rel-19.

# 4	References

RP-240828 “New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, RAN4 Chair (Huawei), 3GPP TSG-RAN Meeting #103, Maastricht, Netherlands, March 18th – 21st, 2024

R4-2420329 “WF on HPUE for CA in TN”, Samsung, 3GPP TSG RAN WG4 Meeting #113, Orlando, USA, November 18th – 22nd, 2024

R4-2420380 “WF on MSD rule”, Skyworks et al, 3GPP TSG RAN WG4 Meeting #113, Orlando, USA, November 18th – 22nd, 2024