3GPP RAN WG4 Meeting #114		R4-2500318

Athens, Greece, February 17th – 21st, 2025

Agenda item:	7.7.1

Source:	Apple

Title:	On PC2 RedCap n1 and n3 1Rx REFSENS

WI/SI:	NR_PC2_RedCap_UE-Core

Release:	Rel-19

Document for:	Approval

# 1	Introduction

A new work item on NR power class 2 (PC2) RedCap UE was approved in RAN #104 meeting [1]. The main objective of the core part is to apply the PC2 RF requirements for non-RedCap UE to RedCap UE based on 1Tx architecture with 26 dBm PA and the following additional clarifications:

- The PC2 RF requirements include: UE maximum output power, Tx power tolerance, MPR, A-MPR and RSD requirements if needed.

- For TDD bands, PC2 RF requirements for non-RedCap UE can be re-used for PC2 RedCap UE in a band agnostic way.

- Check at RAN #106 on whether to define PC2 RF requirements for FDD bands and the applicable form factors for FDD bands (if defined).

In RAN #106 meeting, it was approved to specify RedCap PC2 RF requirements for FDD bands with n1 and n3 as two example bands where the revised objective was captured in the “Revised WID on NR power class 2 RedCap UE” [2]. Unlike TDD bands where PC2 RF requirements for non-RedCap UE can be re-used for PC2 RedCap UE in a band agnostic way, for PC2 FDD bands, the 1Rx REFSENS (or RSD which stands for Reference Sensitivity Degradation) needs to be exclusively specified for RedCap UE on a per-band basis as the 1Rx REFSENS requirement has not been specified for non-RedCap NR UE. In this contribution, we share our PC2 1Rx RSD analysis for n1 and n3 and propose the corresponding requirements for consideration.

# Discussion

The PC2 RSD requirements have already been specified for several FDD bands for non-RedCap UE, including n1 and n3 [3]. The requirements can be directly reused for RedCap UE supporting 2Rx on the same bands. However, the 1Rx requirement has not been specified for non-RedCap UE. Therefore, the PC2 1Rx RSD requirements have to be developed and exclusively specified for RedCap UE FDD bands supporting 1Rx.

Observation 1: The PC2 RSD requirements with 1Tx already specified for FDD bands for non-RedCap UE can be directly reused for RedCap UE supporting 2Rx on the same bands.

Observation 2: The PC2 1Rx RSD requirements have to be developed and exclusively specified for RedCap UE FDD bands supporting 1Rx as they have not been specified for non-RedCap UE.

In this contribution, we share our PC2 1Rx RSD analysis for n1 and n3 with 1Tx and propose the corresponding requirements for consideration. Our analysis was carried out based on the existing PC2 2Rx RSD requirements with 1Tx via reverse engineering, with the assumption that the Tx noise in primary Rx (P-Rx) path is effectively 6dB higher than the diversity Rx (D-Rx) path, where the Tx to Rx band isolation is assumed to be 50dB at Rx port and 46dB at the antenna port, and the main and diversity antenna isolation is assumed to be 10 dB, as conceptually illustrated in Figure 2-1.

Figure 2-1 Tx band to Rx band isolation assumption for FDD bands

For n1, since the duplex spacing is relatively wide at 190 MHz, it is expected that the PC2 Tx noise into the Rx band after the duplexer isolation is negligible for both primary Rx and diversity Rx paths as compared to Rx intrinsic noise floor. As a result, the PC2 RSD for both 2Rx and 1Rx can be reasonable assumed to be 0 dB for all channel bandwidths up to 20 MHz.

Observation 3: For n1, it is expected that the PC2 Tx noise into the Rx band after the duplexer isolation is negligible for both primary Rx and diversity Rx paths as compared to Rx intrinsic noise floor.

Observation 4: The n1 PC2 RSD for both 2Rx and 1Rx can be reasonable assumed to be 0 dB for all channel bandwidths up to 20 MHz.

For n3, the PC2 2Rx RSD with 1Tx has been specified as 0.5 dB for all channel bandwidths up to 20 MHz. Based on the above Tx band to Rx band isolation assumption, we can reverse-engineer the PC2 Tx noise into both primary Rx path and diversity Rx path and then derive the 1Rx RSD from the primary Rx path sensitivity degradation, as shown in Table 2-1.

|  | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| --- | --- | --- | --- | --- |
| PC3 2Rx REFSENS (dBm) | -97.0 | -93.8 | -92.0 | -90.8 |
| PC2 1Tx 2Rx RSD (dB) | 0.5 | 0.5 | 0.5 | 0.5 |
| PC2 1Tx 2Rx REFSENS (dBm) | -96.5 | -93.3 | -91.5 | -90.3 |
| PC3 MRC noise (dBm) | -96.0 | -92.8 | -91.0 | -89.8 |
| PC3 primary Rx noise (dBm) | -93.5 | -89.8 | -88.0 | -86.8 |
| PC3 diversity Rx noise (dBm) | -92.4 | -89.8 | -88.0 | -86.8 |
| PC2 MRC noise (dBm) (REFSENS) | -95.5 | -92.3 | -90.5 | -89.3 |
| PC2 primary Tx noise (dBm) | -101.0 | -97.0 | -95.0 | -94.0 |
| PC2 diversity Tx noise (dBm) | -107.0 | -103.0 | -101.0 | -100.0 |
| PC2 primary Rx + Tx noise (dBm) | -92.8 | -89.0 | -87.2 | -86.0 |
| PC2 diversity Rx + Tx noise (dBm) | -92.3 | -89.6 | -87.8 | -86.6 |
| PC2 MRC noise (dBm) (derived) | -95.5 | -92.3 | -90.5 | -89.3 |
| PC2 1Rx RSD (dB) | 0.7 | 0.8 | 0.8 | 0.8 |

Table 2-1 n3 PC2 1Rx RSD analysis based on PC2 2Rx RSD reverse engineering

Based on the above observations and analysis, we propose the PC2 1Rx RSD for n1 and n3 as in Table 2-2.

| PC2 1Rx RSD (dB) | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| --- | --- | --- | --- | --- |
| n1 | 0 | 0 | 0 | 0 |
| n3 | 0.7 | 0.8 | 0.8 | 0.8 |

Table 2-2 Proposed PC2 1Rx RSD for n1 and n3

Proposal 1: The PC2 RSD requirements with 1Tx already specified for FDD bands for non-RedCap UE are reused for RedCap UE supporting 2Rx on the same bands.

Proposal 2: The PC2 1Rx RSD requirements for n1 and n3 are proposed as in Table 2-2.

# 3	Conclusion

In this contribution, we share our PC2 1Rx RSD analysis for n1 and n3 and propose the corresponding requirements for consideration.

Observation 1: The PC2 RSD requirements with 1Tx already specified for FDD bands for non-RedCap UE can be directly reused for RedCap UE supporting 2Rx on the same bands.

Observation 2: The PC2 1Rx RSD requirements have to be developed and exclusively specified for RedCap UE FDD bands supporting 1Rx as they have not been specified for non-RedCap UE.

Observation 3: For n1, it is expected that the PC2 Tx noise into the Rx band after the duplexer isolation is negligible for both primary Rx and diversity Rx paths as compared to Rx intrinsic noise floor.

Observation 4: The n1 PC2 RSD for both 2Rx and 1Rx can be reasonable assumed to be 0 dB for all channel bandwidths up to 20 MHz.

Proposal 1: The PC2 RSD requirements with 1Tx already specified for FDD bands for non-RedCap UE are reused for RedCap UE supporting 2Rx on the same bands.

Proposal 2: The PC2 1Rx RSD requirements for n1 and n3 are proposed as in the table below.

| PC2 1Rx RSD (dB) | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| --- | --- | --- | --- | --- |
| n1 | 0 | 0 | 0 | 0 |
| n3 | 0.7 | 0.8 | 0.8 | 0.8 |

# 4	References

RP-241665 “New WID on NR power class 2 RedCap (Reduced Capability) UE in FR1”, China Telecom, MediaTek, 3GPP TSG-RAN Meeting #104, Shanghai, China, June 17th – 20th, 2024

RP-243267 “Revised WID on NR power class 2 RedCap UE”, China Telecom, 3GPP TSG RAN Meeting #106, Madrid, Spain, December 9th – 12th, 2024

3GPP TS 38.101-1 V18.8.0 (2024-12)