3GPP TSG-RAN4 Meeting #114	R4-2500320

# Athens, Greece, February 17-21, 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-5 | CR | 0146 | rev | - | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_NTN_solutions) CR to 38.101-5 to clarify applicability of phase continuity requirements in R17 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions-Core |  |  |  |  |  | Date: |  |  | 2025-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | When approving the work scope for the Rel-17 NR NTN work item, RAN judiciously selected which Core NR features are part of the NTN scope and which are not. The work item on Rel-17 NR NTN does not include any objectives related to coverage enhancement [RP-213691]. The Rel-18 work item on NR NTN enhancements [RP-213690], however, does include an objective on coverage enhancements (which also included a study phase). It is further noted that after several meetings the RAN1 discussion of the feature identified a differentiation between GEO (or GSO) scenarios and LEO (or NGSO) scenarios [RP-222019]: Agreement For NR-NTN coverage enhancement, RAN1 concludes that coverage enhancements specifically for GEO and MEO are de-prioritized in Rel-18. • Potential enhancements for LEO can also apply to GEO and MEO When RAN4 started discussing the Rel-17 NR NTN work item (during RAN4 #98), no agenda or discussion related to NR NTN DMRS bundling was allowed, as this was being developed as a Rel-17 feature for the terrestrial NR. The first agreement related to NR NTN DMRS bundling is found in the Rel-18 NR NTN work item agenda from RAN4 #108 [R4-2313864]. The eventual RAN4 agreement on NTN UE phase continuity requirements was reached in RAN4 #109 [R4-2321974], where GSO/NGSO scenario was agreed to map to different requirements. Based on this analysis, it is clear that the DMRS bundling feature does not apply to NR NTN UEs in Rel-17 and can only apply to NR NTN UEs starting from Rel-18. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To exclude phase continuity requirements for NTN UE in Rel-17. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Tx EVM clause in TS 38.101-5 does not correctly specify the applicability of DMRS bundling requirements to the release which is previous to the release in which the feature was introduced to NR NTN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-5 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | This CR is related to a previous proposal in R4-2401784 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<Start of Change>>

### 6.4.2	Transmit modulation quality

The requirements for transmit modulation quality defined in 3GPP TS 38.101-1 [5] clause 6.4.2 shall apply for NTN satellite UE.  In this version of the specification, requirements in Clause 6.4.2.5 are not applicable for NTN satellite UEs.

## <<End of Change>>