3GPP TSG RAN WG4 Meeting#114                  	R4-2500332

Athens, GR, 17th - 21nd February 2025

Title:	RRM requirements on UE-initiated/event-driven beam management

Source:	China Telecom

Agenda item:			7.22.3.1

Document for:	Discussion

- Introduction

In the RAN4 #113 meeting, the RRM requirements for NR_MIMO_Ph5_Part1 were discussed and WF [1] has been approved. In this paper, we present our views on some open issues.

- Discussion

Issue 1-3: Whether RAN4 to specify requirements for case 1 or cover both case1 and case2?

The following agreement and options are captured in the WF:

Case 1: no time window based

Case 2: with time window based, M is configurable

Agreement:

It is RAN4 common understanding that no additional filtering applies for case 2 compared to case 1, if RAN4 will define requirement for case 2.

- Proposals

  - Option 1: (Qualcomm, Apple, vivo, Ericsson, MediaTek)

    - Only define event triggered measurement delay based on without time window

  - Option 2: (Nokia, Xiaomi, Samsung, CMCC, ZTE, HW)

    - RAN4 to specify requirements to cover both cases of no time window based and time window based

In option 2, it specifies requirement for time window based case as well. Although the requirement for time window based case is more like functionality verification, it is preferred to introduce the requirement given it is a useful function to avoid ping-pong.

Regarding the test case design and test applicability rule for UE supporting both case 1 and case 2, we can further discuss in the performance part.

Proposal 1: RAN4 to specify requirements to cover both cases of no time window based and time window based.

Issue 1-4: Reporting delay requirements

The following options are captured in the WF:

- Option 1: (Qualcomm)

  - Tgap + TL1-RSRP-measure; TL1-RSRP-measure in Issue 1-2; The measurement reporting delay excludes a delay which is caused by no UL resources being available for UE to send the measurement report on; In test cases, set Tgap = 0;

- Option 2: (Nokia)

  - The maximum delay requirement for the first PUCCH channel is determined in relation to the reception time instant of the RS that triggered the UEIBM event.

- Option 3: (Xiaomi)

  - If multiple beams will be measured, total measurement time will be maximum measurement period between multiple beams.

- Option 4: (Samsung)

  - Need further RAN1 progress for RSs relationship between current beam and new beams.

- Option 5: (CMCC)

  - The event triggered measurement reporting delay shall be less than TL1-RSRP_Measurement_Period or TL1-RSRP_Measurement_Period + TSSB_time_index. The measurement reporting delay excludes a delay which is caused by no UL resources being available for UE to send the measurement report on

- Option 6: (Apple)

  - The total reporting delay is Tfirst-RS-NB + TL1-RSRP-NB + TPUCCH, Add TPucch in reporting delay

- Option 7: ( MediaTek)

  - The total reporting delay is TL1-RSRP_measure + Tfirst UL channel, TL1-RSRP_measure is base on SSB or CSI-RS

  - Event evaluation period for a SSB or CSI-RS is same as their respective measurement period

- Option 8: (Ericsson)

  - Event triggered measurement reporting delay for SSB is TL1-RSRP_event triggered_measurement_Period_SSB (ms) + TTime to first UL channel and event triggered measurement delay for CSI-RS is TL1-RSRP_event triggered_measurement_Period_CSI-RS (ms) + TTime to first UL channel

  - RAN4 to discuss components and criteria for defining TTime to first UL channel.

To our understanding option 1, 5, 6, 7, 8 are in general similar, which is also in line with our view. That is, event triggered measurement reporting delay is TL1-RSRP_measure_period + Tfirst_PUCCH, TL1-RSRP_measure_period is based on SSB or CSI-RS, and Tfirst_PUCCH is the uncertainty time to UL resource for the 1st PUCCH.

Proposal 2: Event triggered measurement reporting delay is TL1-RSRP_measure_period + Tfirst_PUCCH, TL1-RSRP_measure_period is based on SSB or CSI-RS, and Tfirst_PUCCH is the uncertainty time to UL resource for the 1st PUCCH.

Issue 1-8: Impact of beam on known/unknown conditions for TCI states for TCI states switching based on UEIBM

The following options are captured in the WF:

- Option 1: (Qualcomm, Nokia, Samsung, Apple, Huawei)

  - Reuse known/unknown conditions for Rel-17 unified TCI states when defining requirements for TCI-switching based on UEIBM

- Option 2: (vivo)

  - Regarding the impact to know conditions of TCI state activation from event-triggered L1 reporting, RAN4 waits further RAN1/2 conclusions.

- Option 3: (ZTE)

  - Need to define the condition of known cell for UEIBM. The definition in ICBM can be starting point.

We also support option 1 to reuse known/unknown conditions for Rel-17 unified TCI states when defining requirements for TCI-switching based on UEIBM.

Proposal 3: Reuse known/unknown conditions for Rel-17 unified TCI states when defining requirements for TCI-switching based on UEIBM.

Issue 1-9: Whether to specify new unified TCI switching delay requirements?

The following options are captured in the WF:

- Option 1: (Qualcomm, Nokia, Samsung (For known conditions for TCI states), Apple, ZTE, Ericsson)

  - The existing requirements for TCI state switch delay should be reused for UE-initiated / event-triggered beam management.

- Option 2: (Samsung)

  - No unknown case is applied if the TCI states switching is based on UEIBM.

- Option 3: (ZTE)

  - Need to decide whether to define L1-RSRP requirements for unknown cell

We support option 1 that the existing requirements for TCI state switch delay should be reused for UE-initiated / event-triggered beam management, and it is not necessary to define the new TCI state switch delay requirement.

Proposal 4: The existing requirements for TCI state switch delay should be reused for UE-initiated / event-triggered beam management.

Issue 1-10: Whether to enhance beam sweeping in event triggered L1 beam reporting?

The following options are captured in the WF:

- Option 1: (Nokia)

  - Specify RAN4 requirements for a UE supporting Rel-18 multi-Rx fast beam sweeping and configured with UEIBM reporting.

- Option 2: (Xiaomi, Samsung, Apple, Ericsson, MediaTek)

  - Reuse current beam sweeping factor for UE/event triggered beam reporting

Fast beam sweeping with a smaller beam sweeping factor of 2, 4, or 6 in Rel-18 is applied in single-cell multi-TRP scenario. Considering the single TRP scenario in the Rel-19 MIMO WI, we are fine to not specify the fast beam sweeping factor for FR2.

Proposal 5: Reuse current beam sweeping factor for UE/event triggered beam reporting.

Issue 1-11: Capabilities for Support of Event Triggering and Reporting Criteria?

The following options are captured in the WF:

- Option 1: (Nokia)

  - RAN4 to define UE capabilities in the context of UE initiated beam reporting towards the later part of the work item.

- Option 2: (Apple)

  - Capabilities shall be defined by RAN1, to be discussed later if any RAN4 specific UE capabilities need to be added.

- Option 3: (Ericsson)

  - Define UE capability for total number of events supported for UE-initiated beam management across single and multiple TRP.

- Option 4: (MediaTek)

  - Define a capability for supported number of UE-initiated/event-driven beam reporting for intra-frequency.

Similar issue has been discussed in R19 mobility WI. The point is whether to define L1 measurement capability in TS 38.133 in RAN4 or just capture it in RAN1 feature. For this issue, we prefer to align the agreement on whether to define a capability in RAN4 TS 38.133 for R19 MIMO WI and mobility WI.

Proposal 6: Align the agreement on whether to define the capability for support of L1 event triggering and reporting in TS 38.133 for R19 MIMO WI and R19 mobility WI.

Issue 1-13: Whether to define new accuracy requirements by Enhancement for UE-initiated/event-driven beam management?

The following options are captured in the WF:

- Option 1: (Qualcomm, Nokia, Xiaomi, Apple, MediaTek)

  - The existing accuracy requirements for L1 measurements can be applied. Not to define new accuracy requirements.

- Option 2: (Samsung)

  - Focus on core requirements firstly, FFS on accuracy requirements.

- Option 3: (CMCC)

  - For SSB based beam report, Rel-18 LTM accuracy requirements can be used as baseline.

  - For CSI-RS based beam report, FFS whether to use legacy accuracy requirements which only cover serving cell, or to define new accuracy requirements taking neighbour cell measurement into account.

- Option 4: (ZTE)

  - Discuss the accuracy for the two trigger-event solutions respectively.

    - For the basic trigger-event detection, tight the existing L1-RSRP accuracy requirement;

    - For the optional trigger-event detection, resue the existing L1-RSRP accuracy requirement.

As baseline, the existing accuracy requirements for L1 measurements can be applied, and the measurement period delay requirement can be defined under the assumption of reusing existing accuracy requirements.

Proposal 7: The existing accuracy requirements for L1 measurements can be applied. Not to define new accuracy requirements.

- Summary

In this paper, we provide our views on RRM requirements for UE-initiated/event-driven beam management. From this discussion we have the following proposals:

Proposal 1: RAN4 to specify requirements to cover both cases of no time window based and time window based.

Proposal 2: Event triggered measurement reporting delay is TL1-RSRP_measure_period + Tfirst_PUCCH, TL1-RSRP_measure_period is based on SSB or CSI-RS, and Tfirst_PUCCH is the uncertainty time to UL resource for the 1st PUCCH.

Proposal 3: Reuse known/unknown conditions for Rel-17 unified TCI states when defining requirements for TCI-switching based on UEIBM.

Proposal 4: The existing requirements for TCI state switch delay should be reused for UE-initiated / event-triggered beam management.

Proposal 5: Reuse current beam sweeping factor for UE/event triggered beam reporting.

Proposal 6: Align the agreement on whether to define the capability for support of L1 event triggering and reporting in TS 38.133 for R19 MIMO WI and R19 mobility WI.

Proposal 7: The existing accuracy requirements for L1 measurements can be applied. Not to define new accuracy requirements.

- References

[1] R4-2420096, WF on RRM requirements for NR_MIMO_Ph5_Part1, Samsung