3GPP TSG RAN WG4 Meeting#114                  	R4-2500333

Athens, GR, 17th - 21nd February 2025

Title:	RRM requirements on XR enhancements

Source:	China Telecom

Agenda item:			7.28.2

Document for:	Discussion

- Introduction

In the RAN4 #113 meeting, the RRM requirements for XR enhancements were discussed and WF [1] has been approved. In this paper, we present our views on some open issues.

- Discussion

2.1 Discussion on scenarios for measurement skipping

Issue 1-3: L3 Measurements without gaps with scheduling restrictions

The following options are summarized in the WF:

- Option 1: Skipping of measurements without gaps with scheduling restrictions are supported

  - Option 1a: Support skipping of measurements without gaps

  - Option 1b: Support skipping of measurements without gaps for 100MHz CBW band

- Option 2: Skipping of measurements without gaps with scheduling restrictions are deprioritized or not supported

  - Option 2a: Measurement skipping does not apply for measurements without gaps

  - Option 2b: Deprioritize Tx/Rx in L3 measurement without gap with scheduling restriction.

For L3 measurements without gaps, we can understand that it is typically used for intra-frequency measurement. Meanwhile, for L3 measurements without gaps with scheduling restrictions, it only applies for the following three scenarios as companies discussed in the previous meeting:

- Different numerologies for data and SSB in FR1

- Not support simultaneous Tx and Rx for some band combinations in FR1

- Rx beam sweeping in FR2

For the first one on different numerologies for data and SSB in FR1, it is not a typical deployment scenario, and thus it is not motivated to consider this scenario in the first release of XR measurement skipping.

For the second one, based on TS 38.101-1, only for the band combinations with small frequency separation on different bands/carriers, it is challenging to support simultaneous Tx and Rx. Simultaneous Tx and Rx can be supported for most of the band combinations. In addition, this scheduling restriction only applies for the case when async TDD patterns are configured on different bands/carriers in the network.

For the third one, the scheduling restriction is on symbol level, and it brings limited performance improvement for XR traffic if the restriction is cancelled. Additionally, it will increase the measurement delay given that the FR2 measurement delay is already long.

In summary, the scenarios for L3 measurements without gaps with scheduling restrictions are limited, and the gain by cancelling the restriction is not obvious for XR transmission. To keep reasonable RAN4 workload, we prefer option 2a that “Measurement skipping does not apply for measurements without gaps”.

Observation 1: The scenarios for L3 measurements without gaps with scheduling restrictions are limited, and the gain by cancelling the restriction is not obvious for XR transmission.

Proposal 1: To keep RAN4 workload reasonable, measurement skipping does not apply for L3 measurements without gaps with scheduling restrictions.

Issue 1-4: L1 measurements with scheduling restrictions and procedures

The following options are summarized in the WF:

- Option 1: Support skipping of L1 measurements and procedures.

  - Option 1a: Support skipping of L1 measurements and procedures such as RLM, BFD, L1-RSRP etc

  - Option 1b: it is beneficial and feasible to support data Tx/Rx in occasions of scheduling restriction due to RLM and BFD.

  - Option 1c: At least for 100MHz CBW band, support skipping of L1 measurements and procedures such as RLM, BFD, L1-RSRP etc

- Option 2: Not to support or deprioritize skipping of L1 measurements and procedures.

  - Option 2a: Measurement skipping does not apply for L1 measurements and procedures

  - Option 2b: RAN4 should keep the scheduling restriction impact on XR traffic FFS for now until there are sufficient progress in MG cancellation impact.

For L1 measurements with scheduling restrictions and procedures, we also prefer option 2a. The scenario is limited, and the gain for XR transmission by cancelling these symbol level restrictions is not obvious.

Proposal 2: Measurement skipping does not apply for L1 measurements and procedures.

Issue 1-5: L1/L2 triggered mobility measurements and procedures

The following options are summarized in the WF:

- Option 1: At least for 100MHz CBW band, support skipping of L1 measurements and procedures for LTM within and outside gap

- Option 2: measurement skipping do not apply for L1 measurements and procedures for LTM

- Option 3: Only L1 measurements for LTM within gaps are supported for gap skipping.

- Option 4: For L1 measurement for mobility, apply same decision as normal L1 measurement.

To simply the discussion, we support option 4 to apply decision as normal L1 measurement.

Proposal 3: For L1 measurement for mobility, apply same decision as normal L1 measurement.

2.2 Discussion on XR skipping requirements impact

Issue 3-1: How to capture impact of measurement skipping

The following agreements and options are captured in the WF:

<Agreement>:

- Reuse existing measurement accuracy requirements.

- RAN4 shall keep the number of samples for a frequency layer for L3 measurements unimpacted due to gap skipping

  - For the delay requirements, FFS whether and how to extend the delay

<Way forward>:

Proposals related to measurement delay requirements:

  - Option 1: Proposals without distinction of XR and legacy requirements

    - Option 1a: As minimum requirements, cell detection, index reading, and measurement period is extended.

    - Option 1b: as minimum requirements, measurement period is extended by number of occasions which are cancelled due to XR traffic.

    - Option 1c: extend the L3 requirements with MGs such as cell identification and measurements delays when one or more MGs are deactivated by the network.

    - Option 1d: RRM measurement requirements are enhanced for UEs under XR operation, at least for UEs not equipped with an additional rx chain for XR.

  - Option 2: Proposals with distinction of conditions where measurement impact is not needed

    - Option 2a:

      - For the cases where measurement impact is needed

        - As minimum requirements, cell detection, index reading, and measurement period for NR cells is extended.

          - FFS how to extend.

      - For the cases where measurement impact is not needed

        - FFS the feasible conditions and necessary UE assistance information.

    - Option 2b:

      - Apply legacy measurement delay and accuracy requirements for the following condition:

        - the skipped measurement occasion(s) indicated by network is the unused measurement occasion reported by UAI

      - Otherwise, extend measurement delay while keep legacy accuracy requirements unchanged

On how to capture impact of measurement skipping, we in general support option 2b. If UAI is to be introduced and applied, the measurement delay should not be extended. In our view, if the cell identification and measurement delay still need to extended with UAI, the benefit of introducing UAI is not convinced.

If UAI is not to be introduced in RAN4 or is not applied, the cell identification and measurement delay is extended by number of occasions which are cancelled due to XR traffic.

Proposal 4: On how to capture impact of measurement skipping:

- If UAI is to be introduced and applied, the measurement delay should not be extended.

- If UAI is not to be introduced in RAN4 or is not applied, the cell identification and measurement delay is extended by number of occasions which are cancelled due to XR traffic.

Issue 3-2: Principles for measurement delay calculation

The following agreements and options are summarized in the WF:

- Option 1: Options based on scaling factor

  - Option 1a: Follow the scaling factor (Ntotal/Navailable) from Concurrent MG spec to extend the total measurement delay.

  - Option 1b: Introduce a scaling factor to extend the total measurement delay by a factor of Ntotal/Navailable, where Ntotal is equal to all gap occasions within the total measurements delay for all frequency layers, while Navailable is the number of gap occasions after gap cancellation for the same time window.

  - Option 1c: introduce scaling factor to make sure enough measurement samples. Besides the ratio between applicable chances and total chances as in legacy, some channel quality could be when determining the exact scaling factor.

  - Option 1d: pre-defined fixed extended RRM measurement period for UE under XR operation (e.g., based on a fixed scaling factor).

- Option 2: Follow NR-U like approach to extend the delays by the number of MGs skipped during the measurement period.

  - Option 2a: dynamically extendable RRM measurement period (e.g., similar to NR-U), depending on the number of actually cancelled MGs.

On the principle for measurement delay calculation, for now we prefer option 2, since it is clear on how the extension is reflected in the requirement. For option 1, it is not quite clear to us on how the scaling factor can be calculated, and whether the same scaling factor can be used or different scaling factors can be used for difference cases.

Observation 2: On the principle for measurement delay calculation, option 2 with NR-U approach is clear on how the extension is reflected in the requirement, while in option 1 it is not quite clear on how the scaling factor can be calculated.

Proposal 5: On the principle for measurement delay calculation, we prefer option 2 to follow NR-U like approach to extend the delays by the number of MGs skipped during the measurement period.

- Summary

In this paper, we provide our views on RRM requirements for XR enhancements. From this discussion we have the following observations and proposals:

Observation 1: The scenarios for L3 measurements without gaps with scheduling restrictions are limited, and the gain by cancelling the restriction is not obvious for XR transmission.

Proposal 1: To keep RAN4 workload reasonable, measurement skipping does not apply for L3 measurements without gaps with scheduling restrictions.

Proposal 2: Measurement skipping does not apply for L1 measurements and procedures.

Proposal 3: For L1 measurement for mobility, apply same decision as normal L1 measurement.

Proposal 4: On how to capture impact of measurement skipping:

- If UAI is to be introduced and applied, the measurement delay should not be extended.

- If UAI is not to be introduced in RAN4 or is not applied, the cell identification and measurement delay is extended by number of occasions which are cancelled due to XR traffic.

Observation 2: On the principle for measurement delay calculation, option 2 with NR-U approach is clear on how the extension is reflected in the requirement, while in option 1 it is not quite clear on how the scaling factor can be calculated.

Proposal 5: On the principle for measurement delay calculation, we prefer option 2 to follow NR-U like approach to extend the delays by the number of MGs skipped during the measurement period.

- References

[1] R4-2420103, WF on RRM requirements for XR_Ph3, Nokia