3GPP TSG-RAN4 Meeting #114					          	       R4-2500334

Athens, GR, Feb 17th – 21st, 2025

Source: 	Tejas Networks

Title: 	BS demodulation performance requirement using MMSE-IRC

Agenda Item:	7.18.3

Document for: 	Discussion

# Introduction

In the RAN4 #113 meeting, RAN4 discussed interference modelling, test parameters, and simulation assumptions for the MMSE-IRC receiver, and the WF document [1] was agreed upon. In this contribution, we provide our views on the remaining open issues according to [1] and simulation results for initial performance analysis.

# Discussion

## Interference Modelling

Interference profile

- On how to account for HPUE in existing LTE profiles:

  - For interference profile not to consider HPUE scenario for BS MMSE-IRC requirements for this Rel-19 WI

According to the WF document [1], following option has been proposed on the Interference profile:

NR scenarios differ from LTE not only in HPUE power levels but also in carrier frequency, channel characteristics, delay spread, and Doppler values. Therefore, artificially increasing LTE interference power is unreliable, as no simulations have been conducted to determine a valid artificial INR value. Furthermore, given the infeasibility of new simulations and the fact that HPUE affects only simulation parameters without impacting IRC implementations, we support excluding HPUE scenarios and any artificial interference adjustments from the Rel-19 WI.

Proposal 1: We propose to not consider HPUE scenarios for interference profile for Rel-19 BS MMSE-IRC requirement.

Number of interferers for 2Rx

  - Option 1: Define BS MMSE-IRC requirements for 2Rx

- Option 1A: 1 interferer

- Option 1B: 2 interferers

- Option 1C: Consider 2 interferers for HomNet and 1 interferer for HetNet

  - Option 2: Do not need to specify requirements for 2Rx

  - Companies are encouraged to provide simulation results for 1 or 2 interferers for 2Rx, to check the performance gain.

In the WF document [1], the following options are proposed for the number of interferers:

To highlight the impact of the number of interferers on 2Rx, simulations were performed with the legacy LTE interference profile for a 1-layer, 2-Rx scenario under single and dual interference circumstances. Table 1 illustrates the performance gain of MMSE-IRC over MMSE in the presence of a single interference source, while Table 2 demonstrates the same performance gain when two interference sources are present.

Table 1: PUSCH performance gain for single interference scenario

| SCS | BW | MCS for target cell | Modulation for interferer cell | Propagation condition for target cell | Propagation condition for interference cell | Scenario (DIP1, DIP2) | Passing SNR @ 70% |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | MMSE | MMSE-IRC | SNR Gain |
| 15 KHz | 5 MHz | 2 | 16 QAM | TDL-B 100 400 | TDL-C 1000 100 | HetNet (-0.43, NA) | 6.83 | 4.95 | 1.88 |
| 15 KHz | 5 MHz | 16 | 16 QAM | TDL-C 300 100 | TDL-C 1000 100 | HetNet (-0.43, NA) | 18.72 | 16.22 | 2.5 |
| 15 KHz | 5 MHz | 20 | 16 QAM | TDL-A 30 10 | TDL-C 1000 100 | HetNet (-0.43, NA) | 20.34 | 17.15 | 3.19 |
| 15 KHz | 5 MHz | 2 | 16 QAM | TDL-B 100 400 | TDL-C 1000 100 | HomNet (-1.11, NA) | 2.49 | 1.46 | 1.03 |
| 15 KHz | 5 MHz | 16 | 16 QAM | TDL-C 300 100 | TDL-C 1000 100 | HomNet (-1.11, NA) | 15.09 | 13.79 | 1.3 |
| 15 KHz | 5 MHz | 20 | 16 QAM | TDL-A 30 10 | TDL-C 1000 100 | HomNet (-1.11, NA) | 16.69 | 15.2 | 1.49 |
| 30 KHz | 10 MHz | 2 | 16 QAM | TDL-B 100 400 | TDL-C 1000 100 | HetNet (-0.43, NA) | 5.95 | 3.16 | 2.79 |
| 30 KHz | 10 MHz | 16 | 16 QAM | TDL-C 300 100 | TDL-C 1000 100 | HetNet (-0.43, NA) | 18.56 | 16.11 | 2.45 |
| 30 KHz | 10 MHz | 20 | 16 QAM | TDL-A 30 10 | TDL-C 1000 100 | HetNet (-0.43, NA) | 20.50 | 17.77 | 2.73 |
| 30 KHz | 10 MHz | 2 | 16 QAM | TDL-B 100 400 | TDL-C 1000 100 | HomNet (-1.11, NA) | 2.01 | 0.61 | 1.40 |
| 30 KHz | 10 MHz | 16 | 16 QAM | TDL-C 300 100 | TDL-C 1000 100 | HomNet (-1.11, NA) | 14.66 | 13.52 | 1.14 |
| 30 KHz | 10 MHz | 20 | 16 QAM | TDL-A 30 10 | TDL-C 1000 100 | HomNet (-1.11, NA) | 16.83 | 15.82 | 1.01 |

Table 2: PUSCH performance gain for two interference scenarios

| SCS | BW | MCS for target cell | Modulation for interferer cell | Propagation condition for target cell | Propagation condition for interference cell | Scenario (DIP1, DIP2) | Passing SNR @ 70% |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | MMSE | MMSE-IRC | SNR Gain |
| 15 KHz | 5 MHz | 2 | 16 QAM | TDL-B 100 400 | TDL-C 1000 100 | HetNet (-0.43, -13.78) | 10.99 | 8.35 | 2.64 |
| 15 KHz | 5 MHz | 16 | 16 QAM | TDL-C 300 100 | TDL-C 1000 100 | HetNet (-0.43, -13.78) | 22.6 | 18.97 | 3.63 |
| 15 KHz | 5 MHz | 20 | 16 QAM | TDL-A 30 10 | TDL-C 1000 100 | HetNet (-0.43, -13.78) | 24.32 | 19.75 | 4.57 |
| 15 KHz | 5 MHz | 2 | 16 QAM | TDL-B 100 400 | TDL-C 1000 100 | HomNet (-1.11, -10.91) | 6.65 | 4.92 | 1.73 |
| 15 KHz | 5 MHz | 16 | 16 QAM | TDL-C 300 100 | TDL-C 1000 100 | HomNet (-1.11, -10.91) | 18.66 | 16.17 | 2.49 |
| 15 KHz | 5 MHz | 20 | 16 QAM | TDL-A 30 10 | TDL-C 1000 100 | HomNet (-1.11, -10.91) | 20.22 | 17.06 | 3.16 |
| 30 KHz | 10 MHz | 2 | 16 QAM | TDL-B 100 400 | TDL-C 1000 100 | HetNet (-0.43, -13.78) | 10.07 | 6.40 | 3.67 |
| 30 KHz | 10 MHz | 16 | 16 QAM | TDL-C 300 100 | TDL-C 1000 100 | HetNet (-0.43, -13.78) | 22.6 | 19.28 | 3.32 |
| 30 KHz | 10 MHz | 20 | 16 QAM | TDL-A 30 10 | TDL-C 1000 100 | HetNet (-0.43, -13.78) | 24.35 | 20.58 | 3.77 |
| 30 KHz | 10 MHz | 2 | 16 QAM | TDL-B 100 400 | TDL-C 1000 100 | HomNet (-1.11, -10.91) | 5.84 | 3.12 | 2.72 |
| 30 KHz | 10 MHz | 16 | 16 QAM | TDL-C 300 100 | TDL-C 1000 100 | HomNet (-1.11, -10.91) | 18.47 | 16.06 | 2.41 |
| 30 KHz | 10 MHz | 20 | 16 QAM | TDL-A 30 10 | TDL-C 1000 100 | HomNet (-1.11, -10.91) | 20.32 | 17.72 | 2.60 |

Analysis of the simulation results shows a significantly higher SNR gain in the dual interference scenario, ranging from 1.73 dB to 4.57 dB, whereas the single interference case achieves only 1.01 dB to 3.19 dB. Hence, we support the dual interference scenario for the 2Rx case.

Proposal 2: We support candidate option 1B for selecting the number of interferences in 2Rx.

## Test parameters and simulation assumptions

Channel bandwidth

- Proposals on the CHBW for requirement definition for CP-OFDM:

  - Option 1:

- For 15kHz SCS, cover 5MHz, 10MHz, 20MHz

- For 30kHz SCS, cover 10MHz, 20MHz, 40MHz, 100MHz

  - Option 2:

- For 15kHz SCS, cover 5MHz, 10MHz

- For 30kHz SCS, cover 10MHz, 40MHz

  - Option 3: minimum CBW

- For 15kHz SCS, 5MHz

- For 30kHz SCS, 10MHz

  - Option 4: 5/50MHz BW with 15KHz SCS and 10/100MHz BW with 30KHz SCS

- Proposals for DFT-s-OFDM:

  - Option 1:

- For 15kHz SCS, 5MHz

- For 30kHz SCS, 10MHz

According to the WF document [1], following candidate options are available for proposal on the channel bandwidth of CP-OFDM and DFT-s-OFDM:

The PUSCH performance gain using MMSE-IRC is independent of channel bandwidth, as covariance matrix estimation is performed per RB. Additionally, LTE analysis shows that PUSCH performance is not significantly affected by channel bandwidth. Therefore, to reduce the effort of extensive simulation runs, we support selecting the minimum channel bandwidth per SCS.

Hence, for CP-OFDM, we support option 3 of using minimum bandwidths for each SCS, i.e., 5 MHz for 15 KHz SCS and use of 10 MHz for 30 KHz SCS.

Proposal 3: For CP-OFDM, we support option 3 of selecting minimum bandwidth for each SCS

- For 15KHz, 5MHz bandwidth

- For 30KHz, 10MHz bandwidth

As DFT-s-OFDM has limited use cases, we do not support its inclusion in performance requirements. However, if a consensus is reached to define performance requirements for DFT-s-OFDM, it should follow a single BW per SCS to align with CP-OFDM’s SCS-BW combinations. Therefore, we recommend Option 1, which adopts the minimum bandwidths for each SCS i.e.; 5 MHz for 15 KHz and 10 MHz for 30 KHz.

Proposal 4: For DFT-S-OFDM, we support option 1 for SCS and BW combination i.e.,

- 5 MHz bandwidth for 15 KHz

- 10 MHz bandwidth for 30 KHz

MCS

In the WF document [1], following candidate options are proposed for MCS:

  - Consider the following options for performance evaluation and as candidates for further down-selection:

o    For 2Rx:

- Option 1: MCS 2

- Option 2: MCS 16

o    For 4Rx:

- Option 1: MCS 16

o    For 8Rx:

- Option 1: MCS 20

- Option 2: MCS 16

- Note: Reuse the same FRC A.3, A.4 and A.5 with Additional DM-RS position = pos1 and 1 transmission layer defined in TS38.104 for MCS 2, MCS 16 and MCS 20 respectively.

  - Check whether the existing test applicability rule can be reused if different MCS is configured for different Rx configuration.

TS 38.104 specifies MCS2, MCS16, and MCS20 for each Rx to cover all modulation scenarios; following the same approach, we propose selecting a single MCS per Rx configuration. Hence, we support candidate option 1 for 2Rx to select MCS2, candidate option 1 for 4Rx to select MCS16, and candidate option 1 for 8Rx to select MCS20. Additionally, FRC from TS 38.104 can be re-used.

Proposal 5: For MCS selection, we support candidate option 1 for 2Rx to select MCS2, candidate option 1 for 4Rx to select MCS16, and candidate option 1 for 8Rx to select MCS20 while re-using the FRCs from TS 38.104.

Propagation Condition

–    For serving cell, consider the following options for performance evaluation and as candidates for further down-selection:

o    Option 1: TDLA30-10 HetNet and TDLB100-400 for HomNet

o    Option 2: Consider different channel for different MCS

- Option 2A: MCS 2 with TDLB100-400, MCS 16 with TDLC300-100, MCS 20 with TDLA30-10

–    For the interference cell, use TDLC300-100.

In the WF document [1], following candidate options are proposed for the propagation condition of serving cell and interference cells:

To reduce simulation efforts for PUSCH performance gain evaluation, we acknowledge the need to limit the number of options. However, given the significant impact of delay spread and Doppler values on MMSE-IRC performance, especially in challenging channel conditions that decrease the accuracy of covariance matrix estimation, we support utilizing a combination of delay spread and Doppler. Hence, to be consistent with the traditional strategy of covering multiple channel conditions, we support candidate option 2A for selecting the serving cell's propagation condition and propose employing separate channels for different MCSs.

Proposal 6: For the propagation condition of serving cell, we support candidate option 2A to consider MCS 2 with TDLB100-400, MCS 16 with TDLC300-100, and MCS 20 with TDLA30-10.

For the interference cell, the interference signal is expected to have a higher delay spread than the desired signal. Hence, to account for the worst-case scenario, we propose using TDL-C 1000 100. However, willing to consider TDL-C 300 100 as a reasonable compromise.

Proposal 7: We propose TDL-C 1000 100 for the interference cell, but, based on consensus, are open to consider TDL-C 300 100 as an alternative.

Waveform

- Candidate options:

  - Option 1: Cover both CP-OFDM and DFT-s-OFDM waveform

  - Option 2: Cover CP-OFDM only

In the WF document [1], following candidate options are proposed for waveform:

Considering the limited use case of DFT-s-OFDM, which is primarily for coverage enhancement, we support candidate option 2 to focus exclusively on the CP-OFDM waveform. This would minimize the need to define additional parameters, such as DMRS configuration for DFT-s-OFDM, and reduce the effort of running additional simulations for performance analysis.

Proposal 8: We support candidate option 2 to cover only CP-OFDM waveform for defining the PUSCH performance requirement.

DMRS Configuration

  - Cover DMRS configuration type 1. FFS whether to include DMRS configuration type 2.

  - Use DMRS port #0 for both serving and interfering PUSCH

  - On DMRS sequence generation, use different DMRS sequence for each gNB:

- For CP-OFDM: nSCID =0 for all cells and NID0=0, 1 and 2 for cell #0 (serving), cell #1 and cell#2 (interfering) respectively

- FFS DMRS sequence generation configuration for DFT-s-DFDM

As per the WF document [1], following candidate options for the DMRS configuration are specified.

The DMRS configuration type plays a crucial role in the accuracy of covariance matrix estimation, as it determines the number of samples available for covariance matrix averaging. DMRS type-2, has lesser DMRS REs compared to type-1, hence it can significantly impact the covariance matrix estimation. Therefore, we propose to cover both DMRS configuration types 1 and 2.

Proposal 9: For DMRS configuration, we propose to cover both DMRS configuration type 1 and type 2.

Additionally, for successful mitigation of interference, it is required that the interference signal overlaps exactly with the DMRS REs of the serving cell. Hence it is required to use the same DMRS port for both serving and interference PUSCH.

Proposal 10: For DMRS configuration, we support the use of DMRS port #0 for both serving and interfering PUSCH.

For DMRS sequence generation, nSCID and NID0 play a crucial role. Assigning identical values for both the serving and interfering cells causes ambiguity at the receiver, leading to corrupted channel estimates. To avoid this, the desired cell and interfering cell must use different DMRS sequences. Thus, we support the option that sets nSCID =0 for all cells and assigns distinct NID values to each cell such as NID0=0, 1, and 2 for cell #0 (serving), cell #1, and cell#2 (interfering) respectively.

Proposal 11: For DMRS sequence generation, we support candidate option to generate different DMRS sequence for each gNB:

- For CP-OFDM: nSCID =0 for all cells and NID0=0, 1 and 2 for cell #0 (serving), cell #1 and cell#2 (interfering) respectively.

Test requirement definition

In the WF document [1], following candidate options are proposed for defining test requirements:

- Candidate options:

  - Option 1: FFS whether SINR based definition is needed

  - Option 2: Use the SNR based requirement definition

As it is agreed to use INR instead of DIP for defining performance requirements, we support candidate option 2 to use SNR-based requirement definition.

Proposal 12: For test requirement definition, we support option 2 to use the SNR-based requirement.

# Simulation Results

| Scenario | (DIP1, DIP2) dB | Transferred (INR1, INR2) dB |
| --- | --- | --- |
| HetNet with 1 interferer | (-0,43, N/A) | (9.82, N/A) |
| HetNet with 2 interferers | (-0,43, -13,78) | (12.38, -0.97) |
| HomNet with 1 interferer | (-1,11, N/A) | (5.35, N/A) |
| HomNet with 2 interferers | (-1,11, -10,91) | (7.28, -2.52) |

In this section, initial simulation results are provided to analyse the performance gain of the MMSE-IRC receiver over MMSE. In the WF document [1], following simulation parameters are suggested to obtain initial results:

Table 3: PUSCH performance gain for HetNet scenario – SCS: 15KHz, BW: 5MHz

| Case No. | Tx | Rx | MCS for target cell | Propagation condition for target cell | Modulation for interference cell | Propagation condition for interference cell | No. of interference | (DIP1, DIP2) | Passing SNR @ 70% |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | MMSE | MMSE-IRC | Gain |
| 1 | 1 | 2 | 2 | TDL-B 100 400 | 16QAM | TDL-C 1000 100 | 1 | (-0.43, NA) | 6.83 | 4.95 | 1.88 |
| 2 | 1 | 2 | 16 | TDL-C 300 100 | 16QAM | TDL-C 1000 100 | 1 | (-0.43, NA) | 18.72 | 16.22 | 2.5 |
| 3 | 1 | 2 | 20 | TDL-A 30 10 | 16QAM | TDL-C 1000 100 | 1 | (-0.43, NA) | 20.34 | 17.15 | 3.19 |
| 4 | 1 | 4 | 2 | TDL-B 100 400 | 16QAM | TDL-C 1000 100 | 1 | (-0.43, NA) | 6.83 | 1.78 | 5.05 |
| 5 | 1 | 4 | 16 | TDL-C 300 100 | 16QAM | TDL-C 1000 100 | 1 | (-0.43, NA) | 15 | 10.34 | 4.66 |
| 6 | 1 | 4 | 20 | TDL-A 30 10 | 16QAM | TDL-C 1000 100 | 1 | (-0.43, NA) | 16.68 | 13.5 | 3.18 |
| 7 | 1 | 8 | 2 | TDL-B 100 400 | 16QAM | TDL-C 1000 100 | 1 | (-0.43, NA) | 6.00 | -1.63 | 7.63 |
| 8 | 1 | 8 | 16 | TDL-C 300 100 | 16QAM | TDL-C 1000 100 | 1 | (-0.43, NA) | 12.03 | 7.81 | 4.22 |
| 9 | 1 | 8 | 20 | TDL-A 30 10 | 16QAM | TDL-C 1000 100 | 1 | (-0.43, NA) | 14.51 | 9 | 5.51 |
| 10 | 1 | 2 | 2 | TDL-B 100 400 | 16QAM | TDL-C 1000 100 | 2 | (-0.43, -13.78) | 10.99 | 8.35 | 2.64 |
| 11 | 1 | 2 | 16 | TDL-C 300 100 | 16QAM | TDL-C 1000 100 | 2 | (-0.43, -13.78) | 22.6 | 18.97 | 3.63 |
| 12 | 1 | 2 | 20 | TDL-A 30 10 | 16QAM | TDL-C 1000 100 | 2 | (-0.43, -13.78) | 24.32 | 19.75 | 4.57 |
| 13 | 1 | 4 | 2 | TDL-B 100 400 | 16QAM | TDL-C 1000 100 | 2 | (-0.43, -13.78) | 10.88 | 4.87 | 6.01 |
| 14 | 1 | 4 | 16 | TDL-C 300 100 | 16QAM | TDL-C 1000 100 | 2 | (-0.43, -13.78) | 18.8 | 13.97 | 4.83 |
| 15 | 1 | 4 | 20 | TDL-A 30 10 | 16QAM | TDL-C 1000 100 | 2 | (-0.43, -13.78) | 20.62 | 15.24 | 5.38 |
| 16 | 1 | 8 | 2 | TDL-B 100 400 | 16QAM | TDL-C 1000 100 | 2 | (-0.43, -13.78) | 10.35 | 1.67 | 8.68 |
| 17 | 1 | 8 | 16 | TDL-C 300 100 | 16QAM | TDL-C 1000 100 | 2 | (-0.43, -13.78) | 16.2 | 11.93 | 4.27 |
| 18 | 1 | 8 | 20 | TDL-A 30 10 | 16QAM | TDL-C 1000 100 | 2 | (-0.43, -13.78) | 18.56 | 12.66 | 5.90 |

Table 4: PUSCH performance gain for HomNet scenario – SCS: 15KHz, BW: 5MHz

| Case No. | Tx | Rx | MCS for target cell | Propagation condition for target cell | Modulation for interference cell | Propagation condition for interference cell | No. of interference | (DIP1, DIP2) | Passing SNR @ 70% |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | MMSE | MMSE-IRC | Gain |
| 1 | 1 | 2 | 2 | TDL-B 100 400 | 16QAM | TDL-C 1000 100 | 1 | (-1.11, NA) | 2.49 | 1.46 | 1.03 |
| 2 | 1 | 2 | 16 | TDL-C 300 100 | 16QAM | TDL-C 1000 100 | 1 | (-1.11, NA) | 15.09 | 13.79 | 1.3 |
| 3 | 1 | 2 | 20 | TDL-A 30 10 | 16QAM | TDL-C 1000 100 | 1 | (-1.11, NA) | 16.69 | 15.2 | 1.49 |
| 4 | 1 | 4 | 2 | TDL-B 100 400 | 16QAM | TDL-C 1000 100 | 1 | (-1.11, NA) | 2.65 | -0.52 | 3.17 |
| 5 | 1 | 4 | 16 | TDL-C 300 100 | 16QAM | TDL-C 1000 100 | 1 | (-1.11, NA) | 11.09 | 8.07 | 3.02 |
| 6 | 1 | 4 | 20 | TDL-A 30 10 | 16QAM | TDL-C 1000 100 | 1 | (-1.11, NA) | 13.06 | 10.75 | 2.31 |
| 7 | 1 | 8 | 2 | TDL-B 100 400 | 16QAM | TDL-C 1000 100 | 1 | (-1.11, NA) | 1.60 | -3.37 | 4.97 |
| 8 | 1 | 8 | 16 | TDL-C 300 100 | 16QAM | TDL-C 1000 100 | 1 | (-1.11, NA) | 7.92 | 4.43 | 3.49 |
| 9 | 1 | 8 | 20 | TDL-A 30 10 | 16QAM | TDL-C 1000 100 | 1 | (-1.11, NA) | 10.55 | 7.9 | 2.65 |
| 10 | 1 | 2 | 2 | TDL-B 100 400 | 16QAM | TDL-C 1000 100 | 2 | (-1.11, -10.91) | 6.65 | 4.92 | 1.73 |
| 11 | 1 | 2 | 16 | TDL-C 300 100 | 16QAM | TDL-C 1000 100 | 2 | (-1.11, -10.91) | 18.66 | 16.17 | 2.49 |
| 12 | 1 | 2 | 20 | TDL-A 30 10 | 16QAM | TDL-C 1000 100 | 2 | (-1.11, -10.91) | 20.22 | 17.06 | 3.16 |
| 13 | 1 | 4 | 2 | TDL-B 100 400 | 16QAM | TDL-C 1000 100 | 2 | (-1.11, -10.91) | 6.71 | 1.83 | 4.88 |
| 14 | 1 | 4 | 16 | TDL-C 300 100 | 16QAM | TDL-C 1000 100 | 2 | (-1.11, -10.91) | 14.9 | 10.22 | 4.68 |
| 15 | 1 | 4 | 20 | TDL-A 30 10 | 16QAM | TDL-C 1000 100 | 2 | (-1.11, -10.91) | 16.58 | 13.55 | 3.03 |
| 16 | 1 | 8 | 2 | TDL-B 100 400 | 16QAM | TDL-C 1000 100 | 2 | (-1.11, -10.91) | 6.00 | -1.83 | 7.83 |
| 17 | 1 | 8 | 16 | TDL-C 300 100 | 16QAM | TDL-C 1000 100 | 2 | (-1.11, -10.91) | 11.87 | 7.6 | 4.27 |
| 18 | 1 | 8 | 20 | TDL-A 30 10 | 16QAM | TDL-C 1000 100 | 2 | (-1.11, -10.91) | 14.43 | 8.89 | 5.54 |

Table 5: PUSCH performance gain for HetNet scenario – SCS: 30KHz, BW: 10MHz

| Case No. | Tx | Rx | MCS for target cell | Propagation condition for target cell | Modulation for interference cell | Propagation condition for interference cell | No. of interference | (DIP1, DIP2) | Passing SNR @ 70% |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | MMSE | MMSE-IRC | Gain |
| 1 | 1 | 2 | 2 | TDL-B 100 400 | 16QAM | TDL-C 1000 100 | 1 | (-0.43, NA) | 5.95 | 3.16 | 2.79 |
| 2 | 1 | 2 | 16 | TDL-C 300 100 | 16QAM | TDL-C 1000 100 | 1 | (-0.43, NA) | 18.56 | 16.11 | 2.45 |
| 3 | 1 | 2 | 20 | TDL-A 30 10 | 16QAM | TDL-C 1000 100 | 1 | (-0.43, NA) | 20.50 | 17.77 | 2.73 |
| 4 | 1 | 4 | 2 | TDL-B 100 400 | 16QAM | TDL-C 1000 100 | 1 | (-0.43, NA) | 4.51 | -1.31 | 5.82 |
| 5 | 1 | 4 | 16 | TDL-C 300 100 | 16QAM | TDL-C 1000 100 | 1 | (-0.43, NA) | 15.03 | 11.09 | 3.94 |
| 6 | 1 | 4 | 20 | TDL-A 30 10 | 16QAM | TDL-C 1000 100 | 1 | (-0.43, NA) | 16.61 | 11.7 | 4.91 |
| 7 | 1 | 8 | 2 | TDL-B 100 400 | 16QAM | TDL-C 1000 100 | 1 | (-0.43, NA) | 3.16 | -4.37 | 7.53 |
| 8 | 1 | 8 | 16 | TDL-C 300 100 | 16QAM | TDL-C 1000 100 | 1 | (-0.43, NA) | 12.72 | 6.85 | 5.87 |
| 9 | 1 | 8 | 20 | TDL-A 30 10 | 16QAM | TDL-C 1000 100 | 1 | (-0.43, NA) | 14.79 | 9 | 5.79 |
| 10 | 1 | 2 | 2 | TDL-B 100 400 | 16QAM | TDL-C 1000 100 | 2 | (-0.43, -13.78) | 10.07 | 6.4 | 3.67 |
| 11 | 1 | 2 | 16 | TDL-C 300 100 | 16QAM | TDL-C 1000 100 | 2 | (-0.43, -13.78) | 22.6 | 19.28 | 3.32 |
| 12 | 1 | 2 | 20 | TDL-A 30 10 | 16QAM | TDL-C 1000 100 | 2 | (-0.43, -13.78) | 24.35 | 20.58 | 3.77 |
| 13 | 1 | 4 | 2 | TDL-B 100 400 | 16QAM | TDL-C 1000 100 | 2 | (-0.43, -13.78) | 8.60 | 1.17 | 7.43 |
| 14 | 1 | 4 | 16 | TDL-C 300 100 | 16QAM | TDL-C 1000 100 | 2 | (-0.43, -13.78) | 19.12 | 14.08 | 5.04 |
| 15 | 1 | 4 | 20 | TDL-A 30 10 | 16QAM | TDL-C 1000 100 | 2 | (-0.43, -13.78) | 20.67 | 14.35 | 6.32 |
| 16 | 1 | 8 | 2 | TDL-B 100 400 | 16QAM | TDL-C 1000 100 | 2 | (-0.43, -13.78) | 7.42 | -2.41 | 9.83 |
| 17 | 1 | 8 | 16 | TDL-C 300 100 | 16QAM | TDL-C 1000 100 | 2 | (-0.43, -13.78) | 16.82 | 9.83 | 6.99 |
| 18 | 1 | 8 | 20 | TDL-A 30 10 | 16QAM | TDL-C 1000 100 | 2 | (-0.43, -13.78) | 18.87 | 11.17 | 7.70 |

Table 6: PUSCH performance gain for HomNet scenario – SCS: 30KHz, BW: 10MHz

| Case No. | Tx | Rx | MCS for target cell | Propagation condition for target cell | Modulation for interference cell | Propagation condition for interference cell | No. of interference | (DIP1, DIP2) | Passing SNR @ 70% |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | MMSE | MMSE-IRC | Gain |
| 1 | 1 | 2 | 2 | TDL-B 100 400 | 16QAM | TDL-C 1000 100 | 1 | (-1.11, NA) | 2.01 | 0.61 | 1.40 |
| 2 | 1 | 2 | 16 | TDL-C 300 100 | 16QAM | TDL-C 1000 100 | 1 | (-1.11, NA) | 14.66 | 13.52 | 1.14 |
| 3 | 1 | 2 | 20 | TDL-A 30 10 | 16QAM | TDL-C 1000 100 | 1 | (-1.11, NA) | 16.83 | 15.82 | 1.01 |
| 4 | 1 | 4 | 2 | TDL-B 100 400 | 16QAM | TDL-C 1000 100 | 1 | (-1.11, NA) | 0.27 | -2.89 | 3.16 |
| 5 | 1 | 4 | 16 | TDL-C 300 100 | 16QAM | TDL-C 1000 100 | 1 | (-1.11, NA) | 11.3 | 8.46 | 2.84 |
| 6 | 1 | 4 | 20 | TDL-A 30 10 | 16QAM | TDL-C 1000 100 | 1 | (-1.11, NA) | 12.73 | 9.97 | 2.76 |
| 7 | 1 | 8 | 2 | TDL-B 100 400 | 16QAM | TDL-C 1000 100 | 1 | (-1.11, NA) | -1.15 | -5.91 | 4.76 |
| 8 | 1 | 8 | 16 | TDL-C 300 100 | 16QAM | TDL-C 1000 100 | 1 | (-1.11, NA) | 8.65 | 4.72 | 3.93 |
| 9 | 1 | 8 | 20 | TDL-A 30 10 | 16QAM | TDL-C 1000 100 | 1 | (-1.11, NA) | 10.89 | 8.08 | 2.81 |
| 10 | 1 | 2 | 2 | TDL-B 100 400 | 16QAM | TDL-C 1000 100 | 2 | (-1.11, -10.91) | 5.84 | 3.12 | 2.72 |
| 11 | 1 | 2 | 16 | TDL-C 300 100 | 16QAM | TDL-C 1000 100 | 2 | (-1.11, -10.91) | 18.47 | 16.06 | 2.41 |
| 12 | 1 | 2 | 20 | TDL-A 30 10 | 16QAM | TDL-C 1000 100 | 2 | (-1.11, -10.91) | 20.32 | 17.72 | 2.60 |
| 13 | 1 | 4 | 2 | TDL-B 100 400 | 16QAM | TDL-C 1000 100 | 2 | (-1.11, -10.91) | 4.39 | -1.34 | 5.73 |
| 14 | 1 | 4 | 16 | TDL-C 300 100 | 16QAM | TDL-C 1000 100 | 2 | (-1.11, -10.91) | 14.98 | 11 | 3.98 |
| 15 | 1 | 4 | 20 | TDL-A 30 10 | 16QAM | TDL-C 1000 100 | 2 | (-1.11, -10.91) | 16.51 | 11.67 | 4.84 |
| 16 | 1 | 8 | 2 | TDL-B 100 400 | 16QAM | TDL-C 1000 100 | 2 | (-1.11, -10.91) | 3.10 | -4.36 | 7.46 |
| 17 | 1 | 8 | 16 | TDL-C 300 100 | 16QAM | TDL-C 1000 100 | 2 | (-1.11, -10.91) | 12.62 | 6.8 | 5.82 |
| 18 | 1 | 8 | 20 | TDL-A 30 10 | 16QAM | TDL-C 1000 100 | 2 | (-1.11, -10.91) | 14.73 | 8.94 | 5.79 |

Based on the simulation results presented above, the following observations are made:

Observation 1: Compared to the HomNet scenario, the HetNet scenario exhibits a higher SNR gain, demonstrating the effectiveness of the MMSE-IRC algorithm in high-interference environments, as HetNet has a greater INR than HomNet.

Observation 2: The SNR gain increases with the number of receiver antennas, as additional antennas enhance interference rejection capability and improve diversity.

Observation 3: The SNR gain of MMSE-IRC with two interference sources consistently exceeds that of the single-interference scenario, regardless of the number of receiver antennas.

Observation 4: Channels with higher delay spread result in lower SNR gain, as interference mitigation becomes more challenging due to degraded channel quality.

Observation 5: The SNR gain with MMSE-IRC ranges from approximately 1.01 dB to 8.68 dB. In interference scenarios, this level of improvement justifies the use of MMSE-IRC for interference mitigation at the base station.

These simulation results demonstrate the superior interference suppression capability of the MMSE-IRC receiver over the conventional MMSE in NR scenarios.

# Conclusion

In this contribution, we have given our views and proposals on the remaining issues of MMSE-IRC performance requirement for BS demodulation along with initial simulation results.

Proposal 1: We propose to not consider HPUE scenarios for interference profile for Rel-19 BS MMSE-IRC requirement.

Proposal 2: We support candidate option 1B for selecting the number of interferences in 2Rx.

Proposal 3: For CP-OFDM, we support option 3 of selecting minimum bandwidth for each SCS

- For 15KHz, 5MHz bandwidth

- For 30KHz, 10MHz bandwidth

Proposal 4: For DFT-S-OFDM, we support option 1 for SCS and BW combination i.e.,

- 5 MHz bandwidth for 15 KHz

- 10 MHz bandwidth for 30 KHz

Proposal 5: For MCS selection, we support candidate option 1 for 2Rx to select MCS2, candidate option 1 for 4Rx to select MCS16, and candidate option 1 for 8Rx to select MCS20 while re-using the FRCs from TS 38.104.

Proposal 6: For the propagation condition of serving cell, we support candidate option 2A to consider MCS 2 with TDLB100-400, MCS 16 with TDLC300-100, and MCS 20 with TDLA30-10.

Proposal 7: We propose TDL-C 1000 100 for the interference cell, but, based on consensus, are open to consider TDL-C 300 100 as an alternative.

Proposal 8: We support candidate option 2 to cover only CP-OFDM waveform for defining the PUSCH performance requirement.

Proposal 9: For DMRS configuration, we propose to cover both DMRS configuration type 1 and type 2.

Proposal 10: For DMRS configuration, we support the use of DMRS port #0 for both serving and interfering PUSCH.

Proposal 11: For DMRS sequence generation, we support candidate option to generate different DMRS sequence for each gNB:

- For CP-OFDM: nSCID =0 for all cells and NID0=0, 1 and 2 for cell #0 (serving), cell #1 and cell#2 (interfering) respectively.

Proposal 12: For test requirement definition, we support option 2 to use the SNR-based requirement.

Observation 1: Compared to the HomNet scenario, the HetNet scenario exhibits a higher SNR gain, demonstrating the effectiveness of the MMSE-IRC algorithm in high-interference environments, as HetNet has a greater INR than HomNet.

Observation 2: The SNR gain increases with the number of receiver antennas, as additional antennas enhance interference rejection capability and improve diversity.

Observation 3: The SNR gain of MMSE-IRC with two interference sources consistently exceeds that of the single-interference scenario, regardless of the number of receiver antennas.

Observation 4: Channels with higher delay spread result in lower SNR gain, as interference mitigation becomes more challenging due to degraded channel quality.

Observation 5: The SNR gain with MMSE-IRC ranges from approximately 1.01 dB to 8.68 dB. In interference scenarios, this level of improvement justifies the use of MMSE-IRC for interference mitigation at the base station.

# Reference

- R4-2419780, “Way Forward for [113] [321] NR_demod_Ph5_Part1_General_BS”, China Telcom, Mediatek, RAN#113, Orlando, USA.

- RP-240831, “New WID: WI resulting from discussion on Demodulation performance requirements enhancements”, Vivo, RAN#103, Maastricht, Netherlands.