3GPP TSG-RAN WG4 # 114	R4-2500335

Athens, Greece, 17th Feb 2025 – 21st Feb 2025

Agenda item:	7.16.2

Source:	MediaTek Inc.

Title:	Discussion on Spatial Channel Modelling

Document for:	Discussion

# Introduction

The Rel-19 study item [1] aims to investigate practical spatial channel modelling methodologies for both SU-MIMO and MU-MIMO demodulation and CSI reporting requirements. It has been identified that the current TDL-based models and spatial correlation matrices poorly match with realistic propagation conditions that have been used as the design assumption for the NR downlink precoder (PMI) codebooks. We also believe that the test scenarios and channel models should match with realistic transmit strategies and channel conditions in order to correctly support UE algorithm design. When RAN4 test cases are unrealistic, the UE algorithm designers may need to compromise between real-world behavior/performance and RAN4 tests.

In our view, the problem with the existing RAN4 spatial correlation models is that they amplify the signal only in the broadside direction (AOD/AOA) of the TX and RX antenna arrays. In order to support a higher number of layers, multiple separate spatial signal directions should exist. This can be achieved by introducing spatial clusters, where each cluster has its own separable mean AOD/AOA.

The current RAN4 MIMO tests employ TDL (Tapped Delay Line) channel models with specific spatial correlation matrices. For spatial channel model development, both CDL (Clustered Delay Line) and TDL models can be considered as baseline. In our earlier contribution, we have described how to achieve any desired spatial channel properties by employing simple “multi-cluster TDL” on top of TDL channels [3].

# Discussion

# 2.1 Discussion of common issues for all methodologies

In this section, we share our views of common issues for all methodologies.

Capturing comparison cases and results in TR 38.753

Proposals:

- Option 1: TR 38.753 shall be self-contained and include the following:

  - Option 1a: Focussed descriptions of the agreed channel model

  - Option 1b: Comparison case results with respect to all legacy and SCM candidates

We have noticed that implementing the CDL model is not trivial as the many details of the model are scattered between multiple TRs and their many sections. Therefore, there is a need for a more focused and self-contained description of the link-level model.

Proposal #1: The upcoming TR should provide self-contained and focused descriptions of the agreed channel models.

Metrics for comparison

Proposals:

- Option 1: RAN4 to discuss test metrics to compare the SCM methodologies.

- Option 2: The metric of PDSCH cases could be throughput or condition number, i.e, ratio of the absolute value of the maximum eigenvalue to the absolute value of the minimum eigenvalue of the channel.

It has turned out that result alignment in CDL channels between companies (in terms of tput performance) with different AAV options is very difficult. Therefore, we propose to carry out initial alignment via simpler alternative metrics that omit all uncertainty resulting from different receiver algorithms implementations. These alternative metrics should be based on measured channel statistics only. For example, the following measures can be recorded and cross-checked between companies:

- Spatial domain / multiplexing: Average eigenmode powers, sorted from largest to smallest

- Spatial domain / directionality: Power angular distribution (PAD) of the channel in TX and RX dimensions

- Time domain: Doppler spectrum, or more simply time coherence function (measured as channel correlation between symbols or slots)

- Frequency domain: Frequency coherence function (measured as channel correlation between subcarriers or PRBs)

Note that we already share measured average eigenmode powers for the different AAV options in simulation result document [4].

Proposal #2: We propose following measures to be recorded and cross-checked between interested companies

- Spatial domain/multiplexing: Average eigenmode powers, sorted from largest to smallest

- Spatial domain/directionality: Power angular distribution (PAD) of the channel in TX and RX dimensions

- Time domain: Doppler spectrum, or more simply time coherence function

- Frequency domain: Frequency coherence function

Ability for SCM candidates to be modified

Proposals:

- Option 1: RAN4 to consider how SCM candidates can be modified to allow separately, but spatially consistently, correlated channels for different AODs, AOAs and ZODs corresponding to different multiple UEs.

Proposed Multi-cluster TDL models have solution to easily define configurable spatially consistent, correlated channels for different AODs, AOAs and ZODs. We would be happy to see similar easy spatial configurability implemented also in CDL model.

Proposal #3: We support including easy spatial configurability in SCM candidates.

# 2.2 PDSCH demodulation testing in CDL channels

In this section we discuss PDSCH demodulation testing observations and proposals in CDL channels.

Cases for SU-MIMO

Agreement:

- 8Rx: 8 Layer, MCS 13 on both codewords (Type1 codebook)

- Random PMI as a baseline, FFS Fixed.

- Interested companies invited to choose based on most common PMI in PMI histogram.

  - Note, Nokia's proposal in RAN4#113 for fixed PMI (i1_1=6, i1_2=0, i2=0)

We have simulated PDSCH demodulation performance with random and fixed PMI. Results are available in separate simulation result document [4]. We can make the following observations.

Observation #1: CDL-C UMa with all AAV options is feasible with 4T4R and random Type1 precoding.

Observation #2: CDL-C UMa with all AAV options is feasible with 8T8R and best fixed Type1 precoding.

Observation #3: The choice for the best fixed Type1 precoder depends on the AAV option.

Observation #4: CDL-C UMa with AAV options 827 fully connected and fixed array size (opt X) are feasible with 8T8R and random Type1 precoding.

Observation #5: CDL-C UMa with AAV options passthrough (no AAV) and fixed subarray size (opt Y) are not feasible with 8T8R and random Type1 precoding because of bad second codeword performance.

Observation #6: CDL-C UMa with all AAV options show clear performance differences between codewords with 8T8R and best fixed Type1 precoding.

Observation #7: CDL-C UMa with all AAV options show mostly huge performance differences between codewords with 8T8R and random Type1 precoding.

Cases for MU-MIMO

For background the latest SID (RP-241848) stipulates that the objective is as follows:

- Study practical spatial channel modelling methodology for both SU- and MU-MIMO demodulation requirements and CSI reporting requirements:

Proposals:

- Option 1: Prioritise SU-MIMO PDSCH

  - Option 1a: Baseline agreements for the SU-MIMO PMI and MU-MIMO to be postponed once PDSCH is reviewed.

- Option 2: Prioritise SU-MIMO (PDSCH and PMI)

  - Option 2a: discuss potential MU-MIMO scenarios for the study.

We see SU-MIMO as the priority for alignment and channel model observations. However, when SU-MIMO simulation results and analysis are widely available we are open to study also MU-MIMO scenarios.

Proposal #4: We propose to focus on SU-MIMO alignment first but discuss potential MU-MIMO scenarios for study.

# 2.3 Follow-PMI testing in CDL channels

In this section we discuss PMI testing observations and proposals in CDL channels.

Cases for SU-MIMO

Agreement:

- Single-User PMI

- 4Rx: 4 Layer (type I) – Full Throughput Curves (PMI Follow, PMI Random)

- 4Rx: 4 Layer (eType II) – Full Throughput Curves (PMI Follow)

- 4Rx: 2 Layer (type I) – Full Throughput Curves (PMI Follow, PMI Random)

- 4Rx: 2 Layer (eType II) – Full Throughput Curves (PMI Follow)

- Interested companies can use initially 8 CSI-RS Ports, more ports can be used to identify and show relevant impacts.

- Note: Several companies would prefer to focus on PDSCH until the modelling is stable

We simulated 8TX-4RX follow PMI performance with LMMSE receiver for both 2L and 4L cases with multiple AAV options in CDL-C UMa (827) channel. For the simulation results and parameter details see [4]. These simulations help in identifying feasible AAV options for follow-PMI testing.

For rank 2 with AAV via fixed TX-BF as specified in TR38.827, the SNR gain from Type1 precoding vs random precoding is small (up to 1.4dB), while the gain from eType2 remains modest (up to 3.6dB). Furthermore, simple fixed port selection precoding always outperforms Type1 follow PMI by multiple dBs. This suggests that further precoding on top of the fixed beamforming does not make much sense.

Observation #8: CDL-C UMa with AAV via fixed TX-BF as specified in TR38.827, provides only modest gains for Type1/eType2 follow-PMI when compared to random precoding.

Proposal #5: We propose that AAV via fixed TX-BF as specified in TR38.827 is not used for follow-PMI testing.

For rank 2 without AAV, the SNR gain from both Type1 and eType2 precoding vs random precoding is very high (more than 9 dB at 90% tput). Thus, it can be concluded that using no AAV provides a good setup for testing PMI feedback.

Observation #9: CDL-C UMa without AAV, provides significant gains for both Type1 and eType2 follow-PMI when compared to random precoding.

Proposal #6: We propose to consider no AAV for follow-PMI testing in CDL channels.

# 2.4 Discussion of CDL-C UMa FR1 channel model

In this section we discuss CDL channel model related issues. Figure 1 shows high level work plan of CDL model.

Figure 1: 38.753 CDL model work plan in WF [2].

Angles of Departure and Angles of Arrival

Agreement:

- Option 1: Replace the mean angle μϕ,model in the last position of departure and arrival angles generated equation by the desired angle μϕ,desired as ϕn,m,AOA,scaled=ASdesiredASmodelϕn,AOA+cASAαm-μϕ,model+μϕ,desired. Set μϕ,desired based on the LOS direction between the TX and RX

- Option 2: Following 38.827 (i.e., μϕ,desired = μϕ,model).

Our interpretation of proposed Option 1 is that – in CDL channel models with a LOS component – all tabulated cluster and ray angles (at least AOD and AOA) should be shifted based on the (system level) geometric interpretation of the LOS direction that depends on the gNB and UE locations. We wonder if this is the correct interpretation, and whether there are more general intentions behind this proposal. We believe that clarification is needed. Furthermore, we think that the specification/description of “angular shift” might become clearer if it is separately described instead of combining it with the process of “angular scaling”.

Proposal #7: We propose that related to CDL angular scaling, the purpose and meaning of Option 1 should be better clarified and justified.

Proposal #8: We propose RAN4 to consider describing “angular shift” as a separate feature/process.

Time Varying Beam Direction

Proposals:

- Option 1: Further evaluate the method for time variant beam directions in CDL model.

- Option 2: CDL based link level simulation should focus on predefined and fixed beam direction, i.e., no need to introduce time varying beam direction modelling for CDL based link level simulation.

Existing TDL channel models implement time varying beam direction for more extensive PMI testing. We would like to consider similar possibility available for CDL channels too.

Proposal #9: We propose to evaluate the method for time variant beam directions in CDL model.

BS Antenna Virtualiser choice

Way Forward:

- Broadside for BS AAV options except fully connected.

- Max receive power for standard fully connected AAV.

- Interested companies to study with AAV to use max receive power of single strongest direction.

We have used broadside BS antenna virtualiser with AAV using subarrays, both fixed subarray (opt Y) and fixed array (opt X) size variants, in our PDSCH demodulation and PMI reporting simulations. Additionally, we have used the strongest receive power direction virtualiser with AAV using fixed array (opt X) size in our PDSCH demodulation simulations.

Observation #10: We see broadside antenna virtualiser as feasible option.

Observation #11: We see that max receive power of single strongest direction antenna virtualiser further improves the feasibility, but in the simulated cases the improvement is not crucial.

Proposal #10: We propose to focus on broadside virtualiser in TR 38.753.

Frequency Bands

Proposals:

- Option 1: RAN4 to study if the same CDL channel model can be defined for all carrier frequencies/ bands in given frequency range

We see it valuable feasibility check to verify channel model in more bands and carrier frequencies. However, we see that we should focus first to 3.5GHz carrier frequency and check other frequencies in the end of feasibility study.

Proposal #11: We propose to focus on 3.5GHz carrier frequency first but check other frequencies in the end of feasibility study.

CDL Simplification

Proposals:

- Option 1 RAN4 to study if the CDL channel models can be simplified to smaller number of clusters.

- Option 2: RAN4 to discuss whether the existing simplification procedure in 38.901 Annex B.2.1 is applicable to SCM, and if not how to extend the existing simplification methodology to reduce the number of channel taps for CDL channels based on 38.901.

We are open to study different CDL simplification options if there are credible proposals with support from other companies.

Proposal #12: We are open to study different CDL simplification options.

SNR Definition with CDL

Agreements in the RAN4#113 meeting:

- Define normalised receive signal power as the following:

- The sum of signal power is normalised across all Rx RF chains using a single normalisation factor. The normalisation factor is calculated considering the long term average PDP for the spatial CDL channel and AAV [spatial response/combining gain].

- The normalisation factor will be aligned and can be included in TR 38.753.

- Interested companies are welcome to provide measured normalization factor for CDL channel with different AAV options in next meeting.

In the last meeting we agreed SNR definition with CDL model. We have provided measured normalization factors for CDL channel with different AAV options in simulation results document [4]. Also, we propose to split normalization factor into two parts that would correspond to subarray antenna element normalization and transmit beamforming gain normalization.

Proposal #13: We propose to define SNR normalization factor as

Let normalized AAV beamformer vector of length Ms∙Ns (i.e. sub-array size) for one antenna port be

α1,1⋯αMs,Ns=1GBF1Ms∙Nsexp-j2π∙θ1,1 ⋯ exp⁡(-j2π∙θMs,Ns)

Here, amplitude normalization is separated into two factors

- 1Ms∙Ns : split transmit power of each AP evenly over Ms∙Ns transmit antenna elements

- 1GBF : normalize with measured transmit beamforming (power) gain GBF

Reduction of Randomness

Proposals:

- Option 1: All CDL based approaches to use the 827 randomness reduction framework, i.e., small scale fading changes, fixed subpaths, weak clusters, etc.

We agree that TR 38.827 randomness reduction framework is needed in TR 38.753 to enable result alignment.

Proposal #14: We support TR 38.827 randomness reduction framework to be included in CDL models of TR 38.753.

# 2.5 Multi-cluster TDL channels

The most important requirement for RAN4 channel models is to facilitate testing the correctness of the implementation of various 3GPP features. For example, to test PMI reporting or advanced (MU-MIMO) receiver implementation at the UE, a test scenario and channel model, where the feature provides clearly measurable tput gain, must be found.

It is worth recognizing that the good test channel conditions are not necessarily typical in the sense that they would be dominant in live networks or in the field. Clearly, typical/average/realistic channel conditions are valuable when evaluating the overall benefit or feasibility of a new feature. However, when RAN4 tests are defined, the characteristics of the test channel often need to be specifically adjusted/chosen to meet the testability requirements of the new feature.

During Rel-18 MU-MIMO advanced receiver WI, a plethora of TDL correlation models and MU-MIMO precoding options were evaluated. The main target was to find cases where the advanced interference cancelling receiver clearly outperforms the linear baseline demodulator. If the performance difference of the baseline and advanced receiver were too small, some UEs could pass the RAN4 test even without implementing the advanced receiver. To find suitable test conditions, channel models that are easy to configure, easy to adjust, and easy to align (between companies) are essential.

Our proposed multi-cluster TDL extension with the existing 3GPP correlation models is an easy-to-use and easy-to-configure toolbox to tailor test conditions for the specific testing needs of upcoming or existing features. By setting a handful of parameters, desired spatial channel conditions can be created. On the other hand, it is hard to see how the existing tabulated CDL channels models would be adjusted to meet varying test requirements.

Proposal #15: We propose RAN4 to consider the option of multi-cluster TDL model for spatial channel modelling, to ensure RAN4 can find feasible test cases for upcoming or existing features.

In the following sections, we demonstrate the configurability and use of multi-cluster TDL models.

# 2.5.1 PDSCH demodulation testing in multi-cluster TDL channels

In this section, we demonstrate the easy configurability of the multi-cluster TDL approach as described in R4-2500337 (TP to TR38.753: TDL approaches and related Annex) by simulating PDSCH demodulation with cluster power parameter sweep. All simulations employ LMMSE MIMO demodulation.

In Figure 2, simulation results with random precoding are shown. Figure 2 (Left) shows results for 4TX-4RX-4L in a 2-cluster TDL-C channel, for which the channel parameters are detailed in Table 1. Figure 2 (Right) shows results for 8TX-8RX-8L in a 4-cluster TDL-A channel, for which the channel parameters are listed in Table 2.  Here, variable levels of cluster power imbalance x are experimented by setting [p1, p2] = [0, -x] dB for the 2-cluster channel, and [p1, p2, p3, p4] = [0, -x, 0, -x] dB for the 4-cluster channel. As can be seen, the higher the imbalance, the higher the SNR requirement to achieve any desired tput level. As can also be seen form Figure 2 (Right) for this specific channel setup with 8 layers, random precoding effectively equalizes the CW-specific performances to the same level.

Figure 2: Random precoder in multi-cluster TDL channels, 4TX-4RX-4L with TDL-C, 8T-8RX-8L with TDL-A.

In Figure 3, simulation results for 8TX-8RX-8L PDSCH demodulation in a 4-cluster TDL-C channel for fixed Type1 precoder (i11 = i12 = i2 = 0) are given. The channel parameters are listed in Table 2. Here, the cluster phases are chosen so that the first two clusters correspond to CW0 precoder beams and the last two clusters correspond to CW1 precoder beams. As can be seen in Figure 3 (Left), assigning even cluster powers between CWs results in equal tput performance for both CWs. However, introducing an imbalance between the two beams of each CW results in an overall tput degradation. On the other hand, as can be seen in Figure 3 (Right), assigning imbalanced cluster powers between the CWs results in a clear tput performance deviation between the CWs.

Observation #12: In PDSCH testing, the cluster powers of multi-cluster TDL channel model can be effectively used to separately tune the sum tput and the per-CW tput performance to the desired level.

Figure 3: 8TX-8RX-8L fixed precoder tput in 4-cluster TDL-C channel.

Table 1: 2-cluster TDL channel parameters

| Array dimensions: Ntx1, Ntx2, Nrx1, Nrx2 | Ntx/2, 1, Nrx/2, 1 (1-dim antenna arrays) |
| --- | --- |
| Cluster powers: p1, p2 | Variable |
| Cluster TX steering: θtx1(1),θtx1(2) | 0, π |
| Cluster RX steering: θrx1(1),θrx1(2) | 0, π |
| TDL channel model per cluster | TDL-C-300 / 100 Hz Doppler / X-pol high correlation |

Table 2: 4-cluster TDL channel parameters

| Array dimensions: Ntx1, Ntx2, Nrx1, Nrx2 | Ntx/2, 1, Nrx/2, 1 (1-dim antenna arrays) |
| --- | --- |
| Cluster powers: p1, p2, p3, p4 | Variable |
| Cluster TX steering: θtx1(1),θtx1(2),θtx1(3), θtx1(4) | 0, -π/2, -π, -3π/2 |
| Cluster RX steering: θrx1(1),θrx1(2),θrx1(3), θrx1(4) | 0, -π/2, -π, -3π/2 |
| TDL channel model per cluster | TDL-A-30 / 10 Hz Doppler / X-pol high correlation TDL-C-300 / 100 Hz Doppler / X-pol high correlation |

# 2.5.2 Follow-PMI testing in multi-cluster TDL channels

In this section, we demonstrate the easy configurability of the multi-cluster TDL approach by Type1 follow-PMI simulations with power parameter sweep.

In Figure 4, results for 8TX-4RX follow-PMI and random precoding in a 2-cluster TDL-C channel are shown. The channel parameters are listed in Table 1. Here, variable levels of cluster power imbalance x are experimented by setting [p0, p1] = [0, -x] dB. As can be seen from Figure 4 (Left) for rank 2, the cluster imbalance hardly affects the follow-PMI performance since one cluster is sufficient to support two layers. However, for random precoding the second cluster is useful by improving diversity, and the imbalance somewhat reduces the tput. For rank 4, as can be seen in Figure 4 (Right), the cluster imbalance has a big performance impact for both follow-PMI and random precoding.

Observation #13: In follow-PMI testing, the cluster powers of multi-cluster TDL channel model can be effectively used to tune the tput performance to the desired level.

Figure 4: 8TX-4RX follow-PMI tput in 2-cluster TDL-C channel with ranks 2 (Left) and 4 (Right).

# 2.6 Discussion of TDL channels

In this section we discuss general issues of TDL channels.

Tap Delay Line Correlation Matrix

Proposals:

- Option 1: RAN4 to consider spatially filtered TDL channel models described in 38.901 in the study.

In our view spatially filtered TDL channel models described in 38.901 can be considered as an alternative implementation to generate CDL channels. From these options we prefer using ray based CDL model because it is already widely used. However, we are happy to check if interested companies make study of such TDL models.

Proposal #16: We prefer focusing on ray based CDL models and proposed multi-cluster TDL models.

TDL Model Extensions

Proposals:

- Option 1: RAN4 to consider the option of multi-cluster TX-RX beam steering with TDL model for spatial channel modelling.

- Option 2: RAN4 to introduce enhanced TDL channel with multi-beams as one of candidate solutions for R19 SCM.

In this document we simplified “multi-cluster TX-RX beam steering” naming to “multi-cluster TDL model”. We propose companies to agree alignment simulation configuration for these models and share results in the next meeting. We propose to use configurations presented in this document in Table 1 and 2. Also, we are open to discuss other configurations if other companies have other views.

Proposal #17: We propose companies to agree multi-cluster TDL alignment simulation configurations.

Proposal #18: We propose Table 1 and Table 2 as multi-cluster TDL alignment simulation configurations.

# Conclusion

In this paper we provided the view on the spatial channel modelling. The following observations and proposals are made:

Proposal #1: The upcoming TR should provide self-contained and focused descriptions of the agreed channel models.

Proposal #2: We propose following measures to be recorded and cross-checked between interested companies

- Spatial domain/multiplexing: Average eigenmode powers, sorted from largest to smallest

- Spatial domain/directionality: Power angular distribution (PAD) of the channel in TX and RX dimensions

- Time domain: Doppler spectrum, or more simply time coherence function

- Frequency domain: Frequency coherence function

Proposal #3: We support including easy spatial configurability in SCM candidates.

Observation #1: CDL-C UMa with all AAV options is feasible with 4T4R and random Type1 precoding.

Observation #2: CDL-C UMa with all AAV options is feasible with 8T8R and best fixed Type1 precoding.

Observation #3: The choice for the best fixed Type1 precoder depends on the AAV option.

Observation #4: CDL-C UMa with AAV options 827 fully connected and fixed array size (opt X) are feasible with 8T8R and random Type1 precoding.

Observation #5: CDL-C UMa with AAV options passthrough (no AAV) and fixed subarray size (opt Y) are not feasible with 8T8R and random Type1 precoding because of bad second codeword performance.

Observation #6: CDL-C UMa with all AAV options show clear performance differences between codewords with 8T8R and best fixed Type1 precoding.

Observation #7: CDL-C UMa with all AAV options show mostly huge performance differences between codewords with 8T8R and random Type1 precoding.

Proposal #4: We propose to focus on SU-MIMO alignment first but discuss potential MU-MIMO scenarios for study.

Observation #8: CDL-C UMa with AAV via fixed TX-BF as specified in TR38.827, provides only modest gains for Type1/eType2 follow-PMI when compared to random precoding.

Proposal #5: We propose that AAV via fixed TX-BF as specified in TR38.827 is not used for follow-PMI testing.

Observation #9: CDL-C UMa without AAV, provides significant gains for both Type1 and eType2 follow-PMI when compared to random precoding.

Proposal #6: We propose to consider no AAV for follow-PMI testing in CDL channels.

Proposal #7: We propose that related to CDL angular scaling, the purpose and meaning of Option 1 should be better clarified and justified.

Proposal #8: We propose RAN4 to consider describing “angular shift” as a separate feature/process.

Proposal #9: We propose to evaluate the method for time variant beam directions in CDL model.

Observation #10: We see broadside antenna virtualiser as feasible option.

Observation #11: We see that max receive power of single strongest direction antenna virtualiser further improves the feasibility, but in the simulated cases the improvement is not crucial.

Proposal #10: We propose to focus on broadside virtualiser in TR 38.753.

Proposal #11: We propose to focus on 3.5GHz carrier frequency first but check other frequencies in the end of feasibility study.

Proposal #12: We are open to study different CDL simplification options.

Proposal #13: We propose to define SNR normalization factor as

Let normalized AAV beamformer vector of length Ms∙Ns (i.e. sub-array size) for one antenna port be

α1,1⋯αMs,Ns=1GBF1Ms∙Nsexp-j2π∙θ1,1 ⋯ exp⁡(-j2π∙θMs,Ns)

Here, amplitude normalization is separated into two factors

- 1Ms∙Ns : split transmit power of each AP evenly over Ms∙Ns transmit antenna elements

- 1GBF : normalize with measured transmit beamforming (power) gain GBF

Proposal #14: We support TR 38.827 randomness reduction framework to be included in CDL models of TR 38.753.

Proposal #15: We propose RAN4 to consider the option of multi-cluster TDL model for spatial channel modelling, to ensure RAN4 can find feasible test cases for upcoming or existing features.

Observation #12: In PDSCH testing, the cluster powers of multi-cluster TDL channel model can be effectively used to separately tune the sum tput and the per-CW tput performance to the desired level.

Observation #13: In follow-PMI testing, the cluster powers of multi-cluster TDL channel model can be effectively used to tune the tput performance to the desired level.

Proposal #16: We prefer focusing on ray based CDL models and proposed multi-cluster TDL models.

Proposal #17: We propose companies to agree multi-cluster TDL alignment simulation configurations.

Proposal #18: We propose Table 1 and Table 2 as multi-cluster TDL alignment simulation configurations.

# Reference

- RP-241610, “3GPP Work Item Description: Study on spatial channel model for demodulation performance requirements”, Nokia, BT Plc, AT&T, Bell Mobility, Bouygues Telecom, China Telecom, CMCC, Deutsche Telekom, Ericsson, Intel Corporation, KDDI, Keysight, KT Corp., MediaTek, NTT Docomo, Orange, Qualcomm, Rohde & Schwarz, Samsung, SK Telecom, Spark NZ, Telecom Italia, Telefonica, Telenor, Telia Company, Telstra, T-Mobile USA, Verizon, Vodafone, ZTE Corporation

- R4-2419782, “Way Forward for [113][320] NR_SCM”, Nokia

- R4-2415382, “Discussion on Spatial Channel Modelling”, MediaTek Inc

- R4-2500336, “Simulation results of spatial channel modelling”, MediaTek Inc