3GPP TSG-RAN WG4 Meeting # 114	R4- 2400340

Athens, Greece, 17th – 21st February 2025

Source: 	China Telecom

Title: 	Views on Power domain enhancements for single carrier

Agenda Item:	7.1.1.2.1

Document for:	Discussion

Introduction

In meeting RAN4#113, we finally drew an agreement as below that CBW extension should be introduced for converting RB allocation and further discuss the details. We also have a primary framework for approaches of calculating the new inner RB region and the extended RB numbers at each side.

- Issue 1-1-1: Approaches of converting outer RB allocation to inner RB allocation Agreement in main session: - Extended CBW based approach - where the extended CBW or aggregated CBWs for the DL-only contiguous intra-band CA case is signalled from network to UE - Both scenario 1 and 2 should be covered - The network should ensure the co-existence be met when signaling the extended CBW.

So in this contribution, we bring our perspective on the method of converting inner RB allocation to outer RB allocation and restate our views about applicable requirements.

Discussion

Details on approaches of converting RB allocation

- NW signals and extension at each side of UE CBW

- Issue 1-1-2: Approaches of calculating the new inner RB region - Options - Option 1: The NW signals the extended number of RBs and RB shift towards the UE configured CBW or RRC signalling to indicate the extended RB number at each side of the UE CBW for the UE to calculate the new inner, outer, and edge regions - The extended number of RBs may not necessarily be the same as those in the BS CBWs - The intersection of the Inner region relative to the extended UE CBW and the Outer region relative to the configured UE CBW is where the inner MPR can be expected from UE - Option 2: Based on the fixed extension ratio, it is up to the NW to judge the gap width on both sides of the UE and grant the permission to UE using the reduced MPR for the new inner region - The extension ratio on both sides, 0.5* UE CBW should be stipulated in the spec - The intersection of the Inner region relative to the extended UE CBW and the Outer region relative to the configured UE CBW is where the inner MPR can be expected from UE - Option 3: For a UE supporting [capability] and configured with a UE channel bandwidth within a carrier with a bandwidth greater than the transmission bandwidth configuration of this UE channel bandwidth, NRB = N´RB is the said carrier bandwidth as indicated by carrierBandwidth in [SIB1] and the condition for an inner RB allocation is modified accordingly - The intersection of the Inner region relative to the NW CBWs and the Outer region relative to the configured UE CBW is where the inner MPR can be expected from UE

For the options of calculating the new inner RB region in WF [1], we support option 1 and option 3 that NW should directly indicate a dynamic value about RB shift to UE, and fixed ratio deviates from the original intention of this feature. Not all scenarios can satisfy 0.5* UE CBW extension so few UE could benefit from fixed extension ratio, and NW can determine the shift value from a global perspective to ensure the co-existence problem.

Proposal1: The RB shift and NW signals should both use dynamic value.

If asymmetrical extension is introduced, NW must indicate the numbers of RB shift at each side of UE CBW to UE for calculating new inner region. In another word, we can only apply option 1 for this scenario since the central frequency is unchanged and configuring a new N’RB will cause misunderstanding at UE side.

Proposal2: For asymmetrical extension, NW must indicate the numbers of RB shift at each side of UE CBW to UE.

Fig.1 same new N’RB but different inner RB allocation region.

- Practical solution in the spec

As defined in 38.101-1, the formula of determining RB allocation can be treated as a function that NRB and LCRB are input values and RB allocation region is the output value. Any effort about converting inner to outer will concentrate on this formula eventually since the RB allocation region only depends on it. So if we want to reflect the new inner RB allocation in spec, we can introduce new input value to this formula.

NRB is the maximum number of RBs for a given Channel bandwidth and sub-carrier spacing defined in Table 5.3.2-1. RBStart,Low = max(1, floor(LCRB/2)) where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x. RBStart,High = NRB – RBStart,Low – LCRB The RB allocation is an Inner RB allocation if the following conditions are met RBStart,Low ≤ RBStart ≤ RBStart,High, and LCRB ≤ ceil(NRB/2) where ceil(x) is the smallest integer greater than or equal to x.

Observation1: Introduce new input value to the MPR formula in 38.101-1 can reflect the new inner RB allocation in spec.

The extended UE CBW actually influence the NRB in the above formula, so the RB shift values on each side should be reflected in any part of the formula contains NRB.

For two side extension, the UE will have two values: RBshift_l and RBshift_2, which denote the numbers of RB extension on the left and right side of UE CBW. Then the formula of new inner region can be changed as below:

- RBStart,Low_new = max(0, max(1, floor(LCRB/2)) – RBshift_l);

- RBStart,High_new = min(NRB – LCRB, NRB + RBshift_2 – RBStart,Low_new – LCRB);

- RBStart,Low_new  ≤  RBStart  ≤  RBStart,High_new, and LCRB  ≤  ceil((NRB+ RBshift_l + RBshift_2)/2)；

The max() and min() at the outermost layer in RBStart,Low_new and RBStart,Low_new are to make sure the new inner region is the intersection of the inner MPR region of the extended UE CBW and the original MPR region based on the UE CBW.

Proposal3: The formula of new inner region can be changed as below:

- RBStart,Low_new = max(0, max(1, floor(LCRB/2)) – RBshift_l);

- RBStart,High_new = min(NRB – LCRB, NRB + RBshift_2 – RBStart,Low_new – LCRB);

- RBStart,Low_new  ≤  RBStart  ≤  RBStart,High_new, and LCRB  ≤  ceil((NRB+ RBshift_l + RBshift_2)/2)

Besides, here we offer a feasible solution to rewrite the formula and we also do not exclude other options and we are open to discuss the details.

As for those candidate wordings, we think them all convey the same implication and all cannot be used in spec. We believe we should focus on the reflection in spec rather than the description wording.

Proposal4: We should focus on the reflection in spec rather than the description wording.

Applicable requirements

To relax the requirement and reduce MPR, the principle of determining those values should be use actual UE CBW for integration and extended UE CBW for application. Since introduced the bandwidth extension, we must utilize the shifting to calculate ACLR/SEM/SE with lower PSD. As a consequence, the OOBE, including ACLR/SEM, should be integrated with UE CBW and applied from each edge of extended UE CBW. And correspondingly, the application boundary of SE should be extended UE CBW.

Proposal5: The OOBE, including ACLR/SEM, should be integrated with UE CBW and applied from each edge of extended UE CBW.

Proposal6: The application boundary of SE should be extended UE CBW.

For the IBE, there are indeed some cases the extended UE CBW including the guard band, and the UE CBW can still be extended in case of full RB allocation, which all hinge on the scenarios and NW configuration. The purpose of bandwidth extension is relaxing OOB requirement for MPR reduction but we should use IBE for the shifting range in exchange to make sure the emission compliance.

As a consequence:

- IBE should be used in the guard band of UE CBW if it’s included in extended UE CBW;

- And for full RB allocation the extended range depends on actual spectrum allocation and NW indication, but if the extended UE CBW exists, the IBE should be applied.

Proposal7: IBE should be used in the guard band of UE CBW if it’s included in extended UE CBW.

Proposal8: For full RB allocation, if the extended UE CBW exists, the IBE should be applied.

Conclusion

Observation1: Introduce new input value to the MPR formula in 38.101-1 can reflect the new inner RB allocation in spec.

Proposal1: The RB shift and NW signals should both use dynamic value.

Proposal2: For asymmetrical extension, NW must indicate the numbers of RB shift at each side of UE CBW to UE.

Proposal3: The formula of new inner region can be changed as below:

- RBStart,Low_new = max(0, max(1, floor(LCRB/2)) – RBshift_l);

- RBStart,High_new = min(NRB – LCRB, NRB + RBshift_2 – RBStart,Low_new – LCRB);

- RBStart,Low_new  ≤  RBStart  ≤  RBStart,High_new, and LCRB  ≤  ceil((NRB+ RBshift_l + RBshift_2)/2)

Proposal4: We should focus on the reflection in spec rather than the description wording.

Proposal5: The OOBE, including ACLR/SEM, should be integrated with UE CBW and applied from each edge of extended UE CBW.

Proposal6: The application boundary of SE should be extended UE CBW.

Proposal7: IBE should be used in the guard band of UE CBW if it’s included in extended UE CBW.

Proposal8: For full RB allocation, if the extended UE CBW exists, the IBE should be applied.

References

- R4-2420527, WF on power domain enhancement, RAN4#113, Huawei, HiSilicon