# 3GPP TSG-RAN WG4 Meeting #114	                                R4-2500342

Athens, GR, 17th – 21st February, 2025

Source:                Tejas Networks Pvt. Ltd.

Title:                     Discussion on SBFD General aspects

Agenda Item:       7.23.1

Document for:     Discussion

# Introduction

This contribution presents our analysis of general issues related to SBFD, including DL/UL subband configurations.

# Discussion

## DL/UL subband configurations support for SBFD

In previous RAN4 discussion, specifically, in RAN4#113, the following agreements are achieved [1]:

Agreement: - For Rel-19, option 2b is adopted, with the following restrictions on SBFD configuration: - For DUD case, the UL subband should be placed in the middle of channel bandwidth, i.e., symmetric DUD - DL subband 1 and DL subband 2 shall have the same size or different sizes by only 1RB - For a specific channel BW supported for SBFD, maximum [2 or 3] specific UL subband sizes are allowed to be declared and shall be selected from the channel bandwidth sizes from Table 5.3.2-1 in TS38.104. - FFS different sizes for specific UL subband for DUD and DU/UD. - No further restriction is needed on the range of the ratio of the UL subband size relative to the channel bandwidth. - This restriction shall be applied to FR1 WA BS only, as the starting point - FFS the restriction is also applied to FR1 MR BS - For a specific channel BW and a specific UL subband size supported, only one DL subband size is allowed to be declared and tested for DU/UD case, and only one DL subband size is allowed to be declared and tested for DUD case - This restriction shall be applied to FR1 WA BS only, as the starting point - FFS the restriction is also applied to FR1 MR BS

In RAN4, it was decided that only DUD and DU patterns are permitted for sub-band configurations. The DU configuration has a single guard band between the DL and UL subbands, while the DUD configuration includes two guard bands. The total bandwidth occupied by the guard bands reduces the available transmission bandwidth, and this reduction percentage should be considered when determining the minimum channel bandwidth for SBFD operating bands.  In our view the {D, U, D} case, where the UL subband should be placed in the middle of channel bandwidth with {40MHz,20MHz,40MHz} bandwidth, there is an additional protection of 40MHz for the leakage from the adjacent channel CLI. Since for the SBFD symbol represented by {D, U, D}, Adjacent channel gNB-gNB CLI is seen in the UL sub-band of the SBFD symbol due to DL slots of legacy TDD from neighbouring gNBs overlapping with SBFD symbols. Assuming a channel BW of 100 MHz with [D, U, D] configuration the bandwidth will be 98.28 MHz with 273 RBs and assuming a guard band of 3.6 MHz (1.8*2) there will be five RBs will be required. There will be approximately 3.7% Transmission bandwidth reduction due to SBFD inter-subband guard band will be there. Hence an optimum value ~ 3 to 4% can be considered. Moreover, when defining the RF requirements for UL/DL subband configurations, it is feasible to adopt a configuration-agnostic approach. In this approach, the RF requirements are specified independently of the subband configuration. The supported configurations can be declared without limiting SBFD deployment to a specific set of configurations. From the Table 5.3.2-1 in TS38.104, To start with 50 MHz and 100 MHz BW cases can be selected   specific UL subband sizes are allowed to be declared and shall be selected from the channel bandwidth sizes from Table 5.3.2-1 in TS38.104. Moreover, for declaration and testing point of view DUD and DU/UD can be defined in the following table. In case of DUD the DL subband 1 and DL subband 2 shall have the same size or different sizes by only 1RB, either DL1 can be 1RB higher or lower compared to DL2.

Table 1: Sizes for specific UL subband for DUD and DU/UD.

|  | NRB, SBFD, DL | NRB, SBFD, UL | NRB, SBFD, GB |
| --- | --- | --- | --- |
| DU/UD @ SCS 30 kHz) 50 MHz 1000 MHz | 104 | 24 | 5 |
|  | 217 | 51 | 5 |
| DUD (SCS 30 kHz) 50 MHz 1000 MHz | NRB, SBFD, DL1 =50 NRB, SBFD, DL2 =49 | 24 | 5 |
|  | 106 | 51 | 5 |

Proposal 1: For 50 MHz and 100 MHz bandwidth cases, UL subband sizes must align with those in Table 5.3.2-1 of TS38.104. In DUD configurations, the two downlink subbands (DL1 and DL2) can either be the same size or differ by only 1 RB.

Proposal 2: For FR1 WA BS, the DL subband sizes declared to support SBFD shall be tested, and no restrictions should apply to other FR1 BS classes or FR2 BS.

# Conclusion

In this contribution, we discuss about SBFD general aspects with the following proposals and observations.

Proposal 1: For 50 MHz and 100 MHz bandwidth cases, UL subband sizes must align with those in Table 5.3.2-1 of TS38.104. In DUD configurations, the two downlink subbands (DL1 and DL2) can either be the same size or differ by only 1 RB.

Proposal 2: For FR1 WA BS, the DL subband sizes declared to support SBFD shall be tested, and no restrictions should apply to other FR1 BS classes or FR2 BS.

# Reference

- R4-2419907: Way forward for [113][306] NR_duplex_evo_General, Samsung.

- R4-2416134: SBFD general aspects, Ericsson