3GPP TSG-RAN WG4 Meeting #114										      R4-2500387

Athens, Greece, 17th - 21th February 2025

Agenda item:			7.11.6

Source:	MediaTek Inc.

Title:	Discussion on RRM requirements for Ku bands for NR NTN

Document for:	Discussion

# Introduction

According to WID RP-243314 [1], one objective is to specify RRM requirements including timing requirements for mobile VSAT served by NGSO.

- Specify RF requirements for mobile VSAT served by NGSO and GSO based on existing mobile VSAT type 5 RF requirements

- Investigate and if possible specify common FR1 and FR2 radiated RF requirements for Satellite Access Node (SAN) and NTN VSAT types (excluding types 1 to 3) in 38.101-5 also considering existing regulations on antenna sizes for certain parts of the Ku band.

- Priority 1: for all ITU regions, define Ku band #1a and #1b for the downlink block and uplink block B. Define two sets of bands, one for FR1 numerology and the other for FR2 numerology, including the support of the following:

  - Ku band #1a and #1b as FR1-NTN supporting 10 MHz, 20 MHz, 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing

  - Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing.

- Priority 2: for all ITU regions excluding the US in region 2, using the approach in Priority 1, define Ku band #2a and #2b as appropriate for the downlink block and uplink block A.

- Specify RRM requirements including timing requirements for mobile VSAT served by NGSO

  - NOTE:	RRM requirements are defined for VSAT supporting FR1-NTN operation only, or for FR2-NTN operation only.

- RAN#107 checkpoint: Depending on the feedback from RAN1 and RAN2 on the LS in R4-2419902, further WID updates may be needed to address feedback on specifying bands for both FR1-NTN numerology and FR2-NTN numerology or for other issues such as support for a single SCS per frequency band.

# RRM requirement for Ku band

For Ku band RRM requirement, it can leverage the existing Ka band RRM requirements as the starting point, as UE RX beam needs to be considered in Ku band as well. The current Ka band RRM requirements are summarized in Table 1.

Table 1 Ka band RRM requirements

| RRM requirements | Type 1: Electronic |  | Type 2: Mechanical |  |
| --- | --- | --- | --- | --- |
|  | intra-sat | inter-sat | intra-sat | inter-sat |
| UE uplink timing accuracy | Y* |  |  |  |
| RRC IDLE and INACTIVE mobility | Y | N | Y | N |
| (Conditional/blind) Handover | Y | Y* | Y | Y* |
| RRC Re-establishment | Y | N | Y | N |
| RRC Connection Release with Redirection | N |  |  |  |
| Radio Link Monitoring | Y |  |  |  |
| Link Recovery procedure (BFD/CBD) | N |  |  |  |
| Active TCI switching | N |  |  |  |
| L1-RSRP | Y |  |  |  |
| L3 Measurement Procedure (delay) | Y | N | Y | N |
| L3 Measurement Performance (accuracy) | Y | N | Y | N |

| N | unspecified |
| --- | --- |
| Y | Based on FR1 requirement |
| Y* | Impacted Requirement |

As the current UL Timing error for ka band are specified for FR2 SCS and Fixed VSAT in NGSO, as in Table 7.1C.2-2 and 2-3 in TS 38.133, excerpted as below, RAN4 will need to discuss the UL Timing error requirement for FR1 SCS and to include Mobile VSAT in NGSO.

Proposal 1: For UL transmit timing requirement in Ku band, RAN4 to discuss the UL Timing accuracy requirement for FR1 SCS and to include Mobile VSAT in NGSO for FR2 SCS.

According to the current Ka band RRM requirements, it can also be observed that, for intra-Satellite RRM requirement, most Ka band RRM requirements are based on FR1, by assuming same UE Rx beam is used for both serving and neighboring cells which belong to the same satellite. (i.e. no Rx beam sweeping). The same approach can be used for Ku-band.

Proposal 2: For intra-Satellite RRM requirement for Ku-band, Ka band RRM requirements can be used as the starting point.

However, for inter-Satellite Handover requirement, the interruption time differs upon UE beam steering type (i.e. ‘Electronic’ or ‘Mechanical’), as in 6.1C.1.3.2, excerpted as below, RAN4 will need to discuss whether to reuse the same value of T_sat_beam as in Ka band for Ku band.

Proposal 3: For inter-Satellite RRM requirement for Ku-band, Ka band RRM requirements can be used as the starting point. Besides, RAN4 to discuss whether to reuse the same value of T_sat_beam for Ku band Handover requirement.

# UL timing accuracy requirement for Ku band

For FR1 SCS, the current Te_NTN for FR1, as specified in R17, which has covered mobile UE in NGSO, it can be considered as the baseline for Ku band, where Mobile VSAT in NGSO should be covered. And it will be up to RF session discussion to down-select one of SCS in FR1.

Proposal 4: For FR1 SCS, the current Te_NTN for FR1 can be the baseline for Ku band. The down-select one of SCS in FR1 is up to RF session.

For FR2 SCS, the Mobile VSAT in NGSO was not covered in the current Te_NTN requirement for Ka band (FR2).

In R4#113 meeting, the Te_NTN for Mobile VSAT in GSO was revised as 7.5*Ts (from 10 Ts), while the conditions remain TBD, and it can be the starting point for Mobile VSAT in NGSO and RAN4 further discuss the requirement impact due to the difference between GSO and NGSO.

Proposal 5: For FR2 SCS, the current Te_NTN for FR2 (Mobile VSAT in GSO) can be the starting point for Ku band, and RAN4 further discuss the requirement impact due to the difference between GSO and NGSO. The down-select one of SCS in FR2 is up to RF session.

# Summary

The observations and proposals are summarized as the following:

Proposal 1: For UL transmit timing requirement in Ku band, RAN4 to discuss the UL Timing accuracy requirement for FR1 SCS and to include Mobile VSAT in NGSO for FR2 SCS.

Proposal 2: For intra-Satellite RRM requirement for Ku-band, Ka band RRM requirements can be used as the starting point.

Proposal 3: For inter-Satellite RRM requirement for Ku-band, Ka band RRM requirements can be used as the starting point. Besides, RAN4 to discuss whether to reuse the same value of T_sat_beam for Ku band Handover requirement.

Proposal 4: For FR1 SCS, the current Te_NTN for FR1 can be the baseline for Ku band. The down-select one of SCS in FR1 is up to RF session.

Proposal 5: For FR2 SCS, the current Te_NTN for FR2 (Mobile VSAT in GSO) can be the starting point for Ku band, and RAN4 further discuss the requirement impact due to the difference between GSO and NGSO. The down-select one of SCS in FR2 is up to RF session.

# Reference

- RP-243314, Revised WID: Introduction of Ku bands for NR NTN, 3GPP TSG RAN #106, Dec 2024.

- R4-2420236, Corrections to UE transmit timing for Satellite Access, R4#113 meeting, Nov. 2024.