# 3GPP TSG-RAN WG4 Meeting #114	R4-2500400

# Athens, GR, 17th – 21st February 2025

Agenda Item:	7.14.2.2

Source: 		Qualcomm Incorporated

Title: 	Discussion on FR1 NTN devices OTA

Document for:	Approval

# Introduction

In RAN4#113, WF in [1] capturing agreements and open issues for FR1 NTN OTA testing. In this paper, we provide our further views on OTA test method and performance metric for FR1 NR NTN and IoT NTN devices.

# 2. Discussion

## 2.1 Circular Polarization or Linear Polarization

How to resolve UE circular polarization issue has been discussing since RAN4#112. In RAN4#113, this topic was discussed in topic summary [2] and the following WF was agreed in [1].

Issue 3-2-4: How to resolve UE antenna circular polarization issue Proposals - Proposal 1: Use circular polarisation test antennas to assess the performance of the UE. This enables us to characterise the exact loss seen by the UE when operating in a system that utilises circular polarisation antennas, as all satcom systems do. - If we ignore circular polarisation testing at this stage, it may preclude the ability of an operator to leverage XPD to increase the capacity of their system. - Proposal 2: The NR FR1 NTN conformance testing should focus on LP rather than CP assessments. - WF - FFS

The typical UE and SAN antenna polarizations are limited in Table 1 as below.

Table 1: Typical UE and SAN antenna polarizations

| Type | Polarization | Details |
| --- | --- | --- |
| SAN antenna | circular polarization | RHCP or LHCP |
| VSAT UE | circular polarization | RHCP or LHCP |
|  | Co-phased array | Dual Linear polarization [M,N,P] = [2,4,2] |
| Handheld UE | Quasi Isotropic | Linear polarization omni-directional |
|  | Co-phased array | Dual Linear polarization [M,N,P] = [1,2,2] |

Observation 1: NTN satellite uses either RHCP or LHCP antenna.

It is known that there is 3dB polarization mismatch between each component of linear polarization (LP) and circular polarization (CP). Therefore, the mainly controversial point is whether CP or LP measurement antenna shall be used for NTN UE OTA testing. The main concern of reusing the traditional LP OTA conformance test systems to NTN UE OTA testing is that it would lead to unrealistic radiated performance.

To compare the difference of EIS/TIS results with CP and LP measurement antennas, we conducted the preliminary measurements with the following parameters and results.

- Frequency: n256

- Measurement antenna polarization: LP with H/V pol, CP with RHCP/LHCP

- EIS/TIS Measurement grid

  - Az = 0 to 360deg by 45deg, El = 0 to 90deg by 15deg step size

Table 2: Max EIS between CP (RHCP or LHCP) and Total LP (H-POL + V-POL)

| n256: 1995/2185MHz | Max EIS delta = EIS with LP – EIS with CP (LHCP) | Max EIS delta = EIS with LP – EIS with CP (RHCP) |
| --- | --- | --- |
| NTN UE | -3.07dB | -4.27dB |

It is noted that with LP and CP measurement antennas, the peak direction for EIS is changed. For example, the peak direction with LP is Theta = 0 deg, phi = 315 deg. The peak direction with LHCP is Theta = 0 deg, phi = 90 deg and the peak direction with RHCP is Theta = 15 deg, phi = 135 deg.

Observation 2: The difference between LP and CP (RHCP or LHCP) for EIS/TIS measurement is non-negligible. And the peak directions for EIS are different between LP and CP.

Currently, LP is the main antenna design for the handheld UE. However, since the satellite uses a circularly polarized antenna (e.g., LHCP or RHCP), NTN handheld UE may have more advanced antenna designs in future, such as a hybrid of LP and CP, or even CP.

For TRP measurement, we can assume that NTN satellite uses RHCP for receiving signals from three different UE devices:

- UE #1 transmits its Tx signal using a RHCP antenna

- UE #2 transmits its Tx signal using a LP antenna typically used in today’s cell phones

- UE #3 transmits its Tx signal using a LHCP antenna

Assume that each of these three UEs radiated the same Tx power level, but with a different polarization as mentioned above.

NTN satellite with RHCP receiver would receive: 1) ~ 100% of UE#1 signal, 2) ~ 50% of UE#2 signal, 3) very little of UE #3 signal

Observation 3: The difference between LP and CP (RHCP or LHCP) for EIRP/TRP measurement is non-negligible.

In addition, note that our above measurement was implemented using LP and 90degree hybrid. This does not require significant upgrades or costs to the existing test system, and the calibration process takes about 1-2 hours, which is acceptable.

Observation 4: Implementing CP using LP and 90degree hybrid does not require significant upgrades or costs to the existing test system, and the calibration process takes about 1-2 hours, which is acceptable.

In theory, CP signal and LP signal can be mathematically transformed to each other with provided magnitude and phase information.

For UL/EIRP with active measurement, the magnitude and phase difference can be measured by TE, such as analyzer. With that, the EIRP/antenna gain for the target directions for corresponding CP could be transformed by the LP antennas.

While for DL/EIS with active measurement, the magnitude and phase difference can NOT be measured by TE. Therefore, if the performance metric is the EIS for corresponding CP (LHCP or RHCP), CP measurement antennas are needed.

Therefore, we have the following proposal:

Proposal 1: To verify NTN UE OTA conformance performance, it is recommended that CP measurement antennas is used. CP measurement antennas implemented using LP with 90degree hybrid to be considered for NR NTN UE OTA testing.

## 2.2 NTN handheld UE OTA performance metric

The usage scenario and OTA performance metric for NTN handheld UE has been discussed in several previous RAN4 meetings and the following agreements were reached in RAN4#113 [1].

Issue 3-1-3: Handheld IoT-NTN and NR-NTN performance metric Agreements: - For IoT-NTN and NR-NTN handheld UEs supporting Scenario 1 the performance metrics are full sphere TRP and TRS - For IoT-NTN and NR-NTN handheld UEs supporting Scenario 2 only, further study performance metric

Issue 3-1-3: Handheld IoT-NTN and NR-NTN performance metric Agreements: - For IoT-NTN and NR-NTN handheld UEs supporting Scenario 1 the performance metrics are full sphere TRP and TRS - For IoT-NTN and NR-NTN handheld UEs supporting Scenario 2 only, further study performance metric

Issue 3-1-6: How to determine hand only mode for NR/IoT NTN mobile handheld UE (if we call it as Type 1 NTN UEs, usage-mode restriction) Agreements: - RAN4 to discuss how to distinguish UEs which support Scenario 1 (talk and browsing modes) vs. Scenario 2 only (browsing mode only)

Satellite operators’ preference on the usage of NR-NTN/IoT NTN handheld UE is to make sure NTN UE to be close to normal NR UE. Therefore, for UE supporting Scenario 1, it was agreed to use the legacy full sphere TRP and TRS as the performance metric for NR-NTN and IoT NTN handheld UE.

It is note that for the current TN network, since the base stations typically use dual orthogonal-LP antennas, it makes perfect sense to measure TRP/TIS with dual-LP source antenna for TN conformance testing.

But for the NTN network, typically either LHCP or RHCP is used as the antenna. Mathematically, total TRP/TIS values measured using dual-LP antenna and dual-CP antenna should produce the same value. Therefore, it should be clarified that for Scenario 1, the performance metric of full sphere TRP and TRS is for LHCP, RHCP or dual-CP.

Proposal 2: RAN4 should clarify that the performance metric of full sphere TRP and TRS is for LHCP, RHCP or dual-CP.

For UE only supporting Scenario 2, UE needs to use software to track the satellite, so this UE should be able to align its maximum direction as closely as possible to the satellite. Therefore, the performance metric for such UE could use the maximum EIRP and EIS and/or a conical range centered on the maximum EIRP/EIS direction as the metric. The maximum direction of EIRP/EIS can be based on the declaration of UE. Additionally, appropriate method should be defined to verify the accuracy of tracking satellite.

Proposal 3: RAN4 should consider defining a proper performance metric, e.g., Peak EIRP/EIS and/or a conical range centered on the maximum EIRP/EIS direction for corresponding CP (LHCP or RHCP) for NTN UE only supporting Scenario 2.

Proposal 4: The maximum direction of EIRP/EIS can be based on the declaration of UE. Additionally, RAN4 shall define the appropriate method to verify the accuracy of UE’s capability of tracking satellite.

Regarding how to determine hand only mode for NR/IoT NTN mobile handheld UE,

Proposal 5 The description of the usage scenarios in the product manual for NTN UE should be used to determine what’s the target scenario NTN UE supports.

# 3.	Conclusion

In this paper, we provide our views on OTA test method for FR1 NTN devices. The following observations and proposals were made:

Observation 1: NTN satellite uses either RHCP or LHCP antenna.

Observation 2: The difference between LP and CP (RHCP or LHCP) for EIS/TIS measurement is non-negligible. And the peak directions for EIS are different between LP and CP.

Observation 3: The difference between LP and CP (RHCP or LHCP) for EIRP/TRP measurement is non-negligible.

Observation 4: Implementing CP using LP and 90degree hybrid does not require significant upgrades or costs to the existing test system, and the calibration process takes about 1-2 hours, which is acceptable.

Proposal 1: To verify NTN UE OTA conformance performance, it is recommended that CP measurement antennas is used. CP measurement antennas implemented using LP with 90degree hybrid to be considered for NR NTN UE OTA testing.

Proposal 2: RAN4 should clarify that the performance metric of full sphere TRP and TRS is for LHCP, RHCP or dual-CP.

Proposal 3: RAN4 should consider defining a proper performance metric, e.g., Peak EIRP/EIS and/or a conical range centered on the maximum EIRP/EIS direction for corresponding CP (LHCP or RHCP) for NTN UE only supporting Scenario 2.

Proposal 4: The maximum direction of EIRP/EIS can be based on the declaration of UE. Additionally, RAN4 shall define the appropriate method to verify the accuracy of UE’s capability of tracking satellite.

Proposal 5 The description of the usage scenarios in the product manual for NTN UE should be used to determine what’s the target scenario NTN UE supports.

# 4.	References

- R4-2419808, WF for [113][325] TRP_TRS_Ph3, vivo

- R4-2419612, Topic summary for [113][325] TRP_TRS_Ph3