# 3GPP TSG-RAN WG4 Meeting #114		  R4-2500402

# Athens, Greece, 17– 21 February, 2025

Agenda item:			7.24.1

Source:	Qualcomm Incorporated

Title:	Discussion on Ambient IoT deployment scenario in Rel-19

Document for:	Approval

# 1  Introduction

In RAN#106, the new WI on solutions for Ambient IoT (A-IoT) in NR was approved in [1]. The target deployment scenario of Rel-19 WI on A-IoT is Scenario 1 with Topology 1, according to D1T1-B. The spectrum is FR1 licensed spectrum in FDD, with R2D in DL spectrum and D2R and CW in UL spectrum.

In this paper, we provide our views on A-IoT deployment scenario in Rel-19 RAN4 scope.

# 2  Discussion

In Rel-19 A-IoT SI, RAN4 conducted the coexistence study considering different scenarios listed in Table 6.6.1-1 and Table 6.6.1-2 of [2].

Table 1: Co-existence scenarios [2]

| Deployment scenario No. (Case No.) | Topology | Spectrum |
| --- | --- | --- |
| 1-1(a/b/c/d) | D1T1-A2- NR UE only outdoor | R2D: DL CW2D and D2R: DL |
| 1-2(a/b/c/d) | D1T1-A2- NR UE indoor | R2D: DL CW2D and D2R: DL |
| 2-1(e/f/c/d) | D1T1-B- NR UE only outdoor | R2D: DL CW2D and D2R: UL |
| 2-2(e/f/c/d) | D1T1-B- NR UE indoor | R2D: DL CW2D and D2R: UL |
| 3 (Optional)(a/b/g/h) | D1T1-A2- NR UE indoor | R2D: UL CW2D and D2R: UL |
| 5-1(e/f/g/h) | D2T2-A2- NR UE only outdoor | R2D: UL CW2D and D2R: UL |
| 5-2(e/f/g/h) | D2T2-A2- NR UE indoor | R2D: UL CW2D and D2R: UL |
| 6-1(Optional)(a/b/g/h) | D2T2-B- NR UE only outdoor | R2D: UL CW2D and D2R: DL |
| 6-2(Optional)(a/b/g/h) | D2T2-B- NR UE indoor | R2D: UL CW2D and D2R: DL |

As shown in Table 1, for D1T1-B with R2D in DL spectrum, and D2R&CW in UL spectrum, two topology cases, i.e., case 2-1 D1T1-B NR UE only outdoor and case 2-2 D1T1-B NRUE indoor were evaluated.

Based on the coexistence study, it was concluded that for case 2-1, there is no coexistence issue thanks to the penetration loss between outdoor and indoor.

While for case 2-2, non-negligible performance degradation is observed for case No. f and c

Table 2: Scenario 2-2 co-existence conclusions [2]

| Case No. | Aggressor | Victim | Co-existence conclusion |
| --- | --- | --- | --- |
| e | Device | NR UL | Negligible NR UL throughput degradation for both average throughput and cell edge throughput is observed. |
| f | NR UL | reader | Under the assumption of 10% and 100% NR UE indoor, significant SINR degradation is observed |
| c | Reader | NR DL | Under the assumption of 10% and 100% NR UE indoor, NR DL throughput degradation is observed. |
| d | NR DL | device | SINR degradation is less than 1dB. Inter-system interference is negligible. |

For Case No. f, legacy NR UE ACLR is used as Tx characteristic of aggressor, the ACS of legacy gNB is used as the Rx characteristic of victim. It should be noted that the enhancement for the RF requirements of legacy UE/gNB is not expected. Therefore, the other solutions such as increasing frequency offset between AIoT and NR system, or ensuring legacy NR UE camp on a different frequency carrier from A-IoT system when it moves to indoor.

For Case No. c, three options (30dBc, 17dBc, 12dBc with 1 black RB) was assumed as the Tx characteristic of aggressor, and legacy NR UE ACS is used as the Rx characteristic of victim. Even though ACLR of Reader (A-IoT BS) for D1T1 is still under discussion, in case the ACLR requirements of A-IoT BS is higher than 33dBc, the bottom line for this coexistence scenario is legacy UE ACS (33dBc). Similar as Case No. c, it is not expected to enhance the legacy NR UE ACS requirement in Rel-19 A-IoT WI. Therefore, the similar interference coordination solutions as Case No. f should be considered to avoid/mitigate the inter-system interference.

With above, we have the following proposals:

Proposal 1: If RAN4 agrees to specify RF requirements based on the assumption that NR UE is only outdoor, RAN4 needs to discuss how to address the coexistence issue when legacy NR is indoor.

Proposal 2: The following potential solutions can be considered:

- Specify mechanism to ensure legacy NR UE only outdoor

- Specify enough frequency offset between NR and A-IoT systems

- Specify appropriate RF requirements such as selectivity requirement for the device

# 3  Conclusion

In summary, we have following proposals.

Proposal 1: If RAN4 agrees to specify RF requirements based on the assumption that NR UE is only outdoor, RAN4 needs to discuss how to address the coexistence issue when legacy NR is indoor.

Proposal 2: The following potential solutions can be considered:

- Specify mechanism to ensure legacy NR UE only outdoor

- Specify enough frequency offset between NR and A-IoT systems

- Specify appropriate RF requirements such as selectivity requirement for the device

# 4  References

[1] RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, RAN1 Vice-chair (Huawei)

[2] 3GPP TR 38.769 V2.0.1

[3] 3GPP TS 38.161 V18.1.0