3GPP TSG-RAN WG4 Meeting #114						R4-2500435

Athens Greece, 17th -21st February 2025

Agenda item:			7.11.2

Source:	Intelsat, Gatehouse Satcom

Title:	Ku Band Co-existence Simulation Assumptions (Draft 1)

Document for:	Information and Discussion

# Introduction

In this document, we present first draft of the simulation assumptions for the co-existence study between 3GPP Services in the Ku Band and other services within and adjacent to it.

Unlike previous co-existence studies carried out for the Ka Band, where fictitious cellular services were assumed adjacent to the Ka band, the situation with the Ku Band is very different. Within and adjacent to the Ku Band there already are several non-cellular services active and there is no allocated spectrum for IMT/Cellular services adjacent to the Ku Band.

Although the co-existence studies carried out by 3GPP working groups only deals with 3GPP technologies, we should also be aware of sensitive non-3GPP services which are active adjacent to and within the Ku Band.

Due to the volume of information in this document, and for ease of use, a Table of Content is provided below:

Contents													Page

1	Introduction	1

2	Ku Band Spectrum	3

3	Ku Band Satellite Components’ Summary	4

4	Terrestrial Components’ Summary	4

4.1	3GPP Services	4

5	Simulation Assumptions	4

5.1	Interference Scenarios	4

5.2	Beam Pattern and Frequency Re-use	6

5.3	Polarization	7

6	High Level NTN and TN Assumptions	8

7	Simulation Parameters	9

7.1	Network Layout Model	9

7.1.1	Co-existence between NTN and TN	9

7.2	NTN Parameters	11

7.2.1	SAN/Satellite Parameters	11

7.2.2	Terminal/VSAT Parameters	13

7.2.3	Nosie Figure for NTN SAN and UE	14

7.2.4	Additional VSAT/UE Parameters	14

7.3	Terrestrial Parameters	17

7.3.1	Terrestrial UE Parameters	18

8	Antenna Aperture and Beam Forming Pattern Modelling	18

8.1	Satellite and UE Antenna and Beam Forming Pattern Modelling	18

8.2	TN BS and UE antenna and beam forming pattern modelling	21

8.2.1	BS antenna	21

8.2.2	UE Antenna Parameters	22

9	Propagation Modell	23

10	Transmission Power Control (TPC) Model	24

10.1	UL Transmission Power Control	24

10.2	Downlink Transmission Power Control	24

11	Receiver Power Control (RPC) Model	24

12	Performance Matric	25

13	Throughput- SNR Mapping	25

14	Incomplete Parameters Values	25

15	Conclusion	25

16	References	25

17	Appendix	26

17.1	Other Non-3GPP Services	26

17.2	Overview of existing ITU Radio Regulations applicable to Ku-band	26

17.3	Interference Scenarios	30

17.4	Consideration of Deployment Scenarios	34

# Ku Band Spectrum

The current and planned services of the Ku Band are illustrated in the following Figure 1

As it can be observed from Figure 1, the electromagnetic spectrum around the Ku Band is very crowded and there are established and licensed services operating in the Ku Band.

Figure 1 Electromagnetic Spectrum around the Ku Band

With reference to Figure 1 we can observer that:

- There are Radio Astronomy Bands between 10.68 and 10.70 GHz.

- In Regions 1 and 3, there are Multiple Scientific Services Bands between 10.40 and 10.68 GHz.

- In Regions 1 and 3, spectrum between 10.0-10.40 is reserved for Scientific Services.

- In Region 2, 10.0-10.5 GHz band was identified for IMT(5G/6G) services by WRC-23 in 12 countries. Please note that part of this band (10.0-10,4 GHz) band is shared with Scientific Services.

- The spectrum between 12.75-13.25 GHz is reserved for Fixed Satellite Services.

- The spectrum between 13.25-13.75 GHz is reserved for Scientific Services and military NATO Radar.

- The spectrum between 13.75-14.80 GHz is reserved for Fixed Satellite Services.

- The spectrum between 14.80-15.35 GHz has been selected to be studied for IMT services for WRC-27 meeting: this band is currently under study and no services have yet been assigned.

Thus, we can observe that there are no terrestrial services adjacent to the Ku Band.

For further clarification of ITU Radio Regulations of the Ku Band, please see section 17.2

# Ku Band Satellite Components’ Summary

Satellite Components:

- Architecture Type: Transparent Architecture

- Types of Satellite used: GEO, LEO at 600 Km, LEO at 1200 Km

UE/VSAT Component

- Fixed and Mobile VSAT connected to GSO and NGSO Satellites

- Circular polarization is assumed on antennas

# Terrestrial Components’ Summary

## 3GPP Services

For the purpose of the simulation the following are assumed for the Terrestrial Components

- Non-3GPP Services adjacent to and within the Ku Band Spectrum

- For IMT/cellular services 3GPP ACLR and ACS specification to be used

- For non-3GPP Services ITU protection levels are used

# Simulation Assumptions

## Interference Scenarios

Various interference scenarios are illustrated in the following Figure 2.

Figure 2 Possible Interference Scenarios between VSAT, UE, gNodeB and the Satellite

Bearing in mind the Ku Band spectrum, see Figure 1, there are a few possible interference combinations available which are tabulated in the following Table 1

Table 1 Interference Scenarios

| No. | Combination | Aggressor | Victim | Notes on the Ku Band | Scope of Coexistence Simulation |
| --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN | VSAT UL | UE UL | Ku Band VSAT uplink interfering with gNodeB receiver. | The ACLR of the VSAT to be varied/defined. The ACS of the gNodeB is fixed. |
| 2 | TN with NTN | UE UL | VSAT UL | UE uplink interfering with Ku Band Satellite receiver. | The ACLR of the UE is fixed. The ACS of the satellite to be varied/defined. |
| 3 | TN with NTN | VSAT UL | gNodeB DL | Ku Band VSAT uplink interfering with UE receiver. | The ACLR of the VSAT to be varied/defined. The ACS of the UE is fixed. |
| 4 | TN with NTN | gNodeB DL | VSAT UL | gNodeB downlink interfering with Ku Band Satellite receiver. | The ACLR of the gNodeB is fixed. The ACS of the Satellite to be varied/defined. |
| 5 | TN with NTN | gNodeB DL | Satellite DL | gNodeB downlink interfering with Ku Band VSAT receiver. | The ACLR of the gNodeB is fixed. The ACS of the VSAT to be varied/defined. |
| 6 | TN with NTN | Satellite DL | gNodeB DL | Ku Band Satellite downlink interfering with UE receiver. | The ACLR of the Satellite to be varied/defined. The ACS of the UE is fixed. |
| 7 | TN with NTN | Satellite DL | UE UL | Ku Band Satellite downlink interfering with gNodeB receiver. | The ACLR of the Satellite to be varied/defined. The ACS of the gNodeB is fixed. |
| 8 | TN with NTN | UE UL | Satellite DL | UE uplink interfering with Ku Band VSAT receiver. | The ACLR of the UE is fixed. The ACS of the VSAT to be varied/defined. |
| NOTE 1: For coexistence between Ku Band DL and adjacent TN bands, there are no 3GPP defined/specified TN bands. |  |  |  |  |  |

See also section 17.3 for the illustration of the Interference Scenarios.

The proposed ACLR and ACS values are shown in the following Table 2

Table 2 Proposed ACLR and ACS of the Satellite and Terrestrial Components

| Carrier Frequency (GHz) | Carrier Frequency Bandwidth (MHz) | Satellite gNodeB |  | VSAT UE |  | ACLR of Terrestrial BS |  | ACLR of Terrestrial UE |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | ACLR | ACS | ACLR | ACS | ACLR | ACS | ACLR | ACS |
| 11 DL for Satellite | 200 | Minimum [TBD] Note 1 | Minimum [TBD] Note 1 | Minimum [TBD] Note 1 | Minimum [TBD] Note 1 | [TBD] | [TBD] | [TBD] | [TBD] |
| 14 UL for satellite | 200 | Minimum [TBD] Note 1 | Minimum [TBD] Note 1 | Minimum [TBD] Note 1 | Minimum [TBD] Note 1 | [TBD] | [TBD] | [TBD] | [TBD] |

Note 1: ACLR and ACS values of the NTN Components will be determined from the Co-existence simulations

## Beam Pattern and Frequency Re-use

The following Figure 3 illustrates the beam patterns and Frequency Re-use for the co-existence simulation

Figure 3 Satellite Beam Patterns for the Simulations

## Polarization

Throughout this simulation exercise to provide orthogonality between the beams, we assume the beams to have alternate polarization, such that the adjacent beams have Left-Hand and Right-Hand polarizations.

See following Figure 4.

Figure 4 Polarization of the Satellite Beams for the Simulations

# High Level NTN and TN Assumptions

Table 3 High Level TN and NTN Simulation Parameters

| Item | Variations | Exclusions | Remarks |
| --- | --- | --- | --- |
| NTN SAN | GEO LEO @1200 km LEO @ 600 Km |  |  |
| NTN VSATUE | Fixed and Mobile VSAT for GEO |  |  |
|  | FIXED and Mobile VSAT for NGSO |  |  |
| Simulation Channel Bandwidth for NTN | 200 MHz per beam |  |  |
| Simulation Channel Bandwidth for TN | 200 MHz per beam |  |  |
| Simulation Scenario | Urban Macro Scenario | Rural Macro Scenarios |  |
| NTN and TN Antenna Type | Circular aperture antenna model for SAN and NTN UE |  | Phase Array Antenna may be considered for the Co-existence simulation |

# Simulation Parameters

## Network Layout Model

For the interference calculation between the NTN and TN an environment must initially be setup. In this section the agreed simulation environment parameters are listed:

### Co-existence between NTN and TN

Cellular cell structure is considered for both NTN and TN network layout. See following Figure 5 .

#### Coordination System

Referring to TR 38.811 Section 6.3 and Annex A, a 3D global coordinate system is considered (Earth-Centered Earth Fixed) for simulating NTN beams direction and location on the earth surface. It means the NTN beam location, TN randomly dropping location are generated with a set of three parameters (x,y,z).

#### Simulation Methodology

Following simulation methodology can be used for NTN-TN co-existence study.

Phase 1 Generate Aggressor and Victim Networks.

- FRF=2 with two polarizations (RHCP, LHCP)

Figure 5 Satellite Circular Beam Patterns and Polarization

- Do not consider interference leakage from adjacent beams as starting point for co-ex study.

  - For example, for the figure above, do not consider the interference leakage from green, blue and purple beams when study SINR for the red beam.

- Deployment of TN network (19 cells with wraparound) refers to Section 17.4

Phase 2 Setup UE Associations

- TN UE are generated randomly inside the TN network, make sure enough TN UEs are associated to each TN sectors based on coupling loss.

- Deployment of NTN UE refers to Section 17.4.

- Consider following approaches for the deployment of NTN UE and satellite

- The Satellite should be generated in the visible area/sky of NTN UE.

- NTN UEs point to the satellite accurately.

- The position of the satellite should guarantee that NTN UE vertical angle towards the satellite.

- For calibration, use 30 degree and 90 degrees instead of the range.

- SAN elevation angle value, 25 & 90 degree (computed from the centre of the beam), for co-existence simulation.

- Horizontal angle of NTN UEs should be calculated based on the satellite position

- Use 35m as minimum distance assumed between VSAT and TN BS for co-ex study.

Phase 3 Setup Scheduling

Once association is done, round robin scheduling is used.

BF weights are adjusted to point to the LOS direction between BS-UE. This is done for both victim and aggressor networks.

Phase 4 Calculate Throughput

Throughput is computed in the victim systems without considering ACI as below:

- ThputNO ACIbpshz=fSINRICI=fSN+IICI, where IICI is the inter-cell interference.

For TN-NTN SINR calculation, the satellite receiver off angle should be considered in the satellite receiver gain calculation when calculating SINR. Note that such angle is not considered in TR 38.821 section 6.1.3 equations. Thus, those equations should be used for SINR calculation.

Phase 5 Calculate ACI

Throughput is computed considering ACI as below:

-	ThputACIbpshz=fSINRICI+ACI=fSN+IICI+IACI, where IACI is the adjacent channel interference.

Phase 6 Calculate the RF Parameters

RF parameters are determined based on the degradation cause by ACI as below:

-	LossACI=1-ThputACIThputSINGLE.

To simplify the simulation of interference from TN to NTN UL in Case 2 and 6, following method can be used. Consider the active TN cells from central NTN beam for the ACI evaluation from TN to NTN UL. There is a view that simplifying such coexistence simulation work for Case 2 may even not be required.

- Step 1: to drop NTN UE per NTN beam-print randomly.

- Step 2: to drop N terrestrial clusters consisting of 57 sectors per NTN beam-print randomly

- Step 3: to calculate the total ACI per beam to NTN UL by following scaling factor:

where:

active_TN = active_factor*round (the area per beam/the area of 57 sectors)

active_factor = 20% (or lower, particularly for urban scenarios)

- Step 4: to calculate the total ACI from all beams (e.g. M=7) for NTN:

## NTN Parameters

### SAN/Satellite Parameters

Set-1 satellite parameters are shown in following Table 4 and Table 5 according to TR 38.821.

The satellite max Tx power can be calculated by the equation as below:

maxTx powerdBm=EIRP densitydBWMHz+30+10 log10NRB*SCSMHz*12-Max GaindBi

Table 4 NRB Configuration per Bandwidth Size and SCS

| Configuration | Bandwidth (MHz) | NRB |
| --- | --- | --- |
| FR1 30 kHz SCS | 100 | 273 |
| FR2 120 kHz SCS | 200 | 132 |

Table 5 Set-1 Satellite Parameters for Co-existence Study

| Satellite orbit |  |  | GEO |  | LEO-1200 |  | LEO-600 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Satellite altitude |  |  | 35786 km |  | 1200 km |  | 600 km |  |
| Equivalent satellite antenna aperture |  |  | 9.1m/ 7.14m (DL/UL) |  | 0.91m/00714 m (DL/UL) |  | 0.91m/00714 m (DL/UL) |  |
| Payload characteristics for DL transmissions |  |  |  |  |  |  |  |  |
| Satellite EIRP density |  | Ku-band | 43 dBW/MHz |  | 3 dBW/MHz [ |  | 1 dBW/MHz |  |
| Satellite max TX power in dBm | BW (MHz) |  | 200 | 200 | 200 | 200 | 200 | 200 |
|  | SCS kHz |  | 120 | 30 kHz for FR1 120 kHz for FR2 | 120 | 30 kHz for FR1 120 kHz for FR2 | 120 | 30 kHz for FR1 120 kHz for FR2 0 |
|  | NRB |  | 132 | 264 | 132 | 264 | 132 | 264 |
| Satellite Tx max Gain |  |  | 58.5 dBi |  | 38.5 dBi |  | 38.5 dBi |  |
| 3dB beamwidth or HPBW (Half-Power Bandwidth) of main central beam |  |  | 0.1765 deg |  | 1.7647 deg |  | 1.7647 deg |  |
| ABS (Adjacent Beam Spacing) of adjacent beams from the central beam |  |  | Note 1 |  | Note 1 |  | Note 1 |  |
| Satellite beam diameter |  |  | 110 km |  | 40 km |  | 20 km |  |
| Payload characteristics for UL transmissions |  |  |  |  |  |  |  |  |
| Satellite Rx max Gain |  |  | 58.5 dBi |  | 38.5 dBi |  | 38.5 |  |
| Note 1: ABS[rad] = sqrt(3) x sin(HPBW[degrees]/2) or ABS[rad] = sqrt(3) x sinr(HPBW[rad]/2) - with ABS [degree]=180/pi x ABS[rad] and - with HPBW the Half-Power Bandwidth of the main lobe from the satellite antenna pattern. |  |  |  |  |  |  |  |  |

[above table to be reviewed]

Additional Parameters agreed for the simulation are shown in following Table 6, Table 7 and Table 8

Table 6 Other Parameters for NTN

| Parameters | NTN | Remark |
| --- | --- | --- |
| Carrier frequency | 14 GHz (UL) / 11 GHz (DL) [TBD] |  |
| The number of active UE (UL) | 10 UEs and 13RBs per UE for GEO and LEO |  |
| The number of active UE (DL) | 1 | Same with TN |
| Traffic model | Full buffer |  |
| DL power control | NO |  |
| UL power control | See Session 10.1 |  |
| NTN satellite Noise figure in dB | See Table 7[ |  |
| Handover margin | 3dB |  |

Table 7 NTN Satellite Noise Figure in dB

| Satellite | GEO | LEO 600 | LEO 1200 |
| --- | --- | --- | --- |
| G_Rx (dBi) | 58.5 | 38.5 | 38.5 |
| Noise Figure for calibration is 5.9 for all satellite types. |  |  |  |

Additional greed SAN Parameter Values, from RAN4 meetings [2], are listed in the following

Table 8  Additional Agreed Values for the SAN

| Parameter | Values | Remarks |
| --- | --- | --- |
| SAN/Satellite Equivalent Antenna Aperture | 9.1 m DL, 7.14 UL (GEO), 0.91 m DL, 0.714 UL (LEO DL 600 & 1200) |  |
| SAN/Satellite Antenna Pattern | Section 6.4.1 from 38.811 to be used. |  |
| SAN/Satellite Tx Antenna Maximum Gain | 58.5 dBi (GEO), 38.5 dBi (LEO 600 & 1200) |  |
| SAN/Satellite Rx Antenna Maximum Gain | 58.5 dB (GEO), 38.5 dB (LEO 600 & 1200) |  |
| SAN/Satellite Antenna Beamwidth | 0.177 (GEO), 1.768 (LEO 600 & 1200) |  |
| SAN/Satellite Maximum EIRP | 43 dBW/MHz (GEO), 3 dBW/MHz (LEO 1200), 1 dBW/MHz (LEO 600) | using 100 MHz bandwidth |
| Coexistence TN channel model | For TN channel model, reuse existing version of TR 38.901. |  |
| Noise figure of the SAN/ Satellite | 3 dB |  |
| SAN G/T | 30.9 dB/K and 10.9 dB/K based on 3 dB NF | [need to elaborate] |

### Terminal/VSAT Parameters

UE parameters are shown in following Table 9

Table 9 VSAT/UE Characteristics for System Level Simulations

| Characteristics | Fixed VSAT | L-ESIM |
| --- | --- | --- |
| Frequency band | 14 GHz UL/11 GHz DL | 14 GHz UL/11 GHz DL [TBD] |
| Antenna type and configuration | Directional Section 6.4.1 of [3] with 60 cm equivalent aperture diameter [TBD] | Directional Section 6.4.1 of [3] with 60 cm equivalent aperture diameter [TBD] |
| Polarisation | Circular | circular |
| Rx Antenna gain | 34.3 dBi [TBD] | 39.7 dBi [TBD] |
| Antenna temperature | 150 K | 150 K |
| Tx transmit power | 50 W for GSO 19 W for NGSO | 2 W (33 dBm) |
| Tx antenna gain | 36.6 dBi (option 1 2W), 36.2 dBi (option 2 10W) | 43.2 dBi[TBD] |

[parameters to be reviewed]

The type of phase array antenna and its radiation pattern are yet to be decided

For calibration, the number of active NTN UE (UL) is 10 UEs with divided RBs per UE by channel bandwidth.

For calibration, NTN UE antenna elevation angle is 30 and 90 degrees. For co-ex study, use 25 and 90 degrees.

For calibration, NTN UE NF is 4dB

Table 10 Altitude of VSAT/UE from the ground

| NTN UE scenario | Fixed VSAT (on roof) | M-ESIM | A-ESIM | L-ESIM |
| --- | --- | --- | --- | --- |
| Altitude | 22.5m | 22.5m | 3-14km | 1.5m |

### Nosie Figure for NTN SAN and UE

The following Table 13 illustrates the Satellite SAN and VSAT/UE proposed Noise Figures:

Table 11 Satellite SAN and VSAT/UE Noise Figures

|  | SAN NF (dB) | VSAT/UE NF (dB) |
| --- | --- | --- |
| For GEO | 3 [TBD] | Type 1: 2.5 dB for GEO |
| For LEO (option 1) | 3 [TBD] | Type 1: 2.5 dB for VSAT LEO Type 2: 6 dB for VSAT to LEO |
| For LEO (option 2) | 3 [TBD] | Type 1: 2.5 dB for VSAT LEO Type 2: 6 dB for VSAT to LEO |

### Additional VSAT/UE Parameters

The following Table 12 lists additional VSAT/UE Parameters agreed in RAN4 meetings [2] which will be used in the co-existence simulations:

Table 12 Additional VSAT/UE Parameters

| Parameters | Values | Remarks |
| --- | --- | --- |
| VSAT Equivalent Antenna Aperture | 60 cm Equivalent antenna aperture to be used |  |
| VSAT Polarization | Assume Circular Polarization first and if necessary, derive the linear result |  |
| VSAT Maximum Tx Antenna Gain | 36.6 dBi (option 1 2W), 36.2 dBi (option 2 10W) |  |
| VSAT Maximum Rx Antenna Gain | 34.3 dBi |  |
| VSAT Noise Figure | Type 1: 2.5 dB for VSAT LEO and GEO Type 2: 6 dB for VSAT to LEO | Other options are not precluded |
| VSAT Max Tx Power | 40 dBm (10 W) |  |
| Channel Models for NTN and TN | - For the NTN use 38.811 is the channel model for satellite used for Ka. It can be scaled for Ku. - For TN use 38.863 and change the frequency to 11 & 14 GHz | [ to be reviewed] |
| VSAT Antenna Pattern | - VSAT antenna model, to be used for the co-existence study - Parabolic Antenna to be studied first - Phased Array (flat panel) antenna modelling to progress in parallel - To use TR 38.921 in the co-existence modelling and for su0array to use TR 38.803 | The following Table 4 VSAT parameters were agreed: |

And following Table 13, lists the VSAT Phase Array Parameters

Table 13 VSAT Phased Array Antenna Characteristics.

| Ku band VSAT Phased Array Antenna Characteristics |  |  |
| --- | --- | --- |
| 1 | Ku band VSAT phased array antenna Characteristics |  |
| 1.1 | Antenna pattern | Table 2-2 and 2-3 (TR 38.921) |
| 1.2 | Element gain (dBi) (Note 2) | 3.5 |
| 1.3 | Y axis / Z axis element 3 dB beam width of single element (degree) | 90° for Y axis 90° for Z axis |
| 1.4 | Y axis / Z axis element front-to-back ratio (dB) | 30 dB |
| 1.5 | Antenna polarization | Circular（RHCP or LHCP） |
| 1.6 | Antenna array configuration (Y axis × Z axis) (Note 4) | 45x45 elements |
| 1.7 | Number of supported polarizations, P | 1 |
| 1.8 | Y axis / Z axis radiating element spacing | 0.5 lambda |
| 1.9 | Array Ohmic loss (dB) (Note 2) | 2 |
| 1.10 | Conducted power (before Ohmic loss) per antenna element (dBm) (Note 3) | [4.44 dBm] TBA |
| 1.11 | maximum coverage angle in the horizontal plane (degrees) | 0~360 degrees |
| 1.12 | vertical coverage range (degrees) between beam direction and normal direction | 0~60 degrees |
| 1.13 | normal direction | Toward X+ axis |

Note 1:	Void

Note 2:	The element gains in row 1.2 includes the loss given in row 1.9.

Note 3:	The conducted power per element assumes Y axis × Z axis ×Number of supported polarizations elements (i.e. power per Y axis / Z axis polarized element).

Note 4:	All the Y axis × Z axis elements are assumed as the horizontal radiating elements in the horizontal plane.

Note 5: Similar to subtopic 2-4, other options not precluded e.g. antenna array configuration (Y axis × Z axis).

[Conducted power is undefined]

Following Table 14lists additional VSAT and UE Antenna Parameter agreed during previous RAN4 meetings:

Table 14 Additional VSAT/ UE Parameters

| Parameters | Values | Remarks |
| --- | --- | --- |
| VSAT Max Tx Antenna Gain | The worst case, i.e. the GSO, to be used initially with 100 cm antenna aperture (41.3dBi), 88.2 dBm EIRP. Other options e.g. different size, EIRP and antenna type, are not precluded The antenna model for NTN VSAT uses circular polarization, but since many GEO satellites use linear polarization, evaluations are conducted by converting to linear polarization |  |
| VSAT Noise Figure | 2.5dB and 6-dB Noise Figures to be used, see R4-2419724, as per agreement for the Ka Band |  |
| TN BS/UE antenna patterns for 11 GHz | For the TN BS and UE antenna and beam forming pattern modelling to be used as specified in in TS 38.921 for the 11 GHz Carrier Frequency |  |
| TN BS Antenna Pattern for 14 GHz | For the TN BS antenna and beam forming pattern modelling as specified in TS 38.922 to be used for 14GHz carrier For the TN UE, the isotropic antenna model will be used as specified in TS 38.922 |  |
| VSAT Maximum Tx power | 50 W for GSO and 10 W for NGSO. |  |
| Noise Figure of the VSAT/UE | [to be decided] |  |

.

## Terrestrial Parameters

Table 15 Simulation Assumptions of TN NR

| Parameters | Values |
| --- | --- |
| Carrier Frequency in GHz | 14 GHz UL/11 GHz DL |
| Size of each nominal channel BW in MHz | 200 |
| Transmission bandwidth in MHz | 190.8 |
| Environment | Urban macro |
| Network layout | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |
| Inter-site distance in meter | 300m |
| UE number per cell | Same as the number of BS beam |
| System loading and activity | 20% |
| Network location | See section 17.4 |
| DL subcarrier spacing | 30 kHz SCS for FR1 120 kHz SCS for FR2 |
| UL | OFDMA |
| DL power control | No |
| UL power control | See Section 10.1 |
| Frequency reuse | 1 |
| Number of scheduled UE per cell (DL) | Same as the number of BS beam |
| Number of scheduled UE per cell (UL) | Same as the number of BS beam |
| BS height (m) | 25 |
| UE antenna height (m) | 1.5 |
| UE TX power in dBm | -40 to 23 |
| Building penetration loss | In path loss model, TR 38.901 |
| Cell selection margin in dB | 3 |
| BS-MS min distance (m) | 35 |
| BS noise figure in dB | 10 |
| UE noise figure in dB | 10 |
| BS-UE path-loss model | TR 38.901 |
| Standard deviation of BS-UE log-normal shadow fading in dB | Deployment scenario related, referring to TR 38.901. |
| Shadowing correlation | Between cells: 1.0 Between sites: 0.5 |
| Link-level performance model | [TBD] |
| UE distribution | Uniform |
| Evaluation metrics | See Section 12 |

### Terrestrial UE Parameters

The following Table 16, illustrates the Terrestrial BS and UE ACLR and Acs values, agreed for the simulation

Table 16 ACLR/ACS for TN

| Carrier Frequency (GHz) |  |  | NR |
| --- | --- | --- | --- |
| 14 GHz UL/11 GHz DL | BS | ACLR | [TBD] |
|  |  | ACS | [TBD] |
| 14 GHz UL/11 GHz DL | UE | ACLR | [TBD] |
|  |  | ACS | [TBD] |

# Antenna Aperture and Beam Forming Pattern Modelling

## Satellite and UE Antenna and Beam Forming Pattern Modelling

Satellite and UE Antenna and beam forming pattern modelling of satellite could be referred to section 6.4.1 in TR 38.811.

Satellite antenna pattern

Parabolic antenna pattern for SAN and UE:

In Rel-18, antenna pattern in section 6.4.1 of TR38.811 is reused for NTN SAN and NTN UE. Use of phased array antenna model needs further discussion.

The following normalized antenna gain pattern, corresponding to a typical reflector antenna with a circular aperture, is considered.

1                for θ=0

4J1kasinθπ180kasinθπ1802        for 0<θ≤90°

For UE, 4J1kasinπ2kasinπ22          for θ>90°

where:

- J1(x) is the Bessel function of the first kind and first order with argument ‘x’;

- ais the radius of the antenna's circular aperture.

- k = 2f/c is the wave number.

- f is the frequency of operation.

- c is the speed of light in a vacuum and  is the angle measured from the bore sight of the antenna's main beam.

Note that ka equals to the number of wavelengths on the circumference of the aperture and is independent of the operating frequency. And the sin() function is in radian.

Satellite and UE beam forming pattern

The following table is agreed for the beam layout definition for a single satellite simulation in S-Band.

Table 17 Satellite and Beam Forming Pattern Parameters

| Beam layout definition | Baseline: Hexagonal mapping of the beam bore sight directions on UV plane defined in the satellite reference frame. Only the 3dB beam width parameters should be used. The beam diameter and beam spacing values can be computed directly from the 3 dB beam width assumptions and should be considered as informative. |
| --- | --- |
| Number of beams | Baseline: 7-beam layout (i.e. 6 co-frequency beams surrounding the central beam) |
| UV plane illustration (extracted from [19]) |  |
| UV plane convention | U axis is defined as the perpendicular line to the satellite-earth line on the orbital plane as illustrated here after: The straight line being orthogonal to UV plane is pointing towards the Earth centre. UV coordinates of the nadir of the reference satellite is (0,0) |
| Adjacent beam spacing on UV plane | Baseline: Adjacent beam spacing computation based on 3dB beam width of the satellite antenna pattern: ABS[rad] = sqrt(3) x sin(HPBW[degrees]/2) or ABS[rad] = sqrt(3) x sinr(HPBW[rad]/2) with ABS [degree]=180/pi x ABS[rad] and with HPBW the Half-Power Bandwidth of the main lobe from the satellite antenna pattern. |
| Central beam bore sight direction definition | Baseline: Case 1: Central beam centre is considered at nadir point Case 2: 25° for GEO and LEO Interested companies can bring analysis and results for other values. |
| Frequency-reuse factor | FRF=2 Figure 6 Satellite Beams and Polarization Do not consider interference leakage from adjacent beams as starting point for co-ex study. For example, for the figure above, do not consider the interference leakage from green, blue and purple beams when study SINR for the red beam. |
| Central beam bore sight direction definition | Baseline: Case 1: Central beam centre is considered at nadir point Case 2: 25° for GEO and LEO Interested companies can bring analysis and results for other values. |
| Polarization re-use | Enable Note: Polarization re-use should apply only if circular polarization for terminal antenna is considered |
| UEs outdoor/indoor distribution | 100% outdoor distribution for UEs |
| UE distribution | The cell area associated to a given beam is defined as the Voronoi cell associated with the corresponding beam centres. |
| UE configuration | Fixed and mobile VSAT for GSO Fixed VSAT for NGSO |
| UE orientation | Ideal Tracking serving beam |
| UE attachment | RSRP |

NOTE 1: Typical impairment values (additional frequency error, SNR loss) due to the feeder link except for delay can be negligible. When available, specific values can be considered in the evaluation and should be reported.

NOTE 2: For the calibration purpose, the ionospheric scintillation loss shall be considered equal to zero (i.e., the UEs are located between 20 and 60 degrees of latitude).

## TN BS and UE antenna and beam forming pattern modelling

### BS antenna

Table 18 BS antenna modelling for Urban macro scenario

| Parameter | Macro urban |
| --- | --- |
| Antenna pattern | TR 38.803 |
| Element gain GE,max (dBi) | 5.5 |
| Horizontal j3dB /vertical q3dB 3 dB beam width of single element (degree) | 90º for H 90º for V |
| Horizontal/vertical front-to-back ratio Am (dB) | 30 for both H/V |
| Side lobe suppression SLAv (dB) | 30 |
| Antenna polarization | Linear ±45º |
| Antenna array configuration (Row × Column) | 16 × 8 elements |
| Horizontal/Vertical radiating element spacing | dh = 0.5 λ dv = 0.5λ |
| Array Ohmic loss LE (dB) | 2 |
| Conducted power (before Ohmic loss) per antenna element (dBm) | 22 |
| Mechanical down tilt (degrees) | 10 |

Table 19 BS Antenna Tsdostong PatternSimulation Parameters

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) | AE,Vθ"=-min12θ"-90°θ3dB2, SLAV, θ3dB=90°， SLAV=30dB |
| Antenna element horizontal radiation pattern (dB) | AE,Hφ"=-min12φ"φ3dB2, Am, φ3dB=90°， Am=30dB |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 5.5 dBi |
| (Mg, Ng, M, N, P) note | For [TBD] GHz: (1, 1, 8, 16, 2) |
| (dv, dh) | (0.5λ, 0.5λ) |
| Note: An additional 3dB gain is added to the total beamforming gain to account for the two polarization directions. Boresight direction is horizontal. |  |

### UE Antenna Parameters

Refer to TR 38.803.

The centre frequency for the terrestrial antenna parameters is to be decided.

Table 20 Antenna Parameters for [TBD] GHz

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 5 dBi |
| (Mg, Ng, M, N, P) | (1, 1, 2, 2, 2) |
| (dv, dh) | (0.5λ, 0.5λ) |
| UE orientation | Random orientation in the azimuth domain: uniformly distributed between -90 and 90 degrees* Fixed elevation: 90 degrees |
| NOTE: This is done to emulate two panels: the configuration is equivalent to 2 panels with 180 shift in horizontal orientation and UE orientation uniformly distributed in the azimuth domain between -180 and 180 degrees. |  |

# Propagation Modell

[To be reviewed]

Table 21 Proposed Propagation Model for the Co-existence Simulation

| Link | Propagation model |
| --- | --- |
| TN BS to Fixed VSAT on roof | Free space path loss |
| TN BS to L-ESIM at 1.5 m | UMa as in 3GPP TR 38.803 |
| TN BS to TN UE | UMa as in 3GPP TR 38.803 |
| TN UE to Fixed VSAT on roof | UMa as in 3GPP TR 38.803 (BS is to be replaced with VSAT) |
| TN UE to L-ESIM | Umi |
| Satellite to TN BS/UE | 3GPP TR 38.821 |
| Satellite to VSAT/ESIM | 3GPP TR 38.821 |
| TN BS to Satellite | 3GPP TR 38.821 |
| Note1: For the propagation models which use the 3GPP TR 38.821 [3], to use same assumptions as in [3] and consider the atmospheric losses and the scintillation losses. Note2: TN BS height is 25m |  |

# Transmission Power Control (TPC) Model

## UL Transmission Power Control

Use following UL power control model for both NTN and TN as starting point.

Where:

Pmax = maximum UE Tx power, i.e. 23 dBm for TN UE, and 33 dBm for fixed VSAT NTN UE.

Rmin = minimum power reduction ratio, i.e. -63 dB for TN UE, and [-63 dB by assuming NTN UE min Tx power as -30dBm as starting point] for NTN UE.

γ = 1

CLx-ile = –SNR_target + Pmax – ThermalNoise – BS/SAN_NoiseFigure - 10*log10(BW), considering SNRtarget is 15dB and BW is actual UL BW.

## Downlink Transmission Power Control

For downlink scenario, no power control scheme is applied.

# Receiver Power Control (RPC) Model

The received power in downlink and uplink scenarios is defined as below:

RX_PWR = TX_PWR – Path loss + G_TX + G_RX

Where:

RX_PWR is the received power

TX_PWR is the transmitted power

G_TX is the transmitter antenna gain (directional array gain)

G_RX is the receiver antenna gain (directional array gain).

# Performance Matric

Table 22 Performance Matric for NR and NTN Technologies used for the Simulation

| Technology | Average Throughput Loss | Remarks |
| --- | --- | --- |
| NR | The average throughput loss and 5 percentile throughput loss should be less than 5%. |  |
| NTN | The average throughput loss and 5 percentile throughput loss should be less than 5%. |  |

# Throughput- SNR Mapping

Adopt Section 5.2.7 of TR 38.803[20] as the SINR-Throughput performance metrics for both NTN and TN.

The applicability of the mapping for NTN link is yet to be decided.

# Incomplete Parameters Values

For the Ku Band Co-existence Simulation, the following parameters need to be clarified and their values to be agreed at next RAN4 working group meetings:

# Conclusion

In this contribution we have provided co-existence simulation assumptions of 3GPP Non-Terrestrial services with Terrestrial planned services in the vicinity of the Ku Band. The simulation assumptions are based on the work carried out for the Ka Band and require further discussion and modifications.

# References

- R4-2318202

- R4-2416593

- R4-2419909

- RP-242946

- R4-2316906

# Appendix

## Other Non-3GPP Services

With slight departure from the work undertaken for the Ka Band, as part of the co-existence studies of the Ku Band, we should be aware of sensitive live services running in and adjacent to the Ku band Spectrum

These services are shown in the following Table 23

Table 23 Various non-3GPP Services in and adjacent to the Ku Band as per ITU-R Radio Regulation

| Band | Coverage (GHz) | ITU Region |
| --- | --- | --- |
| Radio Astronomy | 10.68-10.70 | 1,2 and 3 |
| Scientific Services and NATO Radar Band | 13.25-13.75 | 1,2 and 3 |
| Fixed Satellite Services | 14.50-14.80 | 1,2 and 3 |

## Overview of existing ITU Radio Regulations applicable to Ku-band

In band applicable regulation for FSS downlink in 10.7-12.75 GHz:

Section V of Article 21 presents the limits of power flux density from space stations to protect fixed and mobile service. See below a snippet of Table 21-4 of the ITU-R RR for the relevant downlink frequencies:

| Frequency band | Service* | Limit in dB(W/m2) for angles of arrival (δ) above the horizontal plane |  |  | Reference bandwidth |
| --- | --- | --- | --- | --- | --- |
|  |  | 0°-5° | 5°-25° | 25°-90° |  |
| 10.7-11.7 GHz | Fixed-satellite (space-to-Earth) (geostationary-satellite orbit) | −150 | −150 + 0.5(δ − 5) | −140 | 4 kHz |
| 10.7-11.7 GHz | Fixed-satellite (space-to-Earth) (non-geostationary-satellite orbit) 25 | −126 | −126 + 0.5(δ − 5) | −116 | 1 MHz |
| 10.7-11.7 GHz 11.7-12.5 GHz (Region 1) 12.5-12.75 GHz (Region 1 countries listed in Nos. 5.494 and 5.496) 11.7-12.7 GHz (Region 2) 11.7-12.75 GHz (Region 3) | Fixed-satellite (space-to-Earth) (non-geostationary-satellite orbit) 24 | −129 23 | −129 + 0.75(δ − 5) 23 | −114 23 | 1 MHz |
| 11.7-12.5 GHz (Region 1) 12.5-12.75 GHz (Region 1 countries listed in Nos. 5.494 and 5.496) 11.7-12.7 GHz (Region 2) 11.7-12.75 GHz (Region 3) | Fixed-satellite (space-to-Earth) (non-geostationary-satellite orbit) 25 | −124 | −124 + 0.5(δ − 5) | −114 | 1 MHz |

23	21.16.16	The applicability of these limits may need to be reviewed by a future competent conference if the number of co-frequency non-geostationary systems brought into use and simultaneously operating in the same hemisphere is greater than five.     (WRC-03)

24 	21.16.17	These limits apply to non-geostationary fixed-satellite service space stations employing an orbit with an inclination angle between 35 and 145 and apogee altitude greater than 18 000 km.     (WRC-03)

25	21.16.18	These limits apply to non-geostationary fixed-satellite service space stations that are not covered by No. 21.16.17.     (WRC-03)

In band applicable regulation for FSS uplink in 13.75-14.5 GHz

Finally, the ITU-R RR also presents some EIRP limits for the operations of satellite earth stations or VSATs. RR 22.26-22.28 present maximum 
EIRP. levels for earth stations in the band 13.75-14.5 GHz. RR 21.8 also provides EIRP limit of the earth station towards the horizon to protect fixed and mobile services. Finally, a specific IERP limit is also presented under RR 21.13A for the band 13.75-14 GHz.

Protection of Radio Astronomy in 10.68-10.7GHz

For the protection of Radio Astronomy in the band 10.68-10.7 GHz, ITU-R Recommendation RA.769 presents the level of allowable unwanted emissions into the radio astronomy service. The following table is an extract of Table 1 of ITU-R Recommendation RA.769 with the applicable threshold interference level.

| Centre frequency (1) fc (MHz) | Assumed bandwidth f (MHz) | Minimum antenna noise temperature TA (K) | Receiver noise temperature TR (K) |  | System sensitivity(2) (noise fluctuations) |  | Threshold interference levels(2) (3) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Temperature T (mK) | Power spectral density P (dB(W/Hz)) | Input power PH (dBW) | pfd SH f (dB(W/m2)) | Spectral pfd SH (dB(W/(m2 Hz))) |
| (1) | (2) | (3) | (4) |  | (5) | (6) | (7) | (8) | (9) |
| 10 650 | 100 | 12 | 10 |  | 0.049 | –272 | –202 | –160 | –240 |

(1)	Calculation of interference levels is based on the centre frequency shown in this column although not all regions have the same allocations.

(2)	An integration time of 2 000 s has been assumed; if integration times of 15 min, 1 h, 2 h, 5 h or 10 h are used, the relevant values in the Table should be adjusted by +1.7, 1.3, 2.8, 4.8 or 6.3 dB respectively.

(3)	The interference levels given are those which apply for measurements of the total power received by a single antenna.  Less stringent levels may be appropriate for other types of measurements, as discussed in § 2.2. For transmitters in the GSO, it is desirable that the levels be adjusted by 15 dB, as explained in § 2.1.

Additionally, Recommendation ITU-R S.1586 presents the methodology for the calculation of unwanted emission levels produced by a non-geostationary fixed-satellite service system at radio astronomy sites.

General out of band emission ITU recommendations:

Annex 5 of ITU-R Recommendation SM.1541 presents the Out of Band domain emission (OOBE) limits for space services (earth and space stations). Section 2 of Annex 5 presents the OOB masks for FSS earth and space stations:

“The OoB domain emissions of a station operating in the bands allocated to the FSS should be attenuated below the maximum psd, in a reference bandwidth of 4 kHz (for systems operating above 15 GHz a reference bandwidth of 1 MHz may be used in place of 4 kHz) within the necessary bandwidth, by the following:

where F is the frequency offset from the edge of the total assigned band, expressed as a percentage of necessary bandwidth. It is noted that the OoB emission domain starts at the edges of the total assigned band.

FIGURE 14

Example 1: OoB mask assuming the spurious limit is equivalent to 25 dBsd

FIGURE 15

Example 2: OoB mask assuming the spurious limit is equivalent to 40 dBsd

”

The definition of dBsd is also provided in the ITU-R recommendation SM.1541:

“dBsd: decibels relative to the maximum value of power spectral density (psd) within the necessary bandwidth. The maximum value of psd of a random signal is found by determining the mean power in the reference bandwidth when that reference bandwidth is positioned in frequency such that the result is maximized. The reference bandwidth should be the same regardless of where it is centred and is as specified in § 1.4.”

## Interference Scenarios

In this section all eight co-existence scenarios are illustrated.

Figure 7 Coexistence Scenario 1: VSAT uplink interfering with gNodeB receiver.

Figure 8 Co-existence Scenario 2: UE uplink interfering with Satellite receiver.

Figure 9 Co-existence scenario 3: VSAT uplink interfering with UE receiver.

Figure 10 Co-existence Scenario 4: gNodeB downlink interfering with Satellite receiver.

Figure 11 Co-existence Scenario 5: gNodeB downlink interfering with VSAT receiver.

Figure 12 Co-existence Scenario 6: Satellite downlink Interfering with UE receiver.

Figure 13 Co-existence Scenario 7: Satellite downlink interfering with gNodeB receiver.

Figure 14 Co-existence Scenario 8: UE uplink interfering with VSAT receiver.

## Consideration of Deployment Scenarios

Table 24 Various Satellite and Terrestrial Deployment Scenarios for the Co-existence simulation

| No. | Combination | Aggressor | Victim | Which NTN cell/UE to observe? | Which TN/UE to observe? | Which TN cells in a TN to observe? |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN | NTN UL | TN UL | NTN cell: Nadir point. NTN UE: NTN UEs randomly dropped in TN clusters. | TN randomly placed in this NTN beam | Only the active TN clusters which contain NTN UE(s) |
| 2 | TN with NTN | TN UL | NTN UL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: NTN UEs randomly dropped in TN clusters | Consider an active rate of 20% for TN. | Only the active TN cells in central NTN beam |
| 3 | TN with NTN | NTN UL | TN DL | NTN cell: Nadir point NTN UE: NTN UEs randomly dropped in TN clusters. | TN clusters randomly placed in this NTN beam | All active TN clusters which contain NTN UE(s) |
|  |  |  |  | NTN cell: NTN cell with satellite at low elevation (to be further discussed for GEO and LEO，interested companies can bring analysis and results for other values). NTN UE: NTN UEs randomly dropped in TN clusters. | TN clusters randomly placed in this NTN beam | Only the active TN clusters which contain NTN UE(s). |
| 4 | TN with NTN | TN DL | NTN UL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: NTN UEs randomly dropped in TN clusters | Consider the active rate of 20% for TN. | All active TN cells in central NTN beam |
| 5 | TN with NTN | TN DL | NTN DL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: NTN UEs randomly dropped in TN clusters. | One cluster with 19 TN cells (57 sectors) randomly placed in the central NTN beam | Only the active TN clusters which contain the NTN UE(s). |
| 6 | TN with NTN | NTN DL | TN DL | NTN cell: Nadir point. NTN UE: NTN UEs randomly dropped in TN clusters. | TN clusters randomly placed in this NTN beam | All in central NTN beam |
|  |  |  |  | NTN cell: NTN cell with satellite at low elevation (to be further discussed for GEO and LEO，interested companies can bring analysis and results for other values) NTN UE: NTN UEs randomly dropped in TN clusters | TN clusters randomly placed in this NTN beam |  |
| 7 | TN with NTN | NTN DL | TN UL | NTN cell: Nadir point NTN UE: NTN UEs randomly dropped in TN clusters. | TN clusters randomly placed in this NTN beam | Only the active TN clusters which contain NTN UE(s) |
|  |  |  |  | NTN cell: NTN cell with satellite at low elevation (to be further discussed for GEO and LEO，interested companies can bring analysis and results for other values). NTN UE: NTN UEs randomly dropped in TN clusters. | TN clusters randomly placed in this NTN beam | Only the active TN clusters which contain NTN UE(s). |
| 8 | TN with NTN | TN UL | NTN DL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: NTN UEs randomly dropped in TN clusters | Consider the active rate of 20% for TN. | All active TN cells in central NTN beam |