3GPP TSG-RAN WG4 Meeting #114                              						         R4-25xxxxx

# Athens, Greece, 17th – 21th Feburary, 2025

Source: 	Apple

Title: 	On RRM requirement impact for type 4 UE

Agenda Item:	7.5.3

Document for:	Discussion

# Introduction

Type 4 UE capability and new BS signaling have been discussed in the past RAN4 meetings. Though not concluded, it is anticipated that the signaling design will be different from Rel-18. This contribution will present our views on RRM requirement impact by the new signaling design.

# Discussion

In Rel-18, a default UE behaviour in non-collocated condition is assumed for type 2 and introduced BS signalling nonCollocatedTypeNR-CA and nonCollocatedTypeMRDC for type 2 UE for the case of maxMIMO-Layers with value less than or equal to 2. If nonCollocatedTypeNR-CA/ nonCollocatedTypeMRDC is indicated, type 2 UE will reconfigure its RF to type 1 and type 1 requirements for collocated scenario apply. Otherwise, requirements for non-collocated scenario apply. If maxMIMO-layers ＞2, it is collocated by default. The description for nonCollocatedTypeNR-CA in TS 38.331 is reproduced as below.

nonCollocatedTypeNR-CA This field is only present for a UE configured with maxMIMO-Layers with value less than or equal to 2 for all corresponding serving cells, in case of TDD-TDD intra-band NR-CA. If this field is present, the UE applies MRTD according to Table 7.6.4-1 in TS 38.133 [14] and UE RF requirements for intra-band NR-CA except for 7.10A in TS 38.101-1 [15]. If this field is absent, the UE applies MTTD/MRTD requirements according to Table 7.5.4-1/Table 7.6.4-2 in TS 38.133 [14] and UE RF requirements for intra-band non-collocated NR-CA including 7.10A in TS 38.101-1 [15] when indicating support of intraBandNR-CA-non-collocated-r18.

nonCollocatedTypeMRDC

This field is only present for a UE configured with maxMIMO-Layers with value less than or equal to 2 for all corresponding serving cells, in case of TDD-TDD inter-band (NG) EN-DC with overlapping or partially overlapping bands. If this field is present, the UE applies (NG)EN-DC MTTD/MRTD according to clause 7.5.3/7.6.3 in TS 38.133 [14] and inter-band RF requirements. If this field is absent, the UE applies (NG)EN-DC MTTD/MRTD according to clause 7.5.2/7.6.2 in TS 38.133 [14] and inter-band RF requirements.

Hence, Rel-18 requirement for type 2 UE is defined assuming non-collocated requirements apply by default and collocated requirements apply when network command UE to switch to collocated scenario.

As for Rel-19, RRM session concluded that the requirements impact for type 4 UE can follow the same approach as for R18 type 2 UE. While as discussed in [2], for R19 type 4 UE, signalling design should consider different way where the following can be assumed.

When maxMIMO-layers ＞4, similar consideration can be borrowed from Rel-18. In this case the UE is type 1.

When maxMIMO-layers =4, no need to define a default UE behavior since every status is clearly commanded by network signaling. Collocated and non-collocated behavior are both explicitly commanded by network. E.g.

- when network signaling [nonCollocatedTypeNR-CA-R19/ nonCollocatedTypeMRDC-R19] is present with 0, the UE is in non-collocated scenario.

- when network signaling [nonCollocatedTypeNR-CA-R19/ nonCollocatedTypeMRDC-R19] is present with 1, the UE is in collocated scenario.

When maxMIMO-layers =2,

- If Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is present, UE is Type 1.

- If Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is absent, UE is Type 2.

From the above, it is seen that UE behavior in collocated and non-collocated scenario both will be clearly command by network. The requirement should clearly mention that,

- For UE indicating support of [intraBandNR-CA-non-collocated-r19], requirements based on non-collocated scenario apply when network signaling [nonCollocatedTypeNR-CA-R19] is present with 0.

- For UE indicating support of [intraBandNR-CA-non-collocated-r19], requirements based on collocated scenario apply when network signaling [nonCollocatedTypeNR-CA-R19] is present with 1.

Below is an example of requirement impact for MRTD. The other requirements shall also be reviewed once the signalling in RF room is concluded.

Proposal 1: RRM requirement impact should be reviewed again once the signalling design is concluded.

# Summary

This contribution presented our consideration on RRM requirement impact for type 4 UE.

Proposal 1: RRM requirement impact should be reviewed again once the signalling design is concluded.

# References

[1] R4-2500437, On signalling support for type 4 UE, KDDI, RAN4#114