3GPP TSG-RAN WG4 Meeting #114	 R4-2500444

Athens, Greece, 17th – 21st February, 2025

Agenda Item:	4.8

Source: 	Apple

Title:	Discussion on RRM measurement on deactivated SCC

Document for:	Discussion

# Introduction

Requirements for RRM measurement on deactivated SCC was introduced in R15 NR. Both delay requirements and interruption requirements were specified. However, currently delay requirements are specified in the way that it only applies for deactivated SCell, which we believe it is a mistake. In this contribution we will elaborate the issue and propose corresponding update.

# Discussion

Requirements for RRM measurement on deactivated SCell and neighbor cells on deactivated SCC have been specified in RRM spec since LTE. For instance:

Observation 1: RRM measurement delay requirements specified in TS36.133 apply for deactivated SCell and neighbor cells on deactivated SCC.

However, similar requirements defined in NR spec TS38.133 only apply for deactivated SCell. Take PSS/SSS detection in for example:

As can be seen, requirements apply only for deactivated SCell. They don’t apply for neighbor cells on the deactivated SCC. Same problem can also be observed in requirements for time index detection and measurement period.

Observation 2: RRM measurement delay requirements specified in TS38.133 only apply for deactivated SCell, but not for neighbor cells on deactivated SCC.

We believe the spec is unintentionally written in the way that it appears that corresponding requirements only apply for deactivated SCell. Another evidence is that interruption requirements are defined for measurement on deactivated SCC, as captured in TS38.133:

Actually, the similar interruption requirements have been specified since LTE:

Observation 3: requirements of interruption due to RRM measurement on deactivated SCC has been specified in RRM spec since LTE, including measurement on deactivated SCell and neighbor cells on deactivated SCC.

On the other hand, technically speaking if relaxed RRM measurement requirement (based on measCycleSCell) only applies deactivated SCell but not neighbor cells on the deactivated SCC, UE can barely enjoy power saving. Therefore, we propose to fix this.

Proposal 1: requirements of RRM measurement on deactivated SCell also apply for neighbor cells on deactivated SCC.

# Conclusion

In this contribution, we provide discussion on requirements of RRM measurement on deactivated SCell. After discussion, the following conclusions are provided:

Observation 1: RRM measurement delay requirements specified in TS36.133 apply for deactivated SCell and neighbor cells on deactivated SCC.

Observation 2: RRM measurement delay requirements specified in TS38.133 only apply for deactivated SCell, but not for neighbor cells on deactivated SCC.

Observation 3: requirements of interruption due to RRM measurement on deactivated SCC has been specified in RRM spec since LTE, including measurement on deactivated SCell and neighbor cells on deactivated SCC.

Proposal 1: requirements of RRM measurement on deactivated SCell also apply for neighbor cells on deactivated SCC.

# Reference

- TS38.133

- TS36.133