3GPP TSG-RAN WG4 Meeting #114	 R4-2500457

Athens, Greece, 17th – 21st February, 2025

Agenda Item:	7.25.2.2

Source: 	Apple

Title:	Discussion on adaptation of common signal/channel transmission

Document for:	Discussion

# Introduction

RAN4 requirements for adaptation of common signal/channel transmission was widely discussed in the previous RAN4 meeting. The latest agreements can be found in [1]. There are still some open issues pending more RAN1 input.  In the last November meeting, RAN1 approved an LS to RAN4 regarding their new design of adaptation of common signal/channel transmission.

In this contribution we provide some discussion based on RAN1 input. After discussion, corresponding proposals are provided.

# Discussion

## SSB Adaptation requirements

For information, RAN1 LS is duplicated here:

RAN1 would like to thank RAN4 for the LS on SSB relation in On-demand SSB and SSB adaptation on Scell and would like to provide the following replies. Obj 1 Reply to Q1 (What is the relation in terms of periodicity between always-on SSB and OD-SSB?): - The periodicity of on-demand SSB is one of 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms. - The periodicity of on-demand SSB can be configured separately from the periodicity of always-on SSB. - RAN1 is discussing what is the relation between periodicity of always-on SSB and periodicity of on-demand SSB and it has been identified that the main use case is that the periodicity of on-demand SSB is equal to or smaller than that of always-on SSB. Reply to Q2 (What is the relation in terms of time location between always-on SSB and OD-SSB?): - RAN1 understands the time location of OD-SSB in Q2 refers to the time location of possible OD-SSB burst. - RAN1 is still discussing the relation in terms of time location between always-on SSB and OD-SSB. Reply to Q3 (What is the relation in terms of frequency location between the always-on SSB and OD-SSB?): - The frequency location of on-demand SSB is the same as the frequency location of always-on SSB at least for the case where always-on SSB is not CD-SSB. RAN1 is discussing the frequency location of OD-SSB for the case where always-on SSB is CD-SSB. Reply to Q4 (What is the spatial relation between the always-on SSB and OD-SSB?): - SS/PBCH blocks with the same SSB indexes for always-on SSB and on-demand SSB are quasi co-located with respect to Doppler spread, Doppler shift, average gain, average delay, delay spread, and when applicable, spatial RX parameters. - Applies at least for the case when the centre frequency locations of always-on SSB and OD-SSB is same - When a signal/channel is configured to be QCLed with a SSB index, the signal/channel is QCLed with the same SSB index of always-on SSB and on-demand SSB (if transmitted) with the same QCL parameters according to existing specifications - Applies at least for the case when the centre frequency locations of always-on SSB and OD-SSB is same - At least the case where SSB indices within on-demand SSB burst are identical to SSB indices within always-on SSB burst is supported. RAN1 is discussing whether to support the case where SSB indices within on-demand SSB burst can be subset of SSB indices within always-on SSB burst. Obj 3 Reply to Q1(What is the relation in terms of time location before and after SSB adaptation?): - RAN1 agreed that at least SSB burst periodicity is adapted. - There are no RAN1 agreements to adapt the time location of the SSB burst other than the periodicity but RAN1 is still discussing other options. Reply to Q2(What is the relation in terms of frequency location before and after SSB adaptation?): - The frequency location is same before and after SSB adaptation. Reply to Q3(What is the spatial relation before and after SSB adaptation?): - There is no change to spatial relation (in terms of QCL assumption) for the same SSB index before and after SSB adaptation. - At least the case where there is no change to actually transmitted SSBs within a burst before and after SSB adaptation is supported.

SSB adaptation related agreements are highlighted in green above. With these agreement, we can conclude the following issues:

Issue 1-1-2: Scenario clarification- SSB periodicity

- Proposals

  - Option 1: RAN4 to consider legacy SSB burst periodicity as starting point to define the requirements (Samsung, Nokia)

    - Option 1a: RAN4 to maintain the specified range of SSB burst periodicity (5ms to 160ms) for SSB periodicity adaptation. (Nokia)

    - Option 1b:  wait for RAN1 progress on whether longer SSB burst adaptation periodicity can be supported. (Samsung)

- Recommended WF:

  - Postpone the discussion and wait for RAN1 progress.

Since there is no any new agreement for new values of SSB burst periodicity, option 1 is supported, i.e. RAN4 to maintain the specified range of SSB burst periodicity (5ms to 160ms) for SSB periodicity adaptation.

Issue 1-1-3: Scenario clarification-  SSB adaptation triggering

- Proposals

  - Option 1: RAN4 to focus on NW controlled SSB adaptation firstly (Samsung, Ericsson)

    - Option 1a: wait for RAN1 progresses on whether MAC-CE and/or DCI can be supported to indicate SSB adaptation transmission on top of the RRC reconfiguration. (Samsung)

- Recommended WF:

  - RAN4 to focus on NW controlled SSB adaptation firstly.

Issue 1-1-4: Scenario clarification-  SSB adaptation mechanism

- Proposals

  - Option 1: RAN4 to start investigation of RRM impacts for SSB adaptation in time domain for a single active SSB pattern, due to usage of different SSB burst periodicities. Further scenarios can be considered based on further RAN1 agreements (Nokia)

  - Option 2: Suggest to wait for RAN1 agreements on the applicable scenarios and functionalities of option 1 and option 2 (SSB adaptation based on two SSB configurations) for SSB adaptation. (Samsung)

- Recommended WF:

  - Postpone the discussion and wait for RAN1 progress.

As mentioned in RAN1 LS answer to Q1 under objective 3, RAN1 only agreed that at SSB burst periodicity is adapted. Others are still under discussion. therefore, RAN4 only needs to focus on SSB burst periodicity adaptation.

Proposal 1: RAN4 work of this objective is based on legacy SSB burst periodicity (5ms to 160ms).

Proposal 2: RAN4 work of this objective shall focus on SSB burst periodicity adaptation. This can be revisited if RAN1 reaches further agreement.

### Sub-topic 1-2 SSB adaptation – L1 Requirements impacts

Issue 1-2-1: L1 requirements at transition

Agreement:

  - L1 evaluation/measurement period is the maximum of the period corresponding to the SSB before and after adaptation.

    - FFS the applicable L1 requirements.

In general, we think all L1 operations for serving cell need to be considered once SSB adaptation happens, such as RLM/BFD/CBD/L1 measurement. L1 operations for neighbor cell/TRP do not need to be considered in this release. It is unclear whether the adaptation is supported or not in these scenarios. Especially for neighbor cell (L1 measurement of LTM candidate cell), so far it is unclear how this can be supported.

Proposal 3: applicable L1 requirements in SSB adaptation shall include the following L1 operations for serving cell:

- L1-RSRP for serving cell

- L1-SINR

- BFD

- CBD

Proposal 4: not consider L1 impact of SSB adaptation in mTRP and LTM scenario.

# Conclusion

In this contribution, we provide discussion on RAN4 requirements for adaptation of common signal/channel transmission. After discussion, the following conclusions are provided:

Proposal 1: RAN4 work of this objective is based on legacy SSB burst periodicity (5ms to 160ms).

Proposal 2: RAN4 work of this objective shall focus on SSB burst periodicity adaptation. This can be revisited if RAN1 reaches further agreement.

Proposal 3: applicable L1 requirements in SSB adaptation shall include the following L1 operations for serving cell:

- L1-RSRP for serving cell

- L1-SINR

- BFD

- CBD

Proposal 4: not consider L1 impact of SSB adaptation in mTRP and LTM scenario.

# Reference

- R4-2420242, WF on Netw_Energy_NR_enh, Ericsson