3GPP TSG-RAN WG4 Meeting #114	 R4-2500458

Athens, Greece, 17th – 21st February, 2025

Agenda Item:	7.27.2.1

Source: 	Apple

Title:	Discussion on L1 event triggered measurement reporting

Document for:	Discussion

# Introduction

L1 event triggered measurement reporting has been widely discussed in previous RAN4 meeting. The latest agreements can be found in [1], according to which there are quite many open issues. In this contribution, we continue discussing the open issues.

# Discussion

#### Issue 2-5: Event triggered reporting delay for L1 reporting

<Agreement>

- Agree on the following high level proposal:

  - The event triggered measurement reporting delay shall be no larger than the L1-RSRP measurement period [on the cells corresponding to the event or on the candidate cells].

    - FFS reuse L1-RSRP measurement period from the existing spec with Treport = 0.

  - Additional time TSSB_time_index for SSB index acquisition is needed, unless deriveSSB-IndexFromCell is enabled or UE has reported SSB index in L3 measurement report of the same cell.

The yellow highlighted part was raised by some companies in RAN4#113. The intention is to consider both serving cell and neighbor cell in events wherein UE needs to compare measurement results of the two cells, like Event LTM3. That technically makes sense. The event can still be triggered if UE realizes target cell RSRP doesn’t not change but serving cell RSRP somehow drops. Although we didn’t use this approach in legacy L3 measurement, we are open to optimize the similar procedure in LTM.

Regarding the FFS part in reporting delay, it is fine to reuse L1-RSRP measurement period from the existing spec with Treport = 0.

Proposal 1: confirm the following agreement:

  - The event triggered measurement reporting delay shall be no larger than the L1-RSRP measurement period on the cells corresponding to the event or on the candidate cells.

    - Reuse L1-RSRP measurement period from the existing spec with Treport = 0.

#### Issue 2-6: impact of time-to-trigger (TTT) on RRM

<Candidate options>

- Option 1: RAN4 to discuss whether to take TTT (time to trigger) into account in the L1 event-triggered reporting delay requirements once more agreements from RAN1/2 are available. (Nokia, Xiaomi)

<Recommended WF>

- Continue discussion.

Latest agreements in RAN2 regarding TTT is duplicated here for information:

Agreements on L1 event-driven MR:

- TTT is evaluated per beam, and measurement report is only triggered by beam that has satisfied the condition (entering/leaving) for the whole duration of TTT.

- TTT timer is not restarted if the current beam changes and the entering condition is still met with the new current beam.

- TTT is applied to the leaving condition.

- Network can configure which RS type (SSB or CSI-RS) is used for LTM event evaluation.

- Either CSI-RS or SSB could be configured as candidate beam and the measurement RS of the serving cell beam is determined based on the candidate beam to ensure same RS type, i.e. the RS for current beam of serving cell is same as or QCLed with the QCL RS of the indicated TCI state, according to what is agreed in RAN1.

- SSBRI and CRI is used to represent the candidate beam ID in LTM MR MAC CE.

- For event-triggered L1 LTM measurement reporting, max N is total number of all beams included into MR MAC CE.

- For event-triggered L1 LTM measurement reporting, NW controls if the beam(s) not satisfying the event could be reported according to N beams in MR MAC CE.

- A single MAC CE format for the event-triggered L1 measurement report is used for both the SSB and CSI-RS reference signals.

- Support the truncated measurement report MAC CE.

As can be seen, basically it is similar as legacy TTT for L3 measurement. It is only used to control reporting time. From RAN4 RRM measurement point of view, we only need to focus on UE measurement behavior and performance. Therefore in legacy RAN4 didn’t explicitly mention  TTT in requirements. Besides, TTT is always set as zero in legacy test cases design. As long as UE can meet legacy requirement and pass test case without TTT configured, it is rational to assume that UE can also work properly when TTT is configured.

Proposal 2: seems unnecessary to explicitly capture TTT in core requirements.

#### Issue 2-10: whether and how to coordinate with R19 MIMO

<Candidate options>

- Option 1: (MTK)

  - Events configured for mobility and MIMO should be considered as different events even when they are configured for the same beams and the same cell with the same threshold.

  - Define the capability for L1 events shared by mobility L1 events and MIMO L1 events.

- Optoin 2: Define separate capabilities for MIMO and mobility. (Samsung, CMCC, Apple)

- Option 3: Discuss the coordination between LTM and MIMO events at the end of the Wis. (Nokia)

<Recommended WF>

- Continue discussion.

Procedure wise, we don’t think we need to discuss this in this work item. Although they are similar in some extent, they are still separate features. New events introduced by RAN1/2 for these two work items are also different. whether and how to coordinate with R19 MIMO, RAN4 shall develop the two features independently given that UE is allowed to only support of the two features. Optimization such as joint consideration of UE capabilities can be discussed at later stage.

Proposal 3: whether and how to coordinate with R19 MIMO, RAN4 shall develop the two features independently given that UE is allowed to only support of the two features. Optimization such as joint consideration of UE capabilities can be discussed at later stage

#### Issue 2-11: L1 filtering assumption in event evaluation

<Candidate options>

- Option 1: For whether to consider the RRM impact due to L1 filtering for event triggered L1 report, RAN4 needs to wait for more RAN1 input. (CATT, ZTE, CTC, E///)

  - If L1 filtering is introduced in RAN1, RAN4 needs to define additional SSB based L1 measurement delay requirements with filtering for event triggered L1 report.

  - Otherwise, there is no impact on RAN4.

- Option 2: In event triggered L1 measurement in mobility, do not consider L1 filter for requirements. (Samsung, Xiaomi, CMCC, Apple, HW)

  - Option 2a: RAN4 to define RRM requirements for the event triggered L1 report with the assumption of no specific L1 filtering. This decision can be revisited if other working groups overturn their decision on L1 filtering. (QC)

<Recommended WF>

Continue discussion.

There is no any new agreement (not even an open issue) in RAN1 and RAN2 to study L1 filtering for event triggered reporting. Technically, L1 filtering is used, it would become quite similar to L3 measurement, which is an unnecessary design. We understand that some companies conern about robustness. This can be guaranteed by proper configuration of event threshold, TTT and so on. Thus we don’t think RAN4 needs to consider L1 filtering.

Proposal 4: In event triggered L1 measurement in mobility, do not consider L1 filter for requirements.

#### Issue 2-15: Impact of UL resource waiting time.

<Candidate options>

- Option 1: For L1 event triggered reporting, it needs to specify that L1 result which triggers L1 reporting will be included in L1 reporting. (Xiaomi)

  - It means that if L1 reporting procedure starts, UE can continue to measure for L1, however, the following L1 result will not be evaluated for event anymore.

- Option 2: for event triggered L1 measurement reporting, it is proposed that measurement reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on, which is to follow the approach for event triggered L3 measurement reporting. (CMCC, Apple, vivo)

- Option 3: RAN4 to discuss components and criteria for defining TTime to first UL channel (E///)

  - Once the event is met, UE needs to send the indication on the SR or special SR to the NW to indicate to the NW that event is met. It can be denoted as TTime to first UL channel.

<Recommended WF>

- Continue discussion.

This shall be same as legacy. It is already reflected in existing event triggered L3 measurement reporting.

Proposal 5: for event triggered L1 measurement reporting, it is proposed that measurement reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on, which is to follow the approach for event triggered L3 measurement reporting.

# Conclusion

In this contribution, we provide discussion on measurement related enhancements for LTM. After discussion, the following conclusions are provided:

Proposal 1: confirm the following agreement:

  - The event triggered measurement reporting delay shall be no larger than the L1-RSRP measurement period on the cells corresponding to the event or on the candidate cells.

    - Reuse L1-RSRP measurement period from the existing spec with Treport = 0.

Proposal 2: seems unnecessary to explicitly capture TTT in core requirements.

Proposal 3: whether and how to coordinate with R19 MIMO, RAN4 shall develop the two features independently given that UE is allowed to only support of the two features. Optimization such as joint consideration of UE capabilities can be discussed at later stage

Proposal 4: In event triggered L1 measurement in mobility, do not consider L1 filter for requirements.

Proposal 5: for event triggered L1 measurement reporting, it is proposed that measurement reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on, which is to follow the approach for event triggered L3 measurement reporting.

# Reference

- R4-2420101, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple