3GPP TSG-RAN WG4 Meeting #114	R4-2500468

Athens, Greece, Feb 17th – Feb 21st, 2025

Source:	Nokia

Title:	Simulation result for L1-CLI-SRS-RSRP measurement

Agenda item:	7.23.3.1

Document for:	Discussion

- Introduction

In RAN4#112-bis, 3GPP agreed on simulation assumptions for evaluating the L1-SRS-RSRP measurement requirements [1]. In this paper we provide a sub-set of our link-level simulation results. The full dataset can be found in the excel spreadsheet companion to this document [2].

2. Link-level simulation results

  - Case 1: Inter-cell measurements

    - FR1

Figure 1. Inter-cell delta L1-SRS-RSRP for AWGN channel in FR1 (SRS SNR = 1 dB)

Table 1. Summary of the inter-cell L1-SRS-RSRP measurements for AWGN in FR1

| Case | Channel | FR | BW [PRB] | SCS [kHz] | SRS samples [#] | SRS SNR [dB] | 5th percentile of delta L1-SRS-RSRP [dB] | 95th percentile of delta L1-SRS-RSRP [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Case 1 | AWGN | FR1 | 24 | 15 kHz | 1 | 1 | -0.43 | 0.49 |
| Case 1 | AWGN | FR1 | 48 | 15 kHz | 1 | 1 | -0.44 | 0.48 |
| Case 1 | AWGN | FR1 | 24 | 30 kHz | 1 | 1 | -0.51 | 0.36 |
| Case 1 | AWGN | FR1 | 48 | 30 kHz | 1 | 1 | -0.56 | 0.35 |
| Case 1 | AWGN | FR1 | 24 | 60 kHz | 1 | 1 | -0.72 | 0.03 |
| Case 1 | AWGN | FR1 | 48 | 60 kHz | 1 | 1 | -0.71 | 0.08 |

Figure 2. Inter-cell delta L1-SRS-RSRP for TDL-C channel in FR1 (SRS SNR = 1 dB)

Table 2. Summary of the inter-cell L1-SRS-RSRP measurements for TDL-C in FR1

| Case | Channel | FR | BW [PRB] | SCS [kHz] | SRS samples [#] | SRS SNR [dB] | 5th percentile of delta L1-SRS-RSRP [dB] | 95th percentile of delta L1-SRS-RSRP [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Case 1 | TDL-C | FR1 | 24 | 15 kHz | 1 | 1 | -0.26 | 0.36 |
| Case 1 | TDL-C | FR1 | 48 | 15 kHz | 1 | 1 | -0.2 | 0.26 |
| Case 1 | TDL-C | FR1 | 24 | 30 kHz | 1 | 1 | -0.17 | 0.21 |
| Case 1 | TDL-C | FR1 | 48 | 30 kHz | 1 | 1 | -0.35 | 0.44 |
| Case 1 | TDL-C | FR1 | 24 | 60 kHz | 1 | 1 | -0.24 | 0.26 |
| Case 1 | TDL-C | FR1 | 48 | 60 kHz | 1 | 1 | -0.21 | 0.22 |

    - FR2

Figure 3. Inter-cell delta L1-SRS-RSRP for AWGN channel in FR2 (SRS SNR = 1 dB)

Table 3. Summary of the inter-cell L1-SRS-RSRP measurements for AWGN in FR2

| Case | Channel | FR | BW [PRB] | SCS [kHz] | SRS samples [#] | SRS SNR [dB] | 5th percentile of delta L1-SRS-RSRP [dB] | 95th percentile of delta L1-SRS-RSRP [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Case 1 | AWGN | FR2 | 24 | 60 kHz | 1 | 1 | -0.63 | 0.2 |
| Case 1 | AWGN | FR2 | 48 | 60 kHz | 1 | 1 | -0.63 | 0.21 |
| Case 1 | AWGN | FR2 | 24 | 120 kHz | 1 | 1 | -0.88 | -0.32 |
| Case 1 | AWGN | FR2 | 48 | 120 kHz | 1 | 1 | -0.88 | -0.26 |

Figure 4. Inter-cell delta L1-SRS-RSRP for TDLC channel in FR2 (SRS SNR = 1 dB)

Table 4. Summary of the inter-cell L1-SRS-RSRP measurements for TDL-C in FR2

| Case | Channel | FR | BW [PRB] | SCS [kHz] | SRS samples [#] | SRS SNR [dB] | 5th percentile of delta L1-SRS-RSRP [dB] | 95th percentile of delta L1-SRS-RSRP [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Case 1 | TDL-C | FR2 | 24 | 60 kHz | 1 | 1 | -0.43 | 0.21 |
| Case 1 | TDL-C | FR2 | 48 | 60 kHz | 1 | 1 | -0.38 | 0.1 |
| Case 1 | TDL-C | FR2 | 24 | 120 kHz | 1 | 1 | -0.35 | 0.07 |
| Case 1 | TDL-C | FR2 | 48 | 120 kHz | 1 | 1 | -0.54 | 0.21 |

  - Case 2: Intra-cell measurements

    - FR1

Figure 5. Intra-cell delta L1-SRS-RSRP for AWGN channel in FR1 (SRS SNR = 1 dB)

Table 5. Summary of the intra-cell L1-SRS-RSRP measurements for AWGN in FR1

| Case | Channel | FR | BW [PRB] | SCS [kHz] | SRS samples [#] | SRS SNR [dB] | 5th percentile of delta L1-SRS-RSRP [dB] | 95th percentile of delta L1-SRS-RSRP [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Case 2 | AWGN | FR1 | 24 | 15 kHz | 1 | 1 | -0.42 | 0.5 |
| Case 2 | AWGN | FR1 | 48 | 15 kHz | 1 | 1 | -0.47 | 0.47 |
| Case 2 | AWGN | FR1 | 24 | 30 kHz | 1 | 1 | -0.43 | 0.48 |
| Case 2 | AWGN | FR1 | 48 | 30 kHz | 1 | 1 | -0.46 | 0.49 |
| Case 2 | AWGN | FR1 | 24 | 60 kHz | 1 | 1 | -0.45 | 0.43 |
| Case 2 | AWGN | FR1 | 48 | 60 kHz | 1 | 1 | -0.47 | 0.43 |

Figure 6. Intra-cell delta L1-SRS-RSRP for TDL-C channel in FR1 (SRS SNR = 1 dB)

Table 6. Summary of the intra-cell L1-SRS-RSRP measurements for TDL-C channel in FR1

| Case | Channel | FR | BW [PRB] | SCS [kHz] | SRS samples [#] | SRS SNR [dB] | 5th percentile of delta L1-SRS-RSRP [dB] | 95th percentile of delta L1-SRS-RSRP [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Case 2 | TDL-C | FR1 | 24 | 15 kHz | 1 | 1 | -0.27 | 0.36 |
| Case 2 | TDL-C | FR1 | 48 | 15 kHz | 1 | 1 | -0.21 | 0.25 |
| Case 2 | TDL-C | FR1 | 24 | 30 kHz | 1 | 1 | -0.17 | 0.21 |
| Case 2 | TDL-C | FR1 | 48 | 30 kHz | 1 | 1 | -0.34 | 0.42 |
| Case 2 | TDL-C | FR1 | 24 | 60 kHz | 1 | 1 | -0.23 | 0.26 |
| Case 2 | TDL-C | FR1 | 48 | 60 kHz | 1 | 1 | -0.19 | 0.23 |

    - FR2

Figure 7. Intra-cell delta L1-SRS-RSRP for AWGN channel in FR2 (SRS SNR = 1 dB)

Table 7. Summary of the intra-cell L1-SRS-RSRP measurements for AWGN in FR2

| Case | Channel | FR | BW [PRB] | SCS [kHz] | SRS samples [#] | SRS SNR [dB] | 5th percentile of delta L1-SRS-RSRP [dB] | 95th percentile of delta L1-SRS-RSRP [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Case 2 | AWGN | FR2 | 24 | 60 kHz | 1 | 1 | -0.42 | 0.47 |
| Case 2 | AWGN | FR2 | 48 | 60 kHz | 1 | 1 | -0.31 | 0.33 |
| Case 2 | AWGN | FR2 | 24 | 120 kHz | 1 | 1 | -0.27 | 0.25 |
| Case 2 | AWGN | FR2 | 48 | 120 kHz | 1 | 1 | -0.45 | 0.48 |

Figure 8. Intra-cell delta L1-SRS-RSRP for TDL-C channel in FR2 (SRS SNR = 1 dB)

Table 8. Summary of the intra-cell L1-SRS-RSRP measurements in TDL-C FR2

| Case | Channel | FR | BW [PRB] | SCS [kHz] | SRS samples [#] | SRS SNR [dB] | 5th percentile of delta L1-SRS-RSRP [dB] | 95th percentile of delta L1-SRS-RSRP [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Case 2 | TDL-C | FR2 | 24 | 60 kHz | 1 | 1 | -0.29 | 0.38 |
| Case 2 | TDL-C | FR2 | 48 | 60 kHz | 1 | 1 | -0.22 | 0.25 |
| Case 2 | TDL-C | FR2 | 24 | 120 kHz | 1 | 1 | -0.19 | 0.21 |
| Case 2 | TDL-C | FR2 | 48 | 120 kHz | 1 | 1 | -0.36 | 0.38 |

References

- R4-2416910, “Simulation assumption for evaluate L1-SRS-RSRP measurement requirements”, Ericsson, RAN4#112-bis

- R4-2500468, “Updated simulation results for L1-SRS-RSRP”, Nokia