3GPP TSG-RAN WG4 Meeting #113	R4-2500471

Athens, Greece, February 17 – 21, 2025

Source:	Nokia, Nokia Shanghai Bell

Title:	RRM requirements maintenance for network energy saving

Agenda item:	5.27.1

Document for:	Discussion

Introduction

At RAN4#112 meeting, the core part of the WI was further discussed but there are still some open issues as listed in the WF [1]. In this paper, we will continue the discussion on the leftover issues.

SSB-less operation

Neighbor cell measurement on SSB-less SCell carrier

In RAN4#113 meeting, the neighbor cell measurement on the carrier of SSB-less SCell was discussed but could not conclude in terms of intra-frequency vs. inter-frequency and gaps vs. without gaps issues. It was agreed RAN4 provides analysis on the categorization considering if servingCellMO is configure or not.

Issue 1-1-1: Neighbour cells on carrier of SSB-less SCell Sub-issue 1: Whether servingCellMO can be present for inter-band SSB-less SCell. Agreement: RAN4 provide analysis on the categorization of intra-frequency/inter-frequency, whether MG is needed and any other aspects (if identified) for following cases: - When servingCellMO is configured - When servingCellMO is not configured RAN4 provide the above analysis to RAN2 (if consensus can be reached) and wait for RAN2 decision on whether servingCellMO is present for carriers of SSB-less SCell. Sub-issue 2: Definition on intra-frequency or inter-frequency - Proposals - Option 1: Intra-frequency or inter-frequency depends on whether servingCellMO is configured. (Nokia) - If the SSB-less SCell configuration include servingCellMO, the neighbor cell measurement in this MO shall be considered as intra-frequency measurement. - In the MO indicated by ServingCellMO, network may configure ssbFrequency to indicate a carrier frequency for the SSB-less SCell so that UE can determine measurement as intra-frequency or inter-frequency measurement based on the ssbFrequency. - Option 2: Inter-frequency measurement based on existing definition. (CMCC, Apple, Vivo, Huawei, ZTE)

Once an SCell has been configured (added), the SCell is a serving cell. The configured SCell is a serving cell no matter if the SCell is activated, dormant or deactivated. For serving cells, the basic rule is that intra-frequency measurement requirements in section 9.2 applies.

Observation #1: For serving cells, the basic rule is that intra-frequency measurement requirements in section 9.2 applies.

According to current definition of intra-frequency measurement, it is defined based on the center frequency of the SSBs in the serving cell and neighbor cell. As there is no SSB in the serving cell (SSB-less SCell), the current intra-frequency measurement definition does not very well cover the SSB-less SCell scenario. Hence, it was considered whether the measurement requirements applicable for an SSB-less serving should be intra-frequency or inter-frequency measurement requirements.

Observation #2: The current intra-frequency measurement definition does not account when the serving cell is an SSB-less SCell. Hence, it is not clear if it does fulfill the conditions for intra-frequency measurement as defined in TS 38.133.

However, the SSB-less SCell is a serving cell hence the carrier of SSB-less SCell is a serving CC. Although there is no SSB transmission on the SCell, it would be beneficial to consider neighbor cell measurements in the serving cell carrier to be intra-frequency ones – Anyway UE is able to measure those without interruptions.

Observation #3: It would be beneficial to consider serving cell frequency measurements as intra-frequency to allow fluent mobility for SCell.

Signaling allows UE to be configured with servingCellMO on the carrier of SSB-less SCell and the neighbor cell measurement in this MO shall be considered as neighbors on a serving carrier and therefore possibility to be measured according to intra-frequency measurement requirements. In particular, in the MO indicated by servingCellMO, network may configure ssbFrequency to indicate a carrier frequency for the SSB-less SCell so that UE can determine the intra-frequency or inter-frequency measurement based on the ssbFrequency. As there is no actual SSB being transmitted, the ssbFrequency is configured indicating an ARFCN value not associated with any SSB.

Observation #4:  The SSB-less SCell is a serving cell and the carrier of SSB-less SCell is a serving CC.

If the SSB-less SCell configuration include servingCellMO, the neighbor cell measurement in this MO shall be considered as intra-frequency measurement.

In the MO indicated by ServingCellMO, network may configure ssbFrequency to indicate a carrier frequency for the SSB-less SCell so that UE can determine measurement as intra-frequency or inter-frequency measurement based on the ssbFrequency.

We also provide a CR [2] to show the impact on the definition of intra-frequency measurement due to SSB-less SCell operation.

Conclusion

In this paper we have made the following proposals and observations:

Observation #1: For serving cells, the basic rule is that intra-frequency measurement requirements in section 9.2 applies.

Observation #2: The current intra-frequency measurement definition does not account when the serving cell is an SSB-less SCell. Hence, it is not clear if it does fulfill the conditions for intra-frequency measurement as defined in TS 38.133.

Observation #3: It would be beneficial to consider serving cell frequency measurements as intra-frequency to allow fluent mobility for SCell.

Observation #4:  The SSB-less SCell is a serving cell and the carrier of SSB-less SCell is a serving CC.

- If the SSB-less SCell configuration include servingCellMO, the neighbor cell measurement in this MO shall be considered as intra-frequency measurement.

In the MO indicated by ServingCellMO, network may configure ssbFrequency to indicate a carrier frequency for the SSB-less SCell so that UE can determine measurement as intra-frequency or inter-frequency measurement based on the ssbFrequency.

References

[1] R4-2420121, WF on RRM requirements for NR network energy saving, Huawei

[2] R4-2500472, 38133CR on intra-frequency measurement with SSB-less SCell, Nokia, Nokia Shanghai Bell