3GPP TSG RAN WG4 Meeting #120                                                 	      R4-2500474

Athens, Greece, February 17th – 21st, 2025

Title:	Fair coexistence between SBFD and TDD networks

Agenda item:	7.23.3.1 - RRM requirements for UE-to-UE CLI handling

Source:	Charter Communications

Document for:	Discussion/Decision

# Introduction

In Rel 18 study, SBFD performance was evaluated, and it was concluded in TR 38.858 [1] that SBFD provides benefits of reduced latency and increased UL throughput over legacy TDD. It was concluded in the study phase that SBFD may suffer from gNB-to-gNB interference in its UL subbands due to DL transmission from legacy TDD networks deployed in the adjacent frequency bands. SBFD networks may also cause UE-to-UE interference to the UEs of a legacy TDD network operating in the adjacent frequency band. Rel. 19 normative phase started with the specific objective in the WID [2] for adjacent channel coexistence between the two operators.

In this contribution we will present our view on how to mitigate UE-to-UE CLI for the UEs of a legacy TDD operator in the adjacent frequency band. We will also claim that our proposals can mitigate gNB-to-gNb CLI for the SBFD gNBs while improving their spectrum efficiency.

# Discussion

As discussed above, gNBs of SBFD networks will face gNB-to-gNB CLI in the UL subbands of SBFD symbols/slots. To overcome this interference from the neighboring gNBs, the SBFD aware UEs will have to transmit at a higher power level than the legacy TDD UEs to meet the target Rx power level set by gNB. Especially, when the SBFD aware UE is located at cell-edge, the interference at the SBFD gNB receiver may cause it to keep on increasing the UE transmit power which may transmit at its maximum transmit power to meet the gNB’s target Rx power level.

Even at max power level there is no guarantee that cell edge SBFD UEs will benefit from the SBFD feature benefits. They may end up getting retransmission failures during their SBFD symbols/slots, while their TDD slots are still working OK.

This issue will drain their batteries and will waste the SBFD resources of the cell. Since this issue is the result of gNB-to-gNb CLI, we claim that our proposals below could mitigate both gNB-to-gNb CLI and UE-to-UE simultaneously.

## Limiting Maximum UE Transmit power in SBFD symbols/slots.

As mentioned above, the benefit of allowing UEs to transmit at the maximum power level during the SBFD symbols/slots may not be beneficial to the UEs and may increase UE-to-UE CLI in the adjacent networks. This UE-to-UE CLI is worse when the UEs of adjacent legacy TDD network are at their cell edge and SBFD-Aware UEs are close to it as shown in Rel. 18 study, TR 38.858 [1] section 11.3.1-1.

To limit this increase in UE-to-UE CLI in the adjacent legacy TDD networks, it will be more efficient to limit maximum transmit power of UEs during SBFD symbols/slots, while keeping maximum transmit power of UEs during legacy UL symbols/slots same as in case of legacy TDD systems as shown in the following figure:

The figure above shows that SBFD aware UEs will transmit at a higher power level than the legacy TDD UEs. This higher power is caused by the interference factor, caused by gNB-to-gNB interference, where the aggressor is TDD, and the victim is SBFD. To overcome this interference at the SBFD gNB receiver, the UE will need to boost its power. This may cause UE-to-UE CLI, where now the aggressor is the SBFD UE and the victim is TDD UE. To mitigate this UE-to-UE CLI in the adjacent networks the maximum UE transmit power should be limited to a value lower than the maximum transmit power allowed in the legacy TDD networks. Based on the above figure we propose

Transmit power=minPCMAX, Ptx,  &non-SBFD symbols/slotsminPCMAX,SBFD, Ptx,  &SBFD symbols/slots

Where:

Ptx = transmit power as defined in 38.213 [3] for different Channels (PUSCH/PUCCH/RACH/SRS)

PCMAX = UE configured maximum output power as specified in [8-1, 38.101-1], [8-2, 38.101-2], and [8-3. 38.101-3]

PCMAX,SBFD = PCMAX – XdB where XdB could be a backoff as determined by gNB supporting SBFD.

Limiting the UE transmit power during SBFD symbols/slots will help in mitigating UE-to-UE CLI in the adjacent legacy TDD networks. It will also save the SBFD system in unsuccessful retransmissions of screaming UEs at cell edge which cannot benefit from SBFD anyway. The gNB will have full cell range for TDD symbols, and smaller range for SBFD symbols.

The SBFD operator may set XdB = 0 to completely turn off this proposal. Alternatively, the operator may set XdB to a value that will turn off only 5% or 10% of the SBFD UE population by using a dynamic algorithm which is based on Tx power headroom reports from the UEs, as we propose at the end of this TDOC.

Proposal 1: Reduce the maximum UE transmit power in the SBFD symbols/slots to limit UE-to-UE CLI in the adjacent legacy TDD networks. gNB will convey the XdB value in a message to the UE which will be used to back off the maximum UE transmit power during SBFD symbols/slots.

Proposal 2: This message could be defined in 38.101-1, 38.101-2 and 38.101-3.

## Using a threshold to limit the scheduling of cell-edge UEs in SBFD symbols/slots

As mentioned above high power UEs transmitting in the SBFD symbols/slots will increase the UE-to-UE CLI in the adjacent legacy TDD networks. To limit this UE-to-UE CLI in the adjacent networks one way could be to not schedule high power UEs in the SBFD symbols/slots. All the UEs that may be transmitting at high powers, e.g. those UEs that are at the cell edge of an SBFD network, may be scheduled to transmit during the legacy TDD UL slots where there will be no problem of facing any gNB-to-gNB interference and hence these UEs may conserve their battery life by not increasing the transmit power to overcome additional gNB-to-gNB interference that will be present in the SBFD symbols.  This means that UEs that will not face large propagation loss will be granted more opportunity to transmit in the SBFD symbols/slots.

The following figure shows an example that shows the network can specify a power threshold and all the UEs who may use transmit power above that threshold may not be scheduled to transmit in the SBFD symbols/slots.

Max UE Tx power

Distance

[meter]

SBFD

Cell

SBFD

UE

Estimated UE Tx Power

(dBm)

Threshold

No SBFD UEs

Distance

[meter]

SBFD UEs

The SBFD operator may set Threshold = 999 dBm to completely turn off this proposal. Alternatively, the operator may set Threshold to a value that will turn off only 5% or 10% of the SBFD UE population by using a dynamic algorithm which is based on Tx power headroom reports from the UEs, as we propose at the end of this TDOC.

Proposal 3: Set a UL Power threshold, where only UEs with an estimated Tx power level below the threshold will have full access to the SBFD resources. All other UEs will not be scheduled in the SBFD symbols/slots (but still have full access to TDD symbols).

Another variant can be that Power Head Room (PHR) indicator from UEs can be used to figure out at what power level UE is transmitting. All the UEs whose PHR is below a threshold will not be scheduled in the SBFD symbols/slots as shown in the figure below:

Distance

[meter]

SBFD

Cell

SBFD

UE

PHR report

PHR

[dB]

Distance

[meter]

Good PHR

Poor PHR

No SBFD UEs

SBFD UEs

0 dB

Threshold

The SBFD operator may set Threshold = 0 dBm to completely turn off this proposal. Alternatively, the operator may set Threshold to a value that will turn off only 5% or 10% of the SBFD UE population by using a dynamic algorithm which is based on Tx power headroom reports from the UEs, as we propose at the end of this TDOC.

Proposal 4: Alternatively, set a PHR threshold, where only UEs with a PHR level above the threshold will have full access to SBFD resources. All other UEs will not be scheduled in the SBFD symbols/slots (but still have full access to TDD symbols).

## Using two thresholds to limit the scheduling of cell-edge UEs in SBFD symbols/slots

There can still be other possibilities in which multiple thresholds can be specified and UEs will be allowed to transmit at different power levels in different threshold regions. This is illustrated in the next figure by using an example of PHR. There is no restriction of scheduling the UEs if their PHR is above both Threshold2, if their PHR is above Threshold1 but below Threshold2 then these UEs will be scheduled in the SBFD symbols/slots but with limited maximum transmitted power but if PHR is below Threshold1 then those UEs will not be scheduled in SBFD symbols/slots.

SBFD UEs

No SBFD UEs

SBFD UEs limited power

Distance

[meter]

SBFD

Cell

SBFD

UE

PHR report

PHR

[dB]

Distance

[meter]

0 dB

Threshold2

Threshold1

The two thresholds for deciding how to schedule the UEs on SBFD symbols/slots can be constructed from historical PHR values reports fed back by the UEs to the gNB.

As an example of how these thresholds can be determined, the gNB operator could decide to improve UL service to 90% of its served UE population by offering them full access to SBFD symbols/slots, while sacrificing the lowest 5% of the population by denying them the SBFD UL resources. The resulting 5% will be given restricted access to SBFD symbols/slots by reducing their maximum power by XdB. The following dynamic algorithm is an example of how a gNB might achieve that:

Dynamic Algorithm

Step1:	The SBFD gNB will continuously monitor a sliding window of the PHR reports from the most recent 1000 served UEs.

Step2: The SBFD gNB will sort these values in ascending order:

{UE1 = 0 dB, UE2 = 0.5dB, …, UE50 = 1.1dB, …., UE100 = 2.2dB, …, UE1000 = 20dB}

For the lowest 5% of the PHR values, the value at location UE50 will define Threshold1 (example: 1.1dB). Similarly, the value in location UE100 will define Threshold2 (example: 2.2dB), and the remaining 900 UEs values will constitute the 90% population with PHR above both Thresholds.

Step3:	Configure full SBFD UL resources to the 90% of the UEs with PHR > Threshold2 without any limit.

Step4:	Deny SBFD access for the 5% UEs with PHR < Treshold1. (while TDD symbols are still allowed)

Step5:	Restrict the SBFD UL maximum power of all other UEs (with Treshold1 < PHR < Treshold2).

This algorithm can be used by the gNB scheduler to manage the configuration of SBFD resources.

The SBFD operator may set the threshold to 0 dB to completely turn off this proposal

Proposal 5: Alternatively, define 2 PHR Thresholds for managing SBFD UL resources for the served UEs. The thresholds can be found based on historical measurements/reports fed back by the served UEs. These 2 Thresholds will define 3 groups of UE based on their reported PHR levels. A connected UE will be scheduled to the UL SBFD resources based on its specific PHR group level.

# Conclusions

Based on the discussion above we propose the following:

Proposal 1: Reduce the maximum UE transmit power in the SBFD symbols/slots to limit UE-to-UE CLI in the adjacent legacy TDD networks. gNB will convey the XdB value in a message to the UE which will be used to back off the maximum UE transmit power during SBFD symbols/slots.

Proposal 2: This message could be defined in 38.101-1, 38.101-2 and 38.101-3.

Proposal 3: Set a UL Power threshold, where only UEs with an estimated Tx power level below the threshold will have full access to the SBFD resources. All other UEs will not be scheduled in the SBFD symbols/slots (but still have full access to TDD symbols).

Proposal 4: Alternatively, set a PHR threshold, where only UEs with a PHR level above the threshold will have full access to SBFD resources. All other UEs will not be scheduled in the SBFD symbols/slots (but still have full access to TDD symbols).

Proposal 5: Alternatively, define 2 PHR Thresholds for managing SBFD UL resources for the served UEs. The thresholds can be found based on historical measurements/reports fed back by the served UEs. These 2 Thresholds will define 3 groups of UE based on their reported PHR levels. A connected UE will be scheduled to the UL SBFD resources based on its specific PHR group level.

# Reference

[1] 3GPP TR 38.858, “Study on Evolution of NR Duplex Operation”

[2] RP-242772 Revised WID - Evolution of NR duplex operation Sub-band full duplex (SBFD)

[3] 3GPP TS 38.213, “Physical Layer Procedures for Control”

[4] 3GPP TS 38.101-1, “User Equipment (UE) radio transmission and reception: Part 1: Range 1 Standalone”

[5] 3GPP TS 38.101-2, “User Equipment (UE) radio transmission and reception: Part 2: Range 2 Standalone”

[5] 3GPP TS 38.101-3, “User Equipment (UE) radio transmission and reception: Part 3: Range 1 and Range 2 Interworking operation with other radios”