3GPP TSG-RAN4 Meeting #114	R4-2500485

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | 0721 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_demod_enh3-Perf) Correct Measurement Channel for Requirements in Section 5.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_demod_enh3-Perf |  |  |  |  |  | Date: |  |  | 2025-02-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | “Measurement channel” in Table 5.6.2.1.1.0-1 is incorrectly referencing TBS.1-1 and TBS.1-2 under Table A.4-2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct “Measurement channel” in Table 5.6.2.1.1.0-1 as per 38.101-4 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirement definition is not correct |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TR 38.521-4 CR 0937 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of Changes >>

##### 5.6.2.1.1	2Rx FDD FR1 PDSCH absolute physical layer throughput performance with link adaptation

5.6.2.1.1.0	Minimum conformance requirements

The purpose of the requirements is to verify the PDSCH absolute physical layer throughput performance with link adaptation under 2 receive antenna conditions.

The performance requirements are specified in Table 5.6.2.1.1.0-2, with additional test parameters in Table 5.6.2.1.1.0-1 and the downlink physical channel setup according to Annex C.2.1.

Table 5.6.2.1.1.0-1: Test parameters

| Parameter |  |  | Unit | Test 1 |
| --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 10 |
| Subcarrier spacing |  |  | kHz | 15 |
| Duplex Mode |  |  |  | FDD |
| Propagation channel |  |  |  | TDLA30-5 |
| Antenna configuration |  |  |  | 2×2 |
| Correlation configuration |  |  |  | ULA Low |
| Beamforming Model |  |  |  | As defined in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 4 |
|  | CDM Type |  |  | FD-CDM2 |
|  | Density (ρ) |  |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5,(4) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | (9) |
|  | CSI-RS periodicity and offset |  | slot | 5/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 2 |
|  | CDM Type |  |  | FD-CDM2 |
|  | Density (ρ) |  |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 3 (6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | (13) |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 5/1 |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic |
|  | CSI-IM RE pattern |  |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4,9) |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 5/1 |
| ReportConfigType |  |  |  | Aperiodic |
| CQI-table |  |  |  | Table 2 |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  |  | not configured |
| timeRestrictionForInterferenceMeasurements |  |  |  | not configured |
| cqi-FormatIndicator |  |  |  | Wideband |
| pmi-FormatIndicator |  |  |  | Wideband |
| Sub-band Size |  |  | RB | 8 |
| csi-ReportingBand |  |  |  | 1111111 |
| CSI-Report periodicity and offset |  |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  |  | 5 |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |
|  |  | Codebook Mode |  | 1 |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | N/A |
|  |  | CodebookSubsetRestriction |  | 111111 |
|  |  | RI Restriction |  | N/A |
| Physical channel for CSI report |  |  |  | PUSCH |
| CQI/RI/PMI delay |  |  | ms | 6 |
| Maximum number of HARQ transmission |  |  |  | 4 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  | 2 |
| Measurement channel |  |  |  | As specified in Table A.4-2 Rank 1: TBS.2-1 Rank 2: TBS.2-2 |
| Note1: For retransmission including initial transmission, RV {0,2,3,1} with same MCS and rank as initial transmission; follow the latest UE reported PMI whose rank is same as the initial transmission |  |  |  |  |

<< End of changes >>