3GPP TSG-RAN4 Meeting #114	R4-2500489

# Athens, Greece,  16th – 21st February, 2025

Title:	Ambient IoT reader ACS requirements with external CW

Source:	Qualcomm Incorporated

Agenda item:	7.24.2.1

Release:	Rel-19

WI: 	Ambient_IoT_Solutions-Core

Lead WG: 	RAN1

Document for:	Approval

# 1.	Introduction

WI for Ambient IoT in Rel-19 was approved in RAN#106.  The scope is topology 1 and device 1 with external CW node. In this paper we discuss how to ensure system performance with the external CW node.

# 2. 	Discussion

CW cancellation was discussed during the work item and number of conclusions were added to the TR in section 6.7.5. WI RAN4 objective defines that RAN4 will specify requirements for Ambient IoT BS and requirements for CW. WF [2] also agrees to specify number of receiver selectivity and blocking requirements for the A-IoT BS.

CW cancellation is difficult to measure if the DUT is considered as a back box and observed only as the functional box. In order to know how much CW is cancelled, the tester would need an access to the received spectrum after cancellation. Measuring CW cancellation directly would not be meaningful from the system performance and functionality view point either.

Observation 1: Specifying and testing CW cancellation capability explicitly is not meaningful

Since CW can be transmitted from an external node, the reader receiver needs to be able to withstand some level of CW in its input without degrading the received D2R signal SINR. It is more meaningful to define receiver selectivity test with CW as a jammer and D2R as the wanted signal.

Proposal 1: Define reader receiver selectivity test with CW as jammer for the BS reader to stress the CW cancellation capability

This is close to narrowband blocking requirement with parameters adjusted to imitate the actual A IoT deployment.

The values can be derived in deterministic way looking for reader range and worst case CW transmitter distance from the reader. In order to define the weakest D2R signal, the back scattering gain, or as proposed in our companion paper, some form of output power number for the Device is needed. For the level of CW aggressor, RAN4 can define minimum separation of the CW node from the reader and use CW power and pathloss.

Using the feasibility study values, the CW power of 35 dBm and the reader range of 15 meters (pathloss 55 dB) and the backscattering loss is 6 dB and assuming CW node is at 1 meters (pathloss 31.5 dB) from the reader, the CW to D2R signal power ratio becomes to 3.5 dBm to -81 dBm i.e. 84.5 dB. It should be noted that the CW needs to be cancelled further than to the same level with the D2R signal since OOK will need positive SNR unlike NR DL in REFSENS or ACS test where SNR of -1 is sufficient.

Observation 2: Deterministic analysis yields CW selectivity test values to be 3.5 dBm for the aggressor CW and -81 dBm for the wanted signal

Alternatively RAN4 can also use probabilistic approach, in TR 38.769 [3] Figure 6.7.5.2.2.3-2 (copied below as Figure 1 for convenience) D2R received level at the reader. TR also concludes that the CW received power is always -39 dBm in the chosen topology at the reader input.

Figure 1. Received D2R power in T1

Aggressors CW power could be set to -39 dBm and the wanted D2R power at the reader set to -101 dBm for 90% probability or alternatively -112 dBm to cover 100% cases for better reliability.

Observation 3: Probabilistic determination analysis yields CW selectivity test values for aggressor level of -39 dBm and wanted D2R signal power -101 dBm or -112 dBm depending on the chosen probability.

These values may need to be adjusted when reader sensitivity is agreed.

RAN4 can discuss also from reader feasibility point of view what are appropriate values. It also seems in many analysis in the TR, for CW external to the reader, the CW cancellation values of up to 160 dBc were mentioned but system simulation show that this is unnecessary high level. One good way is to pic dBc value between the two methods resulting in to 70 dBc.

Proposal 2: Define selectivity test for the reader with more than 70 dBc CW to D2R signal level ratio

This will ensure at least some minimum system performance is met by the reader and CW aggressor can be placed relatively freely in the deployment.

# Conclusion

We discussed the importance of defining selectivity test for the reader that ensures redeer performance with CW coming outside the topology and made three observations:

Observation 1: Specifying and testing CW cancellation capability explicitly is not meaningful

Observation 2: Deterministic analysis yields CW selectivity test values to be 3.5 dBm for the aggressor CW and -81 dBm for the wanted signal

Observation 3: Probabilistic determination analysis yields CW selectivity test values for aggressor level of -39 dBm and wanted D2R signal power -101 dBm or -112 dBm depending on the chosen probability.

And to ensure RAN4 requirement would ensure proper system performance we made two proposals:

Proposal 1: Define reader receiver selectivity test with CW as jammer for the BS reader to stress the CW cancellation capability

Proposal 2: Define selectivity test for the reader with more than 70 dBc CW to D2R signal level ratio

# Reference

[1]	RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, RAN1 Vice-chair (Huawei), 3GPP TSG RAN Meeting #106, Madrid, Spain, December 9-12, 2024

[2]	R4-2420470 “WF on UE RF requirements for A-IoT”, Huawei, 3GPP TSG-RAN WG4 Meeting # 113, Orlando, United States,18 – 22 Nov., 2024

[3]	TR 38.769, “Study on solutions for Ambient IoT (Internet of Things) in NR”, 3GPP