3GPP TSG-RAN WG4 Meeting #114	R4-2500505

# Athens, Greece, 17th - 21th, Feb 2025

Title:	On OTA performance metric and polarization for handheld NTN UE

Source:	Samsung

Agenda item:	7.14.2.2

Document for:	Discussion

1.	Introduction

In last RAN4 meeting, progress on NTN OTA performance metric for handheld UE was achieved by separating usage scenarios into two categories [1]:

Issue 3-1-2: General view on Usage scenarios for handheld IoT-NTN and NR-NTN Agreements: - For IoT-NTN and NR-NTN, typical usage scenarios are: - Scenario 1: both talk mode (head+hand, and hand only) and browsing mode. - Scenario 2: browsing mode (hand only) - For IoT-NTN UEs not supporting voice call or only supporting voice call with loud speaker etc. are only required and tested with hand only mode. - UEs supporting Scenario 1 are tested with head+hand and hand only modes - UEs supporting Scenario 2 only are only tested with hand only mode Issue 3-1-3: Handheld IoT-NTN and NR-NTN performance metric Agreements: - For IoT-NTN and NR-NTN handheld UEs supporting Scenario 1 the performance metrics are full sphere TRP and TRS - For IoT-NTN and NR-NTN handheld UEs supporting Scenario 2 only, further study performance metric Issue 3-1-6: How to determine hand only mode for NR/IoT NTN mobile handheld UE (if we call it as Type 1 NTN UEs, usage-mode restriction) Agreements: - RAN4 to discuss how to distinguish UEs which support Scenario 1 (talk and browsing modes) vs. Scenario 2 only (browsing mode only)

Meanwhile the polarization issue for NTN OTA testing is still open yet. In this contribution we share our views on performance metric and polarization for handheld NTN UE.

2. 	Discussion

2.1 Performance metric

If we compare the two typical usage scenarios, i.e.

  - For IoT-NTN and NR-NTN,  typical usage scenarios are:

    - Scenario 1: both talk mode (head+hand, and hand only) and browsing mode.

    - Scenario 2: browsing mode (hand only)

It can be observed that most handheld UE support Scenario 1 (both talk mode and browsing mode). There may be some handheld UE only supporting Scenario 2 (hand only) currently, but the trend is that Scenario 1 will be the dominant usage scenario for handheld UE. Even for IoT NTN, in recent RAN plenary December meeting, there shows strong desire to consider IMS voice over IoT NTN, as indicated in the endorsed RAN chair’s summary [2]

IoT NTN - Strong desire to consider IMS Voice over GEO in IoT-NTN for FDD - If so, need to make sure the workload in various WGs is reasonable - Close cooperation with other TSGs is necessary

Observation 1:	it can be expected that Scenario 1 (both talk mode and browsing mode) will be the dominant usage scenario for handheld NTN UE.

So the handheld NTN UE supporting only Scenario 2 (hand only) could be merely kind of temporary UE type in a transition period. It would be reasonable to prioritize the Scenario 1 (both talk mode and browsing mode) in Rel-19. Scenario 2 (hand only) can be considered depending on future available devices in future release or future generation.

Proposal 1:	For handheld NTN UE OTA, prioritize the Scenario 1 (both talk mode and browsing mode) in Rel-19. Scenario 2 (hand only) can be considered depending on future available devices in future release or future generation.

2.2 Polarization

Linear polarization vs circular polarization has been extensively discussed but no conclusion yet. In last meeting, a widely co-signed contribution [2] has comprehensively analysed every aspect of this issue. In addition, it is realized that the discussion should be clearly separated for UE antenna polarization and TE antenna polarization, for the convenience of discussion and move forward.

Proposal 2:	Separate the polarization issue into two aspects: UE antenna polarization and TE antenna polarization.

For UE antenna polarization, it has been always assumed that VSAT antenna is circular polarization and handheld UE antenna is linear polarization, and the polarization mismatch of 3dB has already been taken into account in the link budget computation, as indicated in TR 38.811:

Table 4.4-1 gives the minimum RF characteristics of the terminals operating respectively in Ka band (e.g. very small aperture terminals) and in S band (e.g. handheld terminals). Table 4.4-1: Typical minimum RF characteristics of UE in satellite and aerial access networks Very Small Aperture Terminal (fixed or mounted on moving platforms); Handheld or IoT devices (3GPP class 3, see [2]). Transmit Power; 2 W (33 dBm); 200 mW (23 dBm). Antenna type; 60 cm equivalent aperture diameter (circular polarisation); Omnidirectional antenna (linear polarisation). Antenna gain; Tx: 43.2 dBi Rx: 39.7 dB; Tx and Rx: 0 dBi. Noise figure; 1.2 dB; 9 dB. EIRP; 45.75 dBW; -7 dBW. G/T (NOTE 1); 18.5 dB/K; -33.6 dB/K. Polarisation (NOTE 2); Circular; Linear. NOTE 1: For the computation of G/T or figure of merit, following formula applies in dB: G/T = Ga – NF – 10*LOG (To+(Ta-To)/(100.1*NF)) Where: - Antenna Gain : Ga in dBi - Ambient Temperature : T0 (usually 290 K) - Antenna temperature : Ta (typically 290 K with 0 dBi gain and 150 K with >30 dBi gain) - Noise Figure: NF in dB NOTE 2: For S band, we assume that the User Equipment has an omni-directional antenna of linear polarization, while the antenna on board space-borne or airborne platforms features typically employs circular polarization. Hence a polarization mismatch of 3 dB has to be taken into account for the radio link budget computation. This will impact the UE RF characteristics as below: - Equivalent EIRP of 20 dBm (-10 dBW) under satellite coverage. - Equivalent G/T of -36,6 dB/K under satellite coverage..

Observation 2:	For UE antenna polarization, it has been always assumed that VSAT antenna is circular polarization and handheld UE antenna is linear polarization, and the polarization mismatch of 3dB has already been taken into account in the link budget computation.

Moreover, from practical implementation perspective, it is a question for a mobile handheld device to select Left Hand Circular Polarization or Right Hand Circular Polarization, because a LPCP antenna when one side pointing to satellite would become a RHCP when the opposite side pointing to satellite.

Observation 3:	The mobility characteristic of handheld UE does not allow circular polarization implementation.

Based on above, it should be clear that the UE antenna polarization assumption for handheld UE should be linear polarization.

Proposal 3:	UE antenna polarization assumption for handheld UE should be linear polarization.

For TE antenna polarization, various technical justification has been provided previously. It is not necessarily to adopt circular polarization either.

Proposal 4:	TE antenna polarization for handheld UE NTN OTA testing should also be linear polarization.

3. 	Conclusion

Observation 1:	it can be expected that Scenario 1 (both talk mode and browsing mode) will be the dominant usage scenario for handheld NTN UE.

Proposal 1:	For handheld NTN UE OTA, prioritize the Scenario 1 (both talk mode and browsing mode) in Rel-19. Scenario 2 (hand only) can be considered depending on future available devices in future release or future generation.

Proposal 2:	Separate the polarization issue into two aspects: UE antenna polarization and TE antenna polarization.

Observation 2:	For UE antenna polarization, it has been always assumed that VSAT antenna is circular polarization and handheld UE antenna is linear polarization, and the polarization mismatch of 3dB has already been taken into account in the link budget computation.

Observation 3:	The mobility characteristic of handheld UE does not allow circular polarization implementation.

Proposal 3:	UE antenna polarization assumption for handheld UE should be linear polarization.

Proposal 4:	TE antenna polarization for handheld UE NTN OTA testing should also be linear polarization.

4. 	References

- R4-2419808    “WF for [113][325] TRP_TRS_Ph3”, vivo

- RP-242438    “RAN Chair’s Summary of 5G-Advanced in Rel-20”, RAN Chair