3GPP TSG RAN WG4 Meeting #114	R4-2500510

Athens, Greece, 17th – 21st February 2025

Source:	Rohde & Schwarz

Title:	Inclusion of Plane Wave Synthesizer (PWS) method for Expected EIRP test procedures

Agenda item:	7.13.2

Document for:	Discussion

# Introduction

At the last RAN4 meeting (RAN4#113 in Orlando) it was suggested that a technical paper be brought up to discuss and approve the inclusion of PWS into the 38.908 specification and related documents. This document aims to support the reasons why PWS should be considered.

# Discussion

PWS is a 3GPP approved OTA measurement system. It is described as an extension of a CATR measurement method where the CATR reflector is replaced by an antenna array, or a Plane Wave Synthesizer (PWS). This PWS can approximate a plane wave within a specified quiet zone in front of the antenna in a similar manner as the reflector in a CATR system.

The principle of the Plane Wave Synthesizer for Tx requirements is shown in figure below

PWS measurement setup, Tx requirements

By adjusting and setting the phase and amplitude of the array signals, a quiet zone containing planar waves as a linear superposition of the array elements' spherical radiation waves can be created within the near field of the array.

The PWS can hence achieve far-field testing conditions in a Quiet Zone (QZ), just as in the case of CATRs. Thereby it enables direct measurements of far-field BS performance in a controlled indoor environment as an alternative to CATR.

Observation 1: PWS based system is 3GPP approved and can achieve far-field test conditions with a small measurement distance.

From our real-world experiments in mid FR1 bands, we see that the PWS concept provides quite accurate measurements when compared to commercial chambers. We thoughtfullly foresee that the same should hold good for higher FR1 bands (including n104).

In the above experiment, a high-gain passive antenna array with dimensions of 60 cm × 60 cm was measured in a certified antenna chamber to provide far-field results(pink line). The DUT dimensions correspond to a far-field distance of around 12 m at 2.4 GHz. The verification of the antenna gain with a PWS was performed in the DUT's radiated near field at a distance of 1.5 m(blue line). The realized gain, sidelobe level and location of nulls show excellent agreement. On the contrary when a single antenna is used instead of the PWS, the results are not as good(black line).

Observation 2: PWS based measurements are accurate and a proven OTA measurement system

Many test campaigns have been conducted with base station manufactures and a test house with massive MIMO BS samples. The results showed an excellent correlation of both the radiation pattern and equivalent isotropic radiated power (EIRP) transmit power, compared to measurements within a CATR.

In the above image, we see the radiated field distribution when using PWS. Magnitude (left) and phase (right) at 3600 MHz are within a quiet zone of 1.00 m

# Summary

Observation 1: PWS based system is 3GPP approved and can achieve far-field test conditions with a small measurement distance.

Observation 2: PWS based measurements are accurate and a proven OTA measurement system

Proposal 1: RAN4 agrees on PWS inclusion in test procedures as suggested in tdoc R4-2500548

# References

[1]	Radio Frequency (RF) conformance testing background for radiated Base Station (BS) requirements, 37.941

[2] 	R&S Plane Wave Converter, PWC200

[3]	PWS method inclusion for Expected EIRP test procedures, R4-2500548