3GPP TSG-RAN WG4 Meeting #114									                       R4-2500526

Athens, GR, 17th – 21st February, 2025

Agenda item:			7.5.4

Source:	Moderator (Huawei, HiSilicon)

Title:	Topic summary for [114][212] NonCol_intraB_ENDC_NR_CA_Ph2

Document for:	Information

Introduction

This email thread discusses the RRM core requirements of WI on R19 Support of intra-band non-collocated EN-DC/NR-CA deployment Phase2: new receiver type(s).

Online handling:

(Online) Issue 1-1: Update on RRM requirements for FR1 non-collocated intra-band EN-DC/NR-CA for Type 4 UE in Rel-19

Topic #1:	RRM requirements

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2500440 | Apple | Proposal 1: RRM requirement impact should be reviewed again once the signalling design is concluded. |
| R4-2500441 | Apple | Draft CR |
| R4-2500470 | Nokia, Nokia Shanghai Bell | Proposal 1: To wait for RF conclusion on BS signaling and initial operation type for Type 4 UE. Proposal 2: To discuss the text proposal on MRTD/MTTD. |
| R4-2500705 | ZTE Corporation, Sanechips | Draft CR |
| R4-2501229 | Huawei, HiSilicon | Draft CR |
| R4-2501616 | Ericsson | Observation 1: We have assumed same logic and structure as existing release-18 text, with the addition of signaling element [nonCollocatedTypeNR-CA-r19] in brackets. The final signaling for release-19 is not yet agreed and the CRs will have to be updated accordingly when we reach agreement. Proposal 1: Approve CRs in [2] and [3] according to work split. |
| R4-2501617 | Ericsson | Draft CR |
| R4-2501618 | Ericsson | Draft CR |
| R4-2501681 | Samsung | Draft CR |

Open issues summary

Sub-topic 1: RRM requirements impact

(Online) Issue 1-1: Update on RRM requirements for FR1 non-collocated intra-band EN-DC/NR-CA for Type 4 UE in Rel-19

Background: In RRM session, in RAN4#112bis meeting, the impacted RRM core requirements were identified in [R4-2416850], and in RAN4#113 meeting, CR split among companies was approved in [R4-2420090].

In RF session, the UE capability and network signalling for Type 4 UE are still under discussion.

- Proposals

  - Option 1 (Apple): RRM requirement impact should be reviewed again once the signalling design for Type 4 UE is concluded.

  - Option 2 (Nokia): To wait for RF conclusion on BS signaling and initial operation type for Type 4 UE and discuss the text proposal on MRTD/MTTD.

  - Option 3 (Ericsson): Approve CRs according to work split.

- Recommended WF:

- In draft CR/TP, the UE capability “supporting R19 intra-band non-collocated” and the detailed description of “R19 type 4 capable UE is in non-collocated condition” and “R19 type 4 capable UE is in collocated condition” with regard to BS signalling in the identified RRM requirements will be updated, once the UE capability and BS signalling are concluded in RF session.

- Further discuss on whether to endorse the draft CRs submitted at this meeting.