# 3GPP TSG-RAN WG4 Meeting #114                               R4-2500551

# Athens, Greece, 17th-21st Feb, 2025

# Source:        ZTE Corporation, Sanechips

# Title:           Discussion on system parameters for LP-WUS/WUR

# Agenda Item:   7.26.2.1

# Document for:  Approval

- Introduction

In this contribution, we provide our view on the channel raster for LP-WUR.

- Discussion

In last RAN4 meeting, RAN4 achieved an agreement that no new channel raster is dedicated for LP-WUS/WUR[1]. But whether there is need to limit set of frequencies supported by LP-WUS to reduce the receiver complexity and cost needs further discussion.

Issue 1-2-1: Channel raster for LP-WUR Agreements: - No new channel raster is introduced dedicated for LP-WUS/WUR. - FFS on whether there is need of the limited set of frequencies supported by LP-WUS to reduce the receiver complexity and cost

Thanks to the contribution from CATT [2], it is verified that the center of a flexibly allocated LP-WUS is at fixed frequency points, i.e., n*10kHz, and this is why some companies wanted to introduce channel raster for LP-WUS at the beginning.

However, in our understanding, as long as the center of LP-WUS has been known with fixed potential frequency points in an NR carrier, they form a set of frequency points naturally. On one hand, extending this frequency set is meaningless, because LP-WUS will only be located on this n*10kHz frequency set. On the other hand, if we further reduce this frequency set, the location of an LP-WUS within an NR carrier will be very limited, which will limit the flexibility of network configuration. This is not expected from the perspective of network.

With the considerations above, we think RAN4 should not specifically introduce a limited set of frequencies supported by LP-WUS. The known n*10kHz frequency points in an NR carrier for LP-WUS can already help LP-WUR to reduce complexity and cost.

Proposal 1: RAN4 should not specifically introduce a limited set of frequencies supported by LP-WUS.

- Conclusion

Proposal 1: RAN4 should not specifically introduce a limited set of frequencies supported by LP-WUS.

- Reference

- R4-2420394 WF on UE RF requirements for LP-WUS, vivo, RAN4#113, Orlando, USA, Dec. 2024.

- R4-2415130 Further discussion on system parameters for LP-WUS, CATT, RAN4#112bis, Hefei, China, Oct. 2024.