# 3GPP TSG-RAN WG4 Meeting #114                               R4-2500557

# Athens, Greece, 17th-21st Feb, 2025

# Source:        ZTE Corporation, Sanechips

# Title:           Discussion on system parameters and UE RF requirements to introduce 7MHz for n5 and n26

# Agenda Item:   7.9.2

# Document for:  Approval

- Introduction

In last RAN plenary meeting, a new WI to introduce 7MHz CBW for bands n26 and n5 was approved [1]. The objective of this WI is:

The core part of this WI includes: - Specify the generic core requirements to support 7 MHz NR FR1 channel bandwidth [RAN4] - Introduce the channel bandwidth 7MHz - Only include 15 kHz SCS - Specify the spectrum utilization - Specify UE RF requirements for UL and DL - Specify BS RF requirements for UL and DL - Specify the band specific core requirements to introduce 7 MHz for NR band n5, n26. [RAN4] - BS and UE Channel bandwidth per operating band - UE reference sensitivity - When needed: - MPR (relative bandwidth criteria) - Additional Maximum Power Reduction (A-MPR) - Enable, if feasible, optional UE support of 7 MHz channel bandwidth from Rel-15 onwards in a release independent manner. [RAN2, RAN4] - No impact to 38.307 is expected. - Once RAN4 has introduced the new channel bandwidth, RAN2 will introduce the necessary signalling for this channel bandwidth. [RAN2] Note: This should be similar to the signalling for 35 and 45 MHz channel bandwidths. This signalling should take into account that also additional bands (not considered by this WI) may consider this new channel bandwidth in the future. RAN2 will be informed by RAN4 via LS. No additional RAN2 TUs will be requested for this objective. - Once RAN4 has introduced the new channel bandwidth, RAN3 will introduce the necessary signalling for this channel bandwidth [RAN3] Note: This should be similar to the signalling for 35 and 45 MHz channel bandwidths. This signalling should take into account that also additional bands (not considered by this WI) may consider this new channel bandwidth in the future. RAN3 will be informed by RAN4 via LS. No additional RAN3 TUs will be requested for this objective. NOTE: Once this WI is completed, the 7 MHz bandwidth can be introduced also for other bands (not addressed in this WI) via the appropriate basket WI.

First of all, the system parameters should be defined then we can come to requirements. In this contribution, we provide our views on the system parameters for 7MHz CBW and the impacts on UE RF requirements.

- Discussion

## 2.1 System parameters

For maximum transmission bandwidth configuration, the spectrum utilization is the most important issue. Based on some mathematical calculation and analysis, we derive that 36 RBs are most suitable for 7MHz CBW. The configuration of 7MHz with NRB=36 with 92.57% spectrum utilization is list in Table 1.

Table 1  Maximum transmission bandwidth configuration for 7MHz CBW with NRB=36

| CBW(MHz) | 3MHz | 5MHz | 7MHz | 10MHz | 20MHz |
| --- | --- | --- | --- | --- | --- |
| SCS(kHz) | 15 | 15 | 15 | 15 | 15 |
| NRB | 15 | 25 | 36 | 52 | 106 |
| Spectrum Utilization | 90% | 90% | 92.57% | 93.6% | 95.76% |

Then, the minimum guardband (kHz) can be calculated and listed in Table 2.

Table 2  Minimum guardband (kHz) for 7MHz CBW with NRB=36

| SCS(kHz) | 3MHz | 5MHz | 7MHz | 10MHz | 20MHz |
| --- | --- | --- | --- | --- | --- |
| 15 | 142.5 | 242.5 | 252.5 | 312.5 | 382.5 |
| 30 | N/A | 505 | N/A | 665 | 645 |
| 60 | N/A | N/A | N/A | 1010 | 990 |

Proposal 1: Maximum transmission bandwidth configuration for 7MHz CBW could be 36 RBs.

Proposal 2: Minimum guardband for 7MHz CBW could be 252.5kHz.

## 2.2 UE RF requirements

Then after system parameters are defined, the RF requirements should be defined for 7MHz CBW. The spec impact on TS38.101-1 and our views on the UE RF requirements are listed in Table 3.

Table 3  Spec impact on UE RF to introduce 7MHz CBW

| Clause | Requirements | ZTE’s views on the UE RF requirements |
| --- | --- | --- |
| 6.2.3 | UE additional maximum output power reduction | Evaluate A-MPR for 7MHz CBW for band n5 and n26. Add 7MHz CBW to A-MPR requirements and introduce A-MPR regions and values for NS_12, NS_13, NS_14 and NS_15, related to 6.5.3.3 |
| 6.3.1 | Minimum output power | Add 7MHz CBW, same as legacy 3~20MHz CBW |
| 6.3.2 | Transmit OFF power | Add 7MHz CBW, same as legacy 3~50MHz CBW |
| 6.5.1 | Occupied bandwidth | Add 7MHz CBW, same as legacy 3~50MHz CBW |
| 6.5.2.2 | Spectrum emission mask | Introduce general SEM for 7MHz CBW, including ΔfOOB region division and spectrum emission limit, add 7MHz to legacy 10~45MHz column |
| 6.5.2.4 | Adjacent channel leakage ratio | Add 7MHz CBW, same as legacy 3~50MHz CBW |
| 6.5.3.1 | General spurious emissions | Add 7MHz CBW, same as legacy 5~100MHz CBW |
| 6.5.3.3 | Additional spurious emissions | Add 7MHz CBW to spurious emissions requirements for NS_12, NS_13, NS_14 and NS_15 if needed, related to 6.2.3 |
| 7.3.2 | ### Reference sensitivity power level | Evaluate reference sensitivity for 7MHz CBW for n26 and n5 |
| 7.4 | Maximum input level | Add 7MHz CBW, same as legacy 3~20MHz CBW |
| 7.5 | Adjacent channel selectivity | Add 7MHz CBW, same as legacy 3, 5, 10MHz CBW |
| 7.6.2 | In-band blocking | Add 7MHz CBW, same as legacy 5, 10MHz CBW with FDL_high < 2700 MHz and FUL_high < 2700 MHz |
| 7.6.3 | Out-of-band blocking | Add 7MHz CBW, same as legacy 3, 5, 10MHz CBW |
| 7.6.4 | Narrow band blocking | Add 7MHz CBW, evaluate Pw for 7MHz CBW |
| 7.7 | Spurious response | Add 7MHz CBW, same as legacy 3, 5, 10MHz CBW with FDL_high < 2700 MHz and FUL_high < 2700 MHz |
| 7.8.2 | Wide band Intermodulation | Add 7MHz CBW, same as legacy 5, 10MHz CBW with FDL_high < 2700 MHz and FUL_high < 2700 MHz |

Proposal 3: The UE RF requirements for 7MHz CBW as listed in Table 3 should be discussed in RAN4.

- Conclusion

Proposal 1: Maximum transmission bandwidth configuration for 7MHz CBW could be 36 RBs.

Proposal 2: Minimum guardband for 7MHz CBW could be 252.5kHz.

Proposal 3: The UE RF requirements for 7MHz CBW as listed in Table 3 should be discussed in RAN4.

- Reference

- RP-243323 New WID on 7 MHz Channel Bandwidth for n26 and n5, China Telecom, T-mobile USA, Madrid, ES, Dec. 2024.