# 3GPP TSG-RAN WG4 Meeting # 114		  R4-2500558

# Athens, GR, 17th – 21st February, 2025

# Source:	ZTE Corporation, Sanechips

# Title:	Discussion on BS RF core requirements for supporting 7 MHz NR FR1 channel bandwidth

# Agenda Item:	7.9.3

# Document for:	Approval

- Introduction

During RAN#106 meeting, a new Work Item [1] has been approved on 7 MHz Channel Bandwidth for n26 and n5. In this contribution, some initial views on BS RF requirements and an overview of expected impacts when considering such introduction are provided. The objective of this WI related to BS RF is shown as below:

- Specify the generic core requirements to support 7 MHz NR FR1 channel bandwidth [RAN4]

  - Introduce the channel bandwidth 7MHz

    - Only include 15 kHz SCS

  - Specify the spectrum utilization

  - Specify UE RF requirements for UL and DL

  - Specify BS RF requirements for UL and DL

- Specify the band specific core requirements to introduce 7 MHz for NR band n5, n26. [RAN4]

  - BS and UE Channel bandwidth per operating band

  - UE reference sensitivity

  - When needed:

    - MPR (relative bandwidth criteria)

    - Additional Maximum Power Reduction (A-MPR)

- Discussion

2.1 General aspect

According to the WID, only native 7 MHz channel bandwidths for n5 and n26 in Rel-19 was included, at least single carrier operation need to be considered at the start point. From BS perspective, there are many RF requirements defined related to channel bandwidth, and also there are several features such as NB-IoT and CA , are supported. For example, the CA CACLR requirements are also defined for each channel bandwidth. However, it is unclear whether or not 7MHz is supported for CA from the objectives. It is our understanding that only single carrier operation is considered for this WID, but still we would like to clarify this scope first.

Proposal1: To clarify whether only single carrier is considered for new added 7MHz WID.

To specify the BS RF requirement, a number of Fixed Reference Channels (FRCs) have been defined for almost all Rx requirements such as reference sensitivity level, ACS, in-band blocking, out-of-band blocking, receiver intermodulation, in-channel selectivity, OTA sensitivity, OTA reference sensitivity level, OTA ACS, OTA in-band blocking, OTA out-of-band blocking, OTA receiver intermodulation and OTA in-channel selectivity. The legacy BS channel CB of NR FRCs are 3M, 5M, 10M and 20M, so for 7 MHz CB, RAN4 need to determine whether to define new FRCs for 7 MHz or to reuse the legacy FRCs defined for 5 MHz/15 k SCS CBW. If legacy FRCs defined for 5 MHz/15 k SCS CBW are reused for 7 MHz CBW, then some overlapping of FRC occurrences is needed for conformance testing, which may increase the test burden. And conversely, defining new FRCs for 7 MHz can avoid this situation, but the 7 MHz/15 k SCS specific FRC parameters need to be determined for each Rx requirement.

Proposal2: To decide whether to define new FRCs or to reuse the legacy FRCs defined for 5 MHz/15 k SCS CBW for new added 7MHz/15 k SCS CBW.

2.2 BS RF requirement for 7 MHz channel bandwidths for n5 and n26

The BS RF requirements can be divided into two categories: the first category is band/channel bandwidth agnostic requirements, and the second category is band/channel bandwidth specified requirements For the former one, the requirements such as base station output power, transmit ON/OFF power, transmitted signal quality, occupied bandwidth, etc, are defined as channel bandwidth agnostic way, which means these requirements are not impacted with introduction of  the 7 MHz channel bandwidth. While for the latter one, almost all of the Rx requirements are defined based on channel bandwidth. Thus for these requirement, the newly 7MHz channel bandwidth shall be included.

For Rx requirements, supposing that Fixed Reference Channels defined for 5 MHz/15 k SCS CBW are reused for the newly added 7 MHz/15 k SCS channel bandwidth in this section. In table 1, we give a briefly summarize on the requirements which need to consider 7 MHz channel bandwidths for n5 and n26, and meanwhile give the proposal on how to include the 7MHz:

Table 1: Summary of the proposals for BS RF requirement update for 7 MHz channel bandwidths for n5 and n26

| Tx RF requirements part |  |
| --- | --- |
| Tx requirements to update | Proposals |
| Total power dynamic range | -To add Total power dynamic range requirement for 7 MHz CBW: --Total power dynamic range for 7 MHz/15k SCS can be derived according to transmission bandwidth configuration of 7 MHz CBW, Based on NRB proposed in another contribution[3], we can determine that total power dynamic range for 7 MHz/15k SCS is 15.5 dB. |
| ACLR | -To add ACLR requirement for 7MHz CBW: --Reuse the existing ACLR requirements, add 7MHz into the table. |
| Rx RF requirements part |  |
| Rx requirements to update | Proposals |
| Reference sensitivity level | -To add (OTA) REFSENS requirement for 7MHz CBW: --Reference measurement channel defined for 5 MHz/15k SCS CBW can be reused for 7 MHz/15k SCS CBW. |
| OTA reference sensitivity level |  |
| Dynamic range | -To add (OTA) dynamic range requirement for 7MHz CBW: --Reference measurement channel defined for 5 MHz/15k SCS CBW can be reused for 7 MHz/15k SCS CBW. --for conducted requirement, interfering signal mean power for 7 MHz/15k SCS is -80.9 dBm for WA, -75.9 dBm for MR and -72.9 dBm for LA. --for OTA requirement, interfering signal mean power for 7 MHz/15k SCS is -80.9 - ΔOTAREFSENS dBm for WA, -75.9- ΔOTAREFSENS dBm for MR and -72.9 - ΔOTAREFSENS dBm for LA. |
| OTA dynamic range |  |
| ACS | -To add base station (OTA) ACS requirement associated with interferer frequency offset value for 7MHz CBW. --For 7MHz CBW, we propose to use the same 5MHz DFT-s-OFDM NR signal as the interfering signal, then the （OTA）ACS interferer frequency offset value can be obtained as ±2.5075 MHz. |
| OTA ACS |  |
| In-band blocking | -To add (OTA) In-band blocking requirement for 7MHz CBW: --To add base station general blocking requirement for 7MHz CBW. --For Base Station (OTA) narrowband blocking interferer frequency offsets, ±(355+m*180) kHz, m=0, 1, 2, 3, 4, 9, 14, 19, 24 is proposed for 7MHz/15k SCS CBW. |
| OTA in-band blocking |  |
| RX IMD | -To add (OTA) RX IMD requirement for 7MHz CBW: --for interfering signals for intermodulation requirement, CW and 5 MHz DFT-s-OFDM NR signal are considered for 7 MHz CBW interfering signals type, and then interfering signal centre frequency offset values of ±7.495 MHz@CW, ±17.5 MHz@5 MHz DFT-s-OFDM NR signal are considered for the interfering signal centre frequency offset. --for interfering signals for narrowband intermodulation requirement, CW and 5 MHz DFT-s-OFDM NR signal 1 RB are considered for 7 MHz CBW interfering signals type, and then interfering signal centre frequency offset values of ±400 kHz@CW, ±1240 kHz@5 MHz DFT-s-OFDM NR signal 1 RB are considered for the interfering signal centre frequency offset. |
| OTA RX IMD |  |
| ICS | -To add (OTA) ICS requirement for 7MHz CBW: --reuse the requirements of 5 MHz/15 k SCS for 7MHz CBW. |
| OTA ICS |  |

Proposal3:To consider the proposals in Table 1 for BS RF requirement for 7 MHz channel bandwidths for n5 and n26.

- Conclusion

In this contribution, some initial views on BS RF requirements are provided and the proposals ares made as following:

Proposal1: To clarify whether only single carrier is considered for new added 7MHz WID.

Proposal2: To decide whether to define new FRCs or to reuse the legacy FRCs defined for 5 MHz/15 k SCS CBW for new added 7MHz/15 k SCS CBW.

Proposal3:To consider the proposals in Table 1 for BS RF requirement for 7 MHz channel bandwidths for n5 and n26.

- Reference

RP-243323, 7 MHz Channel Bandwidth for n26 and n5, China Telecom, T-Mobile USA.

3GPP TS38.104 NR Base Station (BS) radio transmission and reception.

R4-2500557 Discussion on system parameters and UE RF requirements to introduce 7MHz for n5 and n26, ZTE, RAN4#114, Athens, GR, Feb 2025.