3GPP TSG-RAN WG4 Meeting #114	R4-2500610

Athens, Greece, 17th – 21st February 2025

# Source: 	ZTE Corporation, Sanechips

# Title: 	Discussion on SAN RF requirements for NR NTN Ku band

# Agenda item:	7.11.5

# Document for:	Approval

# 1 Introduction

In RAN#106 meeting, a revised Work Item [1] was approved on introduction of Ku Band for NR NTN for NR NTN with the following RAN4 objective. In this contribution, we want to share some further views about NR NTN Ku band from system parameters perspective.

The objectives are: - Use the regulation requirements in ITU Regions 1, 2 and 3 to identify relevant adjacent band co-existence scenarios for NTN Ku Band covering the following frequency ranges, considering targeted deployment scenarios: - For ITU regions 1 and 3: - Downlink 10.70 – 12.75 GHz - Uplink 13.75-14.5 GHz - For ITU regions 2 (The US is in scope for the WI using the current allocation of the Ku band. The band introduction in specification is pending FCC decision): - Downlink 10.70 – 12.70 GHz - Uplink 13.75-14.5 GHz - For ITU regions 1 and 3 - Downlink 10.70 – 12.75 GHz - Uplink 12.75 - 13.25 GHz - For ITU regions 2 (The US is in scope for the WI using the current allocation of the Ku band. The band introduction in specification is pending FCC decision): - Downlink 10.70 – 12.70 GHz - Uplink 12.70 - 13.25 GHz - Note: Appropriate Tx/Rx separation for uplink downlink pairing will be required. - Conduct coexistence study. Reuse Ka band coexistence study assumptions as much as possible, with modifications where necessary, e.g. for coexistence scenario, frequency range and subcarrier spacing. Conduct new co-existence studies based on adjacent services. - Specify RF requirements for mobile VSAT served by NGSO and GSO based on existing mobile VSAT type 5 RF requirements - Investigate and if possible specify common FR1 and FR2 radiated RF requirements for Satellite Access Node (SAN) and NTN VSAT types (excluding types 1 to 3) in 38.101-5 also considering existing regulations on antenna sizes for certain parts of the Ku band. - Priority 1: for all ITU regions, define Ku band #1a and #1b for the downlink block and uplink block B. Define two sets of bands, one for FR1 numerology and the other for FR2 numerology, including the support of the following: - Ku band #1a and #1b as FR1-NTN supporting 10 MHz, 20 MHz, 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing - Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing. - Priority 2: for all ITU regions excluding the US in region 2, using the approach in Priority 1, define Ku band #2a and #2b as appropriate for the downlink block and uplink block A. - Specify RRM requirements including timing requirements for mobile VSAT served by NGSO - NOTE: RRM requirements are defined for VSAT supporting FR1-NTN operation only, or for FR2-NTN operation only. - RAN#107 checkpoint: Depending on the feedback from RAN1 and RAN2 on the LS in R4-2419902, further WID updates may be needed to address feedback on specifying bands for both FR1-NTN numerology and FR2-NTN numerology or for other issues such as support for a single SCS per frequency band.

# 2 Discussion

  - 2.1 SAN types

There are three SAN types defined in TS 38.108: SAN type 1-H, SAN type 1-O and SAN type 2-O, and only SAN type 2-O is specified for FR2-NTN. According to the objective of the revised WID, it states that investigate and if possible specify common FR1 and FR2 radiated RF requirements for Satellite Access Node (SAN) and NTN VSAT types (excluding types 1 to 3) in 38.101-5. Besides, it also specifies that define two sets of bands, one for FR1 numerology and the other for FR2 numerology. Hence, we propose that SAN type 1-O and SAN type 2-O should be supported for Ku band.

Proposal 1: It is proposed that SAN type 1-O and SAN type 2-O should be supported for Ku band.

  - 2.2 SAN RF requirements

According to the objective of the revised WID, radiated requirements should be specified for SAN for Ku band. Based on the above proposal for SAN types, we make the following proposals for SAN radiated RF requirements for Ku band.

Table 2.2 SAN RF requirements for NTN Ku band

| NR BS RF Tx/Rx requirements | Proposal for NTN Ku band |
| --- | --- |
| Radiated Tx Requirements |  |
| 9.2 Radiated transmit power | No specification impact. |
| 9.3 OTA SAN output power | No specification impact. |
| 9.4 OTA output power dynamics | For SAN type 1-O, the requirement is same as SAN type 1-H. As for total power dynamic range for SAN type 1-H, it only specifies the requirement for 5 MHz, 10 MHz, 15 MHz, 20 MHz. For Ku band as FR1-NTN, it should support 10 MHz, 20 MHz, 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing. Hence, total power dynamic range for SAN type 1-O should be defined for 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing, and we propose to define same requirement as in Table 6.3.3.2-1 in TS 38.104. For SAN type 2-O, no specification impact. |
| 9.5 OTA transmit ON/OFF power | Not applicable. |
| 9.6 OTA transmitted signal quality | No specification impact. |
| 9.7.2 OTA occupied bandwidth | No specification impact. |
| 9.7.3 OTA Adjacent Channel Leakage Power Ratio | It depends on the outcome of coexistence study. |
| 9.7.4 OTA out-of-band emissions | No specification impact. |
| 9.7.5 OTA transmitter spurious emissions | No specification impact. |
| 9.8 OTA transmitter intermodulation | Not applicable. |
| Radiated Rx Requirements |  |
| 10.2 OTA sensitivity | For SAN type 1-O, it should specify OTA sensitivity requirement for 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing. For SAN type 2-O, no specification impact. |
| 10.3 OTA reference sensitivity level | For SAN type 1-O, it should specify OTA REFSENS requirement for 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing. For SAN type 2-O, no specification impact. |
| 10.4 OTA dynamic range | For SAN type 1-O, it should specify dynamic range requirement for 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing. |
| 10.5.1 OTA adjacent channel selectivity | It depends on the outcome of coexistence study. |
| 10.5.2 OTA in-band blocking | Not applicable. |
| 10.6 OTA out-of-band blocking | For SAN type 1-O, it should specify ∆fOOB offset requirement for Ku band. For SAN type 2-O, no specification impact. |
| 10.7 OTA receiver spurious emissions | Not applicable. |
| 10.8 OTA receiver intermodulation | Not applicable. |
| 10.9 OTA in-channel selectivity | For SAN type 1-O, it should specify OTA ICS requirement for 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing. For SAN type 2-O, no specification impact. |

Proposal 2: We make the proposals as Table 2.2 for SAN radiated RF requirements for NTN Ku band.

# 3 Conclusion

In this contribution, we share some views on SAN RF requirements for NTN Ku band and the proposals are made as follows:

Proposal 1: It is proposed that SAN type 1-O and SAN type 2-O should be supported for Ku band.

Proposal 2: We make the proposals as Table 2.2 for SAN RF requirements for NTN Ku band.

# 4 Reference

- RP-243314, Revised WID: Introduction of Ku bands for NR NTN, Intelsat et al.

- R4-2419790, Way Forward for [112bis][314] NR_NTN_Ku_Band_UE_SAN_RF, Moderator (CHTTL)