3GPP TSG-RAN4 Meeting #114	R4-2500613

# Athens, Greece,  16th – 21st February, 2025

Title:	UE mechanism to trigger one RF RX chain mode

Source:	Qualcomm Incorporated

Agenda item:	7.6.2

Release:	Rel-19

WI: 	FS_NR_FR1_DL_Frag_Carrier

Lead WG: 	RAN4

Document for:	Approval

# 1.	Introduction

Ran4 approved a WF [2] on Dl Fragmented carriers study SI [1]. In this paper we discuss the UE behaviour for this feature.

# 2. 	Discussion

WF [2] discusses Triggering condition to enable ”One RF Rx chain”  and one possibility is that UE measures aggressor power in the gap between wanted CCs in question and compare that to the wanted CC power. This implies an autonomous switch between using two RX chains and one RX chain. Our understanding was that this feature was motivated by saving RX chains so that higher order DL CA can be supported with less number of RX chains. When conditions are unfavourable and then UE would use two RX chains for the fragmented carriers, would UE then disconnect a carrier from another band?

Observation 1: If UE triggers use of signal RX chain, will it then update it capabilities for higher order CA?

If this approach is chosen, and the trigger conditions to enable single RX chain is up to the UE, then we may not need a capability for this feature at all since UE will meet the exiting NC DL CA requirements since they are tested in the presence of an aggressor.

Observation 2: If decision to enable single RX chain is up to the UE, we may not need dedicated capabilities for this feature.

However, the triggering between one and two RX chain would cause an interruption on both of the fragmented DL carriers and this will cause DL performance degradation. Network should know when this happens but if the trigger is autonomous in the UE, then UE needs to send some indication to the network about this. This may be a good reason for a capability.

Proposal: If UE autonomously triggers one RX chain mode, send LS to RAN1 to inform them that UE needs be able to indicate the instance of the DL interruption caused by triggering switch to and from using one RX chain to receive fragmented carriers.

Despite making the above proposals, it may be better approach to indicate that UE will always support band combination with single RX chain when this intra-band NC DL CA is part of higher order CA combination. If you wants to be opportunistic and perhaps for power saving reasons, it can trigger single RX chain on it own transparently from the network and specification.

# Conclusion

We discussed and provide input to the WF [2] issues 2-4: Signalling related and 2-5: Triggering condition to enable ”One RF Rx chain” and 3-3: Discussion of impacts on DL performance for UE enabling fragmented carrier mode.

We made two observations:

Observation 1: If UE triggers use of signal RX chain, will it then update it capabilities for higher order CA?

Observation 2: If decision to enable single RX chain is up to the UE, we may not need dedicated capabilities for this feature.

And one proposal:

Proposal: If UE autonomously triggers one RX chain mode, send LS to RAN1 to inform them that UE needs be able to indicate the instance of the DL interruption caused by triggering switch to and from using one RX chain to receive fragmented carriers.

# Reference

[1]	RP-243002, “Revised SID: Study on NR FR1 DL fragmented carriers”, MediaTek Inc., 3GPP TSG RAN Meeting #106, Madrid, Spain, Dec 09-12, 2024

[2]	R4-2420519, “WF on NR FR1 DL fragmented carriers study”, MediaTek, 3GPP TSG-RAN WG4 Meeting #113, Orlando, US, 18th – 22nd November, 2024