3GPP TSG-RAN WG4 Meeting#114		R4-2500615

Athens, GR, Feb 17th -21st, 2025

Agenda item:	7.24.2.3

Source: 	Spreadtrum,UNISOC

Title: 	Discussion on RF requirements for CW

Document for:	Discussion

Introduction

In RAN4#113 meeting, in SI stage, conclusion in TR 38.769 about CW requirements is agreed. The WID [1] was approved. This contribution discussed CW requirements in the WI stage as follows.

- RAN4 scope: - Specify RF requirements for Ambient-IoT BS, device 1, and CW - RF requirements for Type 1-C Ambient-IoT BS - RF requirements for device 1 - RF requirements for CW - Specify RRM core requirements for device 1, if necessary - Study and develop OTA test methodology for A-IoT device 1 - Consider test methods specified in TR 38.870 as starting point. Take test system reuse, test system complexity and test time into account, when developing test methods suitable for Ambient IoT. - Develop the preliminary Measurement Uncertainty (MU) assessment for the test system - Use band n8 as an example band

Discussion

- Output power

According to the SID, just D1T1-B case 1-4 needs to consider, which means that CW uses UL spectrum comes from outside topology. During the co-existence study, CW output power is 23dBm when UL spectrum is used, CW output power is 33dBm when DL spectrum is used. With regard to the coverage range of device and outside topology. CW output power is 33dBm could be a starting point.

Proposal 1: CW output power is 33dBm could be a starting point.

- ON/OFF power

According to the WID, CW is a single-tone. The main purpose is to provide an external carrier for D2R. In our understanding, there is no need to define ON/OFF power.

Proposal 2: There is no need to define ON/OFF power.

- Frequency error

CW comes from outside topology, in our understanding, frequency error could be similar to NR to be 0.1ppm.

Proposal 3: Frequency error could be reuse 0.1ppm.

- Phase noise

In current spec, there are no phase noise requirements. As to CW, phase noise may infect the D2R receiver. CW cancellation capability needs to consider phase noise. However, there is no need to define phase noise for CW requirements.

Proposal 4: There is no need to define phase noise.

- Unwanted emission

The target output power of CW is 23dBm or 33dBm. The power is equivalent to or higher than normal UE. The band of CW is from licensed FDD spectrum. In our understanding, it should comply with relevant regulatory requirements in the local area. Hence, unwanted emission requirements need to be defined. The specific values need to be further discussed.

Proposal 5:  Unwanted emission requirements need to be defined. The specific values need to be further discussed.

- Intermodulation

CW is a single-tone, there is no intermodulation for CW itself. However, CW may inter modulate with NR signals, thus affecting the performance of NR. In our understanding, it is necessary to consider intermodulation.

Proposal 6: Intermodulation needs to be considered.

Conclusions

In this paper, we provide our views on RF requirements for ambient IoT device, we have made the following proposals.

Proposal 1: CW output power is 33dBm could be a starting point.

Proposal 2: There is no need to define ON/OFF power.

Proposal 3: Frequency error could be reuse 0.1ppm.

Proposal 4: There is no need to define phase noise.

Proposal 5:  Unwanted emission requirements need to be defined. The specific values need to be further discussed.

Proposal 6: Intermodulation needs to be considered.

Reference

- RP-243326, New Work Item: Solutions for Ambient IoT(Internet of Things) in NR, Huawei, TSG RAN-P meeting #106