3GPP TSG-RAN WG4 Meeting # 114	R4-2500618

# Athens, GR, 13th – 17th Feb., 2025

Source: 	Xiaomi

Title: 	Discussion on core requirement for AI beam management

Agenda Item:	7.19.3

Document for:	Discussion

- 1 Introduction

In this contribution, we will core part requirement for AI based beam management.

- Performance metric - RX beam sweeping - Measurement time && prediction time - Impact on TCI state activation - Measurement error

- Discussion

      - 2.1 Performance metric

According to agreement in last meeting, definition of RSRP prediction accuracy is agreed. Currently, there are 3 options for AI based beam management accuracy KPI:

Issue 3-4: Metrics/KPIs for beam prediction Proposals - Option 1: Use Option 1. - Option 2: Use Option 2 - Option 3: Use combined Option 2 and 3: - Option-New: The successful rate for the correct prediction which is considered as "the Top-1 predicted beam is one of the Top-K strongest beams, and the Top-1 predicted beam’s ground truth RSRP value is larger than the RSRP of the strongest beam – x dB” - FFS: K = 2 and x = 3 as baseline - Option 4: Add new metric: - Top-K/N (%): the percentage of "the Top-N genie-aided beam is one of the Top-K predicted beams". FFS how N is derived. - Option 5: Add a requirement based on relative difference between predictions under different conditions/inputs (for example varying signal levels by N dBs but maintain same relative difference). - consider defining multiple tests under different conditions but with the same outcome(same predicted beam) - Option 6: both RSRP accuracy and beam prediction accuracy are used as KPIs for beam prediction - RSRP absolute accuracy applies to all predicted beams For metrics for beam management requirements/tests, the following test metrics are identified and could be considered - Option 1: RSRP accuracy - Option 2: Beam prediction accuracy -Top-1 (%) : the percentage of "the Top-1 strongest beam is Top-1 predicted beam" -Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams" -Top-1/K (%) : the percentage of "the Top-1 predicted beam is one of the Top-K strongest beams" - Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB, -Related measurement accuracy can be considered to determine x

For option 2, it mainly targets for beam index prediction accuracy.

For Top-K/1 metric, it requires that UE can successfully found the ground truth of Top-1 beam index by measurement and sorting. However, we found that it’s difficult for UE to identify the ground truth of Top-1 beam by measurement even at high SNR conditions. That’s because in some cases, the L1-RSRP of Top-1 beam and Top-2 beam are quite close. It requires very tight measurement accuracy.

Suppose that there is total 32 TX beams in set A. Next, we will first provide the CDF curve of ideal L1-RSRP difference between best 1st and nth beams for 32 TX beams in Fig.1.

L1-RSRP delta between 1th and 2th best beams is L1-RSRP difference between the largest and the second largest beams. RSRP delta between 1th and 3th best beams is RSRP difference between the largest and the third largest beams, and so on. It is noted that the CDF in all figures is calculated based on ideal L1-RSRP without any error. The ideal L1-RSRPs are derived from SLS channel model defined in 38.901, which is used to train AI model. The TX beam range is equally divided from -60 degree to 60 degree in azimuth. For each TX beam, there is 8 RX beams. The largest L1-RSRP by RX beam sweeping for each TX beam is selected.

Fig.1 Ideal L1-RSRP difference for 32 TX beams

In Fig.1, we take the CDF curve “RSRP delta between 1st and 2nd best beam” as an example. 10 percentile of CDF curve shows the L1-RSRP delta between 1st and 2nd best beam is smaller than 0.16dB for 10% cases. If UE can successfully distinguish between the 1st and 2nd beams in 90% case, the measurement accuracy has to be within 0.16dB. 20 percentile of CDF is 0.3dB, it means that UE can successfully distinguish between the 1st and 2nd beams in 80% case if measurement accuracy can be within 0.3dB.

As another example as shown in the CDF curve “RSRP delta between 1st and 4th best beam”, 10 percentile results in 1dB L1-RSRP difference. That means the measurement accuracy has to be within 1dB in order to successfully distinguish between the 1st and 4th beams in 90% case.

Besides, the beam resolution is dependent on total Tx beam numbers. The more Tx beams, the more difficult to identify the best beam.

When UE performs measurement to derive ground truth of beams, UE will have baseband(BB) measurement error and RF error. With high SNR, UE can reduce measurement error of BB, however, there will still be some measurement error due to RF. Since UE can’t distinguish the 1st best beam due to limited measurement accuracy, the Top-1 best needs to be extended to Top-N best beams.

Observation 1: For derive ground truth for best Top-1 beam, UE needs to measure for all beams in set A under high SNR. However, there is still some measurement uncertainty due to RF and BB. It’s challenging for UE to detect the best 1st beam even at high SNR due to the small gap between best 1st and 2nd beam.

Observation 2: For beam prediction accuracy, option 2 is not correct since UE can’t distinguish which is the best beam index due to measurement error.

Therefore, if beam index is used as reference, test metric will be:

- Top-K/N (%): the percentage of "the Top-N genie-aided beam is one of the Top-K predicted beams"

Here, Top-1 genie-aided beam is extended to Top-N genie-aided beams. UE will report predicted best K beams and ideal best N beams to TE. RAN4 needs to further discuss how to choose proper number N.

Another option is to use L1-RSPR difference, similar as option 3. Here, we refine option 3 as: the maximum measured L1-RSRP of top-K predicted beams is larger than the maximum measured L1-RSRP – X dB.

Proposal 1: For beam prediction accuracy performance metric,

- If beam index is used as reference, the test metric is:

  - Top-K/N (%): the percentage of "the Top-N genie-aided beam is one of the Top-K predicted beams". FFS how N is derived.

- If RSRP difference is used as reference, the test metric is:

  - The maximum measured L1-RSRP of top-K predicted beams is larger than the maximum measured L1-RSRP – X dB.

      - 2.2 RX beam sweeping

In legacy for FR2, RX beam sweeping is considered for SSB based measurement and CSI-RS with repletion ON.

Measurement period will be scaled by RX beam sweeping factor, e.g. N=8 for SSB.

Table 9.5.4.1-2: Measurement period TL1-RSRP_Measurement_Period_SSB for FR2

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(1.5*M*P*N)*TDRX |
| Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

For AI beam management, two kinds of RX beam are assumed in TR 38.843:

Beam prediction accuracy related KPIs, including: - Top-1 genie-aided Tx beam considers the following definitions: - Option A (baseline), the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx and Rx beams - Option B (optional), the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx beams with specific Rx beam(s) - Specific Rx beam(s) are to be reported. Note: specific Rx beams are a subset of all Rx beams.

For option B, the best beam can be predicted by specific RX beam(s). It means that UE can use subset RX beam to predict best TX beam and UE don’t need to sweep full RX beams. Therefore, it may have impact on RX beam sweeping factor in measurement period. Since RX beam sweeping will apply for spatial domain and time domain beam prediction, the impact to BM case-1 and BM case-2 is common.

Proposal 2: For both BM case-1 and BM case-2, RAN4 to discuss whether there is impact on measurement period due to specific RX beam assumption.

      - 2.3 Measurement time && prediction time

For BM-case 2, RAN1 agrees that NW can configure UE to predict future N L1-RSRP instance.

RAN1 #118 Agreement For UE-sided model for BM-Case 2, for inference results report, support to configure UE with N future time instance(s) for inference by NW when applicable - FFS: how to determinate reference time for the time instance(s) - FFS: duration values of the N time instance(s) that can be predicted.

Besides, TR defined several types for time-domain prediction:

Case A: based on number of measurements/RSs and prediction time. An example is shown in Figure 6.3.1-2. - where T2 is the time duration for beam prediction - where Mt is the number of time instances for measurement as AI/ML inputs with a periodicity of Tper - where Pt is the number of time instance(s) for prediction with a periodicity of Tper in T2 Case B: based on a periodicity T of the required reference signals for measurements to achieve a certain beam prediction accuracy. An example is shown in Figure 6.3.1-3. - For AI, every T=Y ms, reference signals for measurements are needed - In this case, - For Set B = Set A, the RS overhead reduction is 1-X/Y. - For Set B (N beams) is a subset of Set A (M beams), the RS overhead reduction is [1-XN/(YM)].

The figures for case A and case B from TR are shown below:

Figure 6.3.1-2: Example for Case A

Figure 6.3.1-3: Example for Case B

The main difference of the two cases is the length of measurement periodicity and prediction periodicity.

  - Case A: the periodicity for history measurement instance is same as that of predicted future time instance.

  - Case B: the periodicity for history measurement instance is larger than that of predicted future time instance.

From our observation, at least case A will have impact on UE measurement requirement. In legacy, measurements on all instance are needed. Measurement period is defined for each instance periodically. While when AI is applied, for case A, some measurement instance can be skipped and UE can acquire predicted results for these time slots. The legacy measurement period requirement can’t reflect such benefits introduced by AI. Suppose that there are totally 10 instances. In legacy, the total measurement time is 10*TRS. While when AI is applied, suppose that AI can measure for 5 instance and predict for another 5 instances, the total measurement time will be reduced.

Since RAN1 introduce the concept of measurement time and prediction time, RAN4 needs to discuss how to define new requirement for measurement time and prediction time for BM-case 2 either. If measurement time and prediction time is defined, RAN4 can further discuss whether measurement restriction and scheduling restriction can be relaxed in prediction time.

Proposal 3: For BM-case 2, RAN4 to discuss impact on measurement requirement, e.g. measurement time, prediction time, measurement restriction and scheduling restriction, etc.

      - 2.4 TCI state activation

UE will measure for set B first and then predict and report best TX beam index in set A, then NW can configure TCI state activation. There are two possibilities for the reported best predicted TX beam index:

- Case 1: best predicted TX beam is in set B.

- Case 2: best predicted TX beam is in set A but not in set B.

For case 1, UE knows the corresponding best RX beam since UE performs RX beam sweeping for all TX beams in set B. For case 2, UE may know or not know the best RX beam. It will depend on UE implementation. If the input of AI model and output of AI model are based on specific RX beam, UE can know the corresponding RX beam. If the input of AI model and output of AI model are based on RX beam sweeping, UE can first predict the L1-RSRP based on different TX-RX pairs, then UE sort L1-RSRP and report the best beam index. Then it’s possible for UE to know the corresponding best RX beam. If UE directly predict best TX beam index, it’s possible that UE didn’t know the RX beam index.

Observation 3: When best predicted TX beam is in set B, UE knows the best RX beam. When best predicted TX beam is in set A but not in set B, it’s still possible for UE to know the RX beam corresponding to the best TX beam.

Whether UE knows the RX beam or not will have impact on TCI state activation delay. Current known condition only considers whether UE has sent the report within a period and it didn’t consider whether UE has known the RX beam or not. We prefer to define UE capability to indicate whether UE can know the RX beam or not. Then the L1 reporting part in known condition for TCI state will be updated to:

The UE has sent at least 1 L1-RSRP report for the target downlink TCI state before the downlink TCI state switch command. Where, - the target TCI state in L1-RSRP report has been measured, or - the target TCI state in L1-RSRP report is predicted and UE support [RX beam known] capability

Proposal 4: RAN4 may know RX beam based on UE capability.

      - 2.5 Measurement error

L1-RSRP difference for beams in set B

For 8 TX beams in set B, there will be 8 L1-RSRPs. In order to have rough idea what’s the L1-RSRP difference between TX beams in set B, we calculate the ideal L1-RSRP delta for set B based on SLS channel model. L1-RSRP delta between best 1st beam and 8th beam can be larger than 20dB for 10% and larger than 15dB for 50% cases. There is a great SNR difference for different beams.

Observation 4: For set B, L1-RSRP difference between best 1st beam and 8th beam is large.

SNR difference Vs L1-RSRP difference

The L1-RSRP delta between the best 1st beam and other beam can be calculated in dB. Since the noise power for all TX beams is relatively similar, for simplicity, the L1-RSRP difference can be equal to SNR difference.

For SLS L1-RSRP, without considering impact of inference, the L1-RSRP can be compared with noise power to generate corresponding SNR. The impact of interference can be further studied. It can also

Proposal 5: For simplicity, L1-RSRP delta for different TX beams in set B can be equal to SNR difference for beams in set B.

Measurement error for different SNR

The measurement error for different SNR is simulated as below. Here, we only simulate for AWGN channel. For fading channel, the measurement error may be larger. Besides, measurement error is dependent on baseband algorithm.

- Table 2:  Measurement error for AWGN channel

| SNR | -20dB | -15dB | -10dB | -5dB | -3dB | 0dB | 5dB |
| --- | --- | --- | --- | --- | --- | --- | --- |
| L1-RSRP measurement error | 13.5dB | 8.5dB | 4.8dB | 2.2dB | 1.7dB | 1dB | 0.5dB |

Observation 5: Under low SNR conditions, measurement error can be large.

Prediction accuracy degradation due to measurement error

Here, we apply the same measurement error to all TX beams. Where measurement error is modeled as Gaussian distribution.

| Measurement error (dB) | Prediction accuracy % |  |  |
| --- | --- | --- | --- |
|  | K=1 | K=2 | K=4 |
| 0 | 76.35 | 93.85 | 98.17 |
| 2 | 53.89 | 74.92 | 92.46 |
| 4 | 48.17 | 66.39 | 86.75 |
| 6 | 42.94 | 63.02 | 82.34 |

Observation 6: There will be performance degradation due to measurement error.

Since different measurement error can be added for different TX beams, we may need to further analyze the performance degradation by adding different measurement error. If different TX beam will add different measurement error, it will indeed change the distribution of L1-RSRP power between different beams, which will greatly degrade the performance if there is no any pre-processing of input data.

Proposal 6: Impact of measurement error needs to be considered for prediction accuracy requirement.

- Conclusion

In this contribution, we provide the following proposals:

Observation 1: For derive ground truth for best Top-1 beam, UE needs to measure for all beams in set A under high SNR. However, there is still some measurement uncertainty due to RF and BB. It’s challenging for UE to detect the best 1st beam even at high SNR due to the small gap between best 1st and 2nd beam.

Observation 2: For beam prediction accuracy, option 2 is not correct since UE can’t distinguish which is the best beam index due to measurement error.

Proposal 1: For beam prediction accuracy performance metric,

- If beam index is used as reference, the test metric is:

  - Top-K/N (%): the percentage of "the Top-N genie-aided beam is one of the Top-K predicted beams". FFS how N is derived.

- If RSRP difference is used as reference, the test metric is:

  - The maximum measured L1-RSRP of top-K predicted beams is larger than the maximum measured L1-RSRP – X dB.

Proposal 2: For both BM case-1 and BM case-2, RAN4 to discuss whether there is impact on measurement period due to specific RX beam assumption.

Proposal 3: For BM-case 2, RAN4 to discuss impact on measurement requirement, e.g. measurement time, prediction time, measurement restriction and scheduling restriction, etc.

Observation 3: When best predicted TX beam is in set B, UE knows the best RX beam. When best predicted TX beam is in set A but not in set B, it’s still possible for UE to know the RX beam corresponding to the best TX beam.

Proposal 4: RAN4 may know RX beam based on UE capability.

Observation 4: For set B, L1-RSRP difference between best 1st beam and 8th beam is large.

Proposal 5: For simplicity, L1-RSRP delta for different TX beams in set B can be equal to SNR difference for beams in set B.

Observation 5: Under low SNR conditions, measurement error can be large.

Observation 6: There will be performance degradation due to measurement error.

Proposal 6: Impact of measurement error needs to be considered for prediction accuracy requirement.

- Reference

- TR38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface