3GPP TSG-RAN WG4 Meeting # 114	R4-2500619

# Athens, GR, 17th – 21th Feb., 2025

Source: 	Xiaomi

Title: 	Discussion on impacts on RAN4 requirement for AI mobility

Agenda Item:	7.21.2

Document for:	Discussion

# 1 Introduction

In this contribution, we will discuss RRM impact on AI mobility.

- RSRP prediction accuracy requirement - Intra-frequency temporal domain prediction - Inter-frequency inter-cell prediction

- Discussion

      - 2.1 RSRP prediction accuracy requirement

Issue 2-1-9a: L3 RSRP prediction accuracy definition Agreement: - For testing, Absolute L3 Predicted RSRP Accuracy = reported predicted L3-RSRP – ground truth of L3-RSRP - FFS Ground truth definition of L3-RSRP for FR1 and FR2

In last meeting, it’s agreed that absolute L3 predicted RSRP accuracy is the delta between reported L3-RSRP and ground truth of L3-RSRP.

For ground truth of L3-RSRP, there are two possible solutions:

- Option 1: Ground truth of L3-RSRP is calculated by TE

- Option 2: Ground truth of L3-RSRP is reported by UE

Option 1: Ground truth of L3-RSRP is calculated by TE

For Option 1, the approach is largely similar to the legacy method. However, there is a key difference in the channel model used for the L3-RSRP accuracy test. In the legacy test, AWGN channel is employed, which means there is no channel fluctuation in the time domain. As a result, the TE can always determine the ideal L3-RSRP value with certainty.

In contrast, for the AI mobility test metric, a fading channel is introduced. This channel introduces time-domain fluctuations, making it more challenging to calculate the ideal L3-RSRP. If TE would like to know the ideal L3-RSRP, it needs to solve several issues:

- Issue 1: L3-RSRP time instance alignment

- Issue 2: UE L1 filtering sample number

- Issue 3: UE L1/L3 filtering method

- Issue 4: beam consolidation

- Issue 5: RX beamforming gain for FR2

For issue 1:

if the TE and UE have a clear alignment regarding the specific time instance that the reported L3-RSRP corresponds to, the TE can still identify the ideal L3-RSRP value in the time domain. Then the detail time instance for the L3-RSRP results should be indicated. This can be solved by additional signaling.

For issue 2:

In current measurement period requirement in 38.133, 5 samples are defined for L1 filtering as minimum requirement. In fact, some advanced UE can use fewer samples to achieve RSRP accuracy, i.e. 3 samples, and can send filtered L1-RSRP to high layer more quickly. If that is the case, the ideal L3-RSRP will be calculated based on average of 3 samples but not 5 samples. In current AI mobility studied in RAN2, if they consider L3-RSRP to predict L3-RSRP, they don’t have limitation on the L1 filtering sample, it’s up to UE implementation.

If L3-RSRP can be calculated at TE side, it means that TE needs to know how many L1 samples will be used to generate ideal L3-RSRP. It will have highly limitation on UE implementation. It means that UE must average based on 5 samples. Otherwise, there will be mismatch.

Observation 1: L1 filtering sample number will have impact on deriving ideal L3-RSRP value from TE side.

For issue 3:

RAN2 defines L1/L3 sliding filtering and non-sliding filtering as below:

Fig.1: non-sliding L1/L3 filtering

While for L1/L3 filtering sliding filtering case, it’s defined as below:

Fig.2 Sliding L1/L3 filtering

The key difference between non-sliding filtering and sliding filtering is that:

  - For L1/L3 filtering non-sliding window, every 5 L1 samples will generate a L3 RSRP.

  - For L1/L3 filtering sliding window, every 1 L1 sample will generate a L3 RSRP.

If TE didn’t know the L1/L3 filtering method, TE can’t know how to calculate the ideal L3-RSRP. According to 3GPP TS 38.331, the L3 filtering formula is as follows:

Fn​=(1−a)⋅Fn−1​+a⋅Mn​

where:

- Fn​ is the updated filtered measurement result used to evaluate reporting conditions or for measurement reporting.

- Fn−1​ is the previous filtered measurement result, with the initial value F0​ set to the first measurement result M1​ received from the physical layer.

- Mn​ is the latest measurement result received from the physical layer.

- α is the filtering coefficient

Fig.3 different sample number to calculate ideal L3-RSRP

L3-RSRP after L3 filtering depends on both the previous filtered result and the new L3-RSRP measurement. For simplicity, assume two L3-RSRP samples are used to derive the filtered L3-RSRP, as illustrated in Fig. 3.

The choice of L1/L3 filtering method significantly impacts the calculation of ideal L3-RSRP. For instance, consider deriving the ideal L3-RSRP at time T:

- Non-sliding filtering: Uses 10 L1 samples to generate the ground truth of L3-RSRP at time T.

- Sliding filtering: Uses only 6 L1 samples to generate the ground truth of L3-RSRP at time T.

Consequently, the ground truth of L3-RSRP at time T differs between sliding and non-sliding filtering methods.

An alternative approach is to disregard the L3 filtering factor α, meaning L3-RSRP is equivalent to the measurement at point C with α=0, similar to L1-RSRP at point A1. In this scenario, the ideal L3-RSRP is independent of the L1/L3 filtering method. Regardless of the method applied, L3-RSRP equals the average of M instantaneous L1-RSRP samples.

Observation 2: If the L3 filtering factor α is not zero, the L1/L3 filtering method will influence the derivation of the ideal L3-RSRP value from the TE side.

For issue 4:

If L3-RSRP is defined based on cell level, several L1 filtered RSRP of TX beams higher than threshold may need to be combined. Since filtered L1-RSRP can only be obtained at UE side, it’s difficult for TE to know RSRP from which beam will be higher than the threshold. Then TE can’t know the combined cell level L3-RSRP. If TX beam consolidation is not considered, it means that cell-level L3-RSRP equals to filtered L1-RSRP at point A1.

Observation 3: Beam consolidation will have impact on deriving ideal cell level L3-RSRP from TE side. If beam consolidation is not considered, cell-level L3-RSRP at point C equals to filtered L1-RSRP at point A1.

For issue 5

For FR2, since TE didn’t know UE RX beamforming gain, similar as legacy Beam management, a large RSRP margin will be considered. The margin can be as large as 20dB. If the margin is added, it’s hard to evaluate the RSRP prediction accuracy.

Observation 4: if TE will calculate ideal cell-level L3-RSRP, several issues need to be solved and some aspects will have limitation on UE implementation.

Proposal 1: If the TE is capable of calculating the ideal cell-level L3-RSRP, RAN4 should discuss the specific conditions and information required for the TE to accurately determine this ideal value.

Option 2: Ground truth of L3-RSRP is reported by UE

For this case, ground truth of L3-RSRP will be measured by UE and reported to TE, there is no UE implementation limitation. However, it needs to solve UE cheating issue. For FR2, similar scheme of AI BM may be re-used. For FR1, it needs further study.

Proposal 2: If the ground truth of L3-RSRP is reported by the UE, RAN4 should discuss whether and how to prevent the UE from providing cheating reports.

Issue 2-1-1: Absolut and/or Relative RSRP Prediction Accuracy Way Forward: Session Chair: Encourage companies to bring proposals on the definition of relative RSRP and absolute RSRP. Subsequently, companies may also consider following candidate options to introduce RSRP accuracy requirements: - Option 1: RAN4 to consider relative RSRP accuracy requirements only. - Option 2: RAN4 to consider absolute RSRP accuracy requirements only. - Option 3: RAN4 to consider both absolute and relative RSRP accuracy requirements.

For the definition of relative RSRP, it involves two RSRP measurements as references. The legacy relative L3-RSRP accuracy requirement is defined as the delta between the measured L3-RSRP of the serving cell and the measured L3-RSRP of the neighboring cell. Following the same definition, the relative prediction accuracy would be expressed as:

Relative prediction accuracy = Predicted RSRP of cell 1 – Predicted RSRP of cell 2

In the context of AI-based RSRP prediction, there are two scenarios to consider: intra-cell and inter-cell predictions.

-  Inter-cell RSRP Prediction:

- Scenario: FR1 to FR1 inter-frequency

- Process: The UE measures the L3-RSRP of the serving cell and predicts the absolute L3-RSRP of another cell.

- Issue: Since only one predicted RSRP is available (for the neighbouring cell), it is not possible to define a relative RSRP prediction accuracy.

-   Intra-cell RSRP Prediction:

- Scenario: The UE measures several L3-RSRPs at time T and predicts the absolute L3-RSRP at time T+n for the same cell.

- Issue: Only one predicted RSRP is generated, making it impossible to define a relative RSRP prediction accuracy.

Given these scenarios, if RAN4 intends to consider relative predicted RSRP accuracy, it is essential to first establish a clear definition for relative RSRP prediction accuracy.

Proposal 3: RAN4 should clarify the definition of relative RSRP prediction accuracy before determining whether to include it as a requirement.

      - 2.2 Intra-frequency temporal domain prediction

      - 2.2.1 RRM impact

RAN2 prioritize 3 scenarios:

| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| --- | --- | --- | --- | --- |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |

Two scenarios are related to intra-frequency intra-cell prediction:

- FR1 to FR1 intra-frequency temporal domain case B

- FR2 to FR2 intra-frequency temporal domain case A

Case A and case B

In TR[1], RAN2 defines observation window and prediction window, which will have impact on RAN4.

  - Observation window will have impact on measurement duration

  - Prediction window will have impact on scheduling restriction

For legacy L3 RSRP measurement requirement, measurement will be conducted periodically. RAN4 defined measurement period which is required to satisfy the L1 filtered RSRP accuracy requirement. However, when AI is applied, some measurement instance can be skipped. Take case B for example, the target is to reduce measurement in temporal. Therefore, the measured samples will be reduced.

In prediction window, UE can skip measurement. It’s possible that scheduling restriction can be relaxed.

Proposal 4: Observation window and prediction window defined in RAN2 will have RRM impact:

  - Observation window will have impact on measurement duration

  - Prediction window will have impact on scheduling restriction

      - 2.2.2 Observation window and prediction window

Observation window and prediction length is dependent on different sub-cases, different pattern and different L1/L3 filtering method.

Impact of L1/L3 sliding filtering

RAN2 defines L1/L3 sliding filtering and non-sliding filtering. Therefore, the final observation window length for the two cases is different for L3 RSRP to L3 RSRP prediction case.

Observation 5: Observation window and prediction length is dependent on different L1/L3 filtering method.

Impact of different sub-cases

Currently, RAN2 defined 3 subcases for RSRP prediction.

3 sub-use cases are considered for cell-level RRM measurement prediction: Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then cell-level measurement result is generated Sub-use case 2: Cell-level measurement result(s) is predicted based on actual cell-level measurement result(s) Sub-use case 3: Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) Editor Note 1: The RRM measurement refers to L3 cell/beam level measurement and/or L1 beam level measurement

- Sub-case 1: RSRP is predicted at L1

For sub-case 1, the prediction happens at L1. In legacy, 5 L1 samples are needed to be measured in one measurement period, which is shown in Fig.4(a). While when AI is applied, for example, only 3 samples are needed to be measured during observation window. Then UE can predict other samples with grey colour, as shown in Fig.4(b). Then measurement period can be reduced. Here observation window is similar as legacy measurement period with reduced samples. Observation window and prediction window length are based on number of L1 samples.

Fig.4(a) measurement period for non-AI L1 filtered RSRP for FR1

Fig.4(b) observation window and prediction window for AI based L1 filtered RSRP

- Sub-case 2: RSRP is predicted at L3

For sub-case 2, we give examples for non-sliding L1/L3 filtering case.

For sub-case 2, filtered L3 RSRP will be predicted by L3 RSRP. Suppose that every 2 L3 filtered RSRP can predict one L3 filtered RSRP. As shown in Fig.5, UE will measure for 10 samples to get 2 L3 results and skip the following L1 5 samples. For this case, the observation window and prediction window length are based on number of L1 measurement period.

Fig.5 observation window and prediction window for AI based L3 filtered RSRP

The observation window will be different for initial stage and later stage. At initial stage, new L1 filtered RSRP will be obtained very L1 sample.

- Sub-case 3: RSRP is predicted at L3

For sub-case 3, RSRP is also predicted at L3. Then observation window and prediction window are similar as sub-case 2.

Observation 6: Observation window and prediction length is dependent on different sub-cases.

Impact of pattern

RAN2 also defines different patterns for prediction window and observation window as below, it will also have impact on the prediction window length and observation window length

Fig.6 Example 1 of intra-frequency temporal domain case B

Fig.7 Example 2 of intra-frequency temporal domain case B

Observation 7: Observation window and prediction length is dependent on different patterns.

Proposal 5: For intra-frequency intra-cell prediction, suggest RAN4 to select some typical case/scenarios to study observation window and prediction window.

      - 2.3 Inter-frequency inter-cell prediction

In TR[1], RAN2 agrees that:

For both Intra-frequency prediction inter-cell prediction and FR1 to FR1 inter-frequency inter-cell prediction, no measurement is reduced in both temporal and spatial domain for cell to be measured. For FR1 to FR1 inter-frequency inter-cell prediction, focus on the case where cell to be measured and cell to be predicted are located in the same sector of either serving site or same neighboring site. If inter-frequency correlation model is assumed, section 7.6.5 in [4] is taken as baseline for inter-frequency correlation model. FR1 to FR1 inter-frequency inter-cell prediction is applicable for all RRM sub-use cases.

For inter-frequency inter-cell RSRP prediction, UE may skip the measurement for another cell on other frequency. Then it’s possible to reduce CSSF.

Observation 5: Inter-frequency inter-cell RSRP prediction will have impact on CSSF reduction in measurement period.

Proposal 6: For inter-frequency inter-cell RSRP prediction, RAN4 to study the impact on CSSF reduction in measurement period.

- Conclusion

In this contribution, we provide the following proposals:

Observation 1: L1 filtering sample number will have impact on deriving ideal L3-RSRP value from TE side.

Observation 2: If the L3 filtering factor α is not zero, the L1/L3 filtering method will influence the derivation of the ideal L3-RSRP value from the TE side.

Observation 3: Beam consolidation will have impact on deriving ideal cell level L3-RSRP from TE side. If beam consolidation is not considered, cell-level L3-RSRP at point C equals to filtered L1-RSRP at point A1.

Observation 4: if TE will calculate ideal cell-level L3-RSRP, several issues need to be solved and some aspects will have limitation on UE implementation.

Proposal 1: If the TE is capable of calculating the ideal cell-level L3-RSRP, RAN4 should discuss the specific conditions and information required for the TE to accurately determine this ideal value.

Proposal 2: If the ground truth of L3-RSRP is reported by the UE, RAN4 should discuss whether and how to prevent the UE from providing cheating reports.

Proposal 3: RAN4 should clarify the definition of relative RSRP prediction accuracy before determining whether to include it as a requirement.

Proposal 4: Observation window and prediction window defined in RAN2 will have RRM impact:

  - Observation window will have impact on measurement duration

  - Prediction window will have impact on scheduling restriction

Observation 5: Observation window and prediction length is dependent on different L1/L3 filtering method.

Observation 6: Observation window and prediction length is dependent on different sub-cases.

Observation 7: Observation window and prediction length is dependent on different patterns.

Proposal 5: For intra-frequency intra-cell prediction, suggest RAN4 to select some typical case/scenarios to study observation window and prediction window.

Observation 5: Inter-frequency inter-cell RSRP prediction will have impact on CSSF reduction in measurement period.

Proposal 6: For inter-frequency inter-cell RSRP prediction, RAN4 to study the impact on CSSF reduction in measurement period.

- Reference

[1] RP-242393 Revised SID: Study on Artificial Intelligence (AI)/ Machine Learning (ML) for mobility in NR, OPPO

[2] R2-2406309, text proposal on TR 38.744 v0.03

[3] R2-2407849, Draft Summary of [POST127][030][AI mobility] RRM simulation assumptions.