3GPP TSG-RAN WG4 Meeting # 114	R4-2500620

# Athens, GR, 17th – 21th Feb., 2025

Source: 	Xiaomi

Title: 	Discussion on event triggered L1 measurement reporting for mobility

Agenda Item:	7.27.2.1

Document for:	Discussion

# 1 Introduction

In this contribution, we will mainly discuss the following issues for Rel-19 Mobility.

- Event-triggered L1 reporting

- Discussion

Reporting capability

Issue 2-7: whether to introduce/update requirements for reporting criteria per measurement category <Candidate options> - Option 1: Yes (CATT, MTK, ZTE, Apple, OPPO) - Option 2: RAN4 to postpone the detailed discussion on ‘Requirements for reporting criteria per measurement category’ until the RRM requirements on the remaining issues are sorted out (QC, Ericsson) - Option 3: RAN4 to align common understanding that the existing number of reporting criteria per category is defined for L3 measurement and reporting only. For L1 measurements, the number of events to be supported by UE shall be discussed in RAN1 first. (vivo) Issue 2-8: whether to define a separate capability for supported number of events triggered L1 measurement reporting <Candidate options> - Option 1: Yes (MTK, Samsung, CMCC) - Option 2: RAN4 to postpone the detailed discussion on ‘Requirements for reporting criteria per measurement category’ until the RRM requirements on the remaining issues are sorted out (QC) - Option 2a: RAN4 to wait for RAN2 progress on the TTT handling before discussing the UE capability of number of events to support. (E///) - Option 3: RAN4 to align common understanding that the existing number of reporting criteria per category is defined for L3 measurement and reporting only. For L1 measurements, the number of events to be supported by UE shall be discussed in RAN1 first. (vivo)

There are some discussions about the UE capability of supported number of events. Generally, we are fine to define capability for supported number of events for L1 based reporting for LTM. Since CSI-RS based LTM is also considered in L1 event triggered reporting, the relation between L3 reporting and L1 reporting, and relation between CSI-RS based L1 measurement/reporting and SSB based L1 measurement/reporting will have impact on the capability definition. it’s also FFS how to count frequency layer. Therefore, we suggest to discuss the detail of UE capability for in later stage when design is clearer.

Proposal 1: RAN4 to define UE capability for Event Triggering and Reporting Criteria for LTM. Suggest to discuss detail in later stage when design is clearer.

TTT impact

Issue 2-6: impact of time-to-trigger (TTT) on RRM <Candidate options> - Option 1: RAN4 to discuss whether to take TTT (time to trigger) into account in the L1 event-triggered reporting delay requirements once more agreements from RAN1/2 are available. (Nokia, Xiaomi)

From RAN2 agreement, TTT operates based on Timer, which is used similar as L3 event triggered MR. For event triggered L3 reporting, minimum configurable TTT is zero, then the shortest reporting delay is just one measurement period. For L1 reporting, if minimum configured TTT can still be 0, no additional delay needs to be considered. If not, minimum configurable TTT may needs to be added into total evaluation delay. Here, the impact of TTT is not about L1 filtering, TTT make sure that L1 reporting can only be triggered if all L1 results in TTT satisfy the event triggering condition.

Proposal 2: RAN4 to wait for more progress in RAN2 about whether to consider the impact of TTT on event evaluation time.

Measurement delay

Issue 2-5: Event triggered reporting delay for L1 reporting <Agreement> - Agree on the following high level proposal: - The event triggered measurement reporting delay shall be no larger than the L1-RSRP measurement period [on the cells corresponding to the event or on the candidate cells]. - FFS reuse L1-RSRP measurement period from the existing spec with Treport = 0. - Additional time TSSB_time_index for SSB index acquisition is needed, unless deriveSSB-IndexFromCell is enabled or UE has reported SSB index in L3 measurement report of the same cell.

If the measurement periods for different beams are the same, the legacy requirement can be re-used. However, if the RS periodicity is different, we prefer that the maximum measurement period between current beam and candidate beam will be used as measurement delay.

Proposal 3: If multiple beams with different measurement period need to be measured, the maximum measurement period will be considered as measurement delay.

Impact of UL resource waiting time

In legacy L3 event triggered reporting, according to 38.331, if L3 measurement result satisfies the event triggering condition, UE will initiate the message reporting procedure and submit the MeasurementReport message to lower layers for transmission. It means that the L3 measurement result which triggers reporting will be included in the L3 report.

For L1 event triggered reporting, different from L3, L1 measurement is much faster, it’s possible that UE can obtain several new L1 results between the time UE trigger L1 reporting and the time when UL resource arrives. Since this is L1 report, the procedure is better specified in physical layer. We prefer to clarify this issue in RAN4.

Following the same logic of L3, L1 result which triggers L1 reporting will be included in L1 reporting. It means that if L1 reporting procedure starts, UE can continue to measure for L1, however, the following L1 result will not be evaluated for event anymore.

Proposal 4: For L1 event triggered reporting, it needs to specify that L1 result which triggers L1 reporting will be included in L1 reporting.

L1 filtering

Issue 2-11: L1 filtering assumption in event evaluation <Candidate options> - Option 1: For whether to consider the RRM impact due to L1 filtering for event triggered L1 report, RAN4 needs to wait for more RAN1 input. (CATT, ZTE, CTC, E///) - If L1 filtering is introduced in RAN1, RAN4 needs to define additional SSB based L1 measurement delay requirements with filtering for event triggered L1 report. - Otherwise, there is no impact on RAN4. - Option 2: In event triggered L1 measurement in mobility, do not consider L1 filter for requirements. (Samsung, Xiaomi, CMCC, Apple, HW) - Option 2a: RAN4 to define RRM requirements for the event triggered L1 report with the assumption of no specific L1 filtering. This decision can be revisited if other working groups overturn their decision on L1 filtering. (QC)

In RAN1 #118bis, it’s agreed that no L1 specified filtering for time and spatial domain is introduced.

Agreement The agreement “Rel-18 LTM CSI reporting framework is the baseline for CSI-RS based L1-measurement report by gNB scheduled measurement reporting” made in RAN#118 is further clarified for L1-RSRP as follows: - UCI format defined in Table 6.3.1.1.2-8C of TS38.212 can be used by replacing SSBRI with CRI. - Whether the L1-RSRP(s) of serving cell is always included is configurable (in line with Rel-18) - The quantization method defined in clause 5.2.1.4.3 of TS38.214 and bit width defined in Table 6.3.1.1.2-6 of TS38.212 can be used - No L1 specified filtering for time and spatial domain is introduced - No enhancement on how to report L cells x M beams - Periodic reporting on PUCCH is supported - FFS: semi-persistent reporting on PUCCH/PUSCH, and aperiodic reporting on PUSCH

Proposal 5: No L1 based filtering needs to be considered in event evaluation.

- Conclusion

In this contribution, we provide the following proposals：

Proposal 1: RAN4 to define UE capability for Event Triggering and Reporting Criteria for LTM. Suggest to discuss detail in later stage when design is clearer.

Proposal 2: RAN4 to wait for more progress in RAN2 about whether to consider the impact of TTT on event evaluation time.

Proposal 3: If multiple beams with different measurement period need to be measured, the maximum measurement period will be considered as measurement delay.

Proposal 4: For L1 event triggered reporting, it needs to specify that L1 result which triggers L1 reporting will be included in L1 reporting.

Proposal 5: No L1 based filtering needs to be considered in event evaluation.

- Reference