3GPP TSG-RAN WG4 Meeting # 114	R4-2500621

# Athens, GR, 17th – 21th Feb., 2025

Source: 	Xiaomi

Title: 	Discussion on CSI-RS based L1 measurement for mobility

Agenda Item:	7.27.2.2

Document for:	Discussion

# 1 Introduction

In this contribution, we will mainly discuss the following issues for Rel-19 Mobility.

- Known cell condition - Definition of intra-frequency and inter-frequency - Supported measurement types - Pre-requisite of CSI-RS based L1 measurement

- Discussion

2.1 Known cell condition

Issue 3-1-2: Whether to keep or remove L1 measurement for FR2-1 in the 2nd bullet <Candidate solutions> - Option 1: Keep SSB based L1 measurement as one of the pre-requisite conditions for CSI-RS based L1 measurement in FR2. (Apple, Huawei, vivo) - Option 2: Remove Rel-18 LTM SSB-based L1-RSRP measurements. (CMCC, Nokia, Samsung, MTK, Ericsson) - Option 2a: Remove L1 measurement for FR2-1 as side condition and consider “The CSI-RS from the target cell configured for L1 measurement remains measurable” as requirement applicability. (Xiaomi) - Option 3: Keep L1 measurement for FR2-1 with bracket in the 2nd bullet and wait for RAN1/RAN2’s progress. (ZTE)

For 2nd bullet, whether SSB based L1 measurement will be kept for removed is highly related to issue 3-2. In issue 3-2, RAN4 will further discuss whether SSB based L1 measurement can be skipped under certain conditions. While issue 3-2 will also be impacted by issue 3-5, e.g. whether CSI-RS can be configured with repetition ON.

If conditions or UE capabilities are defined to allow skipping SSB-based L1 measurement, it would be inappropriate to always include SSB-based L1 measurement in the known cell condition. Therefore, we propose to defer the discussion on whether Rel-18 LTM SSB-based L1-RSRP measurement should be retained or removed in the known cell condition until the measurement procedure is more clearly defined.

Observation 1: Whether SSB based L1 measurement will be kept for removed is highly related to issue 3-2.

Proposal 1: Postpone the discussion on whether SSB-based L1-RSRP measurement for FR2-1 should be retained or removed in the known cell condition until the measurement procedure is more clearly defined.

Issue 3-1-3: Whether to use detectable or measurable? <Candidate solutions> - Option 1: The difference of the definition between detectable and measurable need to be further clarified. (CATT) - Option 2: Use detectable. (Apple, ZTE) - Option 3: Use measurable. (Xiaomi, OPPO, Ericsson, Nokia)

Take requirement applicability of SSB based LTM as example, measurable is used for SSB resource configured for L1-RSRP measurement.

### 9.14.2 Requirements Applicability The requirements in the clause 9.14 are applicable to FR1 and FR2-1 for LTM. The requirements in clause 9.14 apply, provided for the SSB from the neighbor cell configured for intra-frequency L1-RSRP measurement, the following conditions are met: - The cell is known - The SSB configured for intra-frequency L1-RSRP measurement is on the same carrier frequency of SSB configured for L3 intra-frequency measurement. - The SSB resources configured for L1-RSRP measurements are measurable. An SSB resource configured for L1-RSRP for neigbor cell shall be considered measurable when for each relevant SSB the following conditions are met: - L1-RSRP related side conditions given in clause [10.1.19] for FR1 and [10.1.20] for FR2-1, respectively, for a corresponding band, - SSB_RP and SSB Ês/Iot according to Annex [B.2.4.1] for a corresponding band.

Therefore, we also suggest to use the same wording of SSB based LTM for CSI based LTM, i.e. the CSI-RS resources configured for L1-RSRP measurements are measurable.

Proposal 2: The CSI-RS from the target cell configured for L1 measurement remains measurable.

Issue 3-1-4: Side condition for CSI-RS based L1 measurement for neighbor cell <Candidate solutions> - Option 1: SNR of the CSI-RS from the target cell configured for L1 measurement is above -6dB (MTK) - Option 2: It is proposed to use legacy side condition of SINR=-3dB. (Apple, Ericsson, Nokia, Saumsung)

We suggest to apply the same side condition for both SSB and CSI-RS based L1 measurement for neighbour cell.

Proposal 3: Use legacy side condition SINR = -3dB for CSI-RS based L1 measurement for neighbour cell.

2.2 Definition of intra-frequency and inter-frequency

In last meeting, there is some discussion about how to define intra-frequency and inter-frequency measurement for CSI-RS L1-RSRP.

Issue 3-3: Definition of intra-frequency/inter-frequency for CSI-RS based L1 measurement <Agreement> - A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that: - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of [active DL BWP], and - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of [active DL BWP], and - It is applied for SCS = 60KHz - [the centre frequency of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the centre frequency of any of the CSI-RS resources of the serving cell indicated for L1 measurement (if configured).]

For the 3rd bullet, the current definition is related to whether there is configuration of CSI-RS from serving cell in LTM. If follow legacy Rel-18 LTM SSB based L1 measurement configuration, serving cell configuration is optional. If CSI-RS configuration of serving cell is still optional, it means that there are two cases:

- Case 1: CSI-RS resources of serving cell are not indicated in LTM configuration

- Case 2: CSI-RS resources of serving cell are indicated in LTM configuration

For case 1, if CSI-RS resources of serving cell are not configured in LTM, anyway we still need a reference for center-frequency of serving cell. RAN4 needs to discuss how to consider for this case.

In case 2, as the CSI - RS resources from the serving cell might be located at different center frequencies, it is acceptable to compare the center frequency of the CSI - RS from the neighbor cell with that of any CSI - RS from serving cell. However, if feasible, we would like to devise a unified definition applicable to both cases.

Proposal 4: Regarding the definition of intra - frequency, if possible, RAN4 to develop a unified definition that covers both scenarios: CSI - RS of the serving cell is configured or not configured.

2.3 Supported measurement types

- Issue 2-3: CSI-RS based L1 measurement outside active BWP or with measurement gap - Candidate solutions - Option 1: Deprioritize CSI-RS based L1-RSRP measurement outside active BWP (or with measurement gap) in R19. (MTK, QC, vivo, OPPO, Nokia) - Option 1a: RAN4 can first discuss measurement within active BWP for CSI-RS based L1 measurement and wait for progress in other WG about whether measurement gap will be supported. (Xiaomi) - Option 1b: RAN4 to define intra-frequency CSI-RS measurement requirements first. Support inter-frequency L1-RSRP CSI-RS measurements without gap. FFS for inter-frequency with gap. (Nokia) - Option 2: RAN4 at least support the following measurement types for CSI-RS based L1 measurement. (CATT) - Without gap - With gap - Option 2a: for CSI-RS based LTM, it is proposed to consider intra-frequency measurement, inter-frequency measurement without gap and inter-frequency measurement with gap, which are aligned with Rel-18 SSB based LTM. (CMCC) - Option 2b: prioritize intra-frequency measurement without GAP, inter-frequency measurement with GAP. (Apple, Samsung, HW, OPPO, CATT) - Option 2c: CSI-RS based L1 measurements for candidate cells shall support - Measurement within active BWP, Measurements outside active BWP. (E///, ZTE) - Recommended WF - Continue discussion.

The main difference about measurement requirement is whether the measurement is performed inside or outside active BWP. For intra-f without gap and inter-f without gap, if both are within active BWP, the requirement is similar. Considering the complexity of performing CSI-RS based measurement, we suggest to prioritize CSI-RS based L1-RSRP measurement inside active BWP.

Proposal 5: Deprioritize CSI-RS based L1-RSRP measurement outside active BWP (or with measurement gap) in R19.

2.4 Pre-requisite of CSI-RS based L1 measurement

Issue 3-2: Measurement procedure <Agreement> - Agree the following measurement procedure for CSI-RS based LTM candidate cell measurements: - Step 1: SSB based L3 measurements and reports at least before LTM configuration. - Step 2: For FR2-1, SSB-based L1 measurements and reports on the same candidate cell - Further discuss whether Step 2 can be skipped under certain conditions, or based on UE capability or based on network RRC configuration. FFS the details. - Note: Step 2 is removed for FR1. - Step 3: CSI-RS based L1 measurements on the same candidate cell - Depending on whether step 2 is skipped, the requirements in step 3 may be different. - Note: The CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1 or 2. - [Note: For FR1, the CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1.]

The decision to skip or retain Step 2 (SSB-based L1 measurements) in the measurement procedure for CSI-RS-based LTM candidate cell measurements depends on a trade-off between potential benefits and drawbacks.

When SSB based L1 measurement is performed before step 3:

- Possible Benefit:

  - Reduce UE implementation complexity by skipping RX beam sweeping in step 3.

  - Reduce CSI-RS resource number configured for measurement.

- Drawback:

  - Extended L1 measurement time for neighbour cell. Since SSB sharing between different cells may be needed. Periodicity of SSB may be longer than CSI-RS resource.

  - Limit the NW configuration flexibility.

The key issue is to analyse the benefit and drawback when step 2 is introduced. As there may be many candidate cells, the total number of CSI-RS resource number for RX beam sweeping may be quite large. While SSB is already there, UE can use the existing SSB to find the best RX beam, then to reduce CSI-RS number configured for RX beam sweeping in step 3. For power efficiency aspect, we prefer to keep step 2.

Proposal 6: For step 2, prefer to keep SSB-based L1 measurements and reports on the same candidate cell for FR2-1.

- Conclusion

In this contribution, we provide the following proposals：

Observation 1: Whether SSB based L1 measurement will be kept for removed is highly related to issue 3-2.

Proposal 1: Postpone the discussion on whether SSB-based L1-RSRP measurement for FR2-1 should be retained or removed in the known cell condition until the measurement procedure is more clearly defined.

Proposal 2: The CSI-RS from the target cell configured for L1 measurement remains measurable.

Proposal 3: Use legacy side condition SINR = -3dB for CSI-RS based L1 measurement for neighbour cell.

Proposal 4: Regarding the definition of intra - frequency, if possible, RAN4 to develop a unified definition that covers both scenarios: CSI - RS of the serving cell is configured or not configured.

Proposal 5: Deprioritize CSI-RS based L1-RSRP measurement outside active BWP (or with measurement gap) in R19.

Proposal 6: For step 2, prefer to keep SSB-based L1 measurements and reports on the same candidate cell for FR2-1.

- Reference