3GPP TSG-RAN WG4 Meeting #114										R4-2500625

Athens, Greece, 17th – 21st February, 2025

Agenda item:	7.10.3.2

Source: 	Xiaomi

Title: 	Discussion on UE RF requirements for NTN less than 5MHz

Document for:	Discussion

Introduction

In RAN4#113 meeting, RAN4 discussed the system parameters and UE RF requirements for NTN less than 5MHz, and the related WF was approved in [1]. This contribution discusses the remaining issues for NTN less than 5MHz and provides our proposals.

Discussion

System parameters

Issue 1-1: Whether 3MHz channel bandwidth for NR-NTN in FR1-NTN bands is mandatory or optional? - Option 1: Optional - Option 2: Mandatory Issue 1-2: UE capability Signalling - FFS on re-use of existing rel-18 signalling parameters

In previous RAN4 meetings, RAN4 had extensive discussion on whether the 3MHz channel bandwidth is mandatory or optional. From our perspective, we support 3MHz CBW is introduced as an optional UE capability, since UE may reuse the existing modem and RF components from TN to support NTN operations, and the 3MHz CBW introduced for NR TN is optional in Rel-18. In addition, RAN2 has already define UE capability as optional feature to support 3MHz CBW, and no impact on RAN2.

Proposal 1: RAN4 to define 3MHz channel bandwidth is optional for FR1 NR-NTN bands.

If RAN4 agrees 3MHz CBW is an optional UE capability for NTN bands, the exiting signalling can be reused. In Rel-18, UE capabilities of support3MHz-ChannelBW-Symmetric-r18 and support3MHz-ChannelBW-Asymmetric-r18 were introduced to support 3MHz CBW in symmetrical and asymmetrical configurations.

Proposal 2: If 3MHz CBW is optional for FR1 NTN bands, the exiting signalling of support3MHz-ChannelBW-Symmetric-r18 and support3MHz-ChannelBW-Asymmetric-r18 are reused for NTN operations.

UE RF requirements

Issue 2-2-1: Reference sensitivity - Option 1: - 2.2dB improvement compared with 5 MHz CBW; - Option 2: - 1.7dB improvement compared with 5 MHz CBW; - Encourage companies to consider Rx non-linearity in the reference sensitivity evaluations

RAN4 needs to define new reference sensitivity requirements for NTN 3MHz CBW for band n254, n255 and n256. In theory, for 3MHz channel bandwidth with 15 PRBs, the total received power is reduced by 2.2dB compared with 5MHz channel bandwidth. However, for NR NTN, the maximum input power and in-band blocking interferer signal level are reduced to -40 dBm, the Rx IIP2 effects needs to be considered, and we support to consider option 2, e.g. 1.7dB improvement compared with 5 MHz CBW.

Proposal 3: For reference sensitivity, RAN4 to define new requirements for 3MHz for band n256, n255 and n254, and 1.7dB offset is applied compared with existing 5MHz requirements.

Conclusion

In this contribution, we discussed the UE RF requirements for NTN less than 5MHz and provides our proposals:

Proposal 1: RAN4 to define 3MHz channel bandwidth is optional for FR1 NR-NTN bands.

Proposal 2: If 3MHz CBW is optional for FR1 NTN bands, the exiting signalling of support3MHz-ChannelBW-Symmetric-r18 and support3MHz-ChannelBW-Asymmetric-r18 are reused for NTN operations.

Proposal 3: For reference sensitivity, RAN4 to define new requirements for 3MHz for band n256, n255 and n254, and 1.7dB offset is applied compared with existing 5MHz requirements.

- Reference

[1]	R4-2419789	Way Forward for [113][311] NR_IoT_NTN_less_than_5MHz_UERF		Xiaomi