# 3GPP TSG-RAN WG4 Meeting #114	R4-2500628

# Athens, GR, 17th – 21st February, 2025

Source: 	Xiaomi

Title: 	Discussion on Study of testability and interoperability for AI/ML mobility

Agenda Item:	7.21.3

Document for:	Discussion

# 1Introduction

In the last RAN4#113 meeting, an initial discussion on testability and interoperability for AI/ML mobility occurred and a WF was approved as detailed in [1]. Here below we attach the previous RRM agreements on testability and interoperability:

- Agreements on testability and interoperability in RAN4#112-bis Issue 3-1-1: New testability aspects - Agreement - According to the SID, RAN4 to leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability. - RAN4 will need to consider at least the following testability aspects for AI/ML-enabled mobility SI: - Testing in FR1 - Testing inter-carrier scenario, i.e., inter-frequency inter-cell prediction (for scenario 3) - Testing cell-level L3-RSRP measurements - Consideration of serving and neighbor cells - Issue 3-2-1: Testing in FR1 - What way of testing to consider for FR1: - Candidate Options: - Option 1: OTA testing - Option 2: Conducted testing - Option 3: Both OTA and Conducted - Agreement: - RAN4 to consider Option 2 as a baseline option for testing in FR1. - Option 2: Conducted testing - Agreements on testability and interoperability in RAN4#113 Issue 3-1-1: Testing setup - Agreement - RAN4 to further discuss: - Whether and how legacy testing setup for L3 measurements can be reused for FR1 and FR2 Issue 3-2-1: Testing goal - Agreement - As baseline, the testing goal is to verify whether the minimum performance of AI/ML functionality/feature can be achieved.

Based on the achieved agreements, our discussion focuses on RRM measurement prediction use-case at the present stage. In this contribution, we would like to share our further consideration on the testability and interoperability issues for AI/ML mobility and provide our proposals.

- Discussion

RAN2 has discussed the RRM measurement prediction for several meetings, and has agreed to focus on L3 filtered cell level measurement, while deprioritize L3 filtered beam level prediction cases.

3 sub-use cases listed below are considered for cell level RRM measurement prediction:

- Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then cell-level measurement result is generated

- Sub-use case 2: Cell-level measurement result(s) is predicted based on actual cell-level measurement result(s)

- Sub-use case 3: Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

In RAN4#112bis meeting, it was agreed to focus on the three high priority scenarios as ranked by RAN2.

| scenario number | Priority | Evaluation scenario | Target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |

- To verify the accuracy of the predicted L3 filtered cell-level measurement result, sufficient randomness and variation in time and spatial domain of Tx beam has to be configured in the test and emulated by the test equipment. For the scenarios of FR1 to FR1 inter-frequency intra-cell temporal domain case B and FR1 to FR1 inter-frequency inter-cell frequency domain, we understand that Tx beam number=1 can be assumed as the model input.

- Similar as the discussion in NR_AIML_Air WI, the channel model assumption for RRM measurement prediction need to be agreed first. There are two channel model assumptions: AWGN channel and Fading channel. The AWGN channel is a deterministic channel, where signal power will be constant for both spatial and time domain. While, the Fading channel is a random channel, where variation in spatial and/or time domain can be included. It’s hard to verify the prediction functionality introduced by AI using AWGN, since there is no any uncertainty in both spatial and time domain. Just as the progress achieved in Channel model assumption for AI based BM, we think the fading channel should be assumed in the test. For both the scenarios of FR1 to FR1 inter-frequency intra-cell temporal domain case B and FR1 to FR1 inter-frequency inter-cell frequency domain, we understand the variation in time domain would be enough for FR1 scenario, so the TDL channel model is preferred.

Proposal 1: For testing in FR1 RRM measurement prediction, the configuration of 1 Tx and TDL channel model can be considered.

In last meeting, RAN4 has concluded that conductive testing method could be used for AI/ML mobility test in FR1. We understand that in conductive test, the UE and TE are connected directly with RF cable. The TE can adjust the transmission power to mimic the cell to be measured and the cell to be predicted similar to legacy approach. So, we agree to assume that the actual ground truth is available at the TE. However, we see some potential issues. As we support to use fading channel, the channel conditional is random in time domain. UE and TE need to align the time point when UE is supposed to conduct measurement. Otherwise, it’s hard to get the actual ground truth. We believe this issue will be figured out in L3 RSRP prediction accuracy discussion.

Proposal 2: For testing in FR1 RRM measurement prediction, RAN4 can assume that the actual ground truth is available at TE.

- Specifically, for the FR1 to FR1 inter-frequency inter-cell frequency domain scenario, the multi-cell configuration need to be pay more attention. To verify the accuracy of the predicted L3 filtered cell-level measurement result in this scenario, multi-cell configuration in the test need to be further studied. We think the configuration in this case depends on the number of cells to be predicted, which needs to wait for RAN2 decision. If we only verify that case one cell to be measured and one cell to be predicted, we understand that the legacy test setup can be used as baseline. If we need to verify the case one cell to be measured and multiple cells to be predicted, we think the test setup need to be further studied.

Proposal 3: For the scenario of FR1 to FR1 inter-frequency inter-cell frequency domain scenario, multi-cell setup in the test need to be further studied.

For FR2 RRM measurement prediction, we understand that we face the similar testability issues as in NR_AIML_Air WI. We think the conclusions on testability achieved in NR_AIML_Air WI can be taken as baseline.

Proposal 4: For FR2 measurement prediction, conclusions on testability that RAN4 made for AI/ML BM can be taken as a baseline.

- Conclusion

Proposal 1: For testing in FR1 RRM measurement prediction, the configuration of 1 Tx and TDL channel model can be considered.

Proposal 2: For testing in FR1 RRM measurement prediction, RAN4 can assume that the actual ground truth is available at TE.

Proposal 3: For the scenario of FR1 to FR1 inter-frequency inter-cell frequency domain scenario, multi-cell setup in the test need to be further studied.

Proposal 4: For FR2 measurement prediction, conclusions on testability that RAN4 made for AI/ML BM can be taken as a baseline.

- Reference

- R4-2420095, WF on RRM requirements for FS_NR_AIML_Mob, Nokia