3GPP TSG-RAN WG4 Meeting #114				                                           R4-500635

Athens, GR, 17th – 21st February, 2025

Agenda Item:	7.25.2.1

Title: 	Discussion on OD-SSB requirements

Source: 	Xiaomi

Document for:		Discussion

- Introduction

In RANP #103 meeting, the WID [1,RP-240170] for R19 NES was agreed and the objectives for OD-SSB are self-contained as below.

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum. - Specify the corresponding core requirements, for the above features [RAN4].

And in the last RAN4 meeting, the agreements for NES RRM were achieved in [2,R4-2420098]. Therefore, in this discussion, we provided more analysis on the potential RRM impacts due to these aspects in Rel19 NES.

- Discussion

  - OD-SSB L1 measurements

In the last RAN4 meeting, there was not discussion on whether OD-SSB based L1 neighbor cell measurements should be supported due to the limited time. But the regarding o the further RAN1 agreement below, it is clear that L1 measurement requirement based on OD-SSB needs to be defined in RAN4. Otherwise RAN1 design can’t be tested. But from RAN4 itself it is unclear L1 measurement based on OD-SSB shall be performed before or after SCell activation.

R1#118 discussion Agreement - Update the previous RAN1 agreement as follows. - At least support L1 measurement based on on-demand SSB - For L1 measurement based on on-demand SSB, periodic, semi-persistent, [and aperiodic] L1 measurement reports based on existing CSI framework are supported. - FFS on potential enhancements of CSI report configuration and/or triggering/activation mechanisms for L1 measurement based on on-demand SSB - The support of LTM is a separate discussion point Agreement: Support L3 measurement based on on-demand SSB - Further work on L3 measurement is up to RAN2/RAN4

In our views, for OD-SSB scenario 2/2A agreed RAN4 study the measurement on OD-SSB is possible after OD-SSB triggered. And according to TS38.133 requirements for SSB based SCell activation, L1 measurement based on SSB is needed also for SCell activation.

8.3.2 SCell Activation Delay Requirement for Deactivated SCell ……….. If UE supports shortMeasInterval-r18, then - 6ms + TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced + Trs, enhanced + TL1-RSRP, enhanced_measure + TL1-RSRP, report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP), if semi-persistent CSI-RS is used for CSI reporting, - 3ms + TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced + Trs, enhanced + TL1-RSRP, enhanced_measure + TL1-RSRP ,report + max(THARQ + Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay), if periodic CSI-RS is used for CSI reporting. - Otherwise - 6ms + TFirstSSB_MAX + TSMTC_MAX + Trs + TL1-RSRP, measure + TL1-RSRP,report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP), if semi-persistent CSI-RS is used for CSI reporting, - 3ms + TFirstSSB_MAX + TSMTC_MAX + Trs + TL1-RSRP, measure + TL1-RSRP,report + max(THARQ + Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay), if periodic CSI-RS is used for CSI reporting. ……… TL1-RSRP, enhanced_measure is - SSB based L1-RSRP measurement delay TL1-RSRP_Measurement_Period_SSB ms based on applicability as defined in clause 9.5 assuming M=1 and TReport=0; N is equal to the value reported by the UE in reduceForSSB-L1-RSRP-Meas. Otherwise, if reduceForSSB-L1-RSRP-Meas is absent, N= 8. Or, - CSI-RS based L1-RSRP measurement delay TL1-RSRP_Measurement_Period_CSI-RS ms based on applicability as defined in clause 9.5 assuming M=1 and TReport=0. - If UE supports shortMeasInterval-r18 capability, L1-RSRP measurement for TL1-RSRP, enhanced_measure can be performed based on non-DRX mode even if DRX is configured.

Also the requirements of L1 measurement for other purpose beside SCell activation (e.g. for RLM) was also defined separately in TS38.133 clause 9.5.

9.5 L1-RSRP measurements for Reporting 9.5.1 Introduction When configured by the network, the UE shall be able to perform L1-RSRP measurements of configured CSI-RS, SSB or CSI-RS and SSB resources for L1-RSRP. The measurements shall be performed for a serving cell, including PCell, PSCell, or SCell, on the resources configured for L1-RSRP measurements within the active BWP. For UE supporting bwpOperationMeasWithoutInterrupt-r18, the measurements shall also be performed for a serving cell, including PCell, PSCell, or SCell, on the resources configured for L1-RSRP measurements outside the active BWP. For UE supporting ncd-SSB-BWP-Wor-r18, the SSB and SMTC in this section applies for both CD-SSB and NCD-SSB if it is not additional specified. If SSB in the active DL BWP of serving cell i is NCD-SSB, for serving cell i the requirements in clause 8.1 apply provided that serving cell i is PCell or activated PSCell.

Observation 1: L1 measurement based on OD-SSB is needed also for the other measurements beside SCell activation.

That is the OD-SSB L1 measurement is needed during both SCell activation procedure and other RRM procedure.

On the other hand, in our views L1 measurement would happen only after SCell is activated. For an example, L1 RSRP measurement for BFD is based on the activated SCell indeed.

Observation 2: L1 measurement based on OD-SSB is needed for the activated SCell after SCell activation.

Thus, we can propose that:

Proposal 1: RAN4 shall define L1 measurement requirements based on OD-SSB in both clause 8.3 （for SCell activation delay） and Clause 9.5 （for other measurements procedures on the activated SCell）in TS38.133. And the detailed requirements can be FFS upon further RAN1 agreements.

  - Case 1- OD-SSB based deactivated SCell L3 measurement

When the OD-SSB was triggered for SCell activation, the periodicity of OD-SSB is expected to be short as possible to enable UE can complete the measurement quickly which can save the network power. That is the exact meaning of “fast L3 measurement” in RAN4 agreements [4]. However, for the deactivated SCell, the periodicity of OD-SSB can be much sparser than these for the SCell activated or to be activated. As a result, UE need not to measure the OD-SSB so frequently to minimize UE’s power consumption. This is what “slow measurement” mean in comparison with the former one.

Thus, in the previous RAN4 meeting, the harmonized solution to balance the power consumption and the reliable measurement results via OD-SSB occasions is to restrict the periodicity of measurement based OD-SSB in the fast mode window (“Tfast”) and after that (e.g.“Tslow”) UE can relax the measurement with “measCycleSCell” or stop the measurement as agreements below.

Figure 1. OD-SSB measurement stages [4]

And in the last RAN4 meeting, the agreements on how to define the time window (Tfast) and corresponding UE measurement behavior was achieved in [4].

- Sub-issue: Sub-issue 1 – How to perform measurement after time window within one time OD-SSB activation/deactivation

Sub-issue 1 – How to perform measurement after time window within one time OD-SSB activation/deactivation? Agreement: - After the fast time window, the UE shall meet the legacy deactivated SCell measurement requirement as long as OD-SSB is activated/transmitted from UE’s perspective. - The agreement can be revisited based on further RAN1 agreement about OD-SSB deactivation mechanism

In principle, according to RAN4 agreement in the last meeting above, after Tfast even if there are activated ODSSB available, but UE is not required to measure this SCell based on OD-SSB periodicity but meet the legacy requirements defined based on “measCycleSCell”.

However, according TS38.331, measCycleSCell can be used for the SCell in the deactivated state only.

Observation 3: The “measCycleSCell” parameter used to define the legacy deactivated SCell measurement [3, TS38.133] could not available/used when SCell is activated after the fast mode window.

Table 9.2.5.1-4: Time period for PSS/SSS detection, deactivated SCell (FR1) [3]

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | Ceil(5 x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil(5 x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil(5 x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: The requirements also apply to deactivated SCG SCell. |  |

However, in our view, as illustrated in Figure 2 below, for Scenario 2A SCell can be still activated until OD-SSB deactivation signaling received. That is under SCell activation with L3 reporting, UE needs also to measure SSBs.

Figure 2. OD-SSB Scenario 2A measurement

Observation 4: UE needs to measure the activated SCell with OD-SSB under some scenarios, e.g.

- OD-SSB Scenario 2A, 3A and 3B

Figure 3. OD-SSB Scenario 3A/3B under RAN1 discussion

Therefore, in our view it is possible to measurement SSB of activated SCell after Tfast window. It is unclear whether the legacy measurement can be applicable unless SCell was deactivated.

Proposal 2: RAN4 shall clarify whether the legacy deactivated SCell measurement requirements can be applicable for the measurement after the fast mode window for all OD-SSB scenarios.

Alternatively, in our understanding, the main motivation of RAN4 agreements in the last meeting is to avoid the too frequent measurement after Tfast and more power saving gain achieved. It is straightforward that UE can follow the more spare SSB periodicity to measure. Thus, we can propose:

Proposal 2a: From RAN4 measurement perspective, the measurement cycle which shall be followed by UE after Tfast is

- max⁡(Tc2,  [160ms] )  if measCycleSCell absent

- Sub-issue 3&4: How to define the time window and whether the time window is configured or pre-defined?

R4#113 Agreements [4] Sub-issue 3&4: How to define the time window and whether the time window is configured or pre-defined? Agreement: The time window is based on the number of OD-SSB samples and UE provided one valid report. - The uncertainty time should also be considered, such as T_min, T_processing and T_uncertainy - UE should have enough number of OD-SSB samples to achieve the measurement requirement. - The number of OD-SSB is FFS. - FFS whether UE measurement reporting is needed

Meanwhile in RAN1, there are extensive discussion on how to indicate the OD-SSB periodicity and the agreements below were achieved.

R1#117 discussion Proposal #3-1b (Signalling container): - For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell. - FFS: Whether to support DCI based signaling to indicate on-demand SSB transmission on the cell. - This DCI signaling does not provide SCell activation/deactivation. - If supported, details on DCI including UE-specific or group-common DCI, DCI contents, etc. - FFS: Scenarios where the above signalings are applicable Agreement - For a cell supporting on-demand SSB SCell operation, at least the following for on-demand SSB via higher layer RRC signaling is supported. - Frequency of the on-demand SSB - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst - Periodicity of the on-demand SSB - FFS: Whether more than one on-demand SSB configurations can be configured for the cell to UE - FFS: Whether the RRC is newly introduced or existing RRC is reused Proposal #3-2b (Contents): - For a cell supporting on-demand SSB SCell operation, at least the followings for on-demand SSB are known to UE. - Sub-carrier spacing of the on-demand SSB - Physical Cell ID of the on-demand SSB - Location of on-demand SSB burst - Downlink transmit power of on-demand SSB - FFS: Other parameters - FFS: Whether each of above parameters is configured/indicated explicitly or not R1#118 discussion Agreement (contents) For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. - Periodicity of the on-demand SSB FFS: Any other relevant parameters “R1#119 Agreement For a cell supporting on-demand SSB SCell operation, support at least the following options to deactivate on-demand SSB transmission from a UE perspective. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Deactivation by RRC is up to RAN2 - FFS: Which scenario Option 1 is used - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - FFS: Whether Option 4, 4a is needed in addition to Option 2 - FFS: Whether the value of N can be implicitly determined using a timer

Observation 5: Deactivate on-demand SSB transmission from a UE perspective will be introduced in RAN1. But the exact signaling mechanism is still under RAN1 discussion.

Observation 6: The multiple periodicities for OD-SSB can be configured to UE and the exact applicable one will be indicated timely by MAC-CE.

Therefore, we believe when UE completed the SCell activation via OD-SSB NW should update the more proper OD-SSB periodicity to UE or deactivate OD-SSB for the power saving purpose. Otherwise, the motivation of NES OD-SSB will be vanished.

Observation 7: The more proper OD-SSB periodicity or deactivation of ODSSB shall be indicated by NW to obtain NES power saving gain.

Thus as illustrated in Figure 4 below, UE behavior within Tfast window could be impacted by NW’s updates on OD-SSB periodicity and deactivation indication.

Figure 4.

For an example, if RAN1 the deactivation OD-SSB introduced, the end point of this Tfast shall be reconsidered

Thus, it is naturally to assume the time window (Tfast) can start from the time point of SCell activation command (t1) and end at the MAC CE to update the ODSSB periodicity (t2) or OD-SSB deactivation (t3).

And regarding to the agreements on how to define the time window in which UE performing the fast measurement based on OD-SSB, we can propose that:

Proposal 3: The time window in which UE shall perform the fast measurement once UE receives the OD-SSB activation indication can be defined based on the other additional condition below:

- the first MAC CE to update OD-SSB or deactivated OD-SSB or/and

- deactivation OD-SSB

On the other hand, in order to further reduce UE’s power consumption, UE needs over [>5] measurement samples to guarantee the successful measurement results. Under such circumstances, the total length of this time window Tfast can be also limited by other conditions (e.g. “max5*Tc1, t1,t2).

Thus we also slightly prefer the alternative proposal of “the number of OD-SSB is [5]”. For an example, Tfast = min{5*sample, [t1,t3]} + Tuncertainty

Therefore, in summary we can propose that:

Proposal 4: Tfast can be defined based on

- the number of OD-SSB samples and UE provided one valid report, e.g. [5] samples

- The uncertainty time should also be considered, such as T_min, T_processing and T_uncertainy

- The point of OD-SSB deactivation

  - Case 2 - OD-SSB based deactivated SCell L3 measurement

In the last RAN4 meeting, on how to define the necessary requirement on SCell (de)activation with OD-SSB was discussed also with some high level agreements achieved.

Issue 1-3-1: Scenario 1 - OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered - Option 1: UE performs the OD-SSB based fast L3 measurement for deactivated SCell within a time window. After the time window, UE can perform deactivated SCell measurement with a slow mode. - Option 1-1: The measurement periodicity is OD-SSB periodicity - Option 1-2: The measurement periodicity is based on combining OD-SSB and AO-SSB - Option 2: UE always follow legacy requirements based on measCycleSCell - Option 3: The same approach in Case 1 can be reused if the serving cell is unknown. UE follows legacy requirements based on measCycleSCell if the serving cell is already known.

For case 2, the motivation to trigger OD-SSB is same as that for Case 1. That is UE can perform the measurement based on the OD-SSB in the fast window and the other sparer measurement in the slow window. Thus, we can support Option 1 as well.

Proposal 5: OD-SSB based deactivated SCell L3 measurement for Case 2 can follow the conclusion for Case 1.

  - OD-SSB based activation

In the last RAN4 meeting, on how to define the necessary requirement on SCell (de)activation with OD-SSB was discussed also with some high level agreements achieved.

Issue 1-4-1: General Agreement: - RAN4 focuses on OD-SSB based SCell activation known and unknown requirement for both Case 1 and Case 2 and not specifically differentiate Scenario 2/2A scenarios. Issue 1-4-3: known cell condition design – principle Agreement: - The legacy known condition principle can be reused, which is defined based on if the UE has sent a valid measurement report during a ‘period’ before the reception of the SCell activation command. - When OD-SSB transmission is before/together with the SCell activation command(Scenario #2/Scenario #2A), UE follows the principles of legacy known/unknown SCell activation requirement once the known/unknown condition is met. - FFS how known/unknown condition is defined i.e. based on always-on SSB and/or OD-SSB - Other principles are not precluded. Issue 1-4-5: Which type of SSB is used during SCell activation Agreement: - In Case 1, UE is required to always use OD-SSB for Scell activation. - In Case 2, when both OD-SSB and always-on SSB are transmitted, - RAN4 assumes UE to use OD-SSB to perform SCell activation as a baseline - The agreement can be revisited if different with other WG

For the detailed RRM requirements updates, in our view it needs more discussion under the different conditions and scenarios. For an example, as illustrated in Figure 2 below, for the case of “Scenario #2 and Case #1“ and known Scell activation case, if the first SSB transmitted after OD SSB trigger command can located before the time “n + THARQ+5ms+TfirstSSB +margin”, UE can follow the current behavior for Scell activation and corresponding requirements defined in TS38.133 for known cases can be reused. Otherwise, the requirements in current spec needs to be updated. However, for the unknown case since the more than one SSB needed for the SCell timing and frequency synchronization, the current requirements could not be reused with the condition above for known cases indeed.

Observation 8: When leveraging the current requirements in TS38.133 to the SCell activation/deactivation requirement with on-demanding SSB, whether SCell is known or unknown needs to be updated.

Figure 4.

Proposal 6: For the SCell activation/deactivation requirements with OD SSB, the UE known or unknow conditions shall be redefined as  :

During the period equal to max([5]*max(measCycleSCell, ODSSB_cycle), 5*DRX cycles), or max([5]*max(160ms, ODSSB_cycle), 5*DRX cycles) for FR1 before the reception of the SCell activation command: - the UE has sent a valid measurement report for the SCell being activated and - the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3. - the SSB measured during the period equal to max(5*measCycleSCell, 5*DRX cycles) also remains detectable during the SCell activation delay according to the cell identification conditions specified in clause 9.2 and 9.3. Otherwise, SCell in FR1 is unknown.

Nevertheless, beside the scenarios, RAN1 also discussed the always-on SSB is cell based or not (case 2).

Agreement - For a cell supporting on-demand SSB SCell operation, - Note: It is up to gNB implementation whether always-on SSB (if transmitted) on the cell is cell-defining SSB or not. - For on-demand SSB on the cell, downselect between the following alternatives - Alt-1: It is up to gNB implementation whether on-demand SSB is cell-defining SSB or not. - Alt-2: On-demand SSB is limited to non-cell-defining SSB. - FFS: Further limitations to on-demand SSB

That is it is possible UE can receive both non-cell-defined on-demand SSB and legacy SSB together for case 2. If this is implemented by gNB. From UE perspective, it is also up to UE to perform the measurement based either of them.

Observation 9: In order to allow UE’s implement flexibility (e.g. reduce UE’s power consumption with longer measurement cycle), it needs not require UE performing the SCell measurement with OD-SSB if both of OD-SSB and legacy SSB are available.

Especially, within a successful measurement reporting period in which at least 4 measurement samples required, if there are different types of SSB used, this will significantly impact RAN4 current requirements since there is not any requirements defined by the hybrid measurement cycle in TS38.133.

Figure 5.

Observation 10: In order to simplify RAN4 discussion, RAN4 can define the SCell activation measurement requirements base on a single SSB period cycle instead of hybrid ones.

In summary, we can suggest that:

Proposal 7: RAN4 can further discuss the SCell activation/deactivation requirements with OD SSB upon no hybrid measurement SSBs for case 2 to simplify the specification complexity.

- Conclusion

In this contribution, serval issues related to RRM in Rel19 NES are discussed. The observations and proposals can be summarized as:

Observation 1: L1 measurement based on OD-SSB is needed also for the other measurements beside SCell activation.

Observation 2: L1 measurement based on OD-SSB is needed for the activated SCell after SCell activation.

Proposal 1: RAN4 shall define L1 measurement requirements based on OD-SSB in both clause 8.3 （for SCell activation delay） and Clause 9.5 （for other measurements procedures on the activated SCell）in TS38.133. And the detailed requirements can be FFS upon further RAN1 agreements.

Observation 3: The “measCycleSCell” parameter used to define the legacy deactivated SCell measurement [3, TS38.133] could not available/used when SCell is activated after the fast mode window.

Observation 4: UE needs to measure the activated SCell with OD-SSB under some scenarios, e.g.

- OD-SSB Scenario 2A, 3A and 3B

Proposal 2: RAN4 shall clarify whether the legacy deactivated SCell measurement requirements can be applicable for the measurement after the fast mode window for all OD-SSB scenarios.

Proposal 2a: From RAN4 measurement perspective, the measurement cycle which shall be followed by UE after Tfast is

- max⁡(Tc2,  [160ms] )  if measCycleSCell absent

Observation 5: Deactivate on-demand SSB transmission from a UE perspective will be introduced in RAN1. But the exact signaling mechanism is still under RAN1 discussion.

Observation 6: The multiple periodicities for OD-SSB can be configured to UE and the exact applicable one will be indicated timely by MAC-CE.

Observation 7: The more proper OD-SSB periodicity or deactivation of ODSSB shall be indicated by NW to obtain NES power saving gain.

Proposal 3: The time window in which UE shall perform the fast measurement once UE receives the OD-SSB activation indication can be defined based on the other additional condition below:

- the first MAC CE to update OD-SSB or deactivated OD-SSB or/and

- deactivation OD-SSB

Proposal 4: Tfast can be defined based on

- the number of OD-SSB samples and UE provided one valid report, e.g. [5] samples

- The uncertainty time should also be considered, such as T_min, T_processing and T_uncertainy

- The point of OD-SSB deactivation

Proposal 5: OD-SSB based deactivated SCell L3 measurement for Case 2 can follow the conclusion for Case 1.

Observation 8: When leveraging the current requirements in TS38.133 to the SCell activation/deactivation requirement with on-demanding SSB, whether SCell is known or unknown needs to be updated.

Proposal 6: For the SCell activation/deactivation requirements with OD SSB, the UE known or unknow conditions shall be redefined as :

During the period equal to max([5]*max(measCycleSCell, ODSSB_cycle), 5*DRX cycles), or max([5]*max(160ms, ODSSB_cycle), 5*DRX cycles) for FR1 before the reception of the SCell activation command: - the UE has sent a valid measurement report for the SCell being activated and - the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3. - the SSB measured during the period equal to max(5*measCycleSCell, 5*DRX cycles) also remains detectable during the SCell activation delay according to the cell identification conditions specified in clause 9.2 and 9.3. Otherwise, SCell in FR1 is unknown.

Observation 9: In order to allow UE’s implement flexibility (e.g. reduce UE’s power consumption with longer measurement cycle), it needs not require UE performing the SCell measurement with OD-SSB if both of OD-SSB and legacy SSB are available.

Observation 10: In order to simplify RAN4 discussion, RAN4 can define the SCell activation measurement requirements base on a single SSB period cycle instead of hybrid ones.

Proposal 7: RAN4 can further discuss the SCell activation/deactivation requirements with OD SSB upon no hybrid measurement SSBs for case 2 to simplify the specification complexity.

- References

- RP-240830

- TS38.133 v18.6.0

- TS38.331 v17.1.0

- R4-2420098	WF on RRM requirements for Netw_Energy_NR_enh_Part1

- R4-2418282	Topic summary for [113][224] Netw_Energy_NR_enh_Part1